3GPP TSG-RAN WG4 Meeting #90	R4-1900941

Athens, Greece, 25 February-1 March, 2019

Title: 					CSSF outside gap for NE-DC operation

Source: 	Ericsson

Agenda item:	6.3.3

Document for:	Discussion

# Introduction

Previously, RAN4 has agreed CSSF outside gap for both EN-DC and SA scenarios, as shown in table 1 and table 2

| Scenario | CSSFoutside_gap,i for FR1 PSCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PSCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required |
| --- | --- | --- | --- | --- | --- |
| EN-DC with FR1 only CA | 1 | Number of configured FR1 SCell(s) | N/A | N/A | N/A |
| EN-DC with FR2 only intra band CA | N/A | N/A | 1 | N/A | Number of configured FR2 SCells |
| EN-DC with FR1 +FR2 CA (FR1 PSCell) Note 1 | 1 | 2×(Number of configured SCell(s)-1) | N/A | 2 | 2×(Number of configured SCell(s)-1) |
| EN-DC with FR1 +FR2 CA (FR2 PSCell) Note 1 | N/A | Number of configured SCell(s) | 1 | N/A | Number of configured SCell(s) |
| Note 1: Only one NR FR1 operating band and one NR FR2 operating band are included for FR1+FR2 inter-band EN-DC. |  |  |  |  |  |

Table 1: CSSF outside gap for EN-DC

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required |
| --- | --- | --- | --- | --- | --- |
| FR1 only CA | 1 | Number of configured FR1 SCell(s) | N/A | N/A | N/A |
| FR2 only intra band CA | N/A | N/A | 1 | N/A | Number of configured FR2 SCell(s) |
| FR1 +FR2 CA (FR1 PCell) Note 1 | 1 | 2×(Number of configured SCell(s)-1) | N/A | 2 | 2×(Number of configured SCell(s)-1) |
| Note 1: Only one FR1 operating band and one FR2 operating band are included for FR1+FR2 inter-band CA. |  |  |  |  |  |

Table 2: CSSF outside gap for SA

Now that RAN4 works on late drop features, CSSF outside gap needs to be extended for NE-DC and NR-NR synchronous dual connectivity. In this paper, we provide views on the needed scaling factors for NE-DC.

# Discussion

The underlying assumptions for EN-DC related to searcher complexity are

- The LTE searcher and requirements are specified independently of CSSF outside gap

- There are two NR searchers, one of which is allocated for PSCC, and the other is time shared between SCells

- If an FR2 SCell where neighbor cells are measured is implied by the configuration (i.e. there is FR2 configured, and no FR2 PSCell) then the sharing between SCells is unequal

  - 50% of the SCC searcher is used for the FR2 SCC where neighbor cells are measured

  - The remaining 50% of the SCC searcher is shared between other neighbour cells

In NE-DC, the role of PSCell and PCell are swapped compared with EN-DC (i.e. the PSCell becomes an LTE cell, and the PCell is an NR Cell). Otherwise it would appear that a very similar approach can be used for CSSF outside gap. The basic assumption can still be that there are 2 searchers with one searcher allocated for the PCell, and the other searcher shared between SCells in a similar way as for the SCells in the EN-DC cases. Hence the requirements are

Proposal 1 : Requirements for NE-DC are given by

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required |
| --- | --- | --- | --- | --- | --- |
| NE-DC with FR1 only CA | 1 | Number of configured FR1 SCell(s) | N/A | N/A | N/A |
| NE-DC with FR2 only intra band CA | N/A | N/A | 1 | N/A | Number of configured FR2 SCells |
| NE-DC with FR1 +FR2 CA (FR1 PCell) Note 1 | 1 | 2×(Number of configured SCell(s)-1) | N/A | 2 | 2×(Number of configured SCell(s)-1) |
| NE-DC with FR1 +FR2 CA (FR2 PCell) Note 1 | N/A | Number of configured SCell(s) | 1 | N/A | Number of configured SCell(s) |
| Note 1: Only one NR FR1 operating band and one NR FR2 operating band are included for FR1+FR2 inter-band NE-DC. |  |  |  |  |  |

The validity of note 1 depends on band combinations for NE-DC, however given the time available to complete late drop features, we do not see it feasible to develop RF requirements for 3 NR band FR1+FR2 carrier aggregation within the context of rel 15 NE-DC. Therefore, it seems that note 1 needs to be applicable. If 3 NR band operation of NE-DC is introduced in future, the 2 searcher assumption used to derive the requirements seems invalid

# Conclusion

Proposal 1 : Requirements for NE-DC are given by

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required |
| --- | --- | --- | --- | --- | --- |
| NE-DC with FR1 only CA | 1 | Number of configured FR1 SCell(s) | N/A | N/A | N/A |
| NE-DC with FR2 only intra band CA | N/A | N/A | 1 | N/A | Number of configured FR2 SCells |
| NE-DC with FR1 +FR2 CA (FR1 PCell) Note 1 | 1 | 2×(Number of configured SCell(s)-1) | N/A | 2 | 2×(Number of configured SCell(s)-1) |
| NE-DC with FR1 +FR2 CA (FR2 PCell) Note 1 | N/A | Number of configured SCell(s) | 1 | N/A | Number of configured SCell(s) |
| Note 1: Only one NR FR1 operating band and one NR FR2 operating band are included for FR1+FR2 inter-band NE-DC. |  |  |  |  |  |