3GPP TSG-RAN WG4 Meeting #90	R4-1901020

# Athens, Greece,  25th Feb - 1st Mar 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.171 | CR | 0030 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Request Location Information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Spirent Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 22/01/19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In three places “Request Location Information” is incorrectly written as “Provide Location Information”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | “Provide Location Information” is corrected to “Request Location Information”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Errors will remain causing confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.3, 4.3.2.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 4.2.3	MBS Measurement time

For NR, MBS measurement time is defined as the time starting from the moment that the UE has received the LPP [3] message of type REQUEST LOCATION INFORMATION, and ending when the UE starts sending the LPP message of type PROVIDE LOCATION INFORMATION on the Uu interface. For tests that involve sending MBS assistance data to the UE, the assistance data is sent prior to the REQUEST LOCATION INFORMATION message. The response times specified for all test cases are based on new measurements unless otherwise stated, i.e. the UE shall not reuse any information on measurements or other aiding data that was previously acquired or calculated and stored internally in the UE.

For LTE, MBS measurement time is defined as the time starting from the moment that the UE has received the LPP message of type REQUEST LOCATION INFORMATION, and ending when the UE starts sending the LPP message of type PROVIDE LOCATION INFORMATION on the Uu interface. For tests that involve sending MBS assistance data to the UE, the assistance data is sent prior to the REQUEST LOCATION INFORMATION message. The response times specified for all test cases are based on new measurements unless otherwise stated, i.e. the UE shall not reuse any information on measurements or other aiding data that was previously acquired or calculated and stored internally in the UE. A dedicated test message 'RESET UE POSITIONING STORED INFORMATION' has been defined in TS 36.509 [4] clause 6.9 for the purpose of deleting this information.

For UTRA, MBS measurement time is defined as the time starting from the moment that the UE has received the final RRC measurement control message containing reporting criteria different from "No Reporting" sent before the UE sends the measurement report containing the MBS measured results, and ending when the UE starts sending the measurement report containing the measured result on the Uu interface. The response times specified for all test cases are based on new measurements unless otherwise stated, i.e. the UE shall not re use any information on measurements or other aiding data that was previously acquired or calculated and stored internally in the UE. A dedicated test message 'RESET UE POSITIONING STORED INFORMATION' has been defined in TS 34.109 [8] clause 5.4 for the purpose of deleting this information.

The measurements for n MBS beacons, enabled across the slots of an MBS transmission period [7], shall be available at the UE by, where can be expressed as:

ms

where

is the total time for detecting and measuring n beacons

is the elapsed time from the trigger of the measurement to the start of the first MBS transmission period

is the MBS transmission period (1 second)

is the processing time, an upper-bound for which can be given as  where  is the duration of a MBS slot (100 ms) with continuous MBS transmissions

Figure 4.2.3-1: MBS Measurement Time

For this requirement, the assumption is that there is zero frequency offset for the beacons and the UEs have a minimum of 13 parallel correlators.

The test case for MBS Measurement time requirements are specified in clause A.3.1.

### 4.2.4	RRC states for MBS measurements

For NR, the minimum MBS performance requirements specified in clause 5 apply for RRC_CONNECTED state.

For LTE, the minimum MBS performance requirements specified in clause 5 apply for RRC_CONNECTED state.

For UTRA, the minimum MBS performance requirements specified in clause 5 apply for different RRC states that include Cell_DCH and Cell_FACH.

### 4.2.5	MBS Measurement Error Definitions

The code phase measurement error is defined as the difference between the actual code phase for a given MBS beacon, and the estimated code phase for that beacon, as reported in the TBS-MeasurementInformation IE provided in the LPP message of type PROVIDE LOCATION INFORMATION by NR UE [3], the TBS-MeasurementInformation IE provided in the LPP message of type PROVIDE LOCATION INFORMATION by LTE UE [3], and the UE Positioning AddPos measured results IE in the MEASUREMENT REPORT message by UTRA UE [6]. This difference has to then be adjusted for the measurement bias introduced by the UE clock to provide the final code phase measurement error.

## 4.3	WLAN Measurements

### 4.3.1	General

Clause 4.3 defines the measurement requirements for the measurements performed by the UE for WLAN based positioning.

### 4.3.2	WLAN Access Point Measurements

Editor’s note:	In the WLAN requirements for NR, the NR clauses are separate from LTE, but it is FFS whether separate clauses are needed for SA NR and non-SA NR.

#### 4.3.2.1	E-UTRAN FDD-WLAN Access Point Measurements

##### 4.3.2.1.1	Introduction

The requirements defined in section 4.3.2.1 shall apply provided the E-UTRA FDD UE has received WLAN-RequestLocationInformation message from E-SMLC via LPP requesting the UE to report WLAN measurement for one or more WLAN Access Points [12].

##### 4.3.2.1.2	Measurement Requirements

The measurement delay reporting requirements for WLAN are defined in section 4.3.2.1.3. The WLAN Access Point identification minimum performance requirements are defined in clause 7.

##### 4.3.2.1.3	Measurement Reporting Delay

For LTE, WLAN measurement time is defined as the time starting from the moment that the UE has received the LPP message of type REQUEST LOCATION INFORMATION, and ending when the UE starts sending the LPP message of type PROVIDE LOCATION INFORMATION on the Uu interface. The response times specified for all test cases are based on new measurements unless otherwise stated, i.e. the UE shall not re use any information on measurements or other aiding data that was previously acquired or calculated and stored internally in the UE. A dedicated test message 'RESET UE POSITIONING STORED INFORMATION' has been defined in TS 36.509 [4] clause 6.9 for the purpose of deleting this information. No WLAN assistance data is provided to the UE.

The signals from the WLAN APs shall be available at the UE for the duration of the measurement time. Each WLAN AP transmits a beacon signal with a beacon interval smaller or equal to 102.4 ms. The beacon frames from different access points shall be transmitted in different time slots or non-overlapping frequency channels. The beacon frames have variable time duration of ~1ms.

The WLAN Measurement Reporting Delay is given as:

sec

where

is the total time for detecting and measuring the WLAN Access Points

is the elapsed time from the trigger of the measurement to the start of the first WLAN transmission period and is shown in Figure 4.3.2.1.3-1.

Figure 4.3.2.1.3-1: WLAN Measurement Time

#### 4.3.2.2	E-UTRAN TDD-WLAN Access Point Measurements

##### 4.3.2.2.1	Introduction

The requirements defined in section 4.3.2.2 shall apply provided the E-UTRA TDD UE has received WLAN-RequestLocationInformation message from E-SMLC via LPP requesting the UE to report WLAN measurement for one or more WLAN Access Points [12].