3GPP TSG-RAN4 WG4 Meeting #90	R4-1901061

# Athens, Greece,  Feb.25 - Mar.01, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 5366 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR: Addition of performance requirements for FeNB-IoT TDD NPDSCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh2-Perf |  |  |  |  |  | Date: |  |  | 2019-02-28 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As per agreements R4-1814232, RAN4 agreed to define FeNB-IoT TDD NPDSCH performance requirements with UL/DL configuration#4 and special subframe configuration#4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the performance requirements for FeNB-IoT TDD NPDSCH. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The performance requirements for FeNB-IoT TDD NPDSCH will still be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.12.1.1, 8.12.1.2(New), A.3.12.1, A.3.12.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 8.12	Demodulation of Narrowband IoT

### 8.12.1	NPDSCH

#### 8.12.1.1	Half-duplex FDD

Table 8.12.1.1-1: Common Test Parameters

| Parameter | Unit | Value |
| --- | --- | --- |
| Number of HARQ processes per component carrier (Note 2) | Processes | 1 or 2 |
| Maximum number of HARQ transmission |  | 4 |
| Cyclic Prefix |  | Normal |
| eutraControlRegionSize-r13 |  | 3 for In-band and N/A for Standalone/Guard-band unless otherwise stated |
| downlinkBitmap-r13 and dl-Gap-r13 |  | Not configured |
| dl-GapNonAnchor-r13 and downlinkBitmapNonAnchor-r13 |  | Not configured |
| Unused REs or RB (Note 1) |  | OCNG |
| OCNG pattern |  | NB.OP.1 |
| Note 1: For in-band mode, the REs for transmission of LTE signals including PDCCH, CRS should be filled by OCNG. Note 2: 1 is applicable for both UE Category NB1 and NB2; 2 is only applicable for UE Category NB2 if UE supports it. |  |  |

Table 8.12.1.1-2: Test Parameters of related NPDCCH and NPUSCH format 2 configurations

| Parameter | Unit | Value |
| --- | --- | --- |
| DCI format |  | DCI format N1 |
| scheduling delay field () |  | 1 for the first HARQ process; 2 for the second HARQ process if configured |
| (ack-NACK-NumRepetitions-r13) |  | 1 |
| ACK/NACK resource field |  | 0 |
| Reference channel for NPDCCH |  | R.NB.3 FDD for one NRS antenna port; R.NB.4 FDD for two NRS antenna ports |
| (npdcch-Offset-USS-r13) |  | 0 |

##### 8.12.1.1.1	Minimum Requirements for In-band

The requirements are specified in Table 8.12.1.1.1-2, with the addition of the parameters in Table 8.12.1.1.1 -1 and the downlink physical channel setup according to Annex C.3.2 and C.3.6. The purpose of these tests is to verify the performance.

Table 8.12.1.1.1-1: Test Parameters for NPDSCH under In-band

| Parameter |  | Unit |  | Test 1, 2 |
| --- | --- | --- | --- | --- |
| Downlink power allocation of LTE signal |  |  | dB | -3 |
|  |  |  | dB | -3(Note 1) |
|  |  |  | dB | 0 |
| at antenna port |  |  | dBm/15kHz | -93 (Note 2) |
|  |  |  | dBm/15kHz | -99 (Note 3) |
| LTE CRS port number (eutra-NumCRS-Ports-r13) |  |  |  | 4 |
| NPDCCH repetition number |  |  | subframe | 8 for Test 1; 16 for Test 2; 128 for Test 3. |
| (npdcch-NumRepetitions-r13) |  |  | subframe | 8 for Test 1; 16 for Test 2; 128 for Test 3. |
| (nPDCCH-startSF-USS-r13) |  |  |  | 4 for Test 1; 2 for Test 2; 1.5 for Test 3 |
| Note 1: . Note 2: This noise is applied to all subframes from the end of the NPDCCH to the end of the following NPDSCH transmission; Note 3: This noise is applied to all subframes from the end of the NPDSCH to the end of the following NPDCCH transmission. |  |  |  |  |

Table 8.12.1.1.1-2: Minimum performance under In-band with 2 NRS ports

| Test number | Bandwidth | Carrier Type | Reference Channel | Repetition number | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Anchor | R.NB.5 FDD | 1 | EPA5 | 2x1 Low | 70% | 6.9 | NB1, NB2 |
| 2 | 200kHz | Anchor | R.NB.5 FDD | 32 | EPA5 | 2x1 Low | 70% | -4.8 | NB1, NB2 |
| 3 | 200kHz | Non-anchor | R.NB.5-1 FDD | 256 | ETU1 | 2x1 Low | 70% | -9.8 | NB1, NB2 |

##### 8.12.1.1.2	Minimum Requirements for Standalone/Guard-band

The requirements are specified in Table 8.12.1.1.2-2, with the addition of the parameters in Table 8.12.1.1.2 -1 and the downlink physical channel setup according to Annex C.3.6. The purpose of these tests is to verify the performance.

Table 8.12.1.1.2-1: Test Parameters for NPDSCH under Standalone/Guard-band

| Parameter | Unit |  | Test 1, 2 |
| --- | --- | --- | --- |
| at antenna port |  | dBm/15kHz | -93 (Note 1) |
|  |  | dBm/15kHz | -99 (Note 2) |
| NPDCCH repetition number |  | subframe | 32 for Test 1; 256 for Test 2. |
| (npdcch-NumRepetitions-r13) |  | subframe | 64 for Test 1; 512 for Test 2. |
| (nPDCCH-startSF-USS-r13) |  |  | 1.5 |
| Note 1: This noise is applied to all subframes from the end of the NPDCCH to the end of the following NPDSCH transmission; Note 2: This noise is applied to all subframes from the end of the NPDSCH to the end of the following NPDCCH transmission. |  |  |  |

Table 8.12.1.1.2-2: Minimum performance for NPDSCH under Standalone/Guard-band with 1 NRS port

| Test number | Bandwidth | Carrier Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Anchor | R.NB.6 FDD | 32 | EPA5 | 1 | 70% | -3.4 | NB1, NB2 |
| 2 | 200kHz | Non-anchor | R.NB.6-1 FDD | 256 | ETU1 | 1 | 70% | -10.2 | NB1, NB2 |

##### 8.12.1.1.3	Minimum Requirements for Standalone for UE Category NB2

The requirements are specified in Table 8.12.1.1.3-2, with the addition of the parameters in Table 8.12.1.1.3-1 and the downlink physical channel setup according to Annex C.3.6. The purpose of these tests is to verify the NPDSCH performance.

Table 8.12.1.1.3-1: Test Parameters for NPDSCH under Standalone

| Parameter | Unit |  | Test 1 |
| --- | --- | --- | --- |
| at antenna port |  | dBm/15kHz | -93 (Note 1) |
|  |  | dBm/15kHz | -96 (Note 2) |
| NPDCCH repetition number |  | subframe | 4 for Test 1 |
| (npdcch-NumRepetitions-r13) |  | subframe | 8 for Test 1 |
| (nPDCCH-startSF-USS-r13) |  |  | 1.5 |
| Note 1: This noise is applied to all subframes from the end of the NPDCCH to the end of the following NPDSCH transmission; Note 2: This noise is applied to all subframes from the end of the NPDSCH to the end of the following NPDCCH transmission. |  |  |  |

Table 8.12.1.1.3-2: Minimum performance for NPDSCH under Standalone with 1 NRS port

| Test number | Bandwidth | Carrer Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Non-anchor | R.NB.7 FDD | 1 | EPA5 | 1 | 70% | 9.4 | NB2 |

#### 8.12.1.2	TDD

Table 8.12.1.2-1: Common Test Parameters

| Parameter | Unit | Value |
| --- | --- | --- |
| Uplink downlink configuration (Note 1) |  | 4 |
| Special subframe configuration (Note 2) |  | 4 |
| Number of HARQ processes per component carrier (Note 4) | Processes | 1 or 2 |
| Maximum number of HARQ transmission |  | 4 |
| Cyclic Prefix |  | Normal |
| eutraControlRegionSize-r13 |  | 2 for In-band, N/A for Standalone/Guard-band unless otherwise stated |
| downlinkBitmap-r13 and dl-Gap-r13 |  | Not configured |
| dl-GapNonAnchor-r13 and downlinkBitmapNonAnchor-r13 |  | Not configured |
| Unused REs or RB (Note 3) |  | OCNG |
| OCNG pattern |  | NB.OP.1 |
| Note 1: As specified in Table 4.2-2 in TS 36.211 [4] Note 2: As specified in Table 4.2-1 in TS 36.211 [4] Note 3: For in-band mode, the REs for transmission of LTE signals including PDCCH, CRS should be filled by OCNG. Note 4: 1 is applicable for both UE Category NB1 and NB2; 2 is only applicable for UE Category NB2 if UE supports it. |  |  |

Table 8.12.1.2-2: Test Parameters of related NPDCCH and NPUSCH format 2 configurations

| Parameter | Unit | Value |
| --- | --- | --- |
| DCI format |  | DCI format N1 |
| scheduling delay field () |  | 1 for the first HARQ process; 2 for the second HARQ process if configured |
| (ack-NACK-NumRepetitions-r13) |  | 1 |
| ACK/NACK resource field |  | 0 |
| Reference channel for NPDCCH |  | R.NB.3 TDD for one NRS antenna port; R.NB.4 TDD for two NRS antenna ports |
| (npdcch-Offset-USS-r13) |  | 0 |

##### 8.12.1.2.1	Minimum Requirements for In-band

The requirements are specified in Table 8.12.1.2.1-2, with the addition of the parameters in Table 8.12.1.2.1 -1 and the downlink physical channel setup according to Annex C.3.2 and C.3.6. The purpose of these tests is to verify the performance.

Table 8.12.1.2.1-1: Test Parameters for NPDSCH under In-band

| Parameter |  | Unit |  | Test 1, 2 |
| --- | --- | --- | --- | --- |
| Downlink power allocation of LTE signal |  |  | dB | -3 |
|  |  |  | dB | -3(Note 1) |
|  |  |  | dB | 0 |
| at antenna port |  |  | dBm/15kHz | -93 (Note 2) |
|  |  |  | dBm/15kHz | -99 (Note 3) |
| LTE CRS port number (eutra-NumCRS-Ports-r13) |  |  |  | 4 |
| NPDCCH repetition number |  |  | subframe | 8 for Test 1; 16 for Test 2; 128 for Test 3. |
| (npdcch-NumRepetitions-r13) |  |  | subframe | 8 for Test 1; 16 for Test 2; 128 for Test 3. |
| (nPDCCH-startSF-USS-r13) |  |  |  | 4 |
| Note 1: . Note 2: This noise is applied to all subframes from the end of the NPDCCH to the end of the following NPDSCH transmission; Note 3: This noise is applied to all subframes from the end of the NPDSCH to the end of the following NPDCCH transmission. |  |  |  |  |

Table 8.12.1.2.1-2: Minimum performance under In-band with 2 NRS ports

| Test number | Bandwidth | Carrier Type | Reference Channel | Repetition number | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Anchor | R.NB.5 TDD | 1 | EPA5 | 2x1 Low | 70% | [6.9] | NB1, NB2 |
| 2 | 200kHz | Anchor | R.NB.5 TDD | 32 | EPA5 | 2x1 Low | 70% | [-4.8] | NB1, NB2 |
| 3 | 200kHz | Non-anchor | R.NB.5-1 TDD | 256 | ETU1 | 2x1 Low | 70% | [-9.8] | NB1, NB2 |

##### 8.12.1.2.2	Minimum Requirements for Standalone/Guard-band

The requirements are specified in Table 8.12.1.2.2-2, with the addition of the parameters in Table 8.12.1.2.2 -1 and the downlink physical channel setup according to Annex C.3.6. The purpose of these tests is to verify the performance.

Table 8.12.1.2.2-1: Test Parameters for NPDSCH under Standalone/Guard-band

| Parameter | Unit |  | Test 1, 2 |
| --- | --- | --- | --- |
| at antenna port |  | dBm/15kHz | -93 (Note 1) |
|  |  | dBm/15kHz | -99 (Note 2) |
| NPDCCH repetition number |  | subframe | 32 for Test 1; 256 for Test 2. |
| (npdcch-NumRepetitions-r13) |  | subframe | 32 for Test 1; 256 for Test 2. |
| (nPDCCH-startSF-USS-r13) |  |  | 4 |
| Note 1: This noise is applied to all subframes from the end of the NPDCCH to the end of the following NPDSCH transmission; Note 2: This noise is applied to all subframes from the end of the NPDSCH to the end of the following NPDCCH transmission. |  |  |  |

Table 8.12.1.2.2-2: Minimum performance for NPDSCH under Standalone/Guard-band with 1 NRS port

| Test number | Bandwidth | Carrier Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Anchor | R.NB.6 TDD | 32 | EPA5 | 1 | 70% | [-3.4] | NB1, NB2 |
| 2 | 200kHz | Non-anchor | R.NB.6-1 TDD | 256 | ETU1 | 1 | 70% | [-10.2] | NB1, NB2 |

##### 8.12.1.2.3	Minimum Requirements for Standalone for UE Category NB2

The requirements are specified in Table 8.12.1.2.3-2, with the addition of the parameters in Table 8.12.1.2.3-1 and the downlink physical channel setup according to Annex C.3.6. The purpose of these tests is to verify the NPDSCH performance.

Table 8.12.1.2.3-1: Test Parameters for NPDSCH under Standalone

| Parameter | Unit |  | Test 1 |
| --- | --- | --- | --- |
| at antenna port |  | dBm/15kHz | -93 (Note 1) |
|  |  | dBm/15kHz | -96 (Note 2) |
| NPDCCH repetition number |  | subframe | 4 for Test 1 |
| (npdcch-NumRepetitions-r13) |  | subframe | 8 for Test 1 |
| (nPDCCH-startSF-USS-r13) |  |  | 4 |
| Note 1: This noise is applied to all subframes from the end of the NPDCCH to the end of the following NPDSCH transmission; Note 2: This noise is applied to all subframes from the end of the NPDSCH to the end of the following NPDCCH transmission. |  |  |  |

Table 8.12.1.2.3-2: Minimum performance for NPDSCH under Standalone with 1 NRS port

| Test number | Bandwidth | Carrer Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Non-anchor | R.NB.7 TDD | 1 | EPA5 | 1 | 70% | [9.4] | NB2 |

<Unchanged Sections Skipped>

## A.3.12	Reference measurement channels for NPDSCH performance requirements

### A.3.12.1	In-band

#### A.3.12.1.2	Two-antenna transmission

Table A.3.12.1.2-1: NPDSCH Reference Channel with 2 TX Antennas for FDD

| Parameter | Unit | Value | Value |
| --- | --- | --- | --- |
| Reference channel |  | R.NB.5 FDD | R.NB.5-1 FDD |
| Carrier Type |  | Anchor | Non-anchor |
| Channel bandwidth | KHz | 200 | 200 |
| Allocated subframes per Radio Frame |  | Note 2 | Note 2 |
| Modulation |  | QPSK | QPSK |
| ITBS/ISF |  | 4/0 | 4/0 |
| Target Coding Rate |  | 1/3 | 1/3 |
| Coding Rate |  | 0.4 | 0.4 |
| Information Bit Payload |  |  |  |
| For Sub-Frames 1,2,3,6,7,8 | Bits | 56 | 56 |
| For Sub-Frame 0,5 | Bits | N/A | 56 |
| For Sub-Frame 4,9 | Bits | Note 3 | 56 |
| Number of Code Blocks |  |  |  |
| For Sub-Frames 1,2,3,6,7,8 |  | 1 | 1 |
| For Sub-Frame 0,5 | Bits | N/A | 1 |
| For Sub-Frame 4,9 | Bits | Note 4 | 1 |
| Binary Channel Bits |  |  |  |
| For Sub-Frames 1,2,3,6,7,8 | Bits | 200 | 200 |
| For Sub-Frame 0,5 | Bits | N/A | 200 |
| For Sub-Frame 4,9 | Bits | Note 5 | 200 |
| Max. Averaged Throughput | Bps | Note 6 | Note 6 |
| UE Category |  | NB1,NB2 | NB1,NB2 |
| Note 1: For in-band, the first 3 symbols are used for LTE PDCCH and the number of LTE CRS ports is 4. Note 2: It shall depend on the specific NPDSCH scheduling. Note 3: N/A when mod 2 = 0, otherwise 56. Note 4: N/A when mod 2 = 0, otherwise 1. Note 5: N/A when mod 2 = 0, otherwise 200. Note 6: Maximum Average Throughput equals to sum of TB(i) divided by sum of T(i), where TB(i) is the TB size of NPDSCH over ith NPDSCH scheduling period, and T(i) is the total time consisting of NPDCCH transmission duration, NPDCCH to NPDSCH scheduling delay, NPDSCH transmission duration, NPDSCH to NPUSCH format 2 scheduling delay, NPUSCH format 2 transmission duration, possible delay between NPUSCH format 2 and NPDCCH for next NPDSCH scheduling and subframes used for NPSS/NSSS/NPBCH/NB-SIB1/NB-SIB2 transmission during the ith NPDSCH scheduling period. |  |  |  |

Table A.3.12.1.2-2: NPDSCH Reference Channel with 2 TX Antennas for TDD

| Parameter | Unit | Value | Value |
| --- | --- | --- | --- |
| Reference channel |  | R.NB.5 TDD | R.NB.5-1 TDD |
| Carrier Type |  | Anchor | Non-anchor |
| Channel bandwidth | KHz | 200 | 200 |
| Uplink-Downlink Configuration (Note 7) |  | 4 | 4 |
| Allocated subframes per Radio Frame |  | Note 2 | Note 2 |
| Modulation |  | QPSK | QPSK |
| ITBS/ISF |  | 4/0 | 4/0 |
| Target Coding Rate |  | 1/3 | 1/3 |
| Coding Rate |  | 0.4 | 0.4 |
| Information Bit Payload |  |  |  |
| For Sub-Frames 1,6,7,8 | Bits | 56 | 56 |
| For Sub-Frame 5 | Bits | N/A | Note 3 |
| For Sub-Frame 9 |  | N/A | 56 |
| For Sub-Frame 0 | Bits | Note 3 | Note 3 |
| For Sub-Frame 4 |  | Note 3 | 56 |
| Number of Code Blocks |  |  |  |
| For Sub-Frames 1,6,7,8 |  | 1 | 1 |
| For Sub-Frame 5 | Bits | N/A | Note 4 |
| For Sub-Frame 9 |  | N/A | 1 |
| For Sub-Frame 0 | Bits | Note 4 | Note 4 |
| For Sub-Frame 4 |  | Note 4 | 1 |
| Binary Channel Bits |  |  |  |
| For Sub-Frames 1,6,7,8 | Bits | 200 | 200 |
| For Sub-Frame 5 | Bits | N/A | Note 5 |
| For Sub-Frame 9 |  | N/A | 200 |
| For Sub-Frame 0 | Bits | Note 5 | Note 5 |
| For Sub-Frame 4 |  | Note 5 | 200 |
| Max. Averaged Throughput | Bps | Note 6 | Note 6 |
| UE Category |  | NB1,NB2 | NB1,NB2 |
| Note 1: For in-band, the first 3 symbols are used for LTE PDCCH and the number of LTE CRS ports is 4. Note 2: It shall depend on the specific NPDSCH scheduling. Note 3: N/A when mod 2 = 0, otherwise 56. Note 4: N/A when mod 2 = 0, otherwise 1. Note 5: N/A when mod 2 = 0, otherwise 200. Note 6: Maximum Average Throughput equals to sum of TB(i) divided by sum of T(i), where TB(i) is the TB size of NPDSCH over ith NPDSCH scheduling period, and T(i) is the total time consisting of NPDCCH transmission duration, NPDCCH to NPDSCH scheduling delay, NPDSCH transmission duration, NPDSCH to NPUSCH format 2 scheduling delay, NPUSCH format 2 transmission duration, possible delay between NPUSCH format 2 and NPDCCH for next NPDSCH scheduling and subframes used for NPSS/NSSS/NPBCH/NB-SIB1/NB-SIB2 transmission during the ith NPDSCH scheduling period. Note 7: As per Table 4.2-2 in TS 36.211 [4]. |  |  |  |

### A.3.12.2	Standalone/Guard-band

#### A.3.12.2.1 Single-antenna transmission

Table A.3.12.2.1-1: NPDSCH Reference Channel with 1Tx Antenna for UE Category NB1 and NB2 for FDD

| Parameter | Unit | Value | Value |
| --- | --- | --- | --- |
| Reference channel |  | R.NB.6 FDD | R.NB.6-1 FDD |
| Carrier Type |  | Anchor | Non-anchor |
| Channel bandwidth | KHz | 200 | 200 |
| Allocated subframes per Radio Frame |  | Note 1 | Note 1 |
| Modulation |  | QPSK | QPSK |
| ITBS/ISF |  | 9/3 | 6/3 |
| Target Coding Rate |  | 1/2 | 1/3 |
| Coding Rate |  | 0.5 | 0.33 |
| Information Bit Payload |  |  |  |
| For Sub-Frames 1,2,3,6,7,8 | Bits | 616 | 392 |
| For Sub-Frame 0,5 | Bits | N/A | 392 |
| For Sub-Frame 4,9 | Bits | Note 2 | 392 |
| Number of Code Blocks |  |  |  |
| For Sub-Frames 1,2,3,6,7,8 |  | 1 | 1 |
| For Sub-Frame 0,5 | Bits | N/A | 1 |
| For Sub-Frame 4,9 | Bits | Note 3 | 1 |
| Binary Channel Bits |  |  |  |
| For Sub-Frames 1,2,3,6,7,8 | Bits | 320 | 320 |
| For Sub-Frame 0,5 | Bits | N/A | 320 |
| For Sub-Frame 4,9 | Bits | Note 4 | 320 |
| Max. Average Throughput | Bps | Note 5 | Note 5 |
| UE Category |  | NB1,NB2 | NB1,NB2 |
| Note 1: It shall depend on the specific NPDSCH scheduling. Note 2: N/A when mod 2 = 0, otherwise 616. Note 3: N/A when mod 2 = 0, otherwise 1. Note 4: N/A when mod 2 = 0, otherwise 320. Note 5: Maximum Average Throughput equals to sum of TB(i) divided by sum of T(i), where TB(i) is the TB size of NPDSCH over ith NPDSCH scheduling period, and T(i) is the total time consisting of NPDCCH transmission duration, NPDCCH to NPDSCH scheduling delay, NPDSCH transmission duration, NPDSCH to NPUSCH format 2 scheduling delay, NPUSCH format 2 transmission duration, possible delay between NPUSCH format 2 and NPDCCH for next NPDSCH scheduling and subframes used for NPSS/NSSS/NPBCH/NB-SIB1/NB-SIB2 transmission during the ith NPDSCH scheduling period. |  |  |  |

Table A.3.12.2.1-1a: NPDSCH Reference Channel with 1Tx Antenna for UE Category NB1 and NB2 for TDD

| Parameter | Unit | Value | Value |
| --- | --- | --- | --- |
| Reference channel |  | R.NB.6 TDD | R.NB.6-1 TDD |
| Carrier Type |  | Anchor | Non-anchor |
| Channel bandwidth | KHz | 200 | 200 |
| Uplink-Downlink Configuration (Note 7) |  | 4 | 4 |
| Allocated subframes per Radio Frame |  | Note 1 | Note 1 |
| Modulation |  | QPSK | QPSK |
| ITBS/ISF |  | 9/3 | 6/3 |
| Target Coding Rate |  | 1/2 | 1/3 |
| Coding Rate |  | 0.5 | 0.33 |
| Information Bit Payload |  |  |  |
| For Sub-Frames 1,6,7,8 | Bits | 616 | 392 |
| For Sub-Frame 5 | Bits | N/A | Note 3 |
| For Sub-Frame 9 |  | N/A | 392 |
| For Sub-Frame 0 | Bits | Note 2 | Note 3 |
| For Sub-Frame 4 |  | Note 2 | 392 |
| Number of Code Blocks |  |  |  |
| For Sub-Frames 1,6,7,8 |  | 1 | 1 |
| For Sub-Frame 5 | Bits | N/A | Note 4 |
| For Sub-Frame 9 |  | N/A | 1 |
| For Sub-Frame 0 | Bits | Note 4 | Note 4 |
| For Sub-Frame 4 |  | Note 4 | 1 |
| Binary Channel Bits |  |  |  |
| For Sub-Frames 1,6,7,8 | Bits | 320 | 320 |
| For Sub-Frame 5 | Bits | N/A | Note 5 |
| For Sub-Frame 9 |  | N/A | 320 |
| For Sub-Frame 0 | Bits | Note 5 | Note 5 |
| For Sub-Frame 4 |  | Note 5 | 320 |
| Max. Average Throughput | Bps | Note 6 | Note 6 |
| UE Category |  | NB1,NB2 | NB1,NB2 |
| Note 1: It shall depend on the specific NPDSCH scheduling. Note 2: N/A when mod 2 = 0, otherwise 616. Note 3: N/A when mod 2 = 0, otherwise 392. Note 4: N/A when mod 2 = 0, otherwise 1. Note 5: N/A when mod 2 = 0, otherwise 320. Note 6: Maximum Average Throughput equals to sum of TB(i) divided by sum of T(i), where TB(i) is the TB size of NPDSCH over ith NPDSCH scheduling period, and T(i) is the total time consisting of NPDCCH transmission duration, NPDCCH to NPDSCH scheduling delay, NPDSCH transmission duration, NPDSCH to NPUSCH format 2 scheduling delay, NPUSCH format 2 transmission duration, possible delay between NPUSCH format 2 and NPDCCH for next NPDSCH scheduling and subframes used for NPSS/NSSS/NPBCH/NB-SIB1/NB-SIB2 transmission during the ith NPDSCH scheduling period. Note 7: As per Table 4.2-2 in TS 36.211 [4]. |  |  |  |

Table A.3.12.2.1-2: NPDSCH Reference Channel with 1Tx Antenna for UE Category NB2 for FDD

| Parameter | Unit | Value | Value |
| --- | --- | --- | --- |
| Reference channel |  | R.NB.7 FDD |  |
| Carrier Type |  | Non-anchor |  |
| Channel bandwidth | KHz | 200 |  |
| Allocated subframes per Radio Frame |  | Note 1 |  |
| Modulation |  | QPSK |  |
| ITBS/ISF |  | 9/5 |  |
| Target Coding Rate |  | 1/2 |  |
| Coding Rate |  | 0.5 |  |
| Information Bit Payload |  |  |  |
| For Sub-Frames 1,2,3,6,7,8 | Bits | 936 |  |
| For Sub-Frame 0,5 | Bits | 936 |  |
| For Sub-Frame 4,9 | Bits | 936 |  |
| Number of Code Blocks |  |  |  |
| For Sub-Frames 1,2,3,6,7,8 |  | 1 |  |
| For Sub-Frame 0,5 | Bits | 1 |  |
| For Sub-Frame 4,9 | Bits | 1 |  |
| Binary Channel Bits |  |  |  |
| For Sub-Frames 1,2,3,6,7,8 | Bits | 320 |  |
| For Sub-Frame 0,5 | Bits | 320 |  |
| For Sub-Frame 4,9 | Bits | 320 |  |
| Max. Average Throughput | Bps | Note 2 |  |
| UE Category |  | NB2 |  |
| Note 1: It shall depend on the specific NPDSCH scheduling. Note 2: Maximum Average Throughput equals to sum of TB(i) divided by sum of T(i), where TB(i) is the TB size of NPDSCH over ith NPDSCH scheduling period, and T(i) is the total time consisting of NPDCCH transmission duration, NPDCCH to NPDSCH scheduling delay, NPDSCH transmission duration, NPDSCH to NPUSCH format 2 scheduling delay, NPUSCH format 2 transmission duration, possible delay between NPUSCH format 2 and NPDCCH for next NPDSCH scheduling and subframes used for NPSS/NSSS/NPBCH/NB-SIB1/NB-SIB2 transmission during the ith NPDSCH scheduling period. |  |  |  |

Table A.3.12.2.1-2a: NPDSCH Reference Channel with 1Tx Antenna for UE Category NB2 for TDD

| Parameter | Unit | Value | Value |
| --- | --- | --- | --- |
| Reference channel |  | R.NB.7 TDD |  |
| Carrier Type |  | Non-anchor |  |
| Channel bandwidth | KHz | 200 |  |
| Uplink-Downlink Configuration (Note 7) |  | 4 |  |
| Allocated subframes per Radio Frame |  | Note 1 |  |
| Modulation |  | QPSK |  |
| ITBS/ISF |  | 9/5 |  |
| Target Coding Rate |  | 1/2 |  |
| Coding Rate |  | 0.5 |  |
| Information Bit Payload |  |  |  |
| For Sub-Frames 1,6,7,8 | Bits | 936 |  |
| For Sub-Frame 0,5 | Bits | Note 2 |  |
| For Sub-Frame 4,9 | Bits | 936 |  |
| Number of Code Blocks |  |  |  |
| For Sub-Frames 1,2,3,6,7,8 |  | 1 |  |
| For Sub-Frame 0,5 | Bits | Note 3 |  |
| For Sub-Frame 4,9 | Bits | 1 |  |
| Binary Channel Bits |  |  |  |
| For Sub-Frames 1,2,3,6,7,8 | Bits | 320 |  |
| For Sub-Frame 0,5 | Bits | Note 4 |  |
| For Sub-Frame 4,9 | Bits | 320 |  |
| Max. Average Throughput | Bps | Note 5 |  |
| UE Category |  | NB2 |  |
| Note 1: It shall depend on the specific NPDSCH scheduling. Note 2: N/A when mod 2 = 0, otherwise 936. Note 3: N/A when mod 2 = 0, otherwise 1. Note 4: N/A when mod 2 = 0, otherwise 320. Note 5: Maximum Average Throughput equals to sum of TB(i) divided by sum of T(i), where TB(i) is the TB size of NPDSCH over ith NPDSCH scheduling period, and T(i) is the total time consisting of NPDCCH transmission duration, NPDCCH to NPDSCH scheduling delay, NPDSCH transmission duration, NPDSCH to NPUSCH format 2 scheduling delay, NPUSCH format 2 transmission duration, possible delay between NPUSCH format 2 and NPDCCH for next NPDSCH scheduling and subframes used for NPSS/NSSS/NPBCH/NB-SIB1/NB-SIB2 transmission during the ith NPDSCH scheduling period. |  |  |  |