3GPP TSG-RAN4 Meeting #90	R4-1901192

# Athens, Greece, 25th February – 1st March 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on clarification on UE timing requirements in NE-DC (section 7.1.1) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The In UE transmit timing requirements, the reference cell for uplink trtansmission is not clairfied for NE-DC and and NR-DC operations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To clairfy that the the reference cell under NE-DC and NR-DC operations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The reference cell for uplink trtansmission may be not properly defined under NE-DC and NR-DC operations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

7.1.1	Introduction

The UE shall have capability to follow the frame timing change of the connected gNB. The uplink frame transmission takes place  before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell. The UE shall be configured with a psTAG containing the PSCell and the reference cell is PSCell in case of EN-DC. The UE shall be configured with a pTAG containing the PCell and the reference cell is PCell in case of NR standalone including NE-DC operation.

The UE capable of supporting NR-DC operation shall be configured with one pTAG and may also be configured with one psTAG. UE shall use the PCell as the reference cell for deriving the UE transmit timing for cells in the pTAG. If the UE is configured with a psTAG containing the PSCell, UE shall use the PSCell as the reference cell for deriving the UE transmit timing for cells in the psTAG.UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are defined in the following requirements. The requirements in clause 7 apply to all TAGs.

<End of Change 1>