3GPP TSG-RAN WG4 Meeting #90	R4-1901245

# Athens, Greece, 25 February - 1 March 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.141 | CR | 1203 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of PUSCH 256QAM 8Rx conformance test requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_UL_CAP_enh-Perf |  |  |  |  |  | Date: |  |  | 2019-02-28 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | PUSCH 256QAM 8Rx 10MHz conformance test requirements in TS36.141 was set based on 4Rx requirements in TS36.104. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct PUSCH 256QAM 8Rx 10MHz conformance requirement in TS36.141 from 19.8dB to 16.5dB by adding 0.6dB to TS36.104 (15.9dB). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The comformance requirement is not aligned with core requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change ------------------------------------------------------------

Table 8.2.1.5-4: Test requirements for PUSCH, 10 MHz Channel Bandwidth

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | EPA 5Hz Low | A3-5 | 30% | -3.6 |
|  |  |  |  |  | 70% | 0.2 |
|  |  |  |  | A4-6 | 70% | 11.4 |
|  |  |  |  | A5-5 | 70% | 18.9 |
|  |  |  |  | A17-4 | 70% | 23.2 |
|  |  |  |  | A18-4 | 70% | 9.1 |
|  |  |  |  | A19-4 | 70% | 20.1 |
|  |  |  | EVA 5Hz Low | A3-1 | 30% | -2.1 |
|  |  |  |  |  | 70% | 2.5 |
|  |  |  |  | A4-1 | 30% | 4.9 |
|  |  |  |  |  | 70% | 12.0 |
|  |  |  |  | A5-1 | 70% | 19.4 |
|  |  |  | EVA 70Hz Low | A3-5 | 30% | -3.5 |
|  |  |  |  |  | 70% | 0.7 |
|  |  |  |  | A4-6 | 30% | 5.1 |
|  |  |  |  |  | 70% | 13.2 |
|  |  |  | ETU 70Hz* Low | A3-1 | 30% | -1.9 |
|  |  |  |  |  | 70% | 3.0 |
|  |  |  | ETU 300Hz* Low | A3-1 | 30% | -1.6 |
|  |  |  |  |  | 70% | 3.5 |
|  |  | Extended | ETU 70Hz* Low | A4-2 | 30% | 5.4 |
|  |  |  |  |  | 70% | 14.2 |
|  | 4 | Normal | EPA 5Hz Low | A3-5 | 30% | -6.2 |
|  |  |  |  |  | 70% | -2.9 |
|  |  |  |  | A4-6 | 70% | 8.1 |
|  |  |  |  | A5-5 | 70% | 15.3 |
|  |  |  |  | A17-4 | 70% | 19.8 |
|  |  |  |  | A18-4 | 70% | 5.9 |
|  |  |  |  | A19-4 | 70% | 16.4 |
|  |  |  | EVA 5Hz Low | A3-1 | 30% | -4.4 |
|  |  |  |  |  | 70% | -0.6 |
|  |  |  |  | A4-1 | 30% | 1.8 |
|  |  |  |  |  | 70% | 8.5 |
|  |  |  |  | A5-1 | 70% | 16.1 |
|  |  |  | EVA 70Hz Low | A3-5 | 30% | -6.1 |
|  |  |  |  |  | 70% | -2.3 |
|  |  |  |  | A4-6 | 30% | 1.3 |
|  |  |  |  |  | 70% | 8.6 |
|  |  |  | ETU 70Hz* Low | A3-1 | 30% | -4.2 |
|  |  |  |  |  | 70% | -0.3 |
|  |  |  | ETU 300Hz* Low | A3-1 | 30% | -4.0 |
|  |  |  |  |  | 70% | 0.0 |
|  |  |  | ETU 600Hz** Low | A13-4 | 30% | -0.4 |
|  |  |  |  |  | 70% | 6.8 |
|  |  | Extended | ETU 70Hz* Low | A4-2 | 30% | 2.3 |
|  |  |  |  |  | 70% | 10.9 |
|  | 8 | Normal | EPA 5Hz Low | A3-5 | 30% | -9.2 |
|  |  |  |  |  | 70% | -6.1 |
|  |  |  |  | A4-6 | 70% | 4.8 |
|  |  |  |  | A5-5 | 70% | 12.1 |
|  |  |  |  | A17-4 | 70% | 16.5 |
|  |  |  |  | A18-4 | 70% | 2.7 |
|  |  |  |  | A19-4 | 70% | 13.1 |
|  |  |  | EVA 5Hz Low | A3-1 | 30% | -6.3 |
|  |  |  |  |  | 70% | -3.2 |
|  |  |  |  | A4-1 | 30% | -1.1 |
|  |  |  |  |  | 70% | 5.1 |
|  |  |  |  | A5-1 | 70% | 12.5 |
|  |  |  | EVA 70Hz Low | A3-5 | 30% | -9.1 |
|  |  |  |  |  | 70% | -5.6 |
|  |  |  |  | A4-6 | 30% | -2.0 |
|  |  |  |  |  | 70% | 5.3 |
|  |  |  | ETU 70Hz* Low | A3-1 | 30% | -6.2 |
|  |  |  |  |  | 70% | -3.0 |
|  |  |  | ETU 300Hz* Low | A3-1 | 30% | -6.2 |
|  |  |  |  |  | 70% | -2.7 |
|  |  | Extended | ETU 70Hz* Low | A4-2 | 30% | -0.5 |
|  |  |  |  |  | 70% | 7.1 |
| Note*: Not applicable for Local Area BS and Home BS. Note**: Not applicable for Local Area BS and Home BS, and only applicable for BS supporting ETU600. |  |  |  |  |  |  |

------------------------------------------------------------- End of change ------------------------------------------------------------