3GPP TSG-RAN WG4 Meeting #90	R4-1901248

# Athens, Greece, 25 February - 1 March 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 5373 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of UE demodulation requirements for SPDCCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_sTTIandPT-Perf |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Several SPDCCH demodulation requirements are set to TBD |  |  |  |  |  |  |  |  |
| Summary of change: |  | Set the SPDCCH demodulation requirements Clarification of parameters |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SPDCCH demodulation requirements are incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.14.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS36.521 ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change ------------------------------------------------------------

### 8.14.2	SPDCCH

The receiver characteristics of the SPDCCH are determined by the probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg).

#### 8.14.2.1	FDD

The parameters specified in Table 8.14.2.1-1 and Table 8.14.2.1-2 are valid for all FDD tests unless otherwise stated.

Table 8.14.2.1-1: Test Parameters for SPDCCH

| Parameter |  | Unit | Test 1 Test 3 | Test 2 Test 4 |
| --- | --- | --- | --- | --- |
| Number of PDCCH symbols |  | symbols | 2 | 2 |
| Unused RE-s and PRB-s |  |  | OCNG | OCNG |
| Cell ID |  |  | 0 | 0 |
| Downlink power allocation |  | dB | -6 | -3 |
|  |  | dB | -6 | -3 |
|  |  | dB | 0 | 0 |
|  | δ | dB | N/A | 3 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Cyclic prefix |  |  | Normal | Normal |
| Subframe configuration |  |  | Non-MBSFN | Non-MBSFN |
| Precoder update granularity |  | SREG | N/A | 2 |
|  |  | ms | N/A | 1 |
| Beamforming precoder |  |  | N/A | Annex B.4.4 |
| Cell specific reference signal |  |  | Antenna ports 0,1,2,3 | Antenna ports 0,1 |
| Configured SPDCCH-PRB sets |  |  | {Set 1} | {Set 2} |
| SPDCCH-PRB set used for DCI transmission |  |  | Set 1 | Set 2 |
| PDSCH transmission mode |  |  | TM4 | TM9 |
| DCI format |  |  | 7-1C | 7-1F |
| Note 1: For slot-based transmission, DCI 7-1C and 7-1F are transmitted only on SPDCCH in slot 1. For subslot-based transmission, DCI 7-1C and 7-1F are transmitted on SPDCCH in all the subslots. |  |  |  |  |

Table 8.14.2.1-2: SPDCCH-PRB set parameters

| Parameter | Unit | Set 1 | Set 2 |
| --- | --- | --- | --- |
| Refence symbol |  | CRS | DMRS |
| Transmission type |  | Localized | Distributed |
| Number of OFDM symbols |  | 1 | Note 2 |
| Number of PRBs per SPDCCH-PRB set (Note 1) |  | 16 | 16 |
| Note 1: The two SPDCCH-PRB sets are non-overlapping with PRB = {0, 1, …, 15} for Set 1 and PRB = {0, 1, 6, 7, 12, 13, 18, 19, 28, 29, 34, 35, 40, 41, 46, 47} for Set 2. Note 2: 2 OFDM symbols for slot-based SPDCCH. 2 OFDM symbols for subslot indices 2, 3, and 4, and 3 OFDM symbols for subslot indices 1 and 5 for subslot-based SPDCCH. |  |  |  |

##### 8.14.2.1.1	Mimimum requirement

For the parameters specified in Table 8.14.2.1-1 and Table 8.14.2.1-2 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.14.2.1.1-1. The metric Pm-dsg is calculated across all the slots for slot-based SPDCCH and across all the subslots for subslot-based SPDCCH. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.14.2.1.1-1: Minimum performance SPDCCH

| Test number | Bandwidth | DL TTI length | Aggregation Level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 MHz | Slot | 2 SCCE | R.sTTI.10 FDD | OP.7 FDD | EVA30 | 4 x 2 Low | 1 | [2.9] |
| 2 | 10 MHz | Slot | 8 SCCE | R.sTTI.11 FDD | OP.7 FDD | EPA5 | 2 x 2 Low | 1 | TBD |
| 3 | 10 MHz | Subslot | 2 SCCE | R.sTTI.10 FDD | OP.7 FDD | EVA30 | 4 x 2 Low | 1 | [2.3] |
| 4 | 10 MHz | Subslot | 8 SCCE | R.sTTI.11 FDD | OP.7 FDD | EPA5 | 2 x 2 Low | 1 | TBD |

#### 8.14.2.2	TDD

The parameters specified in Table 8.14.2.2-1 and Table 8.14.2.2-2 are valid for all TDD tests unless otherwise stated.

Table 8.14.2.2-1: Test Parameters for SPDCCH

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Number of PDCCH symbols |  | symbols | 2 | 2 |
| Unused RE-s and PRB-s |  |  | OCNG | OCNG |
| Cell ID |  |  | 0 | 0 |
| Downlink power allocation |  | dB | -6 | -3 |
|  |  | dB | -6 | -3 |
|  |  | dB | 0 | 0 |
|  | δ | dB | N/A | 3 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Cyclic prefix |  |  | Normal | Normal |
| Subframe configuration |  |  | Non-MBSFN | Non-MBSFN |
| Precoder update granularity |  | SREG | N/A | 2 |
|  |  | ms | N/A | 1 |
| Beamforming precoder |  |  | N/A | Annex B.4.4 |
| Cell specific reference signal |  |  | Antenna ports 0,1,2,3 | Antenna ports 0,1 |
| Configured SPDCCH-PRB sets |  |  | {Set 1} | {Set 2} |
| SPDCCH-PRB set used for DCI transmission |  |  | Set 1 | Set 2 |
| PDSCH transmission mode |  |  | TM4 | TM9 |
| DCI format |  |  | 7-1C | 7-1F |
| TDD UL/DL Configuration |  |  | 0 | 0 |
| TDD Special Subframe |  |  | 1 | 1 |
| Note 1: For slot-based transmission, DCI 7-1C and 7-1F are transmitted only on SPDCCH in slot 1. For subslot-based transmission, DCI 7-1C and 7-1F are transmitted on SPDCCH in all the subslots. |  |  |  |  |

Table 8.14.2.2-2: SPDCCH-PRB set parameters

| Parameter | Unit | Set 1 | Set 2 |
| --- | --- | --- | --- |
| Refence symbol |  | CRS | DMRS |
| Transmission type |  | Localized | Distributed |
| Number of OFDM symbols |  | 1 | Note 2 |
| Number of PRBs per SPDCCH-PRB set (Note 1) |  | 16 | 16 |
| Note 1: The two SPDCCH-PRB sets are non-overlapping with PRB = {0, 1, …, 15} for Set 1 and PRB = {0, 1, 6, 7, 12, 13, 18, 19, 28, 29, 34, 35, 40, 41, 46, 47} for Set 2. Note 2: 2 OFDM symbols for slot-based SPDCCH. 2 OFDM symbols for subslot indices 2, 3, and 4, and 3 OFDM symbols for subslot indices 1 and 5 for subslot-based SPDCCH. |  |  |  |

##### 8.14.2.2.1	Mimimum requirement

For the parameters specified in Table 8.14.2.2-1 and Table 8.14.2.2-2 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.14.2.2.1-1. The metric Pm-dsg is calculated across all the slots. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.14.2.2.1-1: Minimum performance SPDCCH

| Test number | Bandwidth | DL TTI length | Aggregation Level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 MHz | Slot | 2 SCCE | R.sTTI.10TDD | OP.7 TDD | EVA30 | 4 x 2 Low | 1 | TBD |
| 2 | 10 MHz | Slot | 8 SCCE | R.sTTI.11 TDD | OP.7 TDD | EPA5 | 2 x 2 Low | 1 | TBD |

------------------------------------------------------------- End of change ------------------------------------------------------------