3GPP TSG-RAN WG4 Meeting #90	R4-1901272

Athens, Greece, 25 February – 1 March 2019

Agenda Item:	6.7.6

Source:	Ericsson

Title:	HARQ-ACK transmission timing for DL RMC for FR1 TDD SCS=60kHz

Document for:	Discussion

# 1	Introduction

RAN4#89 modified the RMC for FR1 UE RF tests so that the NR UL/DL timing is aligned with LTE TDD UL/DL configuration 2 (DSUDD DSUDD) to avoid cross link interference during the test as follows [1][2].

Table 1	TDD UL/DL configuration used for FR1 TDD RF test (TS38.101-1)

| Parameter |  | Value |  |  |
| --- | --- | --- | --- | --- |
|  |  | SCS 15 kHz (µ=0) | SCS 30 kHz (µ=1) | SCS 60 kHz (µ=2) |
| UL-DL configuration | referenceSubcarrierSpacing | 15 kHz | 30 kHz | 60 kHz |
|  | dl-UL-TransmissionPeriodicity | 5 ms | 5 ms | 5 ms |
|  | nrofDownlinkSlots | 3 | 7 | 14 |
|  | nrofDownlinkSymbols | 10 | 6 | 12 |
|  | nrofUplinkSlot | 1 | 2 | 4 |
|  | nrofUplinkSymbols | 2 | 4 | 8 |
| Number of HARQ Processes |  | 8 | 8 | 16 |
| K1 value |  | K1 = 4 if mod(i,5) = 0 K1 =3 if mod(i,5) = 1 K1 =2 if mod(i,5) = 2 where i is slot index per frame; i = {0,…,9} | K1 =8 if mod(i,10) = 0 K1 =7 if mod(i,10) = 1 K1 =6 if mod(i,10) = 2 K1 =5 if mod(i,10) = 3 K1 =4 if mod(i,10) = 4 K1 =3 if mod(i,10) = 5 K1 =2 if mod(i,10) = 6 where i is slot index per frame; i = {0,…,19} | K1 = 15 if mod(i,20) = 0 K1 =14 if mod(i,20) = 1 K1 =13 if mod(i,20) = 2 K1 =12 if mod(i,20) = 3 K1 =11 if mod(i,20) = 4 K1 =10 if mod(i,20) = 5 K1 =9 if mod(i,20) = 6 K1 =8 if mod(i,20) = 7 K1 =7 if mod(i,20) = 8 K1 =6 if mod(i,20) = 9 K1 =5 if mod(i,20) = 10 K1 =4 if mod(i,20) = 11 K1 =3 if mod(i,20) = 12 K1 =2 if mod(i,20) = 13 where i is slot index per frame; i = {0,…,39} |

After the review, it is found it is not possible to configure more than 8 K1 values according to TS38.331 as follows. This means the current RMC for SCS 60kHz in Table 1 cannot be configurable and we need to fix it. In this contribution we propose the new TDD UL/DL configuration which is possible to implement.

PUCCH-Config ::= SEQUENCE { resourceSetToAddModList SEQUENCE (SIZE (1..maxNrofPUCCH-ResourceSets)) OF PUCCH-ResourceSet OPTIONAL, -- Need N resourceSetToReleaseList SEQUENCE (SIZE (1..maxNrofPUCCH-ResourceSets)) OF PUCCH-ResourceSetId OPTIONAL, -- Need N resourceToAddModList SEQUENCE (SIZE (1..maxNrofPUCCH-Resources)) OF PUCCH-Resource OPTIONAL, -- Need N resourceToReleaseList SEQUENCE (SIZE (1..maxNrofPUCCH-Resources)) OF PUCCH-ResourceId OPTIONAL, -- Need N format1 SetupRelease { PUCCH-FormatConfig } OPTIONAL, -- Need M format2 SetupRelease { PUCCH-FormatConfig } OPTIONAL, -- Need M format3 SetupRelease { PUCCH-FormatConfig } OPTIONAL, -- Need M format4 SetupRelease { PUCCH-FormatConfig } OPTIONAL, -- Need M schedulingRequestResourceToAddModList SEQUENCE (SIZE (1..maxNrofSR-Resources)) OF SchedulingRequestResourceConfig OPTIONAL, -- Need N schedulingRequestResourceToReleaseList SEQUENCE (SIZE (1..maxNrofSR-Resources)) OF SchedulingRequestResourceId OPTIONAL, -- Need N multi-CSI-PUCCH-ResourceList SEQUENCE (SIZE (1..2)) OF PUCCH-ResourceId OPTIONAL, -- Need M dl-DataToUL-ACK SEQUENCE (SIZE (1..8)) OF INTEGER (0..15) OPTIONAL, -- Need M spatialRelationInfoToAddModList SEQUENCE (SIZE (1..maxNrofSpatialRelationInfos)) OF PUCCH-SpatialRelationInfo OPTIONAL, -- Need N spatialRelationInfoToReleaseList SEQUENCE (SIZE (1..maxNrofSpatialRelationInfos)) OF PUCCH-SpatialRelationInfoId OPTIONAL, -- Need N pucch-PowerControl PUCCH-PowerControl OPTIONAL, -- Need M ... }

# 2	Proposed configurations

To design the UL/DL configuration for SCS 60kHz, we need to consider:

- TDD UL/DL pattern should be aligned with LTE UL/DL configuration 2, that is, 14D2S4U.

- This configuration should support DL scheduling (PDCCH 1_x + PDSCH) and UL grant (PDCCH 0_x + PUSCH) in the test.

- Number of K1 values are up to 8.

- Minimum impact to the agreed RMC to minimize the impact to RAN5.

Firstly, to ensure PUSCH transmission in UL slots 16 to 19, K2 (UL grant in slot N and corresponding uplink data transmission in slot N+K2) should be 6 or larger. When we set K2, we need to consider HARQ-ACK scheduling delay (K0+K1) should be K2, where K0 is the slot delay between PDCCH DCI and PDSCH reception, and K1 corresponds to dl-DataToUL-ACK. Since K0=0 (DCI reception and PDSCH reception in the same slot), K1 ≥ K2 ≥ 6.

Moreover, when we look the existing DL RMC such as Table A.3.3.2-3, A.3.3.3-3, or A.3.3.4-3, no PDSCH is scheduled in slots 0 and 1 to avoid conflict with SS/PBCH block. This means we don’t need to consider K1 in DL slots 0 and 1.

With these aspects, the proposed UL/DL scheduling is given as Figure 1. For PDSCH in slots 2 to 9, HARQ-ACK is transmitted in special slots 15 where 8 UL symbols can be allocated.

Figure 1	Proposed UL/DL scheduling for FR1 UE RF tests with TDD SCS 60kHz.

# 3	Conclusion

Proposal: Modify the NR TDD UL/DL configuration for FR1 UE RF tests with SCS 60kHz as follows:

| Parameter |  | Value |  |  |
| --- | --- | --- | --- | --- |
|  |  | SCS 15 kHz (µ=0) | SCS 30 kHz (µ=1) | SCS 60 kHz (µ=2) |
| UL-DL configuration | referenceSubcarrierSpacing | 15 kHz | 30 kHz | 60 kHz |
|  | dl-UL-TransmissionPeriodicity | 5 ms | 5 ms | 5 ms |
|  | nrofDownlinkSlots | 3 | 7 | 14 |
|  | nrofDownlinkSymbols | 10 | 6 | 12 |
|  | nrofUplinkSlot | 1 | 2 | 4 |
|  | nrofUplinkSymbols | 2 | 4 | 8 |
| Number of HARQ Processes |  | 8 | 8 | 16 |
| K1 value |  | K1 = 4 if mod(i,5) = 0 K1 =3 if mod(i,5) = 1 K1 =2 if mod(i,5) = 2 where i is slot index per frame; i = {0,…,9} | K1 =8 if mod(i,10) = 0 K1 =7 if mod(i,10) = 1 K1 =6 if mod(i,10) = 2 K1 =5 if mod(i,10) = 3 K1 =4 if mod(i,10) = 4 K1 =3 if mod(i,10) = 5 K1 =2 if mod(i,10) = 6 where i is slot index per frame; i = {0,…,19} | K1 = 15 if mod(i,20) = 0 K1 =14 if mod(i,20) = 1 K1 =13 if mod(i,20) = 2 K1 =12 if mod(i,20) = 3 K1 =11 if mod(i,20) = 4 K1 =10 if mod(i,20) = 5 K1 =9 if mod(i,20) = 6 K1 =8 if mod(i,20) = 7 K1 =7 if mod(i,20) = 8 K1 =6 if mod(i,20) = 9 K1 =6 if mod(i,20) = 10 K1 =6 if mod(i,20) = 11 K1 =6 if mod(i,20) = 12 K1 =6 if mod(i,20) = 13 where i is slot index per frame; i = {0,…,39} |

# References

- R4-1816604, “TDD configuration for UE Tx test in FR1”, Ericsson.

- R4-1816610, “WF on TDD configuration for UE Tx test”, Ericsson.