3GPP TSG-RAN WG4 Meeting #90	R4-1901459

Athens, Greece, 25 February 2019 – 1 March 2019

Source:	Telstra, Ericsson

Title:	TP to TR 37.716-31-11: Addition of EN-DC configuration DC_3C-7A-28A_n78A_BCS0 and DC_3C-7C-28A_n78A_BCS0

Agenda item:	9.5.2

Document for:	Approval

# Background

This contribution provides text proposal to TR 37.716-31-11 for addition of EN-DC configuration DC_3C-7A-28A_n78A_BCS0 and DC_3C-7C-28A_n78A_BCS0 as defined in [1].

# Text Proposal

##### ---Start of changes---

## 5.1.x	DC_3C-7A-28A_n78A_BCS0
DC_3C-7C-28A_n78A_BCS0

#### 5.1.x.1	Operating bands for DC

Table 5.1.x.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-7-28_n781 | CA_3-7-28 | n78 | DC_3_n78 |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability |  |  |  |

#### 5.1.x.2	Configuration for DC

Table 5.1.x.2-1:  Inter-band EN-DC configurations of 3 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3C-7A-28A_n78A | DC_3A_n78A DC_7A_n78A DC_28A_n78A | CA_3C-7A-28A | n78A |
| DC_3C-7C-28A_n78A | DC_3A_n78A DC_7A_n78A DC_28A_n78A | CA_3C-7C-28A | n78A |

#### 5.1.x.3	∆TIB and ∆RIB values

For DC_3-7-28_n78 the TIB,c and RIB,c values are given in the tables below.

Table 5.1.x.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-28_n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 28 | 0.6 |
|  | n78 | 0.8 |

Table 5.1.x.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7-28_n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 28 | 0.2 |
|  | n78 | 0.5 |

### 5.1.x.4	REFSENS requirements

No further MSD are needed to be specified for DC_3C-7A-28A_n78A, DC_3C-7C-28A_n78A.

##### ---End of changes---

# Reference

[1]	RP-1812257, “Revised WID on Dual Connectivity (EN-DC) of 3 bands LTE inter-band CA (3DL/1UL) and 1 NR band (1DL/1UL)”, Ericsson