3GPP TSG RAN WG4 Meeting #90	R4-1901493

Athens, Greece, February 25 – March 1, 2019

Agenda Item:	6.10.2.3

Source:	Huawei, HiSilicon

Title:	Discussion of filling remaining RBs in PDCCH symbols of test models

Document for:	Approval

# Introduction

During the drafting of the TPs for the test models in subclause 4.9.2.2 of 38.141-1 [2] and 38.141-2 [3], there were implementation concerns for filling RBs in the first part of the slot. A way forward was drafted in the last meeting to discuss some options [1]. This contribution examines the options and provides a proposal for the filling of the control region.

# Background

## Description

As shown in Figure 1, a test model slot consists of a 2-symbol region for “control signaling” (orange / green) followed by a 12-symbol shared channel region (gray).

NRB-3 (fill)

3 (PDCCH)

2 symbols

12 symbols

(PDSCH)

Figure 1. Current layout

The PDSCH occupancy as a function of test model is shown in Table 1. For all tests except for TM2, the shared channel region is transmitted at full power.

Table 1. PDSCH region occupancy

| Test Model | FR1 | FR2 |
| --- | --- | --- |
| 1.1 | Fully occupied | Fully occupied |
| 1.2 | Fully occupied; Supports 2 UEs | N/A |
| 2 (TM2a for FR1) | 1 RB | 1 RB |
| 3.1 (TM3.1a for FR1) | Fully occupied | Fully occupied |
| 3.2 | Fully occupied; Supports 2 UEs | N/A |
| 3.2 | Fully occupied; Supports 2 UEs | N/A |

In contrast, in each symbol of the control region, only 3 RBs are used as shown in Figure 1. The remaining (NRB-3) RBs are unfilled (orange in Figure 1). The question is whether more RBs need to be occupied in the control region. The amount of filling is a function of test models. A summary of test models and measured signal features is presented in Table 2.

Table 2. Tested signal features. Green shading is for both FR1 and FR2. Yellow shading is FR1.

| Feature | TM1.1 | TM1.2 | TM2 | TM3.1 | TM3.2 | TM3.3 |
| --- | --- | --- | --- | --- | --- | --- |
| BS output power |  |  |  |  |  |  |
| TAE |  |  |  |  |  |  |
| Occupied bandwidth |  |  |  |  |  |  |
| ACLR |  |  |  |  |  |  |
| Operating band unwanted emissions |  |  |  |  |  |  |
| Transmitter spurious emissions |  |  |  |  |  |  |
| Transmitter intermodulation |  |  |  |  |  |  |
| Total power dynamic range |  |  |  |  |  |  |
| EVM |  |  |  |  |  |  |
| Frequency error |  |  |  |  |  |  |

For TM2, having 3 RBs occupied is consistent with the one RB used for testing. From a power perspective there is only a 4.8 dB (=10 log10 3) change in power from a control symbol and a PDSCH symbol.

Observation 1: no additional RBs for the control region are necessary for TM2.

Although the control symbols are not used for the EVM tests (TM2, 3.1, 3.2, 3.3), a difference in power between the control and data regions may affect those EVM results. For TM1.1 and TM1.2, because measurements may be performed on the control region, any difference in power between the control and data regions may impact test results.

To illustrate the difference in power, assume the power of each RB for the PDCCH is equal to the average power of each RB for the PDSCH region. The power ratio of the occupied control symbols and the occupied PDSCH symbols is shown in Figure 2 for two cases as a function of bandwidth: a) 3 RBs used; b) 6NRB6. Case b arises because the number of RBs allocated to the PDCCH must a multiple of 6.

Observation 2: The power difference between the control and data region exceeds 12 dB when the bandwidth is larger than 50 RBs and 3 PDCCH RBs are used.

When the maximum allowable number of PDCCH RBs is used, the largest power difference of 2.6 dB occurs when the bandwidth is 11 RBs.

Figure 2. 3 PDCCH RBs and  6NRB6 RBs.

## Options for the control region

The current specification uses 3 RBs of PDCCH located in RB0 to 2. The options that were listed in [1] are summarized in Table 3.

Table 3. Comparison of options

| Option | Description | Comments |
| --- | --- | --- |
| 1 | Have PDCCH span entire symbol; satisfying a maximum of 6NRB6 RBs | Number of candidates needed (see Table 7). - Can a subset be used? - Instead of 20 distinct CCEs, if a 6 dB difference is tolerated, then 3 sets are needed (11 RBs, 24 RBs, 96 RBs) - Is boosting needed? |
| 2 | The filler region is occupied by PDSCH corresponding to another user (e.g. third user NRNTI = 3) | Use PDSCH mapping type B [4]. The DMRS would be located in symbol 0 |
| 3 | The filler region is occupied by PDSCH of user 1 (and user 2). Their PDSCH will span the entire slot (all 14 symbols) except for the PDCCH | How to manage boosting is complicated for TM1.2, TM3.2, and TM3.3 because the RBG groups are sizes of 2, 4, 8, and 16 RBs |
| 4 | Create a second PDCCH to occupy the filler region as much as possible within the first two symbols | Same as option 1 |
| 5 | Double the number of CCEs so that the number of occupied RBs is 6 across 2 symbols. | 3 dB reduction in variation |
| 6 | Move PDCCH to center of bandwidth | Should have no impact in power |
| 7 | PDCCH 1 CCE spanning only 1 symbol only | Similar to option 5 but then introduce a power gap |

### Option 1 and variations

The goal of option 1 is to fill the control symbols with PDCCH as much as possible. For many channel bandwidths whose size in RBs is not a multiple of 6, boosting may be needed to ensure the power in the control region is the same as the shared region (see Figure 1). One drawback with this type of filling is that number of PDCCH needed varies from one PDCCH [AL=2 (aggregation level)] to twelve [11 AL=8, 1 AL=2].

Another option is to reduce the number of PDCCH supported (and then rely on boosting if needed). To illustrate the use of three allocations for the PDCCH (a variation of option 1), Figure 3 shows the difference between the PDCCH and PDSCH is between 0 and 6 dB when 3 sizes are used.

- For NRB ≤ 18 RBs, one L=2 CCE is used.

- For 24 ≤ NRB ≤ 93 RBs, one L=8 CCE is used.

- For remaining bandwidths, four L=8 CCEs are used.

Figure 3. 3 CCE values

PAPR results for several cases provided in Figure 4, Figure 5, Figure 6, and Figure 7. The simulation conditions are: 30 kHz SCS for several bandwidths using the number of RBs per symbol in Table 4. Note “full” may not be realizable due to multiple of 6 RB requirements for the PDCCH

Table 4. Number of RBs per symbols tested

| BW (MHz) | Full (RB) | #CCEs / symbol | Current (RB) | Partial (RB) | Partial 2 (RB) | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 100 | 273 | 45 | 3 | 96 (4 L=8 CCEs) | 144 (6 L=8 CCEs) | Worst case partial performance |
| 40 | 103 | 17 | 3 | 96 (4 L=8 CCEs) |  | Partial should meet full performance |
| 30 | 78 | 13 | 3 | 24 (1 L=8 CCEs) |  | Worst case partial performance |
| 15 | 38 | 6 | 3 | 24 (1 L=8 CCEs) |  | Partial should meet full performance |

| Figure 4. PAPR results 100 MHz | Figure 5. PAPR results 40 MHz |
| --- | --- |

| Figure 6. PAPR results 30 MHz | Figure 7. PAPR results 15 MHz |
| --- | --- |

The PAPR results show the degradation due to using 3 RBs as a function of test model bandwidth. Using a 3 size PDCCH allocation reduces the variation to within a 6 dB range over that bandwidth. The resulting PAPR results show less degradation.

Observation 3: A 3 size PDCCH allocation can replace the current 3 RB PDCCH allocation in order to improve PAPR performance.

An example modification of the configuration table for PDCCH for FR1 and FR2 is provided in [5] and [6], respectively. The basic concept is to determine the PDCCH configuration according to the bandwidth.

If NRB < 24: Use 1 PDCCH with aggregation level 2. This will handle the cases for 11 and 18 RBs. Not applicable to FR2

If 24 ≤ NRB < 96: Use 1 PDCCH with aggregation level 8.

If 96 ≤ NRB: Use 4 PDCCH with aggregation level 8.

### Explanation of option 2

With option 2, the intent is to fill the control region with an independent shared channel. This independent shared channel would not be used for EVM measurements but to ensure power levels are the same across the symbols. In Figure 8 (left), the control region has 6 units of power while the shared channel region has 11 units of power (only 2 symbols of the 12 are shown). On the right, each symbol has 11 units of power. The “red” is the independent shared channel. The current PDCCH is occupying 6 RBs (2 CCEs).

#0

#2

#4

#0

#0

#2

#4

#0

Figure 8. Option 2: fill undefined RBs with PDCCH (mapped independently)

With option 2, the issue is mostly implementation: how to specify the independent shared channel. After examining TS 38.211, PDSCH mapping type B can be applicable. An example modification for FR1 and FR2 is provided in [5] and [6], respectively Because the independent shared channel is decoupled from the shared channel region, the boosting in TM1.2, TM3.2, and TM3.3 do not affect the independent channel.

Observation 4: Using PDSCH mapping type B allows an independent shared channel.

### Explanation of option 3

Figure 9 illustrates the alignment of RBGs and CCEs for TM1.2, TM3.2, and TM 3.3 for three bandwidths: 11, 38, and 78 RBs. The last two bandwidths are at the edge of the transition to a larger RBG size.

The alignment is needed in order to compute the power level for the PDCCH and proper extension. There are three cases:

- TM1.1, TM3.1: with no boosting pattern, the power per RB in the PDSCH region is the same. Thus if the PDSCH were to start on symbol 0, then the power boosting per RB for the PDCCH is 0 dB. A single size PDCCH can be used.

- TM2: no PDSCH extension is needed in the PDCCH region

- TM1.2, TM3.2, TM3.3: the boosting patterns are on an RBG basis but the PDCCH is in multiples of 6 RBs. The boosting level for the PDCCH is a function of the number of RBGs and the boosting level of those RBGs over the span of the PDCCH.

To determine the boosting level for the PDCCH, a multi-step procedure is needed: Table 5 captures the details of the computation.

- Number of RBGs in the span of PDCCH: must satisfy lcm(P,6): where P is the number of RBs in a RBG. Although the value of P is a function of the bandwidth, the solution is 3 RBGs. However, the number of RBs for the PDCCH is more complicated: 6 if NRB≤36 (P=2); 12 if 36<NRB ≤72 (P=4); 24 if 72<NRB ≤144 (P=8); 48 otherwise (P=16).

- The number of boosted RBGs in the span of the PDCCH: either 1 or 2. It is 1 if the number of boosted RBGs = 2.

- The total power of the PDSCH in the span of the PDCCH: this is function of the boosting level and the number of boosted RBGs.

Table 5. Formula to compute the boosting level for PDCCH

| Parameter | Value |
| --- | --- |
| Number of RBs for PDCCH | Bandwidth, RB; RBs for PDCCH NRBPDCCH; CCE; AL. NRB ≤ 36; 6; 2; (1) L=2. 36 < NRB ≤ 72; 12; 4; (1) L=4. 72 < NRB ≤ 144; 24; 8; (1) L=8. 144 <NRB; 48; 16; (2) L=8. Or equivalently NRBPDCCH=6×2log2NRB36 |
| Number of boosted RBG in PDCCH | NRBGPDCCH=minNRBG-1,2 |
| Boost level, dB (specified in test model) | B |
| Power of boosted RBs | pRBGPDCCH=PNRBGPDCCH×10B10 |
| Power of other RBs in PDCCH (fraction is specified in test model) | potherPDCCH=P3-NRBGPDCCH×NRB-PNRBG×10B10NRB-PNRBG |
| Total power in PDCCH | pRBGPDCCH+potherPDCCH |
| Boosting level PDCCH RB, dB | 10log10pRBGPDCCH+potherPDCCHNRBPDCCH |

Table 6 captures the boosting levels using the equations in Table 5.

#0

#2

#4

#0

#0

#2

#4

#6

#8

#0

#1

#2

#3

#4

#5

#6

#7

#8

Figure 9. PDCCH and RBG groups for 11, 38, and 78 RBs. Yellow is 1 CCE, light orange is 2 CCEs, pink and purple mark units of 2 CCEs (6 RBs x 2 symbols). Green is occupied RBG while light blue is possible RBG. The right hand side illustrates the extension of the PDSCH to the control region.

Table 6. Boosting level for PDCCH for TM1.2, TM3.2, TM3.3

|  |  |  | TM1.2, B=3 dB, x=40% |  |  | TM3.2, B=-3 dB, x=60% |  |  | TM3.3, B=-6 dB, x=50% |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NRB | P | NRBPDCCH | NRBG | NRBGPDCCH | Boost | NRBG | NRBGPDCCH | Boost | NRBG | NRBGPDCCH | Boost |
| 11 | 2 | 6 | 2 | 1 | -0.21 | 3 | 2 | -0.62 | 2 | 1 | 0.15 |
| 18 | 2 | 6 | 3 | 2 | 1.75 | 5 | 2 | -0.58 | 4 | 2 | -1.55 |
| 24 | 2 | 6 | 4 | 2 | 1.75 | 7 | 2 | -0.46 | 6 | 2 | -1.25 |
| 25 | 2 | 6 | 5 | 2 | 1.59 | 7 | 2 | -0.56 | 6 | 2 | -1.36 |
| 31 | 2 | 6 | 6 | 2 | 1.63 | 8 | 2 | -0.73 | 7 | 2 | -1.51 |
| 38 | 4 | 12 | 3 | 2 | 1.79 | 5 | 2 | -0.69 | 4 | 2 | -1.66 |
| 51 | 4 | 12 | 5 | 2 | 1.61 | 7 | 2 | -0.61 | 6 | 2 | -1.41 |
| 52 | 4 | 12 | 5 | 2 | 1.63 | 7 | 2 | -0.65 | 6 | 2 | -1.46 |
| 65 | 4 | 12 | 6 | 2 | 1.67 | 9 | 2 | -0.59 | 8 | 2 | -1.29 |
| 66 | 4 | 12 | 6 | 2 | 1.68 | 9 | 2 | -0.62 | 8 | 2 | -1.33 |
| 78 | 8 | 24 | 3 | 2 | 1.81 | 5 | 2 | -0.74 | 4 | 2 | -1.71 |
| 79 | 8 | 24 | 3 | 2 | 1.81 | 5 | 2 | -0.77 | 4 | 2 | -1.73 |
| 93 | 8 | 24 | 4 | 2 | 1.73 | 6 | 2 | -0.73 | 5 | 2 | -1.62 |
| 106 | 8 | 24 | 5 | 2 | 1.65 | 7 | 2 | -0.69 | 6 | 2 | -1.50 |
| 107 | 8 | 24 | 5 | 2 | 1.66 | 7 | 2 | -0.71 | 6 | 2 | -1.53 |
| 121 | 8 | 24 | 6 | 2 | 1.60 | 8 | 2 | -0.68 | 7 | 2 | -1.45 |
| 132 | 8 | 24 | 6 | 2 | 1.68 | 9 | 2 | -0.62 | 8 | 2 | -1.33 |
| 133 | 8 | 24 | 6 | 2 | 1.69 | 9 | 2 | -0.64 | 8 | 2 | -1.35 |
| 135 | 8 | 24 | 6 | 2 | 1.70 | 9 | 2 | -0.67 | 8 | 2 | -1.39 |
| 160 | 16 | 48 | 4 | 2 | 1.59 | 6 | 2 | -0.38 | 5 | 2 | -1.25 |
| 162 | 16 | 48 | 4 | 2 | 1.60 | 6 | 2 | -0.41 | 5 | 2 | -1.28 |
| 189 | 16 | 48 | 4 | 2 | 1.74 | 6 | 2 | -0.76 | 5 | 2 | -1.65 |
| 216 | 16 | 48 | 5 | 2 | 1.67 | 7 | 2 | -0.72 | 6 | 2 | -1.55 |
| 217 | 16 | 48 | 5 | 2 | 1.67 | 7 | 2 | -0.73 | 6 | 2 | -1.56 |
| 245 | 16 | 48 | 6 | 2 | 1.61 | 8 | 2 | -0.71 | 7 | 2 | -1.48 |
| 264 | 16 | 48 | 6 | 2 | 1.68 | 9 | 2 | -0.62 | 8 | 2 | -1.33 |
| 270 | 16 | 48 | 6 | 2 | 1.70 | 9 | 2 | -0.67 | 8 | 2 | -1.39 |
| 273 | 16 | 48 | 6 | 2 | 1.71 | 9 | 2 | -0.69 | 8 | 2 | -1.42 |

Observation 5: It is possible to extend the PDSCH to fill the control region even with power boosting in several test models.

However, the complexity of the filling (power boosting vs. no boosting), extending PDSCH into the control region makes option 3 less attractive.

## Discussion

Each option has advantages and drawbacks. One key element is the ease of implementation. Among the options, option 2 is preferable due to it scalability and flexibility for the channels.

Proposal 1: Option 2 “The filler region is occupied by PDSCH corresponding to another user” should be used to ensure power of control symbols is the same as the shared channel symbols.

# Conclusion

A power analysis was performed on the current specification for PDCCH.

Observation 1: no additional RBs for the control region are necessary for TM2.

Observation 2: The power difference between the control and data region exceeds 12 dB when the bandwidth is larger than 50 RBs and 3 PDCCH RBs are used.

There are three options that are viable for filling the control region.

Observation 3: A 3 size PDCCH allocation can replace the current 3 RB PDCCH allocation in order to improve PAPR performance.

Observation 4: Using PDSCH mapping type B allows an independent shared channel.

Observation 5: It is possible to extend the PDSCH to fill the control region even with power boosting in several test models.

The accompanying text proposals in [5] and [6] provide possible standards impact.

Proposal 1: Option 2 “The filler region is occupied by PDSCH corresponding to another user” should be used to ensure power of control symbols is the same as the shared channel symbols.

# Reference

R4-1816736, “WF on filling remaining RBs of PDCCH symbols”, Huawei, RAN4#89, Nov. 11-15, 2018.

TS38.141-1

TS38.141-2

TS38.211

R4-1901494, “TP to TS38.141-1 subclause 4.9.2 filling remaining RBs in PDCCH symbols”, Huawei, HiSilicon, Feb. 25 – Mar. 1, 2019.

R4-1901495, “TP to TS38.141-2 subclause 4.9.2 filling remaining RBs in PDCCH symbols”, Huawei, HiSilicon, Feb. 25 – Mar. 1, 2019.

# Appendix

The minimum number of CCEs to fill the PDCCH region at aggregation levels L=2, L=4, and L=8 is presented in Table 7 as a function of bandwidth. The number of available CCEs is T=2NRB6. The number of L=8 candidates is L8=T8. The number of L=4 candidates is L4=T-8L84. The number of L=2 candidates is L2=T-8L8-4L42. The yellow highlighting indicates duplication in the number of CCEs for different bandwidths (there are 20 unique values). The banding indicates the sets.

Table 7. Number of CCE at aggregation level

| BW, NRB | #CCE | L=8 CCE | L=4 CCE | L=2 CCE |
| --- | --- | --- | --- | --- |
| 11 | 2 | 0 | 0 | 1 |
| 18 | 6 | 0 | 1 | 1 |
| 24 | 8 | 1 | 0 | 0 |
| 25 | 8 | 1 | 0 | 0 |
| 31 | 10 | 1 | 0 | 1 |
| 32 | 10 | 1 | 0 | 1 |
| 38 | 12 | 1 | 1 | 0 |
| 51 | 16 | 2 | 0 | 0 |
| 52 | 16 | 2 | 0 | 0 |
| 65 | 20 | 2 | 1 | 0 |
| 66 | 22 | 2 | 1 | 1 |
| 78 | 26 | 3 | 0 | 1 |
| 79 | 26 | 3 | 0 | 1 |
| 93 | 30 | 3 | 1 | 1 |
| 106 | 34 | 4 | 0 | 1 |
| 107 | 34 | 4 | 0 | 1 |
| 121 | 40 | 5 | 0 | 0 |
| 132 | 44 | 5 | 1 | 0 |
| 133 | 44 | 5 | 1 | 0 |
| 135 | 44 | 5 | 1 | 0 |
| 160 | 52 | 6 | 1 | 0 |
| 162 | 54 | 6 | 1 | 1 |
| 189 | 62 | 7 | 1 | 1 |
| 216 | 72 | 9 | 0 | 0 |
| 217 | 72 | 9 | 0 | 0 |
| 245 | 80 | 10 | 0 | 0 |
| 264 | 88 | 11 | 0 | 0 |
| 270 | 90 | 11 | 0 | 1 |
| 273 | 90 | 11 | 0 | 1 |