3GPP TSG RAN WG4 Meeting #90	R4-1901501

Athens, Greece, February 25 – March 1, 2019

Agenda Item:	6.14.1.5

Source:	Huawei, HiSilicon

Title:	Information for tap generation for the delay profile

Document for:	Approval

# Introduction

A procedure to generate the simplified path delay profiles from TR38.901 was agreed to [1] and later used to generate the path delay profiles for UE/BS demodulation requirements. However, when several companies attempted to reproduce the path delay profiles, it was noted that the procedure had some ambiguities. This contribution clarifies the procedure.

# Discussion

## High level description

The high level procedure to generate involves reducing the number of taps by a process of merging weaker power taps. Situations involving the end points are not clarified and whether merged taps are used for further merging. The following figures show the merging examples.

- Figure 1 illustrates the proposed procedure for merging the first tap and second tap.

- Figure 2 illustrates the proposed procedure for merging the last tap and second-to-last tap.

- Figure 3 illustrates the proposed procedure for merging the tap n and two adjacent taps when the time delay between each adjacent tap and tap n is not the same as the delay between the other adjacent tap and tap n.

- Figure 4 illustrates the proposed procedure for merging the tap n and two adjacent taps when the time delay between each adjacent tap and tap n is the same as the delay between the other adjacent tap and tap n.

1

1

Figure 1. Merging with first tap: green + orange  red located on first tap, orange tap location dropped. Used when first tap is weakest or when second tap is to merge with first tap

last

last

Figure 2. Merging with last tap: green + orange  red located on last tap, orange tap location dropped. Used when last tap is weakest or when second-to-last tap is to merge with last tap

Figure 3. Merging when delays  are equal between the weakest tap and nearest neighbors. The selected tap is neighbor weaker in power. When the selected tap is not an end point: green + blue  red for a new tap located between green and blue taps. Orange and blue tap locations dropped.

m

n

m

Figure 4. Merging when delays  are not equal between the weakest tap and nearest neighbors. The selected tap is neighbor closer in delay. When the selected tap is not an end point: green + blue  red for a new tap located between green and blue taps. Orange and blue tap locations dropped.

## Updated algorithm description

There are several updates to the procedure. The text proposal is presented in [2].

First of all, the search for the weakest tap spans all taps. When there are more than one tap with the weakest value, select the tap with the smaller (-est) delay. Procedures when the first or last tap is used for merging are captured.

Step 1: Use the original TDL model from TR38.901.

Step 2: Re-order the taps in ascending delays

Step 3: Perform delay scaling according to the procedure described in section 7.7.3 in TR38.901.

Step 4: Apply the quantization to the delay resolution 5 ns. This is done simply by rounding the tap delays to the nearest multiple of the delay resolution.

Step 5: If multiple taps are rounded to the same delay bin, merge them by calculating their linear power sum.

Step 6: If there are more than 12 taps in the quantized model, merge the taps as follows

- Find the weakest tap from all taps (both merged and unmerged taps are considered).

  - If there are two or more taps have the same value and are the weakest, select the tap that has the smallest delay as the weakest tap.

- When the weakest tap is the first delay tap, merge taps as follows.

- Update the power of the first delay tap as the linear power sum of the weakest tap and the second delay tap.

- Remove the second delay tap.

- When the weakest tap is the last delay tap, merge taps as follows

- Update the power of the last delay tap as the linear power sum of the second-to-last tap and the last tap.

- Remove the second-to-last tap.

- Otherwise

- For each side of the weakest tap, identify the neighbour tap that is with the smallest delay difference to the weakest tap.

- When the delay difference between the weakest tap and the identified neighbour tap on one side equals the delay difference between the weakest tap and the identified neighbour tap on the other side.

          - Select the neighbour tap that is weaker in power for merging.

- Otherwise, select the neighbour tap that has smaller delay difference for merging.

- To merge, the power of the merged tap is the linear sum of the power of the weakest tap and the selected tap.

- When the selected tap is the first tap, the location of the merged tap is the location of the first tap. The weakest tap is removed.

- When the selected tap is the last tap, the location of the merged tap is the location of the last tap. The weakest tap is removed.

- Otherwise, the location of the merged tap is based on the average delay of the weakest tap and selected tap. If the average delay is on the sampling grid, the location of the merged tap is the average delay. Otherwise, the location of the merged tap is rounded towards the direction of the selected tap (e.g. 10 ns & 25 ns  20 ns is used, if 25 ns had higher or equal power; 15 ns is used, if 10 ns had higher power). The weakest tap and the selected tap are removed.

- Repeat step 6 until the final number of taps is 12.

Step 7: Round the amplitudes of taps to one decimal (e.g. -8.78 dB  -8.8 dB)

Step 8: If the delay spread has slightly changed due to the tap merge, adjust the final delay spread by increasing or decreasing the power of the last tap so that the delay spread is corrected.

Step 9: Re-normalize tap powers such that the strongest tap is at 0dB

## Comparison

The procedure listed below is applied to the agreed channel models: TDLA30 (DS = 30 ns), TDLB100 (DS = 100ns), TDLC60 (DS = 60ns), and TDLC300 (DS = 300 ns). The following subsections compare the results.

### TDLA30

TDLA-30 is used for both FR1 and FR2.

Table 1. TDL-A 30

|  | Agreed Values |  | Revised Algorithm |  |
| --- | --- | --- | --- | --- |
| Tap Number | Tap Delay, ns | Tap Value, dB | Tap Delay, ns | Tap Value, dB |
| 1 | 0 | -15.5 | 0 | -15.4 |
| 2 | 10 | 0 | 10 | 0 |
| 3 | 15 | -5.1 | 15 | -5.1 |
| 4 | 20 | -5.1 | 20 | -5.1 |
| 5 | 25 | -9.6 | 25 | -9.5 |
| 6 | 50 | -8.2 | 50 | -8.2 |
| 7 | 65 | -13.1 | 65 | -13.1 |
| 8 | 75 | -11.5 | 75 | -11.5 |
| 9 | 105 | -11.0 | 90 | -13.3 |
| 10 | 135 | -16.2 | 120 | -14.7 |
| 11 | 150 | -16.6 | 140 | -14.9 |
| 12 | 290 | -26.2 | 290 | -22.7 |

Note the last tap was -16.8 dB prior to reducing it by 5.9 dB.

Observation 1: The differences between the agreed values and values generated by the revised algorithm for TDLA-300 are the last 4 taps.

### TDLB100

TDLB-100 is used for FR1.

Table 2. TDL-B 100

|  | Agreed Values |  | Revised Algorithm |  |
| --- | --- | --- | --- | --- |
| Tap Number | Tap Delay, ns | Tap Value, dB | Tap Delay, ns | Tap Value, dB |
| 1 | 0 | 0 | 0 | 0 |
| 2 | 10 | -2.2 | 10 | -2.2 |
| 3 | 20 | -0.6 | 20 | -0.6 |
| 4 | 30 | -0.6 | 30 | -0.6 |
| 5 | 35 | -0.3 | 35 | -1.7 |
| 6 | 45 | -1.2 | 40 | -3.4 |
| 7 | 55 | -5.9 | 50 | -2.5 |
| 8 | 120 | -2.2 | 110 | -4.8 |
| 9 | 170 | -0.8 | 140 | -3.5 |
| 10 | 245 | -6.3 | 180 | -1.9 |
| 11 | 330 | -7.5 | 245 | -4.7 |
| 12 | 480 | -7.1 | 480 | -6.5 |

Note the last tap was -5.2 dB prior to reducing it by 1.3 dB.

Observation 2: The differences between the agreed values and values generated by the revised algorithm for TDLB-100 are the last 8 taps.

### TDLC300

TDLC-300 is used for FR1.

Table 3. TDL-C 300

|  | Agreed Values |  | Revised Algorithm |  |
| --- | --- | --- | --- | --- |
| Tap Number | Tap Delay, ns | Tap Value, dB | Tap Delay, ns | Tap Value, dB |
| 1 | 0 | -6.9 | 0 | -6.9 |
| 2 | 65 | 0 | 65 | 0 |
| 3 | 70 | -7.7 | 70 | -7.7 |
| 4 | 190 | -2.5 | 190 | -2.5 |
| 5 | 195 | -2.4 | 195 | -2.4 |
| 6 | 200 | -9.9 | 200 | -9.9 |
| 7 | 240 | -8.0 | 240 | -9.6 |
| 8 | 325 | -6.6 | 260 | -10.4 |
| 9 | 520 | -7.1 | 370 | -7.6 |
| 10 | 1045 | -13.0 | 390 | -9.5 |
| 11 | 1510 | -14.2 | 1030 | -8.3 |
| 12 | 2595 | -16.0 | 2595 | -15.7 |

Note the last tap was -12.1 dB prior to reducing it by 3.6 dB.

Observation 3: The differences between the agreed values and values generated by the revised algorithm for TDLC-300 are the last 5 taps.

### TDLC60

TDLC-60 is used for FR2.

Table 4. TDL-C 60

|  | Agreed Values |  | Revised Algorithm |  |
| --- | --- | --- | --- | --- |
| Tap Number | Tap Delay, ns | Tap Value, dB | Tap Delay, ns | Tap Value, dB |
| 1 | 0 | -7.8 | 0 | -7.8 |
| 2 | 15 | -0.3 | 15 | -0.2 |
| 3 | 40 | 0 | 40 | 0 |
| 4 | 50 | -8.9 | 50 | -8.9 |
| 5 | 55 | -14.5 | 55 | -14.5 |
| 6 | 75 | -8.5 | 75 | -8.5 |
| 7 | 80 | -10.2 | 80 | -10.2 |
| 8 | 130 | -12.1 | 130 | -12.1 |
| 9 | 210 | -13.9 | 165 | -16.6 |
| 10 | 300 | -15.2 | 255 | -17.3 |
| 11 | 360 | -16.9 | 275 | -17.3 |
| 12 | 520 | -19.4 | 520 | -8.5 |

Note the last tap was -13 dB prior to increasing it by 4.5 dB.

Observation 4: The differences between the agreed values and values generated by the revised algorithm for TDLC-60 are the last 4 taps.

## Discussion

For simulation results, it agreed to use the full taps. However, some companies may have provided simulation results using the captured values for the taps. As a result, the decision to replace the existing tables with the revised values is difficult. One approach is to not to change the values but note the procedure should be used for future values. Another approach is to update the tables.

Proposal 1: Decide whether to update the tables with the new values or to retain the current values. A note may be needed to indicate the new procedure was not used to generate the current values.

The algorithm is reproduced in the annex of several standards (38.104, 38.141-1, 38.141-2, and 38.101-4). Since the algorithm is a procedure for the generation of tap values and delays, a possible place to locate the algorithm is a technical report.

Proposal 2: Consider placing the algorithm from the technical specification to a technical report.

When more channels are generated, the revised algorithm should be used. However, a note should be captured stating that the agreed values were not generated by this specific algorithm.

Proposal 3: All future tap generation values should use the revised procedure.

# Conclusions

A revision of the tap generation algorithm is proposed in this contribution to ensure reproducibility of the results. Some observations of the generated tap values in comparison to the agreed values are

Observation 1: The differences between the agreed values and values generated by the revised algorithm for TDLA-300 are the last 4 taps.

Observation 2: The differences between the agreed values and values generated by the revised algorithm for TDLB-100 are the last 8 taps.

Observation 3: The differences between the agreed values and values generated by the revised algorithm for TDLC-300 are the last 5 taps.

Observation 4: The differences between the agreed values and values generated by the revised algorithm for TDLC-60 are the last 4 taps.

Some proposals are

Proposal 1: Decide whether to update the tables with the new values or to retain the current values. A note may be needed to indicate the new procedure was not used to generate the current values.

Proposal 2: Consider placing the algorithm from the technical specification to a technical report.

Proposal 3: All future tap generation values should use the revised procedure.

# References

R4-1813735, “Way forward on simplified TDL channel model”, Huawei, RAN4#88B, Oct. 8-12, 2018.

R4-1901554, “Corrections to 38.101-4 subclause B.2.1 Delay profile calculation”, Huawei, RAN#90, Feb. 25 – Mar. 1, 2019.

# Appendix

Using TDLC300 as an example, the following tables illustrate the generation of the taps. Green is the minimum tap value while blue is the selected tap. Yellow in Table 5 is used to indicate which taps should be merged after quantization.

Table 5. Steps 1, 3, 4, 5

|  | TDLC (original) |  | Scaling |  | Quantization |  | Merging |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Delay | Power, dB | Delay, ns | Power, linear | Delay, ns | Power, linear | Delay, ns | Power, linear |
| 1 | 0. | -4.4 | 0 | 0.36308 | 0 | 0.36308 | 0 | 0.36308 |
| 2 | 0.2099 | -1.2 | 62.97 | 0.75858 | 65 | 0.75858 | 65 | 1.7676 |
| 3 | 0.2219 | -3.5 | 65.28 | 0.56234 | 65 | 0.56234 |  |  |
| 4 | 0.2329 | -5.2 | 66.57 | 0.44668 | 65 | 0.44668 |  |  |
| 5 | 0.2176 | -2.5 | 69.87 | 0.302 | 70 | 0.302 | 70 | 0.302 |
| 6 | 0.6366 | 0 | 190.98 | 1 | 190 | 1 | 190 | 1 |
| 7 | 0.6448 | -2.2 | 193.44 | 0.60256 | 195 | 0.60256 | 195 | 1.0099 |
| 8 | 0.656 | -3.9 | 196.8 | 0.40738 | 195 | 0.40738 |  |  |
| 9 | 0.6584 | -7.4 | 197.52 | 0.18197 | 200 | 0.18197 | 200 | 0.18197 |
| 10 | 0.7935 | -7.1 | 238.05 | 0.19498 | 240 | 0.19498 | 240 | 0.19498 |
| 11 | 0.8213 | -10.7 | 246.39 | 0.085114 | 245 | 0.085114 | 245 | 0.085114 |
| 12 | 0.9336 | -11.1 | 280.08 | 0.077625 | 280 | 0.077625 | 280 | 0.077625 |
| 13 | 1.2285 | -5.1 | 368.55 | 0.30903 | 370 | 0.30903 | 370 | 0.30903 |
| 14 | 1.3083 | -6.8 | 392.49 | 0.20893 | 390 | 0.20893 | 390 | 0.20893 |
| 15 | 2.1704 | -8.7 | 651.12 | 0.1349 | 650 | 0.1349 | 650 | 0.1349 |
| 16 | 2.7105 | -13.2 | 813.15 | 0.047863 | 815 | 0.047863 | 815 | 0.047863 |
| 17 | 4.2589 | -13.9 | 1277.7 | 0.040738 | 1280 | 0.040738 | 1280 | 0.040738 |
| 18 | 4.6003 | -13.9 | 1380.1 | 0.040738 | 1380 | 0.040738 | 1380 | 0.040738 |
| 19 | 5.4902 | -15.8 | 1647.1 | 0.026303 | 1645 | 0.026303 | 1645 | 0.026303 |
| 20 | 5.6077 | -17.1 | 1682.3 | 0.019498 | 1680 | 0.019498 | 1680 | 0.019498 |
| 21 | 6.3065 | -16 | 1891.9 | 0.025119 | 1890 | 0.025119 | 1890 | 0.025119 |
| 22 | 6.6374 | -15.7 | 1991.2 | 0.026915 | 1990 | 0.026915 | 1990 | 0.026915 |
| 23 | 7.0427 | -21.6 | 2112.8 | 0.0069183 | 2115 | 0.0069183 | 2115 | 0.0069183 |
| 24 | 8.6523 | -22.8 | 2595.7 | 0.0052481 | 2595 | 0.0052481 | 2595 | 0.0052481 |

Table 6. Steps 6

|  | Iteration 1 |  | Iteration 2 |  | Iteration 3 |  | Iteration 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Delay | Power, linear | Delay, ns | Power, linear | Delay, ns | Power, linear | Delay, ns | Power, linear |
| 1 | 0 | 0.36308 | 0 | 0.36308 | 0 | 0.36308 | 0 | 0.36308 |
| 2 | 65 | 1.7676 | 65 | 1.7676 | 65 | 1.7676 | 65 | 1.7676 |
| 3 | 70 | 0.302 | 70 | 0.302 | 70 | 0.302 | 70 | 0.302 |
| 4 | 190 | 1 | 190 | 1 | 190 | 1 | 190 | 1 |
| 5 | 195 | 1.0099 | 195 | 1.0099 | 195 | 1.0099 | 195 | 1.0099 |
| 6 | 200 | 0.18197 | 200 | 0.18197 | 200 | 0.18197 | 200 | 0.18197 |
| 7 | 240 | 0.19498 | 240 | 0.19498 | 240 | 0.19498 | 240 | 0.19498 |
| 8 | 245 | 0.085114 | 245 | 0.085114 | 245 | 0.085114 | 245 | 0.085114 |
| 9 | 280 | 0.077625 | 280 | 0.077625 | 280 | 0.077625 | 280 | 0.077625 |
| 10 | 370 | 0.30903 | 370 | 0.30903 | 370 | 0.30903 | 370 | 0.30903 |
| 11 | 390 | 0.20893 | 390 | 0.20893 | 390 | 0.20893 | 390 | 0.20893 |
| 12 | 650 | 0.1349 | 650 | 0.1349 | 650 | 0.1349 | 650 | 0.1349 |
| 13 | 815 | 0.047863 | 815 | 0.047863 | 815 | 0.047863 | 815 | 0.047863 |
| 14 | 1280 | 0.040738 | 1280 | 0.040738 | 1280 | 0.040738 | 1280 | 0.040738 |
| 15 | 1380 | 0.040738 | 1380 | 0.040738 | 1380 | 0.040738 | 1380 | 0.040738 |
| 16 | 1645 | 0.026303 | 1645 | 0.026303 | 1660 | 0.045801 |  |  |
| 17 | 1680 | 0.019498 | 1680 | 0.019498 |  |  | 1775 | 0.07092 |
| 18 | 1890 | 0.025119 | 1890 | 0.025119 | 1890 | 0.025119 |  |  |
| 19 | 1990 | 0.026915 |  |  |  |  |  |  |
| 20 | 2595 | 0.012166 | 2595 | 0.039082 | 2595 | 0.039082 | 2595 | 0.039082 |

Table 7. Steps 6

|  | Iteration 5 |  | Iteration 6 |  | Iteration 7 |  | Iteration 8 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Delay | Power, linear | Delay, ns | Power, linear | Delay, ns | Power, linear | Delay, ns | Power, linear |
| 1 | 0 | 0.36308 | 0 | 0.36308 | 0 | 0.36308 | 0 | 0.36308 |
| 2 | 65 | 1.7676 | 65 | 1.7676 | 65 | 1.7676 | 65 | 1.7676 |
| 3 | 70 | 0.302 | 70 | 0.302 | 70 | 0.302 | 70 | 0.302 |
| 4 | 190 | 1 | 190 | 1 | 190 | 1 | 190 | 1 |
| 5 | 195 | 1.0099 | 195 | 1.0099 | 195 | 1.0099 | 195 | 1.0099 |
| 6 | 200 | 0.18197 | 200 | 0.18197 | 200 | 0.18197 | 200 | 0.18197 |
| 7 | 240 | 0.19498 | 240 | 0.19498 | 240 | 0.19498 | 240 | 0.19498 |
| 8 | 245 | 0.085114 | 245 | 0.085114 | 245 | 0.085114 | 260 | 0.16274 |
| 9 | 280 | 0.077625 | 280 | 0.077625 | 280 | 0.077625 |  |  |
| 10 | 370 | 0.30903 | 370 | 0.30903 | 370 | 0.30903 | 370 | 0.30903 |
| 11 | 390 | 0.20893 | 390 | 0.20893 | 390 | 0.20893 | 390 | 0.20893 |
| 12 | 650 | 0.1349 | 650 | 0.1349 |  |  |  |  |
| 13 | 815 | 0.047863 | 815 | 0.047863 | 730 | 0.18276 | 730 | 0.18276 |
| 14 | 1280 | 0.040738 | 1330 | 0.081476 | 1330 | 0.081476 | 1330 | 0.081476 |
| 15 | 1380 | 0.040738 |  |  |  |  |  |  |
| 16 | 2595 | 0.11 | 2595 | 0.11 | 2595 | 0.11 | 2595 | 0.11 |

Table 8. Final Steps

|  | After iteration |  | Log domain |  | Power normalization |  | Delay spread correction |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Delay | Power, linear | Delay, ns | Power, dB | Delay, ns | Power, dB | Delay, ns | Power, dB |
| 1 | 0 | 0.36308 | 0 | -4.4 | 0 | -6.9 | 0 | -6.9 |
| 2 | 65 | 1.7676 | 65 | 2.5 | 65 | 0.0 | 65 | 0.0 |
| 3 | 70 | 0.302 | 70 | -5.2 | 70 | -7.7 | 70 | -7.7 |
| 4 | 190 | 1 | 190 | 0.0 | 190 | -2.5 | 190 | -2.5 |
| 5 | 195 | 1.0099 | 195 | 0.0 | 195 | -2.4 | 195 | -2.4 |
| 6 | 200 | 0.18197 | 200 | -7.4 | 200 | -9.9 | 200 | -9.9 |
| 7 | 240 | 0.19498 | 240 | -7.1 | 240 | -9.6 | 240 | -9.6 |
| 8 | 260 | 0.16274 | 260 | -7.9 | 260 | -10.4 | 260 | -10.4 |
| 9 | 370 | 0.30903 | 370 | -5.1 | 370 | -7.6 | 370 | -7.6 |
| 10 | 390 | 0.20893 | 390 | -6.8 | 390 | -9.3 | 390 | -9.3 |
| 11 | 1030 | 0.26424 | 1030 | -5.8 | 1030 | -8.3 | 1030 | -8.3 |
| 12 | 2595 | 0.11 | 2595 | -9.6 | 2595 | -12.1 | 2595 | -15.7 |

The last value is determined iteratively such that T=300 ns

τs=0

ϵ=0.2

while τs-T>ϵ

Adjust A12 smaller if τs>T

τ=k=112τkAkk=112Ak

τs=k=112τ-τk2Akk=112Ak

Where Ak represents the linear power, k is the tap delay

Through iteration, the rms delay spread is 299.8 ns.