3GPP TSG-RAN WG4 Meeting # 90	R4-1901558

# Athens, Greece,  25 February - 1 March 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.145-2 | CR | 0082 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 37.145-2: additional reference angular step criteria in Annex F.2.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  | Date: |  |  | 2019-02-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | At RAN4 #89 meeting, additional reference angular step criteria were agreed and captured in TR 37.843 and TS 38.141-2 but not in TS 37.145-2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Additional reference angular step criteria, which are based on beam width of wanted signals, are included in the annex. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is incomplete and not aligned with TS 38.141-2 and TR 37.843. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex F.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

Annex F (normative): 
TRP measurement grids

# F.1		General

The annex describes various sampling grids and procedures for OTA TRP performance of AAS BS.

# F.2	Spherical equal angle grid

## F.2.1	General

TRPEstimate is defined as

TRPEstimate=π2NMn=0N-1m=0M-1EIRP(θn,ϕm)sinθn

when EIRP measurements is used or as

TRPEstimate=4πd2π2NMn=1N-1m=0M-1PD(θn,ϕm)sinθn

when power density measurements are used, and d is the test distance. N and M are the number of samples in the θ and ϕ angles. Each (θn,ϕm) is a sampling point. The sampling angular intervals for θ and ϕ angles are Δθ=πN and Δϕ=2πM. The sampling intervals Δθ and Δϕ are described in F.2.2.

## F.2.2	Reference angular step criteria

The reference angular steps Δϕ and Δθ in  radians are defined as

Δϕref=min⁡(λDcyl,[π12])

Δθref=⁡(λD,[π12])

Dcyl and D are calculated as

Dcyl=d2+w2

D=d2+w2+h2

The definition of d, w and h is shown in Figure F2.2-1. The radiation source can be EUT antenna array or the whole of EUT.

Figure F2.2-1: Dimensions of a radiation source: depth (d), width (w) and height (h)

In the case of Uniform Linear Array (ULA) and the EUT is mounted along the yz plane as shown in Figure F2.2-2, the  reference angular step can be determined by

∆θref=180°πarcsin⁡(λ/Dz)

Δϕref=180°πarcsin⁡(λDy)

Where Dy is the length of radiating parts of EUT along y-axis, Dz is the length of radiating parts of EUT along the z-axis and  is wavelength for the measured frequency.

Figure F2.2-2. Spherical coordinate for OTA conformance testing of EUT

Where due to practical reasons such as time constraints or turn-table precision, measurement with the reference steps is not practical, sparser grids can be used. Use of sparse grids can lead to errors in TRP assessment. In order to characterize these errors, the SF (sparsity factor) of the grid is defined as

Where  and  are the actual angular steps used in the measurement.

For each frequency within the downlink operating band including ΔfOBUE, the reference angular steps can be specified in terms of the beamwidth of the wanted signal as

∆ϕref=λλo BeWϕ

Δθref=λλo BeWθ

where λo is the wavelength of the wanted signal, and BeWφ and BeWθ are the beamwidth of the wanted signal in the φ-axis and θ-axis, respectively.

BeWφ and BeWθ may be set to beamwidth declared for the OTA AAS BS radiated transmit power requirement provided the same beam is applied to test in-band TRP requirements.

NOTE:	Beamwidth is approximately equal to half the first-null beam width.

<End of modified section>