3GPP TSG-RAN WG4 Meeting #90	R4-1901763

# Athens, Greece, 25th February – 1st March 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR |  | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SUO correction for DC_1_n77-n257 in Table 5.2B.6.2-1. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Skyworks Solutions, Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  | Date: |  |  | 2019-02-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to endorsed R4-1816247, DC_1_n77-n257 “Single UL allowed” is erroneous. This combination should be tagged “DC_1_n77”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In table 5.2B.6.2-1, replaced “No” in column entitled “Single UL allowed” with “DC_1_n77”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent Single UL allowed configuration. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2B.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... 38.521-3 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### ---------------------------------------Start of Change -------------------------------

#### 5.2B.6.2	EN-DC (three bands)

Table 5.2B.6.2-1: Band combinations inter-band EN-DC including both FR1 and FR2 (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1_n77-n257 | 1 | CA_n77-n257 | DC_1_n77 |
| DC_1_n78-n257 | 1 | CA_n78-n257 | No |
| DC_1_n79-n257 | 1 | CA_n79-n257 | No |
| DC_3_n77-n257 | 3 | CA_n77-n257 | DC_3_n77 |
| DC_3_n78-n257 | 3 | CA_n78-n257 | DC_3_n78 |
| DC_3_n79-n257 | 3 | CA_n79-n257 | No |
| DC_5_n78-n2571 | 5 | CA_n78-n257 | No |
| DC_7-7_n78-n257 | CA_7-7 | CA_n78-n257 | No |
| DC_7_n78-n257 | 7 | CA_n78-n257 | No |
| DC_19_n77-n257 | 19 | CA_n77-n257 | No |
| DC_19_n78-n257 | 19 | CA_n78-n257 | No |
| DC_19_n79-n257 | 19 | CA_n79-n257 | No |
| DC_21_n77-n257 | 21 | CA_n77-n257 | No |
| DC_21_n78-n257 | 21 | CA_n78-n257 | No |
| DC_21_n79-n257 | 21 | CA_n79-n257 | No |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability |  |  |  |

### ---------------------------------------End of Change -------------------------------