3GPPRAN4#90	R4-1901870

Athens, Greece, Feb 25th – Mar 1st

Title: 	Test Parameters for RRM tests in FR2

Source: 	Qualcomm Incorporated

Agenda item:	6.13.3.2	 FR2 test setup including AoA and other configurations	[NR_newRAT-Perf]

Document for:	Discussion

# Introduction

RAN4 has been in the process of defining test cases for RRM requirements. There have been some discussions and agreements on which tests should be using 1 AoA vs 2 AoA. However, RAN4 still needs to decide on those for phase 3 and phase 4 tests. Also, for the 1 AoA tests RAN4 needs to decide between setup 1 (beam peak) and setup 2 (non-beam peak). In this contribution we list out the tests already defined and assign the AoA test setup to each. We also provide input to upcoming phase 3 and phase 4 tests. In addition, we also show how to specify certain parameters such as Noc levels, TDM transmissions etc.

# Discussion

Table 2-1 lists the phase 1 and phase 2 tests along with the test setup that should be used.  Table 2-2 lists the same for phase 3 and 4 test scenarios.

Proposal 1: RRM tests should use the test setup proposed in Table 2-1 and Table 2-2.

Table 2-1: Phase 1 and 2 Test Case List with AoA setup

| Category | Tests | Test Setup |
| --- | --- | --- |
| Cell re-selection to NR | Intra-frequency NR case | 1 AoA, non-beam peak |
|  | Cell reselection to FR2 inter-frequency NR case | 1 AoA, non-beam peak |
| Handover | Inter-frequency handover from FR1 to FR2; unknown target cell | 1 AoA, non-beam peak |
|  | Intra-frequency handover from FR2 to FR2; unknown target cell | 1 AoA, non-beam peak |
|  | Inter-frequency handover from FR2 to FR2; unknown target cell | 2 AoA |
| Random Access | Contention based random access test in FR2 for NR Standalone | 1 AoA, non-beam peak |
|  | Non-contention based random access test in FR2 for NR Standalone | 1 AoA, non-beam peak |
| UE Transmit Timing | NR UE Transmit Timing Test for FR2 | 1 AoA, beam peak |
|  | SA FR2 timing advance adjustment accuracy | 1 AoA, beam peak |
| RLM | Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB based RLM RS in non-DRX mode | 2 AoA |
|  | Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode | 2 AoA |
|  | Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB based RLM RS in DRX mode | 1 AoA, non-beam peak |
|  | Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode | 1 AoA, non-beam peak |
|  | Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSIRS- based RLM in non-DRX mode | 2 AoA |
|  | Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS based RLM in non-DRX mode | 2 AoA |
|  | Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSIRS- based RLM in DRX mode | 1 AoA, non-beam peak |
|  | Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS based RLM in DRX mode | 1 AoA, non-beam peak |
| Interruptions |  | 1 AoA, beam peak |
| Scell Activation | SCell Activation and deactivation for FR2 SCell with the active serving cell in the same FR2 band | 1 AoA, beam peak |
| BFD and Link recovery procedures | Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode | 2 AoA |
|  | Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode | 2 AoA |
|  | Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode | 2 AoA |
|  | Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode | 2 AoA |
| Active BWP Delay | FR2- NR FR2 DL active BWP switch of PCell with non-DRX in SA | 1 AoA, beam peak |
|  | NR FR1- NR FR2 DL active BWP switch of PCell with non-DRX in SA | 1 AoA, beam peak |
|  | RRC NR FR2- NR FR2 DL active BWP switch of PCell with non-DRX in SA | 1 AoA, beam peak |
| Intra-frequency Measurements | SA event triggered reporting test without gap under non-DRX | 2 AoA |
|  | SA event triggered reporting test without gap under DRX |  |
|  | SA event triggered reporting test with per-UE gaps under non-DRX | 2 AoA |
|  | SA event triggered reporting test with per-UE gaps under DRX |  |
| Inter-freq measurement | SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2) | 2 AoA |
|  | SA event triggered reporting tests For FR2 without SSB time index detection when DRX is used (PCell in FR2) |  |
|  | SA event triggered reporting tests For FR2 with SSB time index detection when DRX is not used (PCell in FR2) | 2 AoA |
|  | SA event triggered reporting tests For FR2 with SSB time index detection when DRX is used (PCell in FR2) |  |
|  | SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1) | 2 AoA |
|  | SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (PCell in FR1) |  |
|  | SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1) | 2 AoA |
|  | SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1) |  |
| SS-RSRP | intra-frequency case | 1 AoA, non-beam peak |
|  | SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell | 1 AoA, non-beam peak |
| SS-RSRQ | Intra-frequency case | 1 AoA, non-beam peak |
|  | SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell | 1 AoA, non-beam peak |
| SS-SINR |  | 1 AoA, non-beam peak |
| L1-RSRP measurement for beam reporting | SSB based L1-RSRP measurement | 1 AoA, non-beam peak |
|  | CSI-RS based L1-RSRP measurement | 1 AoA, non-beam peak |
| PSCell addition and release delay | Addition and Release Delay of known NR PSCell | 1 AoA, non-beam peak |

Table 2-2: Phase 3 and 4 test scenarios with AoA setup

| Category | Tests | Test Setup |
| --- | --- | --- |
| EN-DC SFTD measurement delay | SFTD is only measured before NR PSCell addition | 1 AoA, beam peak |
| EN-DC interruptions due to measurement on deactivated SCell | EN-DC CA | 1 AoA, beam peak |
| SA interruptions due to measurement on deactivated SCell | SA CA | 1 AoA, beam peak |
| EN-DC/SA SSB RLM scheduling restriction and impact on mobility | Resource overlapping | 2AoA |
| RRC Re-establishment | TS38.133 A.6.3.2.1/A.7.3.2.1 | 2AoA |
| RRC Release with redirection to NR/E-UTRAN | TS38.133 A.6.3.2.3/A.7.3.2.3 | 1 AoA, non-beam peak |
| EN-DC SFTD measurement accuracy | SFTD | 1 AoA, beam peak |
| EN-DC MTTD | MTTD | 1 AoA, non-beam peak |
| NR CA MTTD | MTTD | 1 AoA, non-beam peak |
| EN-DC/SA beam failure detection and recovery and scheduling restriction | Resource overlapping | 2AoA |
| SA/EN-DC SS-SINR measurement accuracies | SS-SINR | 1 AoA, non-beam peak |
| SA SFTD delay and interruption | Only for measurement before LTE PSCell addition in NE-DC | 1 AoA, beam peak |
| TCI state switch delay |  | 2AoA |

For most test cases with 2AoA we propose to use TDM’ing of transmissions from the two AoA’s. The cleanest way of doing this is by adding a scheduling parameter to the RMC’s which specify when that channel can transmit. As an example, the one AoA could be scheduled in even slots and the other in odd slots.

If simultaneous transmissions will be found as feasible, the tests with multiple cell (e.g. intra-frequency measurements) could use this testing mode.

Proposal 2a: For two AoA tests, use TDM transmissions between the two angles for most tests.

Proposal 2b: Add a scheduling parameter to the RMC’s to specify in which slot each AoA can transmit/receive.

The Noc levels in FR2 would need to be band group dependent. In our companion papers [1,2] we propose the Noc levels (dBm/Hz) for 1AoA and 2AoA cases.

| Band Group | Noc (1 AoA)Note1 | Noc (2 AoA)Note 2 |
| --- | --- | --- |
| NR_TDD_FR2_T (n257, n258, n261) | -158 | -142 |
| NR_TDD_FR2_Y (n260) | -155 | -137.5 |
| Note1: Fine beam and beam peak direction assumed here. Note2: Rough beams with 2 AoA |  |  |

In Table 2-3, we provide an example table for OTA parameters in a 2AoA setup. The assumption is that UE is using fine beams for cell 2 and rough beams for cell 3. The AoA setup reference is taken from draftCR [3]. The assumption here is that config1 is with 120 kHz SSB and config2 with 240 kHz SSB.

Table 2-3: OTA Params Table

| Parameter | Unit | Config | Band Group | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  | 1, 2 |  | A.3.15.3 |  |  |  |
| Angle of Arrival |  | 1,2 |  | Angle1 |  | Angle 2 |  |
|  | dB | 1, 2 |  | 3 | 3 | 3 | 3 |
| Note 2,Note4 | dBm/15 kHz | 1, 2 | NR_TDD_FR2_T | -105.24 |  | -100.24 |  |
|  |  |  | NR_TDD_FR2_Y | -100.7 |  | -95.7 |  |
| Note 2,Note4 | dBm/SCS | 1,2 | NR_TDD_FR2_T | -96.24 |  | -91.24 |  |
|  |  |  | NR_TDD_FR2_Y | -91.7 |  | -86.7 |  |
| SS-RSRPNote3,Note4 | dBm/SCS | 1 | NR_TDD_FR2_T | -93.24 |  | -92.7 |  |
|  |  |  | NR_TDD_FR2_Y | -88.7 |  | -83.7 |  |
|  |  | 2 | NR_TDD_FR2_T | -90.24 |  | -89.7 |  |
|  |  |  | NR_TDD_FR2_Y | -85.7 |  | -80.7 |  |
|  | dB | 1, 2 |  | 3 | 3 | 3 | 3 |
|  | dBm/95.04MHz | 1, 2 |  | TBD |  | TBD |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Noc and SS-RSRP values as seen at the reference point in the quite zone |  |  |  |  |  |  |  |

Proposal 3: Format OTA params table for RRM tests as given in Table 2-3

# Conclusions

Proposal 1: RRM tests should use the test setup proposed in Table 2-1 and Table 2-2.

Proposal 2a: For two AoA tests, use TDM transmissions between the two angles for most tests.

Proposal 2b: Add a scheduling parameter to the RMC’s to specify in which slot each AoA can transmit/receive.

Proposal 3: Format OTA params table for RRM tests as given in Table 2-3

# References

[1] R4-1900752, Noc level definition and SNR range for 1AoA RRM testing

[2] R4-1900753, Noc level definition and SINR range for 2AoA RRM testing

[3] R4-1900597, AoA Setup for FR2 RRM Testing