3GPP TSG-RAN4 WG4 Meeting #90	R4-1902061

# Athens, Greece,  Feb.25 - Mar.01, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.104 | CR | 4834 | rev | 1 | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR: Addition of demodulation performance for new NPRACH formats |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh2-Perf |  |  |  |  |  | Date: |  |  | 2018-12-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The demodulation performance requirements for FDD NPRACH format 2 and TDD NPRACH are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the related demodulation performance requirements for FDD NPRACH format 2 and TDD NPRACH. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The demodulation performance requirements for Release 15 Further NB-IoT enhancements will still be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.141 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 8.5.3	Performance requirements for NPRACH

#### 8.5.3.1	NPRACH False alarm probability

The false alarm requirement is valid for any number of receive antennas, for all repetition numbers and for any number of subcarriers.

The false alarm probability is the conditional total probability of erroneous detection of the preamble (i.e. erroneous detection from any detector) when input is only noise.

##### 8.5.3.1.1	Minimum requirement

The false alarm probability shall be less than or equal to 0.1%.

#### 8.5.3.2	NPRACH detection requirements

The probability of detection is the conditional probability of correct detection of the preamble when the signal is present. There are several error cases – detecting different preamble than the one that was sent, not detecting a preamble at all or correct preamble detection but with the wrong timing estimation. A timing estimation error occurs if the estimation error of the timing of the strongest path is larger than 3.646us. The strongest path for the timing estimation error refers to the strongest path in the power delay profile.

The requirements for TDD are optional and only valid for base stations supporting TDD.

Table 8.5.3.2-1 Test preambles for NPRACH

| Parameter | Value |
| --- | --- |
| Narrowband physical layer cell identity | 0 |
| Initial subcarrier index | 0 |
| Uplink-downlink configuration for TDD | 1 |
| Special subframe configuration for TDD | 7 |

##### 8.5.3.2.1	Minimum requirements

The probability of detection shall be equal to or exceed 99% for the SNR levels listed in table 8.5.3.2.1-1.

Table 8.5.3.2.1-1 NPRACH missed detection requirements for FDD

| Number of TX antennas | Number of RX antennas | Repetition number | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR[dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Preamble format 0 | Preamble format 1 | Preamble format 2 |  |
| 1 | 2 | 8 | AWGN | 0 | -2.1 | -2.1 | [-3.9] |  |
|  |  |  | EPA1 Low | 200 Hz | 6.1 | 6.1 | [6.4] |  |
|  |  | 32 | AWGN | 0 | -6.8 | -6.8 | [-8.2] |  |
|  |  |  | EPA1 Low | 200 Hz | 0.5 | 0.5 | [-0.3] |  |

Table 8.5.3.2.1-2 NPRACH missed detection requirements for TDD

| Number of TX antennas | Number of RX antennas |  | Repetition number |  | Propagation conditions and correlation matrix (Annex B) |  | Frequency offset |  | SNR[dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | Preamble format 0 | Preamble format 1 | Preamble format 0-a | Preamble format 1-a |
| 1 |  | 2 |  | 8 |  | AWGN |  | 0 |  | [-17.8] | [-20.4] | [-19.3] | [-23.0] |
|  |  |  |  |  |  | EPA1 Low |  | 200 Hz |  | [-8.6] | [-11.1] | [-9.5] | [-13.3] |
|  |  |  |  | 32 |  | AWGN |  | 0 |  | [-20.2] | [-22.8] | [-20.1] | [-23.0] |
|  |  |  |  |  |  | EPA1 Low |  | 200 Hz |  | [-13.6] | [-17.5] | [-13.3] | [-17.7] |