3GPP TSG RAN WG4 Meeting #90	R4-1902084

# Athens, Greece, February 25-March 1, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | 1 | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS38.133 subclause 8.6.2 BWP switching restrictions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-02-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Rel-15Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | DCI based switching is restricted to several bandwidth scenarios for the same serving cell |  |  |  |  |  |  |  |  |
| Summary of change: |  | A restriction for a DCI based BWP switch between different numerologies is captured. When one of the SCS-specific carriers is a subset of the other SCS-specific carrier, the switch times for DCI based BWP switch applies. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An incorrect time is assumed for resumption BWP switch or possibly a RLF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

8.6	Active BWP switch delay

8.6.1	Introduction

The requirements in this section apply for a UE configured with more than one BWP on PCell or any activated SCell in standalone NR, or PSCell or any activated SCell in SCG in EN-DC. UE shall complete the switch of active DL and/or UL BWP within the delay defined in this section.

8.6.2	DCI and timer based BWP switch delay

For DCI-based BWP switch, after the UE receives BWP switching request at slot n on a serving cell, UE shall be able to receive PDSCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch occurs no later than at slot n+ TBWPswitchDelay.

The UE is not required to follow the requirements defined in this section when performing a DCI-based BWP switch between the BWPs in disjoint channel bandwidths or in partially overlapping channel bandwidths.

For timer-based BWP switch, the UE shall start BWP switch at slot n, where n is the beginning of a subframe (FR1) or half-subframe (FR2) immediately after a BWP-inactivity timer bwp-InactivityTimer [2] expires on a serving cell, and the UE shall be able to receive PDSCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch occurs no later than at slot n+ TBWPswitchDelay.

The UE is not required to transmit UL signals or receive DL signals during time duration TBWPswitchDelay on the cell where DCI-based BWP switch or timer-based BWP switch occurs.

Depending on UE capability bwp-SwitchingDelay [2], UE shall finish BWP switch within the time duration TBWPswitchDelay defined in Table 8.6.2-1.

Table 8.6.2-1: BWP switch delay

|  | NR Slot length (ms) | BWP switch delay TBWPswitchDelay (slots) |  |
| --- | --- | --- | --- |
|  |  | Type 1Note 1 | Type 2Note 1 |
| 0 | 1 | [1] | [3] |
| 1 | 0.5 | [2] | [5] |
| 2 | 0.25 | [3] | [9] |
| 3 | 0.125 | [6] | [17] |
| Note 1: Depends on UE capability. Note 2: If the BWP switch involves changing of SCS, the BWP switch delay is determined by the larger one between the SCS before BWP switch and the SCS after BWP switch. |  |  |  |

Editor’s note: It is FFS if UL BWP switch delay should include the effect of the timing advance. If UE preparation time for UL BWP switch does not take into account of timing advance, the actual allowed UL BWP switch time will be shortened.

8.6.3	RRC based BWP switch delay

For RRC-based BWP switch, after the UE receives BWP switching request, UE shall be able to receive PDSCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch occurs no later than at slot n+ TRRCprocessingDelay + TBWPswitchDelayRRC, where slot n is the last slot containing the RRC command, and TRRCprocessingDelay is the length of the RRC procedure delay in slots defined in clause 12 in TS 38.331 [2], and TBWPswitchDelayRRC is the BWP switching delay for RRC based BWP switch.

Editor’s Note: TBWPswitchDelayRRC is going to be defined for both cases: 1) BWP switch to another already configured BWP; 2) BWP switch to a newly configured BWP in the same RRC re-configuration massage. FFS whether to define delay requirements for MAC based BWP switch. The UL grant uncertainty for transmitting RRCReconfigurationComplete due to TA and TA_offset during RRC based BWP switching is FFS.