3GPP TSG-RAN WG4 RAN4#90	R4-1902108

Athens, Greece, February 25th – March 1st, 2019

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AoA Setup for FR2 RRM Testing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | AoA setup for FR2 tsting is not finalized |  |  |  |  |  |  |  |  |
| Summary of change: |  | AoA setup#1 descirption is corrected, AoA setup for Setup#2 and Setup#3 are added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | FR2 RRM tests cannot be finalized |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

3GPP TS .133 V15.3.0 (2018-09)

.

## << Start of changes >>

## A.3.15	Angle of Arrival (AoA) for FR2 RRM test cases

This clause specifies the AoA setups for FR2 RRM test cases in section A.5 and A.7. The applicable AoA setup is defined in each test case in section A.5 and A.7.

### A.3.15.1	Setup 1: Single AoA in Rx beam peak direction

There is only one active probe in the test. The DL signals, and noise if applicable, transmitted from the probe, are aligned to the UE Rx beam peak direction (as defined in 38.101-2 [19]).

### A.3.15.2	Setup 2: Single AoA in non Rx beam peak direction

There is only one active probe in the test. The DL signals, and noise if applicable, transmitted from the probe, align to a direction which is from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in 38.101-2 Section 7.3.4 for each UE power class.

Changing the AoAs between iterations of the test increases the test coverage and overall repeatability. The minimum angular change which can be used is given in Table 3.15.2-1.

Table 3.15.2-1: Allowed minimum angular distance between iterations for each power class

| UE Power class | Minimum angular distance (°) |
| --- | --- |
| 1 | FFS |
| 2 | FFS |
| 3 | [20] |
| 4 | FFS |

### A.3.15.3	Setup 3: 2 AoAs

There are 2 active probes in the test. The DL signals, and noise if applicable, transmitted from the two active probes, aligned to directions which are from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in 38.101-2 Section 7.3.4 for each UE power class.

Changing the AoAs between iterations of the test increases the test coverage and overall repeatability. The AoAs set for each power class is given in Table 3.15.3-1.

Table 3.15.3-1: Set of AoAs between probes for each power class

| UE Power class | AoA between active probes(°) |
| --- | --- |
| 1 | FFS |
| 2 | FFS |
| 3 | 30, 60, 90, 120° and 150 |
| 4 | FFS |

## << End of changes >>