3GPP TSG-RAN WG4 Meeting #90                                                                 R4-1902113

Athens, Greece, 25 Feb - 1 Mar 2019

Source:	Huawei, Hisilicon

Title:	TP to TR 37.716.21-21: Addition of CA configuration for DC_7C_n28A-n78A

Agenda item:	9.8.2

Document for:	Approval

Background

This contribution provides text proposal on the EN-DC configuration DC_7C_n28A-n78A as defined in Revised WID on EN-DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) RP-182302 [1].

Text Proposal

---Start of changes---

6.x	DC_7C_n28A-n78A

6.x.1	Operating bands for DC

Table 6.x.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7C_n28A-n78A | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.x.2	Channel bandwidths per operating band for DC

Table 6.x.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_7C_n28A-n78A | DC_7C_n28A DC_7C_n78A | 7 | See CA_7C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 160 |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.x.3	Co-existence studies

Co-existence studies of Band 7 + Band n28 + Band n78 with UL Band 7 +Band n78 are already captured in Table 5.3-1 of TR 37.716-21-21 [2].

-	2nd , 5th  order IMD generated by dual uplink of Band 7 + Band n78 may also fall into own Rx of band n28.

-	4th order IMD generated by dual uplink of Band 7 + Band n78 may also fall into own Rx of band n78. But there is no issue for TDD band.

Co-existence studies of Band 7 + Band n28 + Band n78 with UL Band 7 +Band n28 are already captured in Table 5.3-1 of TR 37.716-21-21 [2].

-	5th order harmonic generated by uplink of Band n28 may also fall into own Rx of band n78.

-	2nd , 4th order IMD generated by dual uplink of Band 7 + Band n28 may also fall into own Rx of band n78.

6.x.4	∆TIB and ∆RIB values

For DC_7C_n28A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n28-n78 | 7 | 0.3 |
|  | n28 | 0.3 |
|  | n78 | 0.8 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n28-n78 | 7 | 0 |
|  | n28 | 0 |
|  | n78 | 0.5 |

6.x.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there are IMD2 and IMD5 product produced by Band 7 and n78 that impact the reference sensitivity of Band n28. Following DC_7A-n28A_n78A requirement in table 5.2-5 of TR 37.864-41-21 [3], reference sensitivity exception requirements are defined in Table 6.x.5-1.

Based on Table 5.3-1 in TR 37.716-21-21 [2], there are IMD2 and IMD4 product produced by Band 7 and n28 that impact the reference sensitivity of Band n78. Following DC_7A-n28A_n78A requirement in table 5.2-5 of TR 37.864-41-21 [3], reference sensitivity exception requirements are defined in Table 6.x.5-1.

It is needed to have MSD added in table 7.3B.2.3.5.2-1 of TS 38.101-3.

Table 6.x.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_7C_n28A-n78A | 7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |
|  | n28 | 745 | 5 | 25 | 800 | 28.8 | FDD | IMD2 ¦f n78-f B7 ¦ |
|  | n78 | 3365 | 10 | 50 | 3365 | N/A | TDD | N/A |
|  | 7 | 2520 | 5 | 25 | 2640 | N/A | FDD | N/A |
|  | n28 | 731 | 5 | 25 | 786 | 4.0 | FDD | IMD5 ¦2*f n78-3*f B7 ¦ |
|  | n78 | 3387 | 10 | 52 | 3387 | N/A | TDD | N/A |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |
|  | n28 | 745 | 5 | 25 | 800 | N/A | FDD | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | 29.7 | TDD | IMD2 ¦fn28+fB7¦ |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |
|  | n28 | 745 | 5 | 25 | 800 | N/A | FDD | N/A |
|  | n78 | 3640 | 10 | 50 | 3640 | 10.3 | TDD | IMD4 ¦2*f B7-2*f n28¦ |

---End of changes---

Reference

[1]	RP-182302, “Revised WID on EN-DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL)”, LG Electronics

[2]	R4-1814769, “TR 37.716-21-21 v0.2.0 update: LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16, 37.716-21-21 v0.2.0”, LG Electronics

[3]	TR 37.864-41-21 v15.0.0: E-UTRA (Evolved Universal Terrestrial Radio Access) - NR Dual Connectivity (EN-DC) of LTE x Down Link (DL) / 1 Up Link (UL) (x=1,2,3,4) and inter-band NR 2 Down Link (DL) / 1 Up Link (UL).