# 3GPP TSG-RAN WG4 Meeting #90	                                                                R4-1902164

# Athens, Greece, 25 Feb – 1st Mar 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR |  | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to reflect agreed MSD analysis of DC_25A-n41A for TS 38.101-3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-2-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to R4-151188 the MSD is defined based on filter performance data. The MSD for DC_25A-n41A was missing in the spec. The draft CR is to include approved MSD in R4-1902457 |  |  |  |  |  |  |  |  |
| Summary of change: |  | MSD for DC_25A-n41A requirements are specified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MSD for DC_25A-n41A requirements are missed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Many |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Unchanged sections omitted>

6.2B.4.2.3	Inter-band EN-DC within FR1

6.2B.4.2.3.1	ΔTIB,c for EN-DC two bands

Table 6.2B.4.2.3.1-1: ΔTIB,c due to EN-DC(two bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1_n28 | 1 | 0.3 |
|  | n28 | 0.6 |
| DC_1_n40 | 1 | 0.5 |
|  | n40 | 0.5 |
| DC_1_n51 | 1 | 0.6 |
|  | n51 | 0.6 |
| DC_1_n77 | 1 | 0.6 |
|  | n77 | 0.8 |
| DC_1_n78 | 1 | 0.3 |
|  | n78 | 0.8 |
| DC_2_n5 | 2 | 0.3 |
|  | n5 | 0.3 |
| DC_2_n66 | 2 | 0.5 |
|  | n66 | 0.5 |
| DC_2_n71 | 2 | 0.3 |
|  | n71 | 0.3 |
| DC_2_n78 | 2 | 0.6 |
|  | n78 | 0.8 |
| DC_3_n7 | 3 | 0.5 |
|  | n7 | 0.5 |
| DC_3_n28 | 3 | 0.3 |
|  | n28 | 0.3 |
| DC_3_n40 | 3 | 0.5 |
|  | n40 | 0.5 |
| DC_3_n51 | 3 | 0.3 |
|  | n51 | 0.3 |
| DC_3_n77 | 3 | 0.6 |
|  | n77 | 0.8 |
| DC_3_n78 | 3 | 0.6 |
|  | n78 | 0.8 |
| DC_5_n40 | 5 | 0.3 |
|  | n40 | 0.3 |
| DC_5_n66 | 5 | 0.3 |
|  | n66 | 0.3 |
| DC_5_n78 | 5 | 0.6 |
|  | n78 | 0.8 |
| DC_7_n28 | 7 | 0.3 |
|  | n28 | 0.3 |
| DC_7_n51 | 7 | 0.3 |
|  | n51 | 0.3 |
| DC_7_n78 | 7 | 0.5 |
|  | n78 | 0.8 |
| DC_8_n40 | 8 | 0.3 |
|  | n40 | 0.3 |
| DC_8_n77 | 8 | 0.6 |
|  | n77 | 0.8 |
| DC_8_n78 | 8 | 0.6 |
|  | n77 | 0.8 |
| DC_11_n77 | 11 | 0.4 |
|  | n77 | 0.8 |
| DC_11_n78 | 11 | 0.4 |
|  | n78 | 0.8 |
| DC_12_n5 | 12 | 0.4 |
|  | n5 | 0.8 |
| DC_12_n66 | 12 | 0.8 |
|  | n66 | 0.3 |
| DC_18_n77 | 18 | 0.3 |
|  | n77 | 0.8 |
| DC_18_n78 | 18 | 0.3 |
|  | n78 | 0.8 |
| DC_19_n77 | 19 | 0.3 |
|  | n77 | 0.8 |
| DC_19_n78 | 19 | 0.3 |
|  | n78 | 0.8 |
| DC_20_n8 | 20 | 0.4 |
|  | n8 | 0.4 |
| DC_20_n28 | 20 | 0.5 |
|  | n28 | 0.5 |
| DC_20_n51 | 20 | 0.5 |
|  | n51 | 0.5 |
| DC_20_n77 | 20 | 0.6 |
|  | n77 | 0.8 |
| DC_20_n78 | 20 | 0.6 |
|  | n78 | 0.8 |
| DC_21_n77 | 21 | 0.4 |
|  | n77 | 0.8 |
| DC_21_n78 | 21 | 0.4 |
|  | n78 | 0.8 |
|  | n77 | 0.8 |
| DC_25_n41 | 25 | 0.5 |
|  | n41 | 0.41 |
|  |  | 0.92 |
| DC_26_n41 | 26 | 0.3 |
|  | n41 | 0.3 |
| DC_26_n77 | 26 | 0.3 |
|  | n77 | 0.8 |
| DC_26_n78 | 26 | 0.3 |
|  | n78 | 0.8 |
| DC_28_n51 | 28 | 0.5 |
|  | n51 | 0.5 |
| DC_28_n77 | 28 | 0.5 |
|  | n77 | 0.8 |
| DC_28_n78 | 28 | 0.5 |
|  | n78 | 0.8 |
| DC_30_n5 | 30 | 0.3 |
|  | n5 | 0.3 |
| DC_30_n66 | 30 | 0.5 |
|  | n66 | 0.8 |
| DC_38_n78 | n78 | 0.5 |
| DC_39_n78 | 39 | 0.3 |
|  | n78 | 0.8 |
| DC_39_n79 | 39 | 0.3 |
|  | n79 | 0.8 |
| DC_40_n77 | n77 | 0.5 |
| DC_41_n77 | 41 | 0.3 |
|  | n77 | 0.8 |
| DC_41_n78 | 41 | 0.3 |
|  | n78 | 0.8 |
| DC_41_n79 | 41 | 0.3 |
|  | n79 | 0.8 |
| DC_42_n51 | 42 | 0.6 |
|  | n51 | 0.8 |
| DC_66_n5 | 66 | 0.3 |
|  | n5 | 0.3 |
| DC_66_n71 | 66 | 0.3 |
|  | n71 | 0.3 |
| DC_66_n78 | 66 | 0.6 |
|  | n78 | 0.8 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

<Unchanged sections omitted>

7.3B.2.3.4	Reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1

Sensitivity degradation is allowed for a band if it is impacted by UL of another band part of the same DC configuration due to cross band isolation issues. Reference sensitivity exceptions are specified in Table 7.3B.2.3.4-1 with uplink configuration specified in Table 7.3B.2.3.4-2.

Table 7.3B.2.3.4-1: Reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1

| UL band |  | DL band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 |  | n41 |  |  | [4.3] | [4.0] | [3.9] |  |  | [3.5] | [3.3] | [3.2] | [3.1] | [3.0] |
| n41 |  | 3 |  | [3.0] | [3.0] | [3.0] | [3.1] | [3.0] | [3.0] |  |  |  |  |  |
| n41 |  | 25 |  | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |  |
| n77 |  | 411 |  | -93.5 | -90.5 | -88.7 | -87.5 |  |  |  |  |  |  |  |
| n78 |  | 411 |  | -93.5 | -90.5 | -88.7 | -87.5 |  |  |  |  |  |  |  |
|  | NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.4-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1

| UL band | DL band |  | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | n41 |  |  |  | 50 | 501 | 501 |  |  | 501 | 501 | 501 | 501 | 501 |
| n41 | 3 |  |  | 25 | 50 | 75 | 100 | 128 | 160 |  |  |  |  |  |
| n41 | 25 |  | 30 | 160 | 160 | 160 | 160 |  |  |  |  |  |  |  |
| n77 | 41 |  |  | 30 | 273 | 273 | 273 | 273 | N/A | N/A | N/A | N/A | N/A | N/A |
| n78 | 41 |  |  | 30 | 273 | 273 | 273 | 273 | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | NOTE 1: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. |  |  |  |  |  |  |  |  |  |  |  |  |

<Unchanged sections omitted>