3GPP TSG-RAN WG4 Meeting #90	R4-1902262

Athens, Greece, 25 February – 1 March 2019

Source: 	Huawei

Title: 	AH Minutes for NR BS conformance testing

Agenda Item:	6.10.2.3

Document for:	Approval

# TM1 description update

## Contributions

R4-1900549, “Draft CR for TS 38.141-1: Correction on TM applicability”, NTT DoCoMo

R4-1900550, “Draft CR for TS 38.141-2: Correction on TM applicability”, NTT DoCoMo

## Background

For clarification on the applicability of NR-FR1-TM1.1, “radiated transmit power” and “transmit ON/OFF power” and “receiver spurious emissions” would be added to the list of the tests in which the test model is used.

## Options

Option 1: add following to TM1 description

-	Radiated transmit power

-	Transmit ON/OFF power

-	Receiver spurious emissions

Option 2: keep description unchanged

Discussion:

R4-1900549

ZTE: why do we need RX spurious emission?

Keysight: we get requests to add it for clarify.

R4-1900550:

Nokia: previously we only associated one modulation scheme with a TM. If we want to add more modulation schemes, it is better to add more TMs

Huawei: is the addition of more modulation schemes necessary given that there are notes added already

WF:

R4-1900549: to be revised. Removed “Radiated transmit power”

R4-1900550: to be revised. Removed the addition of “16QAM or QPSK”

# PAPR reduction, Multi-carrier support

## Contributions

R4-1900496, “Further discussion on Data generation for NR TM”, ZTE

R4-1900537, “Draft CR to TS 38.141-1: Data Content of physical channels and signals for NR-FR1-TM (Section 4.9.2.3)”, ZTE

R4-1900539, “Draft CR to TS 38.141-1: Data Content of physical channels and signals for NR-FR1-TM (Section 4.9.2.3) ”, ZTE

R4-1900541, “Draft CR to TS 38.141-2: Data Content of physical channels and signals for NR-TM (Section 4.9.2.3) ”, ZTE

R4-1901322, “Draft CR to 38.141-1 clause 4.9.2.3 Data content of Physical channels and Signals for NR-FR1-TM”, Nokia, Nokia Shanghai Bell

R4-1901323, “Draft CR to 38.141-2 clause 4.9.2.3 Data content of physical channels and signals for NR-TM”, Nokia, Nokia Shanghai Bell

R4-1901363, “Improving CCDF (Peak to Average Radio) of Test Models”, Keysight Technologies

R4-1901367, “NR Test Model and Scrambling sequence”, Ericsson

R4-1901368, “CR to TS 38.141-1: NR TM Multicarrier Configuration”, Ericsson

R4-1901369, “CR to TS 38.141-2: NR TM Multicarrier Configuration”, Ericsson

## Background

Two problems identified:

- Parameters used to configure the seed for PDSCH scrambling may not be appropriate: the seed is 0. This may cause randomness issues (R4-1900496, R4-1901363, R4-1901367_

  - Ncellid=0, q=0, nRNTI=0

- No support for multiple carriers in a band (R4-1900496, R4-1901367, R4-1901322, R4-1901323, R4-1901368, R4-1901369)

## Summary

- Change Ncellid=i for the ith cell, i=1,…. (Not beginning with 0)

ZTE (R4-1900496, R4-1900539, R4-1900541)

Keysight (R4-1901363)?

Ericsson (R4-1901367, R4-1901368, R4-1901369)

Nokia (R4-1901322, R4-1901323)

- Consider changing nRNTI (Ericsson, Keysight)

WF:

R4-1900539 can be returned to

R4-1901369 can be returned to

# Synchronization

## Contributions

R4-1900303, “View on BS TM2 syncronization”, Samsung

R4-1900464, “Discussion on Synchronization design for TM2 for FR1 and FR2”, CATT

R4-1900470, “Discussion on Synchronization design for TM2 for FR1 and FR2”, CATT

R4-1900494, “Further discussion on sync design for NR TM2”, ZTE

R4-1901327, “On synchronization design for TM2”, Nokia, Nokia Shanghai Bell

R4-1901365, “Test Model TM2 Synchronization”, Keysight Technologies

R4-1901370, “Performance Evaluation of Synchronization designs for TM2”, Ericsson

R4-1901496, “Discussion of synchronization procedures for TM2 test model”, Huawei, HiSilicon

R4-1901497, “TP for synchronization procedures for NR-FR1-TM2 test model in TS 38.141-1”, Huawei, HiSilicon,

R4-1901498, “TP for synchronization procedures for NR-FR2-TM2 test model in TS 38.141-2”, Huawei, HiSilicon

## Background

R4-1816728, [“WF on Synchronization design for TM2 for FR1 and FR2” Ericsson] discussed several options for TM2 synchronization.

Based upon investigation and analysis, the RAN4 group should decide next meeting one of the following approaches to be taken for NR-FR1-TM2 and NR-FR2-TM2.

- Option 1: The synchronization in NR-FR1TM2 and NR-FR2-TM2 current design is sufficient for measure for all applicable tests

- Option 2: Create a synchronization slot(s) with full bandwidth DM-RS which will not be used for measurement but only for synchronization purposes, an example illustration shown below

- Option 3: Create similar full bandwidth allocation of DMRS and PDCCH as in other full bandwidth TMs. Null any REs which are not part of NR-FR1-TM2 and NR-FR2-TM2 allocation.

- Option 4: Create more single PRB used for testing, illustrated example shown below.

- Option 5: other options are not precluded for single PRB sync issue.

## Contribution review

|  | Option 1 | Option 2 | Option 3 | Option 4 | Others | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| Samsung | No issue identified | No issue identified | Additional effort to construct this pattern | Additional effort to construct this pattern |  | - Configuration shall be aligned with physical layer design, i.e., special pattern for TM only shall be forbidden as much as possible. - Both synchronization performance and test system design simplicity shall be considered. Provides a TP for TM2 Provide a description of TRS in Table 2 |
| CATT |  | preferred |  |  |  | Observation 1: The option 1 (current design) is not sufficient for measuring frequency error. |
| ZTE | residual freq error are within 20Hz and residual timing error is within 0.6Ts assuming SNR as 20dB. | residual freq error are within 2Hz and residual timing error is within 0.02Ts assuming SNR as 20dB | residual freq error should be the same as that of option 2 assuming SNR as 20dB from baseband perspective, however from RF perspective, option 2 is better than option 3 | residual freq and timing error could be improved |  | Performance curves provided |
| Nokia |  |  |  |  |  | We propose to use the updated configuration of the PDCCH that is discussed in [6] to obtain the synchronization for Test Model 2. Draft CRs that included these update for conducted conformance test specification 38.141-1 are submitted in [7] and for OTA provided in [8]. |
| Keysight | No additional change is needed for Test model 2 design for synchronization improvement | This idea cause problem with power level settings. With having same power level per RE, which causes measurement target resource blocks to be lower in power level because total power of full bandwidth portion becomes much higher power. This results in poor EVM result. With having lowering power level of full bandwidth portion to balance the power, then reference signals in this portion put in lower power which adding noisy synchronization data to make problem in EVM result (result became poor) |  | doesn’t contribute to issue because doesn’t improve number of reference signals in one time, also more work for finding right distribution even if this is picked up as possible option. |  | For option 1 which is to use current design as is, turned out that it’s sufficient enough because of DM-RS in allocated PDCCH contribute to have enough level of synchronization. Because use model of TM2 is to make EVM and frequency error measurement under power dynamic range test, noise impairment should be assumed then having PDCCH on lower end of frequency should be a problem rather than having wider distribution nor both end. |
| Ericsson | synchronization performance based on single PRB DMRS allocation is extremely poor because of inaccurate frequency and timing offset estimation | close to the ideal performance under perfect timing and frequency synchronization | invalid from physical layer design perspective and as such this option cannot be used | good technique for single PRB receiver synchronization | Employ frequency and frame synchronization using a separate synchronization frame before transmission of the waveform for NR-FR1-TM2 or NR-FR2-TM2 measurement | Prefer option 5, then option 2 |
| Huawei |  | Provide design, concern about power of synchronization | Difficult to implement |  |  | TP provided for option 2 |

## Proposals

Summary of positions

Option 1:

- Keysight: fine with current design

- If no problem, keep it (Samsung)

- Ericsson: their option 5 includes option 1

Option 2:

- Several companies find option 2 acceptable (Samsung, CATT, Ericsson)

- Keysight: concerns about power variations (“With having lowering power level of full bandwidth portion to balance the power, then reference signals in this portion put in lower power which adding noisy synchronization data to make problem in EVM result”)

Option 3:

- No support

Option 4

- No support

Others:

- Nokia: Extend PDCCH and use its DMRS

- Ericsson: synchronization frame before testing

How to proceed: discussion points:

- If option 1 selected: how to align TE perspective and simulations

- If option 2 selected: how to reduce power while maintaining complexity

- If option 5 (Nokia): can power in PDCCH be an issue?

- If option 5 (Ericsson): how to design a synchronization frame

- Observation: if a small enhancement on PDCCH filler is made (like Nokia hinted): going from 3 PRBs to 6 PRBs: would that be helpful?

Discussion:

It seems agreed upon that the testing environment setup mentioned in ZTE paper is commonly used.

Keysight still wants to have time to further investigating if TM2 needs to be improved for synchronization.

WF:

Companies to confirm if the testing environment setup mentioned in ZTE paper is commonly used

Leave the current design unchanged while further investigating if TM2 needs to be improved for synchronization

Close this issue by the April meeting.

# Filler

## Contributions

R4-1900304, “View on PDCCH filler”, Samsung

R4-1901328, “On filling remaining RBs of PDDCH in test models”, Nokia, Nokia Shanghai Bell

R4-1900497,” Further discussion on NR TM configuration”, ZTE

R4-1901337, “Draft CR to 38.141-1 clause 4.9.2 Test models - filling PDCCH”, Nokia, Nokia Shanghai Bell

R4-1901338, “Draft CR to 38.141-2 clause 4.9.2 Test models - filling PDCCH”, Nokia, Nokia Shanghai Bell

R4-1901364, “Proposal on filling remaining RBs of PDCCH symbols in Test Model definitions”, Keysight Technologies

R4-1901371, “NR Test Model PDSCH Arrangement”, Ericsson

R4-1901493, “Discussion of filling remaining RBs in PDCCH symbols of test models”, Huawei, HiSilicon

R4-1901494, “TP to TS38.141-1 subclause 4.9.2 filling remaining RBs in PDCCH symbols”, Huawei, HiSilicon

R4-1901495, TP to TS38.141-2 subclause 4.9.2 filling remaining RBs in PDCCH symbols”, Huawei,

## Background

WF R4-1816736 [“WF on filling remaining RBs of PDCCH symbols” Huawei, HiSilicon] discussed several options

- (option 1) Have PDCCH span entire symbol; satisfying a maximum of  6⌊N_RB⁄6⌋ RBs

- (option 2) The filler region is occupied by PDSCH corresponding to another user (e.g. third user N_RNTI = 3)

- (option 3) The filler region is occupied by PDSCH of user 1 (and user 2). Their PDSCH will span the entire slot (all 14 symbols) except for the PDCCH

- (option 4) Create a second PDCCH to occupy the filler region as much as possible within the first two symbols

- (option 5) Double the number of CCEs so that the number of occupied RBs is 6 across 2 symbols.

- (option 6) Move PDCCH to center of bandwidth

- (option 7) PDCCH 1 CCE spanning only 1 symbol only

- (option 8) keep current design

- Other options not precluded

## Summary

|  | Opt. 1 | Opt. 2 | Opt. 3 | Opt. 4 | Opt. 5 | Opt. 6 | Opt. 7 | Opt. 8 | Others | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Samsung | More AL | No | No | No | No | No | No |  |  | If no evaluation can show the significant impact on RF performance testing with existing control region configuration, it is acceptable to retain current TM design as option 8 |
| CATT |  |  |  |  |  |  |  |  |  |  |
| ZTE | Ok |  | Impl. concerns | ok |  |  |  |  |  | Implementation is an underlying concern |
| Nokia | Pref. |  |  |  |  |  |  |  |  |  |
| Keysight |  | ok |  |  |  |  |  |  |  |  |
| Ericsson |  | Ok | Ok |  |  |  |  |  |  |  |
| Huawei |  | ok | Impl. concerns |  |  |  |  |  |  |  |

## Proposals

Summary of positions

Two main options appear

  - More PDCCH or a third user PDSCH

Discussion:

ZTE: option 2 requires BS supporting PDSCH mapping type B

WF:

The power problem is well accepted and RAN4 needs to solve it.

Further investigate the following options:

- (option 1) Have PDCCH span entire symbol; satisfying a maximum of  6NRB6 RBs

- (option 2) The filler region is occupied by PDSCH corresponding to another user (e.g. third user N_RNTI = 3)

- (option 3) The filler region is occupied by PDSCH of user 1 (and user 2). Their PDSCH will span the entire slot (all 14 symbols) except for the PDCCH.

  - DMRS is always located at the third symbol or beyond

- (option 4) Create a second PDCCH to occupy the filler region as much as possible within the first two symbols

# Misc. corrections

## Contributions

R4-1901492, “Comparison of TS38.141-1 and TS38.141-2 base conformation test models”, Huawei, HiSilicon

R4-1901530, “Corrections to 38.141-1 subclause 4.9.2 base conformation test models”, Huawei, HiSilicon

R4-1901534, “Corrections to 38.141-2 subclause 4.9.2 base conformation test models”, Huawei, HiSilicon

## Proposals

Make editorial changes to ensure consistency

WF:

R4-1901530 and R4-1901534 can be revised.

# EVM corrections

## Contributions

R4-1901553, “Corrections to EVM frame structure in subclauses 6.5.2.3 and 9.5.2.3 in 38.104”, Huawei, HiSilicon

R4-1901500, “Discussion of clarifying EVM measurements for test models”, Huawei, HiSilicon

From AI 6.9.2.4

R4-1901326 	draft_cr 	Draft CR to 38.104: EVM frame structure correction 	Nokia, Nokia Shanghai Bell

WF:

R4-1901500 seems agreeable subject to Keysight’s further check, with wording change on “downlink slot” and “flexible slot”.