3GPP TSG-RAN WG4 Meeting #90	R4-1902278

Athens, Greece February 25 – March 1 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR |  | rev |  | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.141-2: Inclusion of subclause 4.8 reference in test procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-02-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For test configuration reference in test procedures, reference to the description and also the applicibilty tables are needed. This is also what is done for other specifications, and therefore an alignment for clairity is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Subclause 4.8 is added to test procedure when referencing test configurations |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not an inclusive reference to test configurations information |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6.4.2.1, 7.6.4.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 7.6.4.2	Procedure

7.6.4.2.1	BS type 1-O procedure for out-of-band blocking

- Place NR BS and the test antenna(s) according to annex E.2.4.1.

- Align the NR BS and test antenna(s) according to the directions to be tested.

- Connect test antenna(s) to the measurement equipment as shown in annex E.2.4.1.

- The test antenna(s) shall be dual (or single) polarized covering the same frequency ranges as the BS and the blocking frequencies. If the test antenna does not cover both the wanted and interfering signal frequencies, separate test antennas for the wanted and interfering signal are required.

- The OTA blocking interferer is injected into the test antenna, with the blocking interferer producing specified interferer field strength level for each supported polarization. The interferer shall be polarization matched in-band and the polarization maintained for out-of-band frequencies.

- Generate the wanted signal in receiver target reference direction, according to the applicable test configuration (see subclause 4.8) using applicable reference measurement channel to the RIB, according to TS 38.104 [2], annex A.1.

- Set the transmitter unit associated with the RIB under test to transmit in receiver target reference direction with the carrier set-up and power allocation according to the applicable test configuration(s) (see subclause 4.8). The transmitter may be turned OFF for the out-of-band blocker tests when the frequency of the blocker is such that no IM2 or IM3 products fall inside the bandwidth of the wanted signal.

- Adjust the signal generators to the type of interfering signals, levels and the frequency offsets as specified for general test requirements in table 7.6.5.1.1-1. The distance between the test object and test antenna injecting the interferer signal is adjusted when necessary to ensure specified interferer signal level to be received.

- The CW interfering signal shall be swept with a step size of 1 MHz within the specified range.

- Measure the performance of the wanted signal at the receiver unit associated with the RIB, as defined in the subclause 7.6.5, for the relevant carriers specified by the test configuration in subclause 4.7 and 4.8.

- Repeat for all supported polarizations.

In addition, for multi-band RIB, the following steps shall apply:

- For multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

7.6.4.2.2	BS type 1-O procedure for co-location blocking

- Place NR BS and CLTA as specified in subclause 4.12.2.3.

- Several CLTA are required to cover the whole co-location blocking frequency ranges. The CLTA shall be selected according to subclause 4.12.2.2.

- Align the NR BS and test antenna(s) according to the directions to be tested.

- Connect test antenna and CLTA to the measurement equipment as depicted in annex E.2.4.2.

- The NR BS receives the wanted signal in all supported polarizations, in the receiver target reference direction from the test antenna.

- The OTA co-location blocking interferer is injected via the CLTA. The CLTA is fed with the specified co-location blocking interferer power per supported polarization.

- Generate the wanted signal in receiver target reference direction, all supported polarizations, from the test antenna, according to the applicable test configuration (see subclause 4.8) using applicable reference measurement channel to the RIB, according to TS 38.141 [2], annex A.1.

- Set the transmitter unit associated with the RIB under test to transmit in receiver target reference direction with the carrier set-up and power allocation according to the applicable test configuration(s) (see subclause 4.8). The transmitter may be turned OFF for the out-of-band blocker tests when the frequency of the blocker is such that no IM2 or IM3 products fall inside the bandwidth of the wanted signal.

- Adjust the signal generators to the type of interfering signals, levels and the frequency offsets as specified for general test requirements in table 7.6.5.1.1-1 and, when applicable, for co-location test requirements in table 7.6.5.1.2-1.

- The CW interfering signal shall be swept with a step size of 1 MHz within the specified range.

- Measure the performance of the wanted signal at the receiver unit associated with the RIB, as defined in the subclause 7.6.5, for the relevant carriers specified by the test configuration in subclause 4.7 and 4.8.

[Unchanged Sections]

7.6.4.2.3	BS type 2-O procedure for out-of-band blocking

- Place NR BS and the test antenna(s) according to annex E.2.4.1.

- Align the NR BS and test antenna(s) according to the directions to be tested.

- Connect test antenna(s) to the measurement equipment as shown in annex E.2.4.1.

- The test antenna(s) shall be dual (or single) polarized covering the same frequency ranges as the BS and the blocking frequencies. If the test antenna does not cover both the wanted and interfering signal frequencies, separate test antennas for the wanted and interfering signal are required.

- The OTA blocking interferer is injected into the test antenna, with the blocking interferer producing specified interferer field strength level for each supported polarization. The interferer shall be polarization matched in-band and the polarization maintained for out-of-band frequencies.

- Generate the wanted signal, according to the applicable test configuration (see subclause 4.7 and 4.8) using applicable reference measurement channel to the RIB, according to TS 38.104 [2], annex A.1.

- Set the transmitter unit associated with the RIB under test to transmit in OTA REFSENS receiver target reference direction with the carrier set-up and power allocation according to the applicable test configuration(s) (see subclause 4.7 and 4.8). The transmitter may be turned OFF for the out-of-band blocker tests when the frequency of the blocker is such that no IM2 or IM3 products fall inside the bandwidth of the wanted signal.

- Adjust the signal generators to the type of interfering signals, levels and the frequency offsets as specified for general test requirements in table 7.6.5.2.1-1. The distance between the test object and test antenna injecting the interferer signal is adjusted when necessary to ensure specified interferer signal level to be received.

- The interfering signal shall be swept within the frequency range and step size specified in table 7.6.4.2.3-1.

- Measure the performance of the wanted signal at the receiver unit associated with the RIB, as defined in the subclause 7.6.5, for the relevant carriers specified by the test configuration in subclause 4.7 and 4.8.