3GPP TSG-RAN WG4 Meeting #90  	R4-1902507

Athens, Greece, 25 Feb. – 1 March, 2019

Agenda item: 	8.2.1

Source: 	LG Electronics, Ericsson, Huawei, Nokia, Qualcomm

Title: 	WF on Simulation assumptions of Co-existence study for CLI

Document for:	Approval

Introduction

The contribution summarizes simulation assumptions of co-existence study for CLI.

Simulation assumptions for FR1

Indoor

| Layout for nodes | Indoor to Indoor : 6 BSs per 120m X 50m Indoor to Macro : the number of Indoor per macro cell (drop randomly) = 1 |
| --- | --- |
| Inter-BS distance | Indoor to Indoor: 20m |
|  | The minimum distance between Macro to Indoor: [35]m |
| Minimum BS-UE (2D) distance | Indoor to Indoor: 0m |
| Minimum UE-UE (2D) distance | Indoor to Indoor: 1m~3m |
| Carrier frequency | 4GHz |
| BS TX power | 24 dBm per TRX i.e. local area BS, 4T4R 30dBm total |
| UE TX power | 23 dBm |
| Path-loss model | - Indoor (Aggressor) – Macro (Victim): - BS-to-BS: ITU InH in TR 38.803 + penetration loss - BS-to-UE: ITU InH in TR 38.803 + penetration loss - UE-to-UE: Outdoor UE – Outdoor UE from TR 36.828 + penetration loss - Indoor (Aggressor) – Indoor (Victim) - BS-to-BS: InH in TR 38.803 - BS-to-UE: InH in TR 38.803 - UE-to-UE: InH in TR 38.803 |
| BS antenna | FR1 BS antenna element pattern for Indoor scenario from 3.12.3 / ceiling |
| BS antenna height: | 3m |
| UE antenna | Omni |
| UE antenna height | 1.5m |
| eNB antenna element gain | 5dBi |
| Antenna gain of UE | 0 dBi |
| Cell selection criteria | Cell selection is based on RSRP |
| BS receiver noise figure | 5dB |
| UE receiver noise figure | 9dB |
| UE power control | Power control as defined in Section 3.6 |
| Multi operators layout | uncoordinated operation (100% Grid Shift) |

Urban macro

| Layout | Single layer with 19 hexagonal cell with wrap around |
| --- | --- |
| Inter-BS distance | 500m |
| Carrier frequency | 4GHz |
| Path-loss model | - Macro(Aggressor) – Macro(Victim) - Macro-to-UE: Uma - Macro-to-Macro: UMa (h_UE=25m) - UE-to-UE: Outdoor UE – Outdoor UE from TR 36.828 + penetration loss |
| BS Tx power | 49 dBm |
| UE Tx power | 23dBm |
| BS antenna configurations | (Mg,Ng,M,N,P)=(1,1,8,8,2) (dH,dV)=(0.5,0.8)λ |
| BS antenna height | 25m |
| BS antenna element gain + connector loss | 5dBi (assuming antenna 1.8dB loss) |
| BS receiver noise figure | 5dB |
| UE antenna configuration | Omni |
| UE antenna height | hUT=3(nfl-1)+1.5 nfl for outdoor UEs: 1 nfl for indoor UEs: nfl~uniform(1,Nfl) where Nfl = 1 |
| UE antenna gain | 0 dBi |
| UE receiver noise figure | 9dB |
| Multi operators layout | uncoordinated operation (100% Grid Shift) |

ACIR

| Parameter | Assumption/Value |
| --- | --- |
| ACIR BS-BS | 43dB |
| ACIR BS-UE | 33dB |
| ACIR UE-BS | 30dB |
| ACIR UE-UE | 28dB |

Simulation assumptions for FR2

Urban macro

| Network layout | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |
| --- | --- |
| Inter-site distance | 200m |
| BS antenna height | 25 m |
| Path-loss model | - Macro (Aggressor) – Macro (Victim) - Macro-to-Macro: 5GCM UMa (h_UE=25m) - Macro-to-UE (V) UE: 5GCM Uma + penetration loss - UE-to-UE: UMi (h_BS=1.5m ~ 22.5m) + penetration loss |
| Shadowing correlation | Between cells: 1.0 Between sites: 0.5 |
| Multi operators layout | uncoordinated operation (100% Grid Shift) |

Dense urban

| Parameters | Values | Remark |
| --- | --- | --- |
| Network layout | Fixed cluster circle within a macro cell. | note1 |
| Number of micro BSs per macro cell | 3 | 3 cluster circles are in a macro cell. 1 cluster circle has 1 micro BS. |
| Radius of UE dropping within a micro cell | < 28.9 m |  |
| BS antenna height | 10 m |  |
| Channel model | UMi - Micro (A) – Micro (V) - Micro-to-Micro: UMi (h_UE=10m) - Micro-to-Micro UE: UMi + penetration loss - Micro UE to Micro UE: 5GCM UMi (h_BS=1.5m ~ 22.5m) + - penetration loss between UEs |  |
| Shadowing correlation | Between cite: 0.5 |  |
| Note 1: Micro BS is randomly dropped on an edge of the cluster circle. All UEs communicate with micro BS, i.e. macro cell is only used for determining position of micro BS. As a layout of macro cell, hexagonal grid, 19 macro sites, 3 sectors per site model with wrap around with ISD = 200m is assumed. |  |  |

| Parameters | Values | Remark |
| --- | --- | --- |
| Multi operator layout | Cluster circle is coordinated | Note 1 |
| Minimum distance between micro BSs in different operator | 10 m |  |
| Note 1: Macro cell is collocated. Micro BS itself is randomly dropped. |  |  |

Indoor

| Parameters | Values | Remark |
| --- | --- | --- |
| Network layout | Indoor-Indoor : 50m x 120m, 6BSs Macro – Indoor: Indoors are placed at different locations | Indoor-Indoor Macro-Indoor |
| Inter-site distance | Indoor – Indoor = 20m |  |
|  | The minimum distance between Macro to Indoor: [35]m |  |
| BS antenna height | 3 m | ceiling |
| Path-loss model | Indoor - Indoor InH-office LOS InH-office NLOS Indoor (A) – Macro (V) - BS-to-BS: 5GCM Indoor-office (h_UE=3m) + penetration loss - BS-to-UE: 5GCM Indoor-office (h_UE=3m) + penetration loss - UE-to-UE: 5GCM Indoor-office (h_BS=1.5m) + penetration loss |  |
| Shadowing correlation | NA |  |
| Multi operators layout for indoor | Uncoordinated operation (100%) |  |

UE Distribution

<FR1 Indoor and macro to indoor>

| UE distribution | - Indoor -> Indoor = 1 user per TRP (Transmission Reception Point); 100% indoor - Indoor <-> macro = 1 user per TRP; Indoor has 100% indoor UE. Macro victim has 50% indoor UE and 50% outdoor. - Indoor <-> macro = Aggressor: 1 user per TRP, 100% indoor. Victim: 1 user per TRP, 100% outdoor |
| --- | --- |

<FR1 Urban Macro>

| UE distribution | 20% indoor, 80% outdoor |
| --- | --- |

<FR2 Urban Macro>

| UE location | Outdoor/indoor | Outdoor and indoor |
| --- | --- | --- |
|  | Indoor UE ratio | 0% |
|  | LOS/NLOS | LOS and NLOS |
|  | UE antenna height | 1.5 m ≦ hUT ≦ 22.5 m |
| UE distribution (horizontal) |  | Uniform |
| Minimum BS - UE distance (2D) |  | 35 m |

<FR2 Dense Urban>

| UE location | Outdoor/indoor | Outdoor and indoor |
| --- | --- | --- |
|  | Indoor UE ratio | 80 % |
|  | 50% low loss, 50% high loss | Low/high Penetration loss ratio |
|  | LOS/NLOS | LOS and NLOS |
|  | UE antenna height | Same as 3D-UMi in TR 36.873 |
| UE distribution (horizontal) |  | Uniform |
| Minimum BS - UE distance (2D) |  | 3m |

<FR2 Indoor to indoor and macro to indoor>

| UE distribution | - Indoor -> Indoor = 1 user per TRP (Transmission Reception Point); 100% indoor - Indoor <-> macro = Aggressor: 1 user per TRP, 100% indoor. Victim: 1 user per TRP, 100% outdoor |
| --- | --- |

<FR2 Indoor>

| LOS/NLOS | LOS and NLOS |
| --- | --- |
| UE antenna height | 1 m |
| UE distribution (horizontal) | Uniform |
| Minimum BS - UE distance (2D) | 0 m |

Propagation model

Path loss

The Path loss model is summarized in Table 5.2.2.1-1 and the distance definitions are indicated in Figure 5.2.2.1-1. Note that the distribution of the shadow fading is log-normal, and its standard deviation for each scenario is given in Table 5.2.2.1-1.

Table 5.2.2.1-1: Pathloss models

| Scenario | Pathloss [dB], fc is in GHz and d is in meters (6) | Shadow fading std [dB] | Applicability range, antenna height default values |
| --- | --- | --- | --- |
| UMa LOS |  | σSF=4.0 σSF=4.0 | 10m < d2D < d'BP 1) d'BP < d2D <5000m 1.5m ≦ hUT≦ 22.5m hBS = 25 m |
| UMa NLOS |  | σSF =6 | 10 m < d2D < 5 000 m 1.5 m ≦ hUT ≦ 22.5 m hBS = 25 m Explanations: see note 3 |
| UMi - Street Canyon LOS |  | σSF=4.0 σSF=4.0 | 10m < d2D < d'BP 1) d'BP < d2D <5000m 1.5m ≦ hUT≦ 22.5m hBS = 10 m |
| UMi – Street Canyon NLOS |  | σSF=7.82 | 10 m < d2D < 5000m 1.5m ≦ hUT≦ 22.5m hBS = 10 m Explanations: see note 4 |
| InH - Office LOS |  | σSF=3.0 | 1<d3D<100m |
| InH - Office NLOS |  | σSF=8.03 | 1<d3D<86m |
| Note 1: d'BP = 4 h'BS h'UT fc/c, where fc is the centre frequency in Hz, c = 3.0108 m/s is the propagation velocity in free space, and h'BS and h'UT are the effective antenna heights at the BS and the UT, respectively. In UMi scenario the effective antenna heights h'BS and h'UT are computed as follows: h'BS = hBS – 1.0 m, h'UT = hUT–1.0 m, where hBS and hUT are the actual antenna heights, and the effective environment height is assumed to be equal to 1.0 m. In UMa scenario the effective antenna heights h'BS and h'UT are computed as follows: h'BS = hBS – hE, h'UT = hUT – hE, where hBS and hUT are the actual antenna heights, and the effective environment height hE is a function of the link between a BS and a UT. In the event that the link is determined to be LOS, hE=1m with a probability equal to 1/(1+C(d2D, hUT)) and chosen from a discrete uniform distribution uniform(12,15,…,(hUT-1.5)) otherwise. Note 2: The applicable frequency range of the PL formula in this table is 0.8 < fc < fH GHz, where fH = 30 GHz for RMa and fH = 100 GHz for all the other scenarios. It is noted that RMa pathloss model for >7 GHz is validated based on a single measurement campaign conducted at 24 GHz. Note 3: UMa NLOS pathloss is from TR36.873 with simplified formatand and PLUMa-LOS = Pathloss of UMa LOS outdoor scenario. Note 4: PLUMi-LOS = Pathloss of UMi-Street Canyon LOS outdoor scenario. Note 5: Break point distance dBP = 2π hBS hUT fc/c, where fc is the centre frequency in Hz, c = 3.0 108 m/s is the propagation velocity in free space, and hBS and hUT are the antenna heights at the BS and the UT, respectively. Note 6: fc denotes the center frequency normalized by 1GHz, all distance related values are normalized by 1m, unless it is stated otherwise. |  |  |  |

LOS probability

The Line-Of-Sight (LOS) probabilities are given in Table 5.2.2.2-1.

Table 5.2.2.2-1: LOS probability

| Scenario | LOS probability (distance is in meters) |
| --- | --- |
| UMi – Street canyon | Outdoor users: Indoor users: Use d2D-out in the formula above instead of d2D |
| UMa | Outdoor users: where and Indoor users: Use d2D-out in the formula above instead of d2D |
| Indoor – Open office |  |
| Note: The LOS probability is derived with assuming antenna heights of 3m for indoor, 10m for UMi, and 25m for Uma |  |

O-to-I penetration loss

The Path loss incorporating O-to-I building penetration loss is modelled as in the following:

PL = PLb + PLtw + PLin + N(0, σP2)

where PLb is the basic outdoor path loss given in Section 5.1.2.2.1. PLtw is the building penetration loss through the external wall, PLin is the inside loss dependent on the depth into the building, and σP  is the standard deviation for the penetration loss.

PLtw is characterized as:

is an additional loss is added to the external wall loss to account for non-perpendicular incidence;

, is the penetration loss of material i, example values of which can be found in Table 5.2.2.3-1.

pi is proportion of i-th materials, where ; and

N is the number of materials.

Table 5.2.2.3-1: Material penetration losses

| Material | Penetration loss [dB] |
| --- | --- |
| Standard multi-pane glass | Lglass=2+0.2⋅f |
| IRR glass | LIRRglass=23+0.3⋅f |
| Concrete | Lconcrete=5+4⋅f |
| Wood | Lwood=4.85+0.12⋅f |
| Note: f is in GHz |  |

Table 5.2.2.3-2 gives PLtw, PLin and σP  for two O-to-I penetration loss models. The O-to-I penetration is UT-specifically generated, and is added to the SF realization in the log domain.

Table 5.2.2.3-2 O-to-I penetration loss model

|  | Path loss through external wall: PLtw [dB] | Indoor loss: PLin [dB] | Standard deviation: σP [dB] |
| --- | --- | --- | --- |
| Low-loss model | 5-10log100.3⋅10-Lglass10+0.7⋅10-Lconcrete10 | 0.5d2D-in | 4.4 |
| High-loss model | 5-10log100.7⋅10-LIRRglass10+0.3⋅10-Lconcrete10 | 0.5d2D-in | 6.5 |

d2D-in is minimum of two independently generated uniformly distributed variables between 0 and 25 m for RMa, UMa and UMi-Street Canyon. d2D-in shall be UT-specifically generated.

Both low-loss and high-loss models are applicable to UMa and UMi-Street Canyon.

Only the low-loss model is applicable to RMa.

The composition of low and high loss is a simulation parameter that should be determined by the user of the channel models, and is dependent on the use of metal-coated glass in buildings and the deployment scenarios. Such use is expected to differ in different markets and regions of the world and also may increase over years to new regulations and energy saving initiatives. Furthermore, the use of such high-loss glass currently appears to be more predominant in commercial buildings than in residential buildings in some regions of the world.

The pathloss incorporating O-to-I car penetration loss is modelled as in the following:

PL = PLb + N(μ, σP2)

where PLb is the basic outdoor path loss given in Section 7.4.1. μ = 9, and σP = 5. Optionally, for metallized car windows, μ = 20 can be used. The O-to-I car penetration loss models are applicable for at least 0.6-60 GHz.

Transmission power control model

For downlink scenario, no power control scheme is applied.

For uplink scenario, TPC model specified in Section 9.1 TR 36.942 is applied with following parameters.

-	CLx-ile = 88 + 10*log10(200/X), where X is UL transmission BW (MHz)

-	γ = 1

Received signal power model

The following model is applied.

RX_PWR = TX_PWR – Path loss + G_TX + G_RX

where:

RX_PWR is the received power

TX_PWR is the transmitted power

G_TX is the transmitter antenna gain (directional array gain)

G_RX is the receiver antenna gain (directional array gain).

Other simulation parameters

| Parameters | Indoor | Urban macro | Dense urban |
| --- | --- | --- | --- |
| Channel bandwidth | 200MHz | 200MHz | 200MHz |
| Scheduled channel bandwidth per UE (DL) | 200MHz | 200MHz | 200MHz |
| Scheduled channel bandwidth per UE (UL) | 200MHz | 200MHz | 200MHz |
| Traffic model | Low (RU 10%) and Full buffer | Low (RU 10%) and Full buffer | Low (RU 10%) and Full buffer |
| DL power control | NO | NO | NO |
| UL power control | YES | YES | YES |
| BS max TX power in dBm TRP (Total Radiated Power) | 23dBm | 43dBm | 33dBm |
| UE Peak EIRP in dBm | 22.4 dBm | 22.4 dBm | 22.4 dBm |
| UE min TX power in dBm | -40dBm | -40dBm | -40dBm |
| BS Noise figure in dB | 10 (note 1) | 10 (note 1) | 10 (note 1) |
| UE Noise figure in dB | 10 (note 1) | 10 (note 1) | 10 (note 1) |
| Handover margin | 3dB | 3dB | 3dB |
| Note 1: ITU WP5D response |  |  |  |

Simulation description

Adopt following simulation steps.

1.	Aggressor and victim network are generated.

-	UEs are distributed randomly across the network.

2.	UE associations: UEs are associated to base station based on coupling loss.

-	Associations are made assuming a single element at both UE and BS.

3.	Once association is done, round robin scheduling is used. BF weights are adjusted to point to the LOS direction between BS-UE. This is done for both victim and aggressor networks.

4.	(optional) Throughput is computed in the victim systems without considering ACI as below:

-	ThputNO ACIbpshz=fSINRICI=fSN+IICI, where IICI is the inter-cell interference.

5.	Throughput is computed considering ACI as below:

-	ThputACIbpshz=fSINRICI+ACI=fSN+IICI+IACI, where IACI is the adjacent channel interference.

Note. Simulation results should be in the form of the throughput with adjacent network with DTDD and without DTDD and the relative difference between the two can be compared at 50% and 5% points.

ACLR and ACS

| Parameter | Assumption/Value |
| --- | --- |
| BS ACLR | 28 dB |
| UE ACLR | 17 dB |
| BS ACS | 23.5 dB |
| UE ACS | 23 dB |

Evaluation metric

Parameters describing baseline Link Level performance for 5G NR

| Parameter | DL | UL | Notes |
| --- | --- | --- | --- |
| α, attenuation | 0.6 | 0.4 | Represents implementation losses |
| SNIRMIN, dB | -10 | -10 | Based on QPSK, 1/8 rate (DL) & 1/5 rate (UL) |
| SNIRMAX, dB | 30 | 22 | Based on 256QAM 0.93(DL) & 64QAM 0.93 (UL) |

Antenna configuration

BS Antenna modelling (section 5.2.3.2 in TR 38.803)

Urban macro scenario

Table 5.2.3.2.1-1: FR2 BS antenna modelling for Urban macro scenario

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 3 dBi (assuming 1.8dB loss) |
| (Mg, Ng, M, N, P) note | For 30GHz: (1, 1, 8, 16, 2) |
| (dv, dh) | (0.5λ, 0.5λ) |
| Note: An additional 3dB gain is added to the total beamforming gain to account for the two polarization directions. Boresight direction is horizontal. |  |

Dense urban scenario

Table 5.2.3.2.2-1: FR2 BS antenna element pattern for Dense urban scenario

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 3 dBi (assuming 1.8dB loss) |
| (Mg, Ng, M, N, P) note | For 30GHz: (1, 1, 8, 16, 2) |
| (dv, dh) | (0.5λ, 0.5λ) |
| Note: An additional 3dB gain is added to the total beamforming gain to account for the two polarization directions. Boresight direction is horizontal. |  |

Indoor scenario

Table 5.2.3.2.3-1: FR1 BS antenna element pattern for Indoor scenario

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 3.5 dBi (assuming 1.8dB loss) |

Table 5.2.3.2.3-2: FR2 BS antenna element pattern for Indoor scenario

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 3 dBi (assuming 1.8dB loss) |
| (Mg, Ng, M, N, P) note | For 30GHz: (1, 1, 4, 8, 2) |
| (dv, dh) | (0.5λ, 0.5λ) |

- UE antenna element pattern

Table 5.2.3.3-1: FR2 UE antenna element pattern

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 3 dBi (assuming 1.8dB loss) |
| (Mg, Ng, M, N, P) | (1, 1, 2, 2, 2) |
| (dv, dh) | (0.5λ, 0.5λ) |
| UE orientation | Random orientation in the azimuth domain: uniformly distributed between -90 and 90 degrees* Fixed elevation: 90 degrees |
| NOTE: This is done to emulate two panels: the configuration is equivalent to 2 panels with 180 shift in horizontal orientation and UE orientation uniformly distributed in the azimuth domain between -180 and 180 degrees. |  |

On antenna modelling

Note the above gives the correct antenna array radiation pattern, however the correct gain is only achieved if the element pattern  is selected for the exact element spacing. For other element spacing’s, the element pattern  must be separately calculated such that it is correct for the element spacing (dg,H and dg,V). If  is not linked to the element spacing then the calculated absolute gain may diverge from the correct value in a manner that varies as the beam is steered.

The correct composite array radiation pattern directivity(D) is given by:

,

The composite array radiation pattern gain can then be calculated as:

Where L is the Loss associated with the antenna. This is currently included in the estimate for element gain , and is 1.8dB.