3GPP TSG-RAN4 Meeting #90	R4-1902640

# Athens, Greece, 25th February – 1st March 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for L1-RSRP based beam reporting requirements (section 9.5) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The measurement period for L1-RSRP measurement for beam reporting are incomplete. The same change as agreed in R4-1816111 should be added to the scheduling restriction due to L1-RSRP measurement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Complete the measurement period for L1-RSRP measurement. Update the scheduling restriction due to L1-RSRP measurement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no accuracy requirement for L1-RSRP. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

## 9.5	L1-RSRP measurements for Reporting

### 9.5.1	Introduction

When configured by the network, the UE shall be able to perform L1-RSRP measurements of configured CSI-RS, SSB or CSI-RS and SSB resources for L1-RSRP. The measurements shall be performed on the resources configured for L1-RSRP measurements within the active BWP.

The UE shall be able to measure all CSI-RS resources and/or SSB resources of the CSI Resource set(s) within the CSI Resource settings configured for L1-RSRP for the active BWP, provided that the number of resources does not exceed the UE capability indicated by beamManagementSSB-CSI-RS.

Unless the reporting quantity is set to ‘none’, the UE shall report the quantity for the CSI reporting configuration associated by the reporting quantity.

### 9.5.2	Requirements applicability

The requirements in Section 9.5 apply, provided:

-	The CSI-RS or SSB or CSI-RS and SSB resources configured for L1-RSRP measurements are measurable.

An CSI-RS and/or SSB resource configured for L1-RSRP shall be considered measurable when for each relevant CSI-RS and/or SSB:

-	L1-RSRP related side conditions given in Sections 10.x.x for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to Annex B.2.2 for a corresponding Band.

Requirements are defined for periodic, semi-persistent and aperiodic resources.

### 9.5.3	Measurement Reporting Requirements

The UE shall only send L1-RSRP reports for report configurations configured for the active BWP.

The UE shall report the L1-RSRP value as a 7-bit value in the range [-140, -44] dBm with 1dB step size according to section 10.1.x if nrofReportedRS is configured to one. Otherwise, and additionally if groupBasedBeamReporting is enabled the UE shall use differential L1-RSRP based reporting as defined in section 10.1.x.

#### 9.5.3.1	Periodic Reporting

Reported L1-RSRP measurements contained in periodic L1-RSRP reports shall meet the requirements in sections 10.x, 10.y and 10.z, respectively.

The UE shall not send any periodic L1-RSRP reports for a non-active BWP.

The L1-RSRP reporting delay is FFS.

The periodic L1-RSRP reporting delay shall be less than [TBD].

#### 9.5.3.2	Semi-Persistent Reporting

Reported L1-RSRP measurements contained in Semi-Persistent L1-RSRP reports shall meet the requirements in sections 10.x, 10.y and 10.z, respectively. This requirement applies for semi-persistent L1-RSRP reports send on PUSCH or PUCCH.

The UE shall not send any semi-persistent L1-RSRP reports on PUSCH, as long as no DCI request have been received.

The UE shall not send any semi-persistent L1-RSRP reports on PUCCH, as long as no activation command [38.321] have been received.

The L1-RSRP reporting delay is defined as the time between a request or command that will trigger an L1-RSRP report is received by the UE and the point when the UE starts to transmit the L1-RSRP report over the air interface.

The Semi-persistent L1-RSRP reporting delay shall be less than [TBD] for a DCI requsted semi-persistent L1-RSRP report and less than [TBD] for an L1-RSRP requested by activation command [36.321].

#### 9.5.3.3	Aperiodic Reporting

Reported L1-RSRP measurements contained in Aperiodic triggered, Aperiodic triggered periodic and Aperiodic triggered semi-persistent L1-RSRP reports shall meet the requirements in sections 10.x, 10.y and 10.z, respectively.

The UE shall not send any aperiodic L1-RSRP reports, as long as no DCI trigger have been received.

The L1-RSRP reporting delay is defined as the time between a DCI request that will trigger an L1-RSRP report and the point when the UE starts to transmit the L1-RSRP report over the air interface.

The aperiodic L1-RSRP reporting delay shall be less than [TBD].

### 9.5.4	L1-RSRP measurement requirements

#### 9.5.4.1	SSB based L1-RSRP Reporting

The UE shall be capable of performing L1-RSRP measurements based on the configured SSB resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TBM_Measurement_Period_SSB.

The value of TBM_Measurement_Period_SSB is defined in Table 9.5.4.1-1 for FR1 and Table 9.5.4.1-2 for FR2, where

-	M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise

- 	N= 8.

For FR1,

-	P=1/(1 – TSSB/MGRP), when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the SSB; and

-	P=1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the SSB.

For FR2,

-	P=1/(1 – TSSB/TSMTCperiod), when BM-RS is not overlapped with measurement gap and BM-RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod).

-	P is Psharing factor, when BM-RS is not overlapped with measurement gap and BM-RS is fully overlapped with SMTC period (TSSB = TSMTCperiod).

-	P is 1/(1- TSSB/MGRP - TSSB/TSMTCperiod), when BM-RS is partially overlapped with measurement gap and BM-RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and

-	TSMTCperiod ≠ MGRP or

-	TSMTCperiod = MGRP and TSSB < 0.5*TSMTCperiod

-	P is 1/(1- TSSB /MGRP)* Psharing factor, when BM-RS is partially overlapped with measurement gap and BM-RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and TSMTCperiod = MGRP  and TSSB = 0.5*TSMTCperiod

-	P is 1/{1- TSSB /min (TSMTCperiod ,MGRP)}, when BM-RS is partially overlapped with measurement gap (TSSB <MGRP) and BM-RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is partially or fully overlapped with measurement gap.

-	P is 1/(1- TSSB /MGRP)* Psharing factor, when BM-RS is partially overlapped with measurement gap and BM-RS is fully overlapped with SMTC occasion (TSSB = TSMTCperiod) and SMTC occasion is partially overlapped with measurement gap (TSMTCperiod < MGRP)

-	Psharing factor = 3.

If the high layer in TS 38.331 [2] signaling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1.

Longer evaluation period would be expected if the combination of BM-RS, SMTC occasion and measurement gap configurations does not meet pervious conditions.

Editor’s Note: FFS what evaluation period would be expected if BM-RS are in the same OFDM symbols with RLM/BFD/CBD-RS, or other BM-RS.

Table 9.5.4.1-1: Measurement period TBM_Measurement_Period_SSB for FR1

| Configuration | TBM_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note: TSSB is the periodicity of SSB configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

Table 9.5.4.1-2: Measurement period TBM_Measurement_Period_SSB for FR2

| Configuration | TBM_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note: TSSB is the periodicity of SSB configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

#### 9.5.4.2	CSI-RS based L1-RSRP Reporting

The UE shall be capable of performing L1-RSRP measurements based on the configured CSI-RS resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TBM_Measurement_Period_CSI-RS.

The value of TBM_Measurement_Period_CSI-RS is defined in Table 9.5.4.2-1 for FR1 and in Table 9.5.4.2-2 for FR2, where

-	For periodic and semi-persistent CSI-RS resources, M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise

-	For aperiodic CSI-RS resources M=1

-	N=

TBD

For FR1,

-	P=1/(1 – TCSI-RS/MGRP), when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the CSI-RS; and

-	P=1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the CSI-RS.

For FR2,

-	P=1, when BM-RS is not overlapped with measurement gap and also not overlapped with SMTC occasion.

-	P=1/(1 – TCSI-RS/MGRP) , when BM-RS is partially overlapped with measurement gap and BM-RS is not overlapped with SMTC occasion (TCSI-RS < MGRP)

-	P=1/(1 – TCSI-RS /TSMTCperiod), when BM-RS is not overlapped with measurement gap and BM-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod).

-	P is Psharing factor, when BM-RS is not overlapped with measurement gap and BM-RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod).

-	P is 1/(1- TCSI-RS /MGRP - TCSI-RS /TSMTCperiod), when BM-RS is partially overlapped with measurement gap and BM-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and

-	TSMTCperiod ≠ MGRP or

-	TSMTCperiod = MGRP and TCSI-RS < 0.5*TSMTCperiod

-	P is 1/(1- TCSI-RS /MGRP)* Psharing factor, when BM-RS is partially overlapped with measurement gap and BM-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and TSMTCperiod = MGRP  and TCSI-RS = 0.5*TSMTCperiod

-	P is 1/{1- TCSI-RS /min (TSMTCperiod ,MGRP)}, when BM-RS is partially overlapped with measurement gap (TCSI-RS < MGRP) and BM-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is partially or fully overlapped with measurement gap.

-	P is 1/(1- TCSI-RS /MGRP)* Psharing factor, when BM-RS is partially overlapped with measurement gap and BM-RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod) and SMTC occasion is partially overlapped with measurement gap (TSMTCperiod < MGRP)

-	Psharing factor is 3.

If the high layer in TS 38.331 [2] signaling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1.

Note: The overlap between CSI-RS for L1-RSRP measurement and SMTC means that CSI-RS for L1-RSRP measurement is within the SMTC window duration.

Longer evaluation period would be expected if the combination of BM-RS, SMTC occasion and measurement gap configurations does not meet pervious conditions.

Editor’s Note: FFS what evaluation period would be expected if BM-RS are in the same OFDM symbols with RLM/BFD/CBD-RS, or other BM-RS.

Table 9.5.4.2-1: Measurement period TBM_Measurement_Period_CSI-RS for FR1

| Configuration | TBM_Measurement_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TCSI-RS) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. |  |

Table 9.5.4.2-2: Measurement period TBM_Measurement_Period_CSI-RS for FR2

| Configuration | TBM_Measurement_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TCSI-RS) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320ms | ceil(M*P*N)*TDRX |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. |  |

### 9.5.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement

The UE is required to be capable of measuring CSI-RS for L1-RSRP without measurement gaps. The UE is required to perform the CSI-RS measurements as described in the following clauses.

#### 9.5.5.1	UE performing CSI-RS measurements with a same subcarrier spacing as SSB on FR1

When the SSB is within the active BWP and has same SCS than CSI-RS, the UE shall be able to perform CSI-RS measurement without restrictions when CSI-RS measurement are performed with same subcarrier spacing as the SSB on FR1.

#### 9.5.5.2	CSI-RS measurement restrictions of UE performing CSI-RS measurements with a different subcarrier spacing than SSB on FR1

When the SSB is within the active BWP and has different SCS than CSI-RS, the UE shall be able to performs CSI-RS measurement with restrictions according to its capabilities:

-	If CSI-RS and SSB are FDM’ed. the UE measurement capability depends on the whether the UE supports simultaneousRxDataSSB-DiffNumerology.

-	If the UE supports simultaneousRxDataSSB-DiffNumerology the UE shall be able to performs CSI-RS measurement without restrictions.

-	If the UE does not support simultaneousRxDataSSB-DiffNumerology the UE is not expected to perform simultaneous FDM’ed SSB and CSI-RS measurements,

-	If CSI-RS and SSB are TDM’ed, the UE shall be able to performs CSI-RS measurement with restrictions: the UE is not expected to measure CSI-RS on symbols on 1 data symbol before each consecutive SSB symbols and 1 data symbol after each consecutive SSB symbols within the SMTC window duration.

### 9.5.6	Scheduling availability of UE during L1-RSRP measurement

Scheduling availability restrictions when the UE is performing L1-RSRP measurement are described in the following clauses.

#### 9.5.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1

There are no scheduling restrictions due to L1-RSRP measurement performed on SSB and CSI-RS configured as RS for L1-RSRP measurement with the same SCS as PDSCH/PDCCH in FR1.

#### 9.5.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1

For UEs which support simultaneousRxDataSSB-DiffNumerology [14] there are no restrictions on scheduling availability due to L1-RSRP measurement based on SSB as RS for L1-RSRP measurement. For UEs which do not support simultaneousRxDataSSB-DiffNumerology [14] the following restrictions apply due to L1-RSRP measurement based on SSB configured as RS for L1-RSRP measurement.

-	The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on SSB symbols to be measured for L1-RSRP measurement.

When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions apply to all SCells that are aggregated in the same band as the PCell or PSCell. When inter-band carrier aggregation within FR1 is configured, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands than the bands in which PCell or PSCell is configured.

#### 9.5.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2

The following scheduling restriction applies due to L1-RSRP measurement.

-	For the case where RS for L1-RSRP measurement is QCLed with active TCI state for PDCCH/PDSCH, and N=1 applies for the RS for L1-RSRP measurement as specified in section 9.4.5.1 if the reference signal is SSB and in section 9.4.5.2 if the reference signal is CSI-RS

-	There are no scheduling restrictions due to L1-RSRP measurement performed based on SSB or CSI-RS with a same SCS as PDSCH/PDCCH.

-	When performing L1-RSRP measurement based on SSB with a different SCS than PDSCH/PDCCH, for UEs which support simultaneousRxDataSSB-DiffNumerology [14] there are no restrictions on scheduling availability due to L1-RSRP measurement. For UEs which do not support simultaneousRxDataSSB-DiffNumerology [14] the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on SSB symbols to be measured for L1-RSRP measurement.

-	Otherwise

-	The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on RS for L1-RSRP measurement symbols to be measured for L1-RSRP measurement.

When intra-band carrier aggregation is performed, the scheduling restrictions apply to all serving cells on the band due to L1-RSRP measurement performed on any serving cell in the same band.

Editor’s Note: FFS scheduling restrictions for inter-band carrier aggregation will be defined depending on band combination in future.

#### 9.5.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA

There are no scheduling restrictions on FR1 serving cell(s) due to L1-RSRP measurement performed on FR2 serving PCell and/or PSCell.

There are no scheduling restrictions on FR2 serving cell(s) due to L1-RSRP measurement performed on FR1 serving PCell and/or PSCell.

<End of Change 1>