3GPP TSG-RAN WG4 Meeting #90												  	         R4-1900066

Athens, Greece, 25th of February – 1st of March, 2019

Source: 	Nokia, Nokia Shanghai Bell

Title: 					How do we group EN-DC combinations in spec?

Agenda Item: 			6.7.2

Document for:	Approval

# 1	Introduction

In this contribution we discuss the fact that currently there are several different ways how EN-DC configurations are listed in 38.101-3 clause 5.5B. It would be important to agree uniform format to avoid miss understandings and guide proponents to draft the TPs and CRs.  In this contribution we focus only interband EN-DC as there are not that many intraband EN-DC configurations in current specification and also as intraband EN-DC is much more complicated that interband EN-DC it many need different approach anyways.

# 2	Discussion

## 2.1	Current situation

EN-DC configurations are captured in table format in 38.101-3 clause 5.5B. Tables have four columns, first column tells which EN-DC configurations the row is about, second column indicates valid uplink EN-DC configurations and then third and fourth columns indicates the corresponding E-UTRA and NR configurations. Uplink EN-DC configuration column is perhaps most important as not all uplink permutations are allowed due to technical reasons such as filtering agreements made in RAN4. Information in columns for E-UTRA and NR configurations is rather obvious, but the merit is that those spell out the baseline E-UTRA and NR configurations and can be used when digging up the requirements for these.

Discrepancy that this contribution discusses is how the EN-DC configurations are grouped or not grouped in the first column, information in other columns just follow accordingly.

As an example, in Table 5.5B.5.4 looking band combination 1+19+21+42+n257 we can see that each DL EN-DC combination is on separate row, see below

Table 5.5B.5.4-1: Inter-band EN-DC configurations including FR2 (five bands)

| DC_1A-19A-21A-42A_n257A | DC_1A_n257A DC_19A_n257A DC_21A_n257A DC_42A_n257A | CA_1A-19A-21A-42A | n257A |
| --- | --- | --- | --- |
| DC_1A-19A-21A-42A_n257D | DC_1A_n257A DC_19A_n257A DC_21A_n257A DC_42A_n257A | CA_1A-19A-21A-42A | n257A |
| DC_1A-19A-21A-42A_n257E | DC_1A_n257A DC_19A_n257A DC_21A_n257A DC_42A_n257A | CA_1A-19A-21A-42A | n257A |
| DC_1A-19A-21A-42A_n257F | DC_1A_n257A DC_19A_n257A DC_21A_n257A DC_42A_n257A | CA_1A-19A-21A-42A | n257A |
| DC_1A-19A-21A-42C_n257A | DC_1A_n257A DC_19A_n257A DC_21A_n257A DC_42A_n257A | CA_1A-19A-21A-42C | n257A |
| DC_1A-19A-21A-42C_n257D | DC_1A_n257A DC_19A_n257A DC_21A_n257A DC_42A_n257A | CA_1A-19A-21A-42C | CA_n257D |
| DC_1A-19A-21A-42C_n257E | DC_1A_n257A DC_19A_n257A DC_21A_n257A DC_42A_n257A | CA_1A-19A-21A-42C | CA_n257E |
| DC_1A-19A-21A-42C_n257F | DC_1A_n257A DC_19A_n257A DC_21A_n257A DC_42A_n257A | CA_1A-19A-21A-42C | CA_n257F |

But when looking Table 5.5B.5.3 and band combination 1+19+21+n257 (fall-back of the above) four different EN-DC configurations are grouped into one row, see below

Table 5.5B.5.3-1: Inter-band EN-DC configurations including FR2 (four bands)

| DC_1A-19A-21A_n257A DC_1A-19A-21A_n257D DC_1A-19A-21A_n257E DC_1A-19A-21A_n257F | DC_1A_n257A DC_19A_n257A DC_21A_n257A DC_21A_n257D | CA_1A-19A-21A | n257A CA_n257D CA_n257E CA_n257F |
| --- | --- | --- | --- |

Logic in above table for 4 bands is that E-UTRA configuration is the same in all EN-DC configurations. This approach is used widely in many tables for FR1 and FR2. An extreme example from Table 5.5B.5.1 see below.

Table 5.5B.5.1-1: Inter-band EN-DC configurations including FR2 (two bands)

| DC_5A_n257A | DC_5A_n257A | 5A | n257A |
| --- | --- | --- | --- |
| DC_5A_n260A DC_5A_n260B DC_5A_n260C DC_5A_n260D DC_5A_n260E DC_5A_n260F DC_5A_n260G DC_5A_n260H DC_5A_n260I DC_5A_n260J DC_5A_n260K DC_5A_n260L DC_5A_n260M DC_5A_n260O DC_5A_n260P DC_5A_n260Q DC_5A_n260(2A) DC_5A_n260(3A) DC_5A_n260(4A) DC_5A-n260(A-I) DC_5A-n260(D-G) DC_5A-n260(D-H) DC_5A-n260(D-I) DC_5A-n260(D-O) DC_5A-n260(D-P) DC_5A-n260(D-Q) DC_5A-n260(E-O) DC_5A-n260(E-P) DC_5A-n260(E-Q) DC_5A-n260(G-I) | DC_5A_n260A | 5A | n260A CA_n260B CA_n260C CA_n260D CA_n260E CA_n260F CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M CA_n260O CA_n260P CA_n260Q CA_n260(2A) CA_n260(3A) CA_n260(4A) CA_n260(A-I) CA_n260(D-G) CA_n260(D-H) CA_n260(D-I) CA_n260(D-O) CA_n260(D-P) CA_n260(D-Q) CA_n260(E-O) CA_n260(E-P) CA_n260(E-Q) CA_n260(G-I) |

There exists inconsistency also within many tables. As an example, below when CA configuration is CA_3A-21A-42A then grouping is used but when it is CA_3A-21A-42C then not.

Table 5.5B.4.3-1: Inter-band EN-DC configurations within FR1 (four bands)

| DC_3A-21A-42A_n77A DC_3A-21A-42A_n77C | DC_3A_n77A DC_21A_n77A | CA_3A-21A-42A | n77A CA_n77A |
| --- | --- | --- | --- |
| DC_3A-21A-42A_n78A DC_3A-21A-42A_n78C | DC_3A_n78A DC_21A_n78A | CA_3A-21A-42A | n78A CA_n78A |
| DC_3A-21A-42A_n79A DC_3A-21A-42A_n79C | DC_3A_n79A DC_21A_n79A | CA_3A-21A-42A | n79A CA_n79A |
| DC_3A-21A-42C_n77A | DC_3A_n77A DC_21A_n77A | CA_3A-21A-42C | n77A |
| DC_3A-21A-42C_n78A | DC_3A_n78A DC_21A_n78A | CA_3A-21A-42C | n78A |
| DC_3A-21A-42C_n79A | DC_3A_n79A DC_21A_n79A | CA_3A-21A-42C | n79A |
| DC_3A-21A-42C_n77C | DC_3A_n77A DC_21A_n77A | CA_3A-21A-42C | CA_n77C |
| DC_3A-21A-42C_n78C | DC_3A_n78A DC_21A_n78A | CA_3A-21A-42C | CA_n78C |
| DC_3A-21A-42C_n79C | DC_3A_n79A DC_21A_n79A | CA_3A-21A-42C | CA_n79C |

In Table 5.5B.5.1 as above we see another approach how to group EN-DC configurations and in this method all EN-DC configurations that have same band combination (42+n257) are grouped, see below. Although looking the example in Table 4 perhaps it would not have been necessary to repeat E-UTRA configuration four times. These two cases are the only ones using this approach.

Table 5.5B.5.1-1: Inter-band EN-DC configurations including FR2 (two bands)

| DC_41A_n257A DC_41C_n257A | DC_41A_n257A | 41A CA_41C | n257A |
| --- | --- | --- | --- |
| DC_42A_n257A DC_42C_n257A DC_42A_n257D DC_42A_n257E DC_42A_n257F | DC_42A_n257A | 42A CA_42C 42A 42A 42A | n257A n257A CA_n257D CA_n257E CA_n257F |

From NR-DC clause we can find another approach for compression where number of rows is not compressed but uplink NR DC configurations are merged, see Table 5.5B.7-1.

Table 5.5B.7-1: Inter-band NR-DC configurations between FR1 and FR2 (two bands)

| Downlink NR DC configuration | Uplink NR DC configuration | NR configuration for FR1 | NR configuration for FR2 |
| --- | --- | --- | --- |
| DC_n77A-n257A | DC_n77A-n257A | n77A | n257A |
| DC_n77A-n257D |  | n77A | CA_n257D |
| DC_n77A-n257E |  | n77A | CA_n257E |
| DC_n77A-n257F |  | n77A | CA_n257F |
| DC_n77A-n257G |  | n77A | CA_n257G |
| DC_n77A-n257H |  | n77A | CA_n257H |
| DC_n77A-n257I |  | n77A | CA_n257I |
| DC_n77A-n257J |  | n77A | CA_n257J |
| DC_n77A-n257K |  | n77A | CA_n257K |
| DC_n77A-n257L |  | n77A | CA_n257L |
| DC_n77A-n257M |  | n77A | CA_n257M |
| DC_n77C-n257A |  | CA_n77C | n257A |
| DC_n77C-n257D |  | CA_n77C | CA_n257D |
| DC_n77C-n257E |  | CA_n77C | CA_n257E |
| DC_n77C-n257F |  | CA_n77C | CA_n257F |

As there inevitable will be massive amount of EN-DC configuration coming to RAN4 perhaps none of the methods used today is not good enough for keeping the spec manageable sized.  One method not yet used in 38,101-3 spec could be that grouping is based on combination of LTE CA + NR CA. Only highest LTE CA is captured into spec as fallbacks are mandatory and similarly only highest NR CA is captured into spec because also for NR fallbacks are mandatory. In case of NR attention is needed for FBG concept. Let’s call this highest fallback method.

As an example, DC_1A_n78C-n257F would include

- DC_1A_n78C-n257E

- DC_1A_n78C-n257D

- DC_1A_n78C-n257A

- DC_1A_n78A-n257E

- DC_1A_n78A-n257D

- DC_1A_n78A-n257A

At the moment all above are listed separately

| DC_1A_n78A-n257A | DC_1A_n78A DC_1A_n257A DC_1A_n78A-n257A | 1A | CA_n78A-n257A |
| --- | --- | --- | --- |
| DC_1A_n78A-n257D | DC_1A_n78A DC_1A-n257A DC_1A_n78A-n257A | 1A | CA_n78A-n257D |
| DC_1A_n78A-n257E | DC_1A_n78A DC_1A_n257A DC_1A_n78A-n257A | 1A | CA_n78A-n257E |
| DC_1A_n78A-n257F | DC_1A_n78A DC_1A_n257A DC_1A_n78A-n257A | 1A | CA_n78A-n257F |
| DC_1A_n78C-n257A | DC_1A_n78A DC_1A_n257A DC_1A_n78A-n257A | 1A | CA_n78C-n257A |
| DC_1A_n78C-n257D | DC_1A_n78A DC_1A_n257A DC_1A_n78A-n257A | 1A | CA_n78C-n257D |
| DC_1A_n78C-n257E | DC_1A_n78A DC_1A_n257A DC_1A_n78A-n257A | 1A | CA_n78C-n257E |
| DC_1A_n78C-n257F | DC_1A_n78A DC_1A_n257A DC_1A_n78A-n257A | 1A | CA_n78C-n257F |

## 2.2	How to move forward

In our view this is a major problem that needs to be resolved. RAN4 needs to agree common way how to list EN-DC and NR-DC configuration in clause 5.5B. Currently the same E-UTRA CA based grouping is by far the most common way to group the EN-DC configurations out of the three used and presented in this paper. Next we look for pros and cons of these three methods

Grouping based on common E-UTRA CA

- Widely used already both for FR1 and FR2

- Reduces separate rows in tables quite much especially in FR2

Grouping based on common band combination

- Used only in two occasions

- Reduces separate rows even more that same E-UTRA CA approach

Grouping based common uplink configuration

- Used only for NR-DC

- Least amount of information compression from these three at least in current format

- Could be further optimized by merging E-UTRA CA

Grouping based highest fallback method

- Not in use at the moment

- Biggest compression of the listed

- leads to re-grouping of some common E-UTRA CA based cases like DC_5A_n260X

- When a new configuration is introduced which has higher BW class than the one in spec then an update would be needed

- Fallback rules needs to crystal clear in spec (should be anyways)

We do not have clear preference at the moment and welcome discussion on this topic. RAN4 needs to find a solution before REL-16 specs are created.

One simplification that RAN4 could consider at this point is the removal or 3rd and 4th columns which are for EN-DC E-UTRA configuration and NR configuration and for NR DC NR configuration for FR1 and NR configuration for FR2. This action would net make tables shorter but reduces the maintenance effort as well as creation of TPs effort. Information in those columns can be deducted from first column.

Proposal; RAN4 needs to agree rules how the EN-DC and NR-DC configurations are grouped in clause 5.5B of 38.101-3 and agree a REL-15 CR before REL-16 specs are implemented.

# 3	Conclusion

In this contribution we have discussed the fact that at the moment there is not general way how the EN-DC and NR-DC configurations are grouped in clause 5.5B of 38.101-3. This needs to be re-solved.

Proposal; RAN4 needs to agree rules how the EN-DC and NR-DC configurations are grouped in clause 5.5B of 38.101-3 and agree a REL-15 CR before REL-16 specs are implemented.