3GPP TSG-RAN4 Meeting #90	R4-1900474

# Athens, Greece, 25th Feb – 1st Mar, 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on TCI state switching delay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2019-2-15 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The delay requirements for TCI state switching shall be provided. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the TCI state switching delay requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The TCI state switching delay requirements are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 8.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 38.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of Change >>

8.7	Active TCI state switching delay

8.7.1	Introduction

The requirements in this section apply for a UE configured with one or more TCI state configurations on serving cell in MR-DC or standalone NR. UE shall complete the switch of active TCI state within the delay defined in this section.

8.7.2	RRC based TCI state switch delay

For RRC-based TCI state switch, after the UE receives TCI state switching request, UE shall be able to receive PDSCH  or transmit PUSCH  on the new beam of the serving cell on which TCI state switch occurs no later than at slot n+ TRRCprocessingDelay, where slot n is the last slot containing the RRC command, and TRRCprocessingDelay is the length of the RRC procedure delay in slots defined in clause 12 in TS 38.331 [2].

8.7.3	MAC-CE based TCI state switch delay

For MAC-CE-based TCI state switch, after the UE receives TCI state switching request, UE shall be able to receive PDSCH  or transmit PUSCH  on the new beam of the serving cell on which TCI state switch occurs no later than at slot n+3ms, where slot n is the last slot containing the MAC-CE command.

8.7.4	DCI based TCI switch delay

For DCI-based TCI state switch, the UE can be configured with a list of up to M TCI-State configurations within the higher layer parameter PDSCH-Config to decode PDSCH according to a detected PDCCH with DCI intended for the UE and the given serving cell, where M depends on the UE capability. The UE receives an activation command used to map up to 8 TCI states to the codepoints of the DCI field 'Transmission Configuration Indication'. If the target TCI state is one of the activated TCI state configuration,  UE shall be able to receive PDSCH or transmit PUSCH  on the new beam of the serving cell on which TCI state switch occurs no later than at slot n+0.5ms, where slot n is the last slot containing the DCI command. Otherwise, UE shall be able to receive PDSCH or transmit PUSCH  on the new beam of the serving cell on which TCI state switch occurs no later than at slot n+3.5ms, where slot n is the last slot containing the DCI command.

<< End of Change >>