3GPP TSG-RAN WG4 Meeting #90	R4-1900787

# Athens, GR, 25th  Feb – 1st March, 2019

Agenda item:		6.12.8.1.2

Source:		MediaTek Inc.

Title:		Discussion on requirements for candidate beam detection

Document for:	Discussion

- Introduction

In this the paper, our view on the N factor and scheduling restriction for CSI-RS based CBD in FR2 is provided.

- N=1 conditions and scheduling restrictions for CSI-RS based CBD in FR2

In the last meeting, for SSB based CBD, it has been agreed that N=1 cannot be applied for SSB based CBD.  One remaining issue is that whether N=1 can apply for CSI-RS based CBD. However, it has made some conclusions on the N=1 condition for CSI-RS based BFD and has been endorsed in 38.133 [1]. In general, the conclusion can be applied to the CSI-RS based CBD.

Observation 1: RX beam can be fixed for CSI-RS based CBD if the QCL source is given.

Proposal 1: For CSI-RS based CBD N=1 can apply, and the N=1 condition of CSI-RS BFD is reused.

- Conclusion

In the contribution, we discuss the requirement for candidate beam detection. We have the following observations and proposals:

Observation 1: RX beam can be fixed for CSI-RS based CBD if the QCL source is given.

Proposal 1: For CSI-RS based CBD N=1 can apply, and the N=1 condition of CSI-RS BFD is reused.

- Reference

[1] TS38.133, NR; Requirements for support of radio resource management, V15.4.0.