3GPP TSG-RAN4 Meeting #90	R4-1900819

# Athens, EU, 25th February – 1st March 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR | 0009 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TR 37.843: Addition of RC MU evaluation and description in clause 2 and sub-clause 10.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  | Date: |  |  | 2019-02-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR add references in clause 2 and add subclause 10.5.2.3A for RC technical description and measurement uncertinaty evaluation. In this CR the MU table for spurious emission is added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Addition of RC references in subclause 2 - A new subclause is created 10.5.2.3A following the structure used for other test methods. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not approved, the TR will not capture essential background informantion for the RC test method. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 2, Sub-clause 10.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications"

[2]	RP-170844: "Work Item Description for Enhancements of Base Station (BS) RF and EMC requirements for Active Antenna System (AAS)"

[3]	3GPP TS 37.105: "Active Antenna System (AAS) Base Station (BS) transmission and reception"

[4]	3GPP TR 37.842: "Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS)"

[5]	3GPP TR 36.942: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Frequency (RF) system scenarios"

[6]	3GPP TR 37.840: "Study of Radio Frequency (RF) and Electromagnetic Compatibility (EMC) requirements for Active Antenna Array System (AAS) base station"

[7]	3GPP TS 37.104: "E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) radio transmission and reception"

[8]	3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception"

[9]	Recommendation ITU-R SM.328-11: "Spectra and bandwidth of emissions", ITU/R

[10]	3GPP TR 25.951: "FDD Base Station (BS) classification"

[11]	3GPP TR 37.809: "Evolved Universal Terrestrial Radio Access (E-UTRA) medium range and Multi-Standard Radio (MSR) medium range / local area Base Station (BS) class requirements"

[12]	3GPP TR 36.931: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Frequency (RF) requirements for LTE Pico Node B"

[13]	3GPP TS 37.114: "Active Antenna System (AAS) Base Station (BS) Electromagnetic Compatibility (EMC)"

[14]	3GPP TS 37.113: "E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) Electromagnetic Compatibility (EMC)"

[15]	3GPP TS 36.113: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) and repeater ElectroMagnetic Compatibility (EMC)"

[16]	3GPP TS 25.113: "Base Station (BS) and repeater ElectroMagnetic Compatibility (EMC)"

[17]	Conversion of power to field strength in the book "Automotive Antenna Design and Applications", by Victor Rabinovich, Nikolai Alexandrov, Basim Alkhateeb; 2010

[18]	Recommendation ITU-R SM.329-10: "Unwanted emissions in the spurious domain"

[19]	3GPP TS 25.104: "Universal Terrestrial Radio Access (UTRA); Base Station (BS) radio transmission and reception (FDD)"

[20]	3GPP TS 25.105: "Universal Terrestrial Radio Access (UTRA); Base Station (BS) radio transmission and reception (TDD)"

[21]	3GPP TS 25.141: "Base Station (BS) conformance testing (FDD)"

[22]	3GPP TS 36.141: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) conformance testing"

[23]	3GPP TS 37.145-1: "Active Antenna System (AAS) Base Station (BS) conformance testing; Part 1: Conducted conformance testing"

[24]	3GPP TS 37.145-2: "Active Antenna System (AAS) Base Station (BS) conformance testing; Part 2: Radiated conformance testing"

[25]	IEEE Std 149: "IEEE Standard Test Procedures for Antennas", IEEE

[26]	TSGR4#8(99)631: "Antenna-to-Antenna Isolation Measurements", Allgon

[27]	Report ITU-R M.2244: "Isolation between antennas of IMT base stations in the land mobile service", ITU-R

[28]	IEC 61000-4-3: 2006+AMD1:2007+AMD2:2010: “Electromagnetic compatibility (EMC) - Part 4-3: Testing and measurement techniques - Radiated, radio-frequency, electromagnetic field immunity test”

[29]	Hald, J.; Hansen, J. E.; Jensen, F. & Holm Larsen, F. Hansen, J. (Ed.) Spherical Near-Field Antenna Measurements, Peter Peregrinus Ltd., 1998, vol 26.

[30]	"Sparse Sampling Analysis Tool" Matlab code: https://se.mathworks.com/matlabcentral/fileexchange/67143-sparse-sampling-analysis-tool?s_tid=srchtitle

[31]	J. Fridén, A. Razavi, and A. Stjernman, “Angular sampling, Test Signal, and Near-Field Aspects for Over-the-Air Total Radiated Power Assessment in Anechoic Chambers”, IEEE Access, 2018, https://ieeexplore.ieee.org/stamp/stamp.jsp?arnumber=8470084.

[32]	IEEE Std 145: “IEEE Standard Definitions of Terms for Antennas”, IEEE

[33]	IEC 61000-4-21. “Electromagnetic compatibility (EMC) - Part 4-21: Testing and measurement techniques - Reverberation chamber test methods” Edition 2.0 2011-01, The International Electrotechnical Commision (IEC), 2011

[34]	Hill, D.A., “Boundary fields in reverberation chambers,” IEEE Transactions on Electromagnetic Compatibility, vol. 47, no. 2, pp. 281-290, May 2005

[35]	Krauthäuser, H. G.; Winzerling, T.; J., N.; Eulig, N. & Enders, A. “Statistical interpretation of autocorrelation coefficients for fields in mode-stirred chambers” 2005 International Symposium on Electromagnetic Compatibility, 2005. EMC 2005., 2005, 2, 550-555

## 10.5	Measurement uncertainty for Out-of-band TRP requirements

### 10.5.1	General

The TRP MU consists of an MU per point and a Summation Error (SE) which allows for errors in the calculation of the TRP from multiple directional power measurements and allows for a sparse grid to be used to reduce measurement time. The total MU is calculated as follows:

Refer to subclause 10.8 for the SE value.

### 10.5.2	Transmitter mandatory spurious emissions

#### 10.5.2.1	General

The conducted spurious emission requirement MU is split up into a number of frequency ranges

| Transmitter spurious emissions, Mandatory Requirements | dB | MU |
| --- | --- | --- |
| 30 MHz ≤ f ≤ 4 GHz |  | 2.0 |
| 4 GHz < f ≤ 19 GHz |  | 4.0 |

The conducted analysis based on UTRA/E-UTRA frequencies which were all below 4.2GHz (at the time), the break point in the MU is hence somewhat related to the in-band and out of band MU analysis. As in-band MU analysis is now being done up to 6GHz (for the LAA and NR bands) it is sensible to change the frequency break point to 6GHz.

The conducted and OTA test set up can be compared as follows:

Figure 10.5.2.1-1:	Conducted and OTA spurious emissions test set up

As the spurious emissions requirements cover a  large frequency span from 30MHz to 12.75GHz it is difficult to use the same measurement chambers as the in band measurements. The CATR for example is not suitable for low frequency measurements.

The chamber analysis is hence done on a general OTA chamber which is capable of all frequency ranges.

Other chambers may be used if their MU is within the final value and they are suitable for the frequencies being tested.

#### 10.5.2.2	General chamber

##### 10.5.2.2.1	General

As the AAS BS antenna radiating dimensions are fixed then the far field distance increases (FF≈2d2/λ). At 12.75 GHz the far field distance for a 1.5m AAS BS antenna array is almost 200m, this is clearly impractical in an indoor chamber (and the path loss would also make measurement difficult), so spurious emission testing will not always be in the far field. This is acceptable as the requirement is TRP and hence it is not necessary to measure in the far field however it needs to be considered when looking at MU.

Considerations of the large frequency range must also be considered, including the chamber performance (quiet zone), the calibration effectiveness and the available reference and test antennas over the frequency range.

##### 10.5.2.2.2	Calibration

Each frequency point can be calibrated using the EIRP calibration approach in sub-clause 10.2.3.2.2 .

The calibration should be repeated for each frequency being tested and each test antenna.

##### 10.5.2.2.3	Procedure

1)	Place the AAS BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (see table 4.10-1, D9.2) of the AAS BS with the test system.

3)	Measurements shall use a measurement bandwidth in accordance to the conditions in subclause 6.7.6.

4)	The measurement device characteristics shall be:

-	Detection mode: True RMS.

5)	Set the AAS BS to transmit according to the applicable test configuration

6)	Align the BS and the test antenna such that measurements to determine TRP can be performed (see subclause 10.8.3)

7)	Measure the emission at the specified frequencies with specified measurement bandwidth

8)	Repeat step 6-7 for all directions in the appropriated TRP measurement grid needed for full TRP estimation (see subclause 10.8.3) and for frequency points to be tested.

Note 1: the TRP measurement grid may not be the same for all measurement frequencies.

Note 2: the frequency sweep or the TRP measurement grid sweep may be done in any order

9)	Calculate TRP at each specified frequency using the directional measurements.

##### 10.5.2.2.4	MU assessment

###### 10.5.2.2.4.1	MU Budget

Table 10.5.2.2.4.1-1: General  Chamber uncertainty contributions
Tx spurious emissions

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | E4-1 |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna | E4-2 |
| 3 | Quality of quiet zone | E4-3 |
| 4 | Polarization mismatch between the AAS BS and the receiving antenna | E4-4 |
| 5 | Mutual coupling between the AAS BS and the receiving antenna | E4-5 |
| 6 | Phase curvature | E4-6 |
| 7 | Conducted measurement uncertainty (minus mismatch) | E4-22, F.2 |
| 8 | Impedance mismatch in the receiving chain | E4-8 |
| 9 | Random uncertainty | E4-9 |
| 23 | Measurement antenna frequency variation | E4-20 |
| 24 | FSPL estimation error | E4-21 |
| 25 | Test system frequency flatness | E4-19 |
| Stage 1: Calibration measurement |  |  |
| 10 | Impedance mismatch between the receiving antenna and the network analyzer | E4-10 |
| 11 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | E4-11 |
| 12 | Impedance mismatch between the reference antenna and the network analyzer. | E4-12 |
| 13 | Quality of quiet zone | E4-3 |
| 14 | Polarization mismatch for reference antenna | E4-4 |
| 15 | Mutual coupling between the reference antenna and the receiving antenna | E4-5 |
| 16 | Phase curvature | E4-6 |
| 17 | Uncertainty of the network analyzer | E4-13, F.1 |
| 18 | Influence of the reference antenna feed cable | E4-14 |
| 19 | Reference antenna feed cable loss measurement uncertainty | E4-15 |
| 20 | Influence of the receiving antenna feed cable | E4-16 |
| 21 | Uncertainty of the absolute gain of the reference antenna | E4-17 |
| 22 | Uncertainty of the absolute gain of the receiving antenna | E4-18 |

###### 10.5.2.2.4.2	MU Value

Table 10.5.2.2.4.2-1: General  Chamber uncertainty assessment
Tx spurious emissions

| UID | Uncertainty source | Uncertainty value |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 30MHz<f≤6 GHz | 6GHz<f ≤19GHz |  |  |  | 30MHz<f≤6 GHz | 6GHz<f ≤19GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | 0.03 | 0.03 | Rectangular | √3 | 1 | 0.02 | 0.02 |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna | 0 | 0 | Rectangular | √3 | 1 | 0 | 0 |
| 3 | Quality of quiet zone | 0.1 | 0.1 | Gaussian | 1 | 1 | 0.1 | 0.1 |
| 4 | Polarization mismatch between the AAS BS and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 5 | Mutual coupling between the AAS BS and the receiving antenna | 0 | 0 | Rectangular | √3 | 1 | 0 | 0 |
| 6 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 | 1 | 0.05 | 0.05 |
| 7 | Conducted measurement uncertainty (minus mismatch | 1.02 | 2.04 | Gaussian | 1 | 1 | 1.02 | 2.04 |
| 8 | Impedance mismatch in the receiving chain | 0.2 | 0.45 | U-shaped | √2 | 1 | 0.14 | 0.32 |
| 9 | Random uncertainty | 0.1 | 0.1 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| 23 | Measurement antenna frequency variation | 0.1 | 0.1 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| 24 | FSPL estimation error | 0 | 0 | Gaussian | 1 | 1 | 0 | 0 |
| 25 | Test system frequency flatness | 0.25 | 0.25 | Gaussian | 1 | 1 | 0.25 | 0.25 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 10 | Impedance mismatch between the receiving antenna and the network analyzer | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 11 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 12 | Impedance mismatch between the reference antenna and the network analyzer. | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 13 | Quality of quiet zone | 0.1 | 0.1 | Gaussian | 1 | 1 | 0.1 | 0.1 |
| 14 | Polarization mismatch for reference antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 15 | Mutual coupling between the reference antenna and the receiving antenna | 0 | 0 | Rectangular | √3 | 1 | 0 | 0 |
| 16 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 | 1 | 0.05 | 0.05 |
| 17 | Uncertainty of the network analyzer | 0.13 | 0.2 | Gaussian | 1 | 1 | 0.13 | 0.2 |
| 18 | Influence of the reference antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 19 | Reference antenna feed cable loss measurement uncertainty | 0.06 | 0.06 | Gaussian | 1 | 1 | 0.06 | 0.06 |
| 20 | Influence of the receiving antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 21 | Uncertainty of the absolute gain of the reference antenna | 0.5 | 0.5 | Rectangular | √3 | 1 | 0.29 | 0.29 |
| 22 | Uncertainty of the absolute gain of the receiving antenna | 0 | 0 | Rectangular | √3 | 1 | 0 | 0 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 1.12 | 2.11 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 2.20 | 4.15 |

#### 10.5.2.3	Void

#### 10.5.2.3A	Reverberation chamber

##### 10.5.2.3A.1	General

A reverberation chamber (RC) is an electrically large shielded metal enclosure that employs one or several “stirring” methods to randomize the fields, such as moving paddles, turntables, etc. In this way, a large number of uncorrelated samples is obtained. The volume in the room where the field is well-stirred is the working volume. Here the E-field, averaged over an entire stirring cycle, is independent of the location in the room, i.e. the field is spatially uniform.

For a proper analysis of the measured data a sufficient number of uncorrelated samples is required. The auto-correlation function is used to calculate the offset between statistically uncorrelated samples using the following expression [33]:

rk=1σ2N-1n=0N-1xn- x xmodn+k,N- x

where the modulus operator mod(x,y) is the remainder of x/y, here performing  a circular shift of the measurement samples over a distance k. The symbols ⟨x ⟩ and σ= "std" (x) denote the average value and standard deviation. The threshold value for uncorrelated samples is defined as [33]:

rlim≤0.371-7.22/N0.64

The distance klimbetween uncorrelated samples is calculated as the minimum k-value satisfying rk≤rlim. The number of uncorrelated samples is calculated as:

Nind=Nklim

When properly designed, this facility can be used for non-directional antenna measurements, such as TRP. When measuring TRP of sources with a directive pattern, special care must be taken to characterize the working volume of the chamber, see subclause 10.5.2.3A.2.

##### 10.5.2.3A.2	Chamber characterization

The purpose of the chamber characterization is to ensure that the effect of a non-uniform field distribution in the chamber has a negligible influence on the measurement result when the EUT is placed in the working volume. Lack of chamber uniformity is a major contributor to measurement uncertainty in reverberation chambers and should be handled with care.

The uniformity test can be quite time consuming and the test can be performed separate from the EUT measurement. Due to the non-negligible size of BS equipment the EUT can have a significant influence on the uniformity. To take this effect into account, an absorber with dimensions equal or larger than the EUT must be placed at the EUT’s location in the room.

The characterization procedure consists of placing a reference transmitter antenna (REF TX ant) at different locations and with different orientations in the room. The actual mode of RC operation shall be used, including stirrer movement, EUT movement, diversity antenna usage, etc. The directivity of the REF TX antenna will influence the spatial uniformity of the room. Therefore, the directivity of the reference antenna shall be at least as high as the directivity of the EUT emissions to be measured and a directivity of [10 dBi] can be assumed for spurious emissions.

Figure 10.5.2.3A.2-1:	Setup for characterization of a reverberation chamber

The working volume shall be at least half a wavelength from the chamber walls and other electromagnetic reflective objects according to [34]. According to [33] this distance may be restricted to 0.75 m below 100 MHz. The number of positions and orientations to use depends on the chamber size and the directivity of the REF TX. Measurements made at positions and orientations at the edges ofthe working volume are used to characterize the chamber and derive certain components of uncertainty.

The exact number of positions and orientations remain for further study, but at least (3) uncorrelated locations should be used and (6) uncorrelated orientations per position when directive spurs are to be detected.

The REF TX antenna and the measurement receive antenna (RX) are connected to a Vector Network Analyzer (VNA).  For each location/orientation n of the REF TX, RC sample, and desired frequency f, the power transfer function

Pn=<S21>2

is measured. The explicit dependence on RC sample and frequency is not written out here. At least 250 uncorrelated samples shall be used per position/orientation. Using a lower number is not compatible with the underlying analysis on measurement uncertainty, see [33, 35].

The following tests are performed and shall be verified for each frequency:

- Uniformity of transfer function: For each location/orientation evaluate Pn. The standard deviation of these average values shall be below (1.5 dB).

- Dynamic range: The dynamic range of each Pn shall be at least 20 dB. The method to compute it is defined below.

- Uncorrelated samples: At least 250 uncorrelated samples shall be used.

It is important to note that spatial uniformity (and number of uncorrelated samples) in an RC is harder to achieve for more directive antenna patterns. As such, the measurement uncertainty of a sub-optimally configured room will be higher and additional measures to randomize the fields should be considered.

##### 10.5.2.3A.3	Calibration

To correctly determine the TRP by the EUT a calibration step to account for losses in cables, antennas, etc. is required. A reference PTF is determined at each frequency of interest by using a relevant reference TX antenna (REF TX). The chamber should be set up identically to the actual set up of the TRP measurement, which means the EUT has to be in the room, the same stirring procedure has to be used, etc. T

A minimum Nref = 1 reference measurement is required, but it is noted that more measurements can be used to estimate the reference PTF and that this will lead to a reduction of the uncertainty by a factor 1/Nref. The radiation efficiency, η, of the reference antenna can be assumed to be the value declared by the manufacturer or can be determined in a separate antenna efficiency measurement.

Calibration procedure

- Place the EUT and the REF TX in the working volume of the RC. Turn off EUT power and EUT controls. See Figure 10.5.2.3A.4-1 (a). If more positions are used to determine the reference PTF the REF TX should be placed at uncorrelated positions within the test volume.

- Set the stirrers and turntables in the mode of operation used in the chamber characterization.

- Set the sampling rate as in the chamber characterization.

- Connect the REF TX and the RX antenna with a calibrated Network analyser (NA)

- Measure the scattering parameters over a complete stirring cycle for each frequency of interest and for each position and orientation.

- Calculate the reference transfer function Pr=S21r2

- Calculate the mis-match efficiency 	M1r=1-S11r2.

##### 10.5.2.3A.4	Procedure

- (b)											(c)

Figure 10.5.2.3A.4-1:	The procedure for TRP measurements uses three consecutive setups: (a) Reference measurement, (b) ambient noise measurement (b), and (c) TRP measurement. The blue dots indicate the measurement planes.

Calculate Ambient Power Level

- Connect the RX antenna to a calibrated Spectrum analyser (SA) using the same cables as in the calibration step. Turn on the EUT control. Keep the EUT RF power off. Terminate the REF TX in a 50Ω load. See Figure 10.5.2.3A.4-1 (b).

- Measure the voltage data Uamb and calculate the ambient power level as Pamb=Uamb250

- Calculate the ambient TRP level as TRPamb=ηM1rPrPamb

Calculate EUT TRP

- Turn on the EUT RF power and measure the received voltage UEUT, see Figure 10.5.2.3A.4-1 (c).

- Calculate the EUT power as PEUT=UEUT250

- Calculate the TRP from the EUT as TRPEUT=η M1rPrPEUT.

- The following tests shall be performed on the measurement data:

  - The dynamic range TRPEUTTRPamb  must be at least 20 dB

  - The number of uncorrelated samples, calculated via the auto-correlation function, see [35], shall be at least 250.

Note: A reverberation chamber does not represent a real-life deployment for the EUT and can be considered as a hostile environment due to the potentially high field strengths. Therefore, it is important to make sure that the EUT is operating properly throughout the entire measurement.

##### 10.5.2.3A.5	MU assessment

###### 10.5.2.3A.5.1	MU Budget

Table 10.5.2.2.4.1-1: Reverberation Chamber uncertainty contributions Tx spurious emissions

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Uncertainty of the measurement equipment | E5-1 |
| 2 | Impedance mismatch in the receiving chain | E5-2 |
| 3 | Random uncertainty | E5-3 |
| Stage 1: Calibration measurement |  |  |
| 4 | Reference antenna radiation efficiency | E5-4 |
| 5 | Mean value estimation of reference antenna mis-match efficiency | E5-5 |
| 6 | Uncertainty of the Network Analyzer | E5-7 |
| 7 | Influence of the reference antenna feed cable | E5-8 |
| 8 | Mean value estimation of transfer function | E5-9 |
| 9 | Uniformity of transfer function | E5-10 |

###### 10.5.2.3A.5.2	MU Value

Table 10.5.2.2.4.2-1: Reverberation Chamber uncertainty assessment Tx spurious emissions

| UID | Uncertainty source | Uncertainty value 380 MHz < f ≦ 3 GHz | Uncertainty value 3 GHz < f ≦ 3.8 GHz | Uncertainty value 3.8 GHz < f ≦ 12.75 GHz | Uncertainty value 12.75 GHz < f ≦ 19 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] 380 MHz < f ≦ 3 GHz | Standard uncertainty ui [dB] 3 GHz < f ≦ 3.8 GHz | Standard uncertainty ui [dB] 3.8 GHz < f ≦ 12.75 GHz | Standard uncertainty ui [dB] 12.75 GHz < f ≦ 19 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Uncertainty of the measurement equipment | 0.14 | 0.26 | 0.26 | 0.37 | Gaussian | 1 | 1 | 0.14 | 0.26 | 0.26 | 0.37 |
| 2 | Impedance mismatch in the receiving chain | 0.2 | 0.2 | 0.45 | 0.45 | U-shaped | √2 | 1 | 0.14 | 0.14 | 0.32 | 0.32 |
| 3 | Random uncertainty | 0.1 | 0.1 | 0.1 | 0.1 | Rectangular | √3 | 1 | 0.06 | 0.06 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | Reference antenna radiation efficiency | 0.50 | 0.50 | 0.50 | 0.50 | Gaussian | 1 | 1 | 0.50 | 0.50 | 0.50 | 0.50 |
| 5 | Mean value estimation of reference antenna radiation efficiency | 0.27 | 0.27 | 0.27 | 0.27 | Gaussian | 1 | 1 | 0.27 | 0.27 | 0.27 | 0.27 |
| 6 | Uncertainty of the Network Analyzer | 0.13 | 0.20 | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.13 | 0.20 | 0.20 | 0.20 |
| 7 | Influence of the reference antenna feed cable | 0.20 | 0.20 | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 | 0.20 | 0.20 |
| 8 | Mean value estimation of transfer function | 0.27 | 0.27 | 0.27 | 0.27 | Gaussian | 1 | 1 | 0.27 | 0.27 | 0.27 | 0.27 |
| 9 | Uniformity of transfer function | 1.5 | 1.5 | 1.5 | 1.5 | Gaussian | 1 | 1 | 1.5 | 1.5 | 1.5 | 1.5 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  |  |  | 1.66 | 1.68 | 1.70 | 1.72 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  |  |  | 3.25 | 3.29 | 3.33 | 3.37 |
| Note: This MU budget is applicable if the data tests in procedure step 11) of Clause 10.5.2.3A.4 are fulfilled. |  |  |  |  |  |  |  |  |  |  |  |  |

#### 10.5.2.4	Test Tolerance

The conduced test tolerance for the mandatory spurious emissions requirements is zero. As the requirements are set by regulatory limits the same test tolerance is used for OTA.

TT=0.

#### 10.5.2.5	Summary

For general chamber the total MU is calculated by adding the MUperpoint and the SE (see subclause 10.8) we have:

,	30MHz<f≤6 GHz

,	6GHz<f≤19 GHz

By adding the MU per point and the SE values together we have the following total MU:

MUtotal = 2.3dB, 	30MHz<f≤6GHz

MUtotal = 4.2dB, 	6GHz<f≤19GHz

The test tolerance for mandatory spurious emissions is zero.

### 10.5.3	Receiver spurious emissions

#### 10.5.3.1	General

The conducted receiver spurious emission requirement MU is the same as for the TX spurious emissions, the measurement technique is the same and the power level is >-60dBm (where there is a break point for conducted power measurement accuracy), so this is reasonable.

For the OTA receiver emissions requirements however the lower power level of the requirement reduces the dynamic range of the TRP measurement and reduces measurement accuracy.

Considering that the loss in the chamber is based on the wanted signal being in the far field the per point noise floor is assumed to be approx -100dBm and the receiver emissions level translated to the test equipment is approx -90dBm. Hence the TRP calculation has only a 10dB dynamic range.

An uncertainty of 1dB is added to the TRP uncertainty budget to account for this additional uncertainty.

#### 10.5.3.2	General chamber

##### 10.5.3.2.1	General

As the AAS BS antenna radiating dimensions are fixed then the far field distance increases (FF≈2d2/λ). At 12.75 GHz the far field distance for a 1.5m AAS BS antenna array is almost 200m, this is clearly impractical in an indoor chamber (and the path loss would also make measurement difficult), so spurious emission testing will not always be in the far field. This is acceptable as the requirement is TRP and hence it is not necessary to measure in the far field however it needs to be considered when looking at MU.

Considerations of the large frequency range must also be considered, including the chamber performance (quiet zone), the calibration effectiveness and the available reference and test antennas over the frequency range.

##### 10.5.3.2.2	Calibration

The calibration is the same as described in sub-clause 10.5.2.2.2.

##### 10.5.3.2.3	Procedure

The measurement procedure is the same as described in sub-clause 10.5.2.2.3.

##### 10.5.3.2.4	MU assessment

###### 10.5.3.2.4.1	MU Budget

Table 10.5.3.2.4.1-1: General  Chamber uncertainty contributions
Rx spurious emissions

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | E4-1 |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna | E4-2 |
| 3 | Quality of quiet zone | E4-3 |
| 4 | Polarization mismatch between the AAS BS and the receiving antenna | E4-4 |
| 5 | Mutual coupling between the AAS BS and the receiving antenna | E4-5 |
| 6 | Phase curvature | E4-6 |
| 7 | Conducted measurement uncertainty | E4-22, F.2 |
| 8 | Impedance mismatch in the receiving chain | E4-8 |
| 9 | Random uncertainty | E4-9 |
| 23 | Measurement antenna frequency variation | E4-20 |
| 24 | FSPL estimation error | E4-21 |
| 25 | Test system frequency flatness | E4-19 |
| 26 | Measurement system dynamic range uncertainty | E4-21 |
| Stage 1: Calibration measurement |  |  |
| 10 | Impedance mismatch between the receiving antenna and the network analyzer |  |
| 11 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | E4-10 |
| 12 | Impedance mismatch between the reference antenna and the network analyzer. | E4-11 |
| 13 | Quality of quiet zone | E4-12 |
| 14 | Polarization mismatch for reference antenna | E4-3 |
| 15 | Mutual coupling between the reference antenna and the receiving antenna | E4-4 |
| 16 | Phase curvature | E4-5 |
| 17 | Uncertainty of the network analyzer | E4-6 |
| 18 | Influence of the reference antenna feed cable | E4-13, F.1 |
| 19 | Reference antenna feed cable loss measurement uncertainty | E4-14 |
| 20 | Influence of the receiving antenna feed cable | E4-15 |
| 21 | Uncertainty of the absolute gain of the reference antenna | E4-16 |
| 22 | Uncertainty of the absolute gain of the receiving antenna | E4-17 |

###### 10.5.3.2.4.2	MU Value

Table 10.5.3.2.4.2-1: General Chamber uncertainty assessment
Rx spurious emissions

| UID | Uncertainty source | Uncertainty value |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 30MHz<f≤6 GHz | 6GHz<f ≤19GHz |  |  |  | 30MHz<f≤6 GHz | 6GHz<f ≤19GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna | 0.03 | 0.03 | Rectangular | √3 | 1 | 0.02 | 0.02 |
| 2 | Pointing misalignment between the AAS BS and the receiving antenna | 0 | 0 | Rectangular | √3 | 1 | 0 | 0 |
| 3 | Quality of quiet zone | 0.1 | 0.1 | Gaussian | 1 | 1 | 0.1 | 0.1 |
| 4 | Polarization mismatch between the AAS BS and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 5 | Mutual coupling between the AAS BS and the receiving antenna | 0 | 0 | Rectangular | √3 | 1 | 0 | 0 |
| 6 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 | 1 | 0.05 | 0.05 |
| 7 | Conducted measurement uncertainty (minus mismatch) | 1 | 1.99 | Gaussian | 1 | 1 | 1 | 1.99 |
| 8 | Impedance mismatch in the receiving chain | 0.2 | 0.45 | U-shaped | √2 | 1 | 0.14 | 0.32 |
| 9 | Random uncertainty | 0.1 | 0.1 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| 23 | Measurement antenna frequency variation | 0.1 | 0.1 | Rectangular | √3 | 1 | 0.06 | 0.06 |
| 24 | FSPL estimation error | 0 | 0 | Gaussian | 1 | 1 | 0 | 0 |
| 25 | Test system frequency flatness | 0.25 | 0.25 | Gaussian | 1 | 1 | 0.25 | 0.25 |
| 26 | Measurement system dynamic range uncertainty | 0.51 | 0.51 | Gaussian | 1 | 1 | 0.51 | 0.51 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |
| 10 | Impedance mismatch between the receiving antenna and the network analyzer | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 11 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 12 | Impedance mismatch between the reference antenna and the network analyzer. | 0.05 | 0.05 | U-shaped | √2 | 1 | 0.04 | 0.04 |
| 13 | Quality of quiet zone | 0.1 | 0.1 | Gaussian | 1 | 1 | 0.1 | 0.1 |
| 14 | Polarization mismatch for reference antenna | 0.01 | 0.01 | Rectangular | √3 | 1 | 0.01 | 0.01 |
| 15 | Mutual coupling between the reference antenna and the receiving antenna | 0 | 0 | Rectangular | √3 | 1 | 0 | 0 |
| 16 | Phase curvature | 0.05 | 0.05 | Gaussian | 1 | 1 | 0.05 | 0.05 |
| 17 | Uncertainty of the network analyzer | 0.13 | 0.2 | Gaussian | 1 | 1 | 0.13 | 0.2 |
| 18 | Influence of the reference antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 19 | Reference antenna feed cable loss measurement uncertainty | 0.06 | 0.06 | Gaussian | 1 | 1 | 0.06 | 0.06 |
| 20 | Influence of the receiving antenna feed cable | 0.05 | 0.05 | Rectangular | √3 | 1 | 0.03 | 0.03 |
| 21 | Uncertainty of the absolute gain of the reference antenna | 0.5 | 0.5 | Rectangular | √3 | 1 | 0.29 | 0.29 |
| 22 | Uncertainty of the absolute gain of the receiving antenna | 0 | 0 | Rectangular | √3 | 1 | 0 | 0 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 1.22 | 2.13 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 2.39 | 4.18 |

#### 10.5.3.3	Void

#### 10.5.3.4	Test Tolerance

The conduced test tolerance for the receiver spurious emissions requirements is zero. However for OTA AAS BS the receiver spurious emissions requirements only apply to TTD in OFF mode. As such the limit is set by RAN4 to be considerably lower than the equivalent regulatory requirement.

In addition due to the difficulty in measuring low levels of TRP close to the measurement system noise floor the risk of false failures is high. As the risk is due to the noise floor of the measurement system it cannot be mitigated by BS design

Hence it has been agreed that for receiver spurious emissions the TT=MU.

#### 10.5.3.5	Summary

Adding the MUperpoint and the SE (see subclause 10.8) we have:

,	30MHz<f≤6 GHz

,	6GHz<f≤19 GHz

By adding the MU per point and the SE values together we have the following total MU:MUtotal = 2.5dB,	30MHz<f≤6GHz

MUtotal = 4.2dB,	6GHz<f≤19GHz

The TT values are as follows:

TT = 2.5dB,	30MHz<f≤6GHz

TT = 4.2dB,	6GHz<f≤19GHz

### 10.5.4	Additional (co-existence) spurious emissions

#### 10.5.4.1	General

The additional spurious emissions requirements consist of the co-existence emissions requirements, and some additional regional requirements such as the protection of PHS and 700 and 800 public safety.

The conducted MU are consistent with the mandatory spurious emissions MU but are split into different frequency sub-divisions

Table 10.5.4.1-1: Conducted additional spurious emissions requirements

| Name | Unit | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz |
| --- | --- | --- | --- | --- |
|  |  | AAS | AAS | E-UTRA |
| 6.7.6.4 Transmitter spurious emissions, Additional spurious emission requirements | dB |  |  |  |
| > -60dBm, f ≤ 3.0GHz |  | 2.0 |  |  |
| > -60dBm, 3.0GHz < f ≤ 4.2GHz |  | 2.5 |  |  |
| > -60dBm, 4.2GHz < f ≤ 6.0GHz |  | 3.0 |  |  |
| > -60dBm, 6.0GHz < f ≤ 26.0GHz |  | x | x | x |
| ≤ -60dBm, f ≤ 3.0GHz |  | 3.0 |  |  |
| ≤ -60dBm, 3.0GHz < f ≤ 4.2GHz |  | 3.5 |  |  |
| ≤ -60dBm, 4.2GHz < f ≤ 6.0GHz |  | 4.0 |  |  |
| ≤ -60dBm, 6.0GHz < f ≤ 26.0GHz |  | x | x | x |

The lower power levels (≤ -60dBm ) are only needed for home BS, as we have no home BS in AAS these can be ignored.

The test set up for the OTA additional emissions requirements is the same as that for the mandatory spurious emissions in Figure 10.5.2.1-1. However, the additional spurious emissions are at a much lower level than the mandatory requirements so the additional effect of the test system dynamic range must be considered in the same way as the receiver emissions requirements.

Unlike the other spurious emissions requirements the additional (co-existence) requirements are specified for other 3GPP bands and as such can be measured in the same chambers as the in-band measurements.

As the CATR MU budget results is the largest MU and is used for setting the in-band MU values only the CATR MU is analysed below. However any suitable IAC or near field) chamber can be used.

#### 10.5.4.2	CATR

##### 10.5.4.2.1	General

The CATR method only is described as it provides the worst MU budget for the additional requirements MU analysis.

##### 10.5.4.2.2	Calibration

The same calibration is carried out as the EIRP calibration described in TR 37.82 [4]

Note: This stage may be omitted provided calibration stage has been performed already during output power measurement

##### 10.5.4.2.3	Procedure

The reference procedure can be found in subclause 10.4.1.3.3.

The appropriate parameters in step 5 is the mean power of additional spurious emissions test over the measurement BW described in the test requirement

##### 10.5.4.2.4	MU assessment

###### 10.5.4.2.4.1	MU Budget

Table 10.5.4.2.4.1-1: Compact antenna test range uncertainty contributions for AAS BS OTA additional spurious emissions measurement

| UID | Description of uncertainty contribution | Details in annex of [1] |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Misalignment DUT & pointing error | E2-1 |
| 2 | Conducted measurement uncertainty (minus mismatch) | E4-22, F.2 |
| 3 | Standing wave between DUT and test range antenna | E2-3 |
| 4 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) | E2-4 |
| 5 | QZ ripple DUT | E2-5 |
| 19 | Miscellaneous uncertainty | E2-14 |
| 25 | Measurement system dynamic range uncertainty | E4-22 |
| 26 | Test system frequency flatness | E2-16 |
| Stage 1: Calibration measurement |  |  |
| 6 | Uncertainty of network analyser | F.1 |
| 7 | Mismatch of receiver chain | E2-7 |
| 8 | Insertion loss variation of receiver chain | E2-8 |
| 9 | RF leakage, (SGH connector terminated & test range antenna connector cable terminated) | E2-4 |
| 10 | Influence of the calibration antenna feed cable: a) Flexing cables, adapters, attenuators, connector repeatability | E2-9 |
| 11 | Uncertainty of the absolute gain of the calibration antenna | F.1 |
| 12 | Misalignment positioning system | E2-11 |
| 13 | Misalignment of calibration antenna and test range antenna | E2-1 |
| 14 | Rotary Joints | E2-12 |
| 15 | Standing wave between reference calibration antenna and test range antenna | E2-3 |
| 16 | Quality of quiet zone | E2-5 |
| 20 | Switching uncertainty | E2-15 |

###### 10.5.4.2.4.2	MU Value

Table 10.5.4.2.4.2-1: Compact antenna test range uncertainty assessment for AAS BS OTA additional spurious emissions measurement

| UID | Uncertainty source | Uncertainty value |  |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | f≤3 GHz | 3GHz<f ≤4.2GHz | 4.2GHz<f ≤6GHz |  |  |  | f≤3 GHz | 3GHz<f ≤4.2GHz | 4.2GHz<f ≤6GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |  |
| 1 | Misalignment DUT & pointing error | 0 | 0 | 0 | Exp. normal | 2 | 1 | 0 | 0 | 0 |
| 2 | Conducted measurement uncertainty (minus mismatch) | 1.02 | 1.28 | 1.53 | Gaussian | 1 | 1 | 1.02 | 1.28 | 1.53 |
| 3 | Standing wave between DUT and test range antenna | 0.21 | 0.21 | 0.21 | U-shaped | √2 | 1 | 0.15 | 0.15 | 0.15 |
| 4 | RF leakage, test range antenna cable connector terminated. | 0.0012 | 0.0012 | 0.0012 | Normal | 1 | 1 | 0.0012 | 0.0012 | 0.0012 |
| 5 | QZ ripple with DUT | 0.0928 | 0.0928 | 0.0928 | Normal | 1 | 1 | 0.0928 | 0.0928 | 0.0928 |
| 25 | Measurement system dynamic range uncertainty | 0.51 | 0.51 | 0.51 | Normal | 1 | 1 | 0.51 | 0.51 | 0.51 |
| 26 | Test system frequency flatness | 0.25 | 0.25 | 0.25 | Normal | 1 | 1 | 0.25 | 0.25 | 0.25 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |
| 6 | Network Analyzer | 0.13 | 0.2 | 0.2 | Normal | 1 | 1 | 0.13 | 0.2 | 0.2 |
| 7 | Uncertainty of return loss (S11) measurement of SGH and test receiver (VNA) ports | 0.127 | 0.325 | 0.325 | U-shaped | √2 | 1 | 0.09 | 0.23 | 0.23 |
| 8 | Insertion loss variation in receiver chain | 0.18 | 0.18 | 0.18 | Rectangular | √3 | 1 | 0.1 | 0.1 | 0.1 |
| 9 | RF leakage, test range antenna cable connector terminated. | 0.0012 | 0.0012 | 0.0012 | Normal | 1 | 1 | 0.0012 | 0.0012 | 0.0012 |
| 10 | Influence of the calibration antenna feed cable | 0.022 | 0.022 | 0.022 | U-shaped | √2 | 1 | 0.015 | 0.015 | 0.015 |
| 11 | SGH Calibration uncertainty | 0.5 | 0.433 | 0.433 | Rectangular | √3 | 1 | 0.29 | 0.25 | 0.25 |
| 12 | Misalignment positioning system | 0 | 0 | 0 | Exp. normal | 2 | 1 | 0 | 0 | 0 |
| 13 | Misalignment SGH and pointing error | 0.5 | 0.5 | 0.5 | Exp. normal | 2 | 1 | 0.25 | 0.25 | 0.25 |
| 14 | Rotary joints | 0.048 | 0.048 | 0.048 | U-shaped | √2 | 1 | 0.034 | 0.034 | 0.034 |
| 15 | Standing wave between SGH and test range antenna | 0.09 | 0.09 | 0.09 | U-shaped | √2 | 1 | 0.06 | 0.06 | 0.06 |
| 16 | QZ ripple with SGH | 0.009 | 0.009 | 0.009 | Normal | 1 | 1 | 0.009 | 0.009 | 0.009 |
| 17 | Switching uncertainty | 0.26 | 0.26 | 0.26 | Rectangular | √3 | 1 | 0.15 | 0.15 | 0.15 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  |  | 1.27 | 1.50 | 1.72 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  |  | 2.48 | 2.93 | 3.37 |

#### 10.5.4.3	Void

#### 10.5.4.4	Test Tolerance

The conduced test tolerance for the additional spurious emissions requirements is zero.

However for OTA AAS BS, the difficulty in measuring TRP of the additional spurious emissions requirements at low levels close to the measurement system noise floor means the high risk of false failures. As the risk is due to the noise floor of the measurement system it cannot be mitigated by BS design.

As the 3GPP to 3GPP co-existence requirements are not regulatory but set by RAN4 to assist with co-existence of 3GPP systems in the same geographical area it is acceptable for RAN4 to set the TT value to be non-zero.

Hence it has been agreed that for 3GPP to 3GPP co-existence spurious emissions the TT=MU.

Some additional requirements such as the protection of PHS and the 700 and 800MHz  public safety bands, are regulatory so it is not possible to have anon zero TT, hence for these requirements the TT is zero.

#### 10.5.4.4	Summary

Adding the MUperpoint and the SE (see subclause 10.8) we have:

,	f≤3 GHz

,	3GHz<f≤4.2 GHz

,	4.2GHz<f≤6 GHz

By adding the MU per point and the SE values together we have the following total MU:

MUtotal = 2.6dB,								f≤3 GHz

MUtotal = 3.0dB,	3GHz<f≤4.2 GHz

MUtotal = 3.5dB,	4.2GHz<f≤6 GHz

For additional requirements for co-existence between 3GPP bands the TT values are as follows:

TT = 2.6dB,	f≤3 GHz

TT = 3.0dB,	3GHz<f≤4.2 GHz

TT = 3.5dB,	4.2GHz<f≤6 GHz

For PHS, and public safety additional requirements the TT=0dB.