3GPP TSG-RAN WG4 Meeting #90                     	R4-1900867

Athens, Greece, Feb 25th – Mar 1th, 2019

Agenda item:            6.14.1.2.1

Source:	MediaTek Inc.

Title:	NR PDSCH simulation result

Document for:          Discussion

# 1	Introduction

In this paper, we present the simulation results of PDSCH.

# 2	Discussion

Based on the simulation assumption in [1][2], we provide the simulation result of PDSCH test cases. Table A.1, Table A.2 and Table A.3 (in Appendix) are the test case list for FR1 FDD, FR1 TDD and FR2 TDD respectively. Table A.4 contains other detailed parameter setting in the simulation.

Table 2.1 is the 70% Tput SNR (30% in case 10) for FR1 FDD tests, Table 2.2 is the 70% Tput SNR for FR1 TDD tests, and Table 2.3 is the 70% Tput SNR (30% in case 7) for FR2 TDD tests. Figure A.2.1~Figure A.2.41 (in Appendix) are the corresponding performance curves.

Table 2.1 70% Tput SNR of FR1 FDD tests

| Case Number | 70% Tput SNR – 2RX | 70% Tput SNR – 4RX |
| --- | --- | --- |
| FR1-FDD1 | -2.0099 | -5.2086 |
| FR1-FDD2 | -0.7849 | -4.0569 |
| FR1-FDD3 | 22.4589 | 18.3386 |
| FR1-FDD4 | 13.1098 | 7.1205 |
| FR1 FDD5a | 17.5748 | 10.8475 |
| FR1 FDD5b | 17.6746 | 10.8522 |
| FR1-FDD6 | NA | 9.3984 |
| FR1-FDD7 | NA | 13.5549 |
| FR1-FDD8 | 15.7061 | NA |
| FR1-FDD9 | NA | 19.6849 |
| FR1-FDD10 (30% Tput) | 1.4743 | -1.4163 |
| FR1-FDD11 | -2.2245 | -5.0797 |
| FR1-FDD12 | -2.6395 | -5.5615 |
| FR1-FDD13 | -2.4661 | -5.5408 |
| FR1-FDD14 | 17.3031 | 11.0972 |

Table 2.2 70% Tput SNR of FR1 TDD tests

| Case Number | 70% Tput SNR – 2RX | 70% Tput SNR – 4RX |
| --- | --- | --- |
| FR1-TDD1 | -2.0148 | -5.1253 |
| FR1-TDD2 | -0.6085 | -3.9187 |
| FR1-TDD3 | 22.8593 | 18.7417 |
| FR1-TDD4 | 13.1469 | 7.1937 |
| FR1-TDD5a | 17.7643 | 11.0524 |
| FR1-TDD5b | 17.9304 | 11.2149 |
| FR1-TDD6 | NA | 9.6823 |
| FR1-TDD7 | NA | 13.7587 |
| FR1-TDD8 | 15.9013 | NA |
| FR1-TDD9 | NA | 20.0346 |
| FR1-TDD10 | 1.8403 | -1.1024 |
| FR1-TDD11 | -2.0361 | -5.0998 |
| FR1-TDD12 | -2.4535 | -5.3532 |
| FR1-TDD13 | -2.341 | -5.113 |
| FR1-TDD14 | 17.7147 | 11.1355 |

Table 2.3 70% Tput SNR of FR2 TDD tests

| Case Number | 70% Tput SNR |
| --- | --- |
| FR2-1 | -1.862 |
| FR2-2 | 9.7842 |
| FR2-3 | 2.9965 |
| FR2-4a | 12.2874 |
| FR2-4b | 12.3125 |
| FR2-5 | 16.4734 |
| FR2-6 | 16.556 |
| FR2-7 (30% Tput) | 1.6231 |
| FR2-8 | 12.5862 |
| FR2-9 | 12.4138 |
| FR2-10 | 12.6893 |

# 3	Conclusions

In the contribution, we provide our simulation results of PDSCH tests.

# 4	Reference

# [1] R4-1814239, “Simulation assumptions for NR PDSCH demodulation performance requirements”, Intel.

[2] R4-1816488, “Way forward on NR UE PDSCH demodulation requirements and General aspects”, Intel.

Appendix

Table A.1 PDSCH FR1 FDD test cases

| Case Number | CHBW/ SCS | MIMO | MCS | Number of layer | Number of PDSCH PRBs | Channel Model | Receiver | Max number of HARQ process |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR1-FDD1 | 10MHz/15kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | Type A, QPSK, MCS 4 | 1 Layer | 52 | TDL-B 100ns, 400 Hz | MMSE-IRC | 4 |
| FR1- FDD 2 | 10MHz/15kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | Type A, QPSK, MCS 4 | 1 Layer | 6 (in the middle of CBW) | TDL-C 300ns, 100 Hz | MMSE-IRC | 4 |
| FR1- FDD 3 | 10MHz/15kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | Type A, 256QAM, MCS 24 | 1 Layer | 52 | TDL-A 30ns, 10Hz | MMSE-IRC | 4 |
| FR1- FDD 4 | 10MHz/15kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | Type A, 16QAM, MCS 13 | 2 Layers | 52 | TDL-C 300ns, 100 Hz | MMSE-IRC | 4 |
| FR1- FDD 5a | 10MHz/15kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | Type A, 64QAM, MCS 19 | 2 Layers | 52 | TDL-A 30ns, 10Hz | MMSE-IRC | 4 |
| FR1- FDD 5b | 10MHz/15kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | Type A, 64QAM, MCS 19 | 2 Layers | 52 | TDL-A 30ns, 10Hz | MMSE-IRC | 8 |
| FR1-FDD6 | 10MHz/15kHz | 4Tx 4Rx ULA Low | Type A, 16QAM, MCS 13 | 3 Layers | 52 | TDL-A 30ns, 10Hz | MMSE-IRC | 4 |
| FR1-FDD7 | 10MHz/15kHz | 4Tx 4Rx ULA Low | Type A, 16QAM, MCS 13 | 4 Layers | 52 | TDL-A 30ns, 10Hz | MMSE-IRC | 4 |
| FR1-FDD8 | 10MHz/15kHz | 2Tx 2Rx ULA Med | Type A, 16QAM, MCS 13 | 2 Layers | 52 | TDL-A 30ns, 10Hz | R-ML | 4 |
| FR1-FDD9 | 10MHz/15kHz | 4Tx 4RX ULA Med A | Type A, 16QAM, MCS 13 | 3 Layers | 52 | TDL-A 30ns, 10Hz | R-ML | 4 |
| FR1- FDD10 | 10MHz/15kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | Type A, 16QAM, MCS 13 | 1 Layer | 52 | TDL-C 300ns, 100 Hz | MMSE-IRC | 8 |
| FR1- FDD11 | 10MHz/15kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | Type B, QPSK, MCS 4 | 1 Layer | 52 | TDL-A 30ns, 10Hz | MMSE-IRC | 4 |
| FR1- FDD12 (LTE-NR test #1) | 10MHz/15kHz | 4Tx 2Rx ULA Low 4Tx 4Rx ULA Low | Type A, QPSK, MCS 4 | 1 Layer | 52 | TDL-A 30ns, 10Hz | MMSE-IRC | 4 |
| FR1- FDD13 (LTE-NR test #2) | 10MHz/15kHz | 4Tx 2Rx ULA Low 4Tx 4Rx ULA Low | Type A, QPSK, MCS 4 | 1 Layer | 52 | TDL-A 30ns, 10Hz | MMSE-IRC | 4 |
| FR1- FDD14 | 20MHz/30kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | Type A, 64QAM, MCS 19 | 2 Layers | 51 | TDL-A 30ns, 10Hz | MMSE-IRC | 4 |

Table A.2 PDSCH FR1 TDD test cases

| Case Number | CHBW/ SCS | MIMO | MCS | Number of layer | Number of PDSCH PRBs | Channel Model | Receiver | TDD Configuration | Max number of HARQ process |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR1- TDD1 | 40MHz/30kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | Type A, QPSK MCS 4 | 1 Layer | 106 | TDL-B 100ns, 400 Hz | MMSE-IRC | 1 | 8 |
| FR1- TDD2 | 40MHz/30kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | Type A, QPSK MCS 4 | 1 Layer | 6 (in the middle of CBW) | TDL-C 300ns, 100 Hz | MMSE-IRC | 1 | 8 |
| FR1- TDD3 | 40MHz/30kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | Type A, 256QAM MCS 24 | 1 Layer | 106 | TDL-A 30ns, 10Hz | MMSE-IRC | 1 | 8 |
| FR1- TDD4 | 40MHz/30kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | Type A, 16QAM MCS 13 | 2 Layers | 106 | TDL-C 300ns, 100 Hz | MMSE-IRC | 1 | 8 |
| FR1- TDD5a | 40MHz/30kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | Type A, 64QAM MCS 19 | 2 Layers | 106 | TDL-A 30ns, 10Hz | MMSE-IRC | 1 | 8 |
| FR1- TDD5b | 40MHz/30kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | Type A, 64QAM MCS 19 | 2 Layers | 106 | TDL-A 30ns, 10Hz | MMSE-IRC | 1 | 16 |
| FR1- TDD6 | 40MHz/30kHz | 4Tx 4Rx ULA Low | Type A, 16QAM MCS 13 | 3 Layers | 106 | TDL-A 30ns, 10Hz | MMSE-IRC | 1 | 8 |
| FR1- TDD7 | 40MHz/30kHz | 4Tx 4Rx ULA Low | Type A, 16QAM MCS 13 | 4 Layers | 106 | TDL-A 30ns, 10Hz | MMSE-IRC | 1 | 8 |
| FR1- TDD8 | 40MHz/30kHz | 2Tx 2Rx ULA Med | Type A, 16QAM MCS 13 | 2 Layers | 106 | TDL-A 30ns, 10Hz | R-ML | 1 | 8 |
| FR1- TDD9 | 40MHz/30kHz | 4Tx 4RX ULA Med A | Type A, 16QAM MCS 13 | 3 Layers | 106 | TDL-A 30ns, 10Hz | R-ML | 1 | 8 |
| FR1- TDD10 | 40MHz/30kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | Type A, 16QAM MCS 13 | 1 Layer | 106 | TDL-C 300ns, 100 Hz | MMSE-IRC | 1 | 16 |
| FR1- TDD11 | 40MHz/30kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | Type A, QPSK MCS 4 | 1 Layer | 106 | TDL-A 30ns, 10Hz | MMSE-IRC | 2 | 8 |
| FR1- TDD12 | 40MHz/30kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | Type A, QPSK MCS 4 | 1 Layer | 106 | TDL-A 30ns, 10Hz | MMSE-IRC | 3 | 8 |
| FR1- TDD13 | 40MHz/30kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | Type B, QPSK, MCS 4 | 1 Layer | 106 | TDL-A 30ns, 10Hz | MMSE-IRC | 1 | 8 |
| FR1- TDD14 | 20MHz/30kHz | 2Tx 2Rx ULA Low 2Tx 4Rx ULA Low | Type A, 64QAM, MCS 19 | 2 Layers | 51 | TDL-A 30ns, 10Hz | MMSE-IRC | 1 | 8 |

Table A.3 PDSCH FR2 TDD test cases

| Case Number | CHBW/ SCS | MIMO | MCS | Number of layer | Number of PDSCH PRBs | Channel Model | Receiver | TDD Configuration | Max number of HARQ process |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR2-1 | 100MHz/120kHz | 2Tx 2Rx ULA Low | Type A, QPSK MCS 4 | 1 Layer | 66 | TDL-C 60ns, 300Hz | MMSE-IRC | 1 | 8 |
| FR2-2 | 100MHz/120kHz | 2Tx 2Rx XPL Med A | Type A, 64QAM MCS 18 | 1 Layer | 66 | TDL-A 30ns, 300Hz | MMSE-IRC | 1 | 8 |
| FR2-3 | 100MHz/120kHz | 2Tx 2Rx ULA Low | Type A, QPSK MCS 4 | 2 Layers | 6 (in the middle of CBW) | TDL-A 30ns, 75Hz | MMSE-IRC | 2 | 10 |
| FR2-4a | 100MHz/120kHz | 2Tx 2Rx ULA Low | Type A, 16QAM MCS 13 | 2 Layers | 66 | TDL-A 30ns, 300Hz | MMSE-IRC | 1 | 8 |
| FR2-4b | 100MHz/120kHz | 2Tx 2Rx ULA Low | Type A, 16QAM MCS 13 | 2 Layers | 66 | TDL-A 30ns, 300Hz | MMSE-IRC | 1 | 16 |
| FR2-5 | 100MHz/120kHz | 2Tx 2Rx ULA Low | Type A, 64QAM MCS 17 | 2 Layers | 66 | TDL-A 30ns, 75Hz | MMSE-IRC | 2 | 10 |
| FR2-6 | 100MHz/120kHz | 2Tx 2Rx ULA Med | Type A, 16QAM MCS 13 | 2 Layers | 66 | TDL-A 30ns, 75Hz | R-ML | 2 | 10 |
| FR2-7 | 100MHz/120kHz | 2Tx 2Rx ULA Low | Type A, 16QAM MCS 13 | 1 Layer | 66 | TDL-A 30ns, 300Hz | MMSE-IRC | 1 | 16 |
| FR2-8 | 50MHz/120kHz | 2Tx 2Rx ULA Low | Type A, 16QAM MCS 13 | 2 Layers | 32 | TDL-A 30ns, 75Hz | MMSE-IRC | 2 | 10 |
| FR2-9 | 200MHz/120kHz | 2Tx 2Rx ULA Low | Type A, 16QAM MCS 13 | 2 Layers | 132 | TDL-A 30ns, 300Hz | MMSE-IRC | 1 | 8 |
| FR2-10 | 50MHz/60kHz | 2Tx 2Rx ULA Low | Type A, 16QAM MCS 13 | 2 Layers | 66 | TDL-A 30ns, 75Hz | MMSE-IRC | 2 | 10 |

Table A.4 PDSCH simulation parameter setting

| Items | Setting | Notes |
| --- | --- | --- |
| TDD UL/DL configurations | - FR1 TDD - Configuration 1: 7D1S2U, S = 6D:4G:4U - Configuration 2: DDDSUDDSUU, S1(D,GP,U)=[10, 4, 0], S2(D, GP, U) = [10,4,0] - Configuration 3: DDDSU, S=10D:2G:2U - FR2 TDD - Configuration 1: DDDSU, S=10D:2G:2U - Configuration 2: DDSU, S=11D+3G |  |
| DMRS | - Type 1 single symbol front loaded - 1 additional DMRS for Low (5/10Hz) and Medium (100 Hz) Speed scenarios - OFDM symbols with DMRS: #2 and #9 - 2 additional DMRSs for High (400 Hz) Speed scenarios - OFDM symbols with DMRS: #2, #6 and #9 - FDM is applied between DMRS and data for test cases with 1 Layer and 2 Layers |  |
| Impairments model | - FR1 - TX EVM = 6% for QPSK/16QAM/64QAM - TX EVM = 3% for 256QAM - FR2 - TX EVM = 6% for QPSK/16QAM/64QAM - Phase noise is explicitly modelled for Rx. Use phase noise model #2 (UE side) as defined in TR 38.803 for FR2 demodulation performance tests. - CF = 29 GHz |  |

PDSCH simulation performance curve: X-label (SNR) / Y-label (Tput ratio)

Figure A.2.1 PDSCH Tput – Test case FR1-FDD1

Figure A.2.2 PDSCH Tput – Test case FR1-FDD2

Figure A.2.3 PDSCH Tput – Test case FR1-FDD3

Figure A.2.4 PDSCH Tput – Test case FR1-FDD4

Figure A.2.5 PDSCH Tput – Test case FR1-FDD5a

Figure A.2.6 PDSCH Tput – Test case FR1-FDD5b

Figure A.2.7 PDSCH Tput – Test case FR1-FDD6

Figure A.2.8 PDSCH Tput – Test case FR1-FDD7

Figure A.2.9 PDSCH Tput – Test case FR1-FDD8

Figure A.2.10 PDSCH Tput – Test case FR1-FDD9

Figure A.2.11 PDSCH Tput – Test case FR1-FDD10

Figure A.2.12 PDSCH Tput – Test case FR1-FDD11

Figure A.2.13 PDSCH Tput – Test case FR1-FDD12

Figure A.2.14 PDSCH Tput – Test case FR1-FDD13

Figure A.2.15 PDSCH Tput – Test case FR1-FDD14

Figure A.2.16 PDSCH Tput – Test case FR1-TDD1

Figure A.2.17 PDSCH Tput – Test case FR1-TDD2

Figure A.2.18 PDSCH Tput – Test case FR1-TDD3

Figure A.2.19 PDSCH Tput – Test case FR1-TDD4

Figure A.2.20 PDSCH Tput – Test case FR1-TDD5a

Figure A.2.21 PDSCH Tput – Test case FR1-TDD5b

Figure A.2.22 PDSCH Tput – Test case FR1-TDD6

Figure A.2.23 PDSCH Tput – Test case FR1-TDD7

Figure A.2.24 PDSCH Tput – Test case FR1-TDD8

Figure A.2.25 PDSCH Tput – Test case FR1-TDD9

Figure A.2.26 PDSCH Tput – Test case FR1-TDD10

Figure A.2.27 PDSCH Tput – Test case FR1-TDD11

Figure A.2.28 PDSCH Tput – Test case FR1-TDD12

Figure A.2.29 PDSCH Tput – Test case FR1-TDD13

Figure A.2.30 PDSCH Tput – Test case FR1-TDD14

Figure A.2.31 PDSCH Tput – Test case FR2-TDD1

Figure A.2.32 PDSCH Tput – Test case FR2-TDD2

Figure A.2.33 PDSCH Tput – Test case FR2-TDD3

Figure A.2.34 PDSCH Tput – Test case FR2-TDD4a

Figure A.2.35 PDSCH Tput – Test case FR2-TDD4b

Figure A.2.36 PDSCH Tput – Test case FR2-TDD5

Figure A.2.37 PDSCH Tput – Test case FR2-TDD6

Figure A.2.38 PDSCH Tput – Test case FR2-TDD7

Figure A.2.39 PDSCH Tput – Test case FR2-TDD8

Figure A.2.40 PDSCH Tput – Test case FR2-TDD9

Figure A.2.41 PDSCH Tput – Test case FR2-TDD10