3GPP TSG-RAN WG4 Meeting #90                     	R4-1900873

Athens, Greece, Feb 25th – Mar 1th, 2019

Agenda item:            6.14.1.4.4

Source:	  MediaTek Inc.

Title:	  NR CSI RI simulation result

Document for:          Discussion

# 1	Introduction

In this paper, we present the simulation results of CSI RI.

# 2	Discussion

Based on the simulation assumption in [1], we provide the simulation result of CSI RI test cases. Table A.1 (in Appendix) is the simulation assumption for RI. Table A.2~Table A.5 contain other detailed parameter setting in the simulation.

Performance metric:

Gamma1 = The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 1

Gamma 2 = The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 2

Test Point:

| Test Number | Test 1 (2X2)/Test 2 (2X2) |  |  | Test 3 (2X2) |
| --- | --- | --- | --- | --- |
| MIMO correlation | ULA Low | ULA Low | ULA High | XP High |
| Metric | Gamma 2 | Gamma 1 | Gamma 1 | Gamma 1 |

| Test Number | Test 1 (2X4)/Test 2 (2X4) |  |  | Test 1 (4X4)/Test 2 (4X4) |
| --- | --- | --- | --- | --- |
| MIMO correlation | ULA Low | ULA Low | ULA High | ULA Low |
| Metric | Gamma 2 | Gamma 1 | Gamma 1 | Gamma 2 |

## 2.1 FR1 FDD

Table 2.1.1~Table 2.1.3 are the simulation results of RI Test 1.

Table 2.1.1 Test 1 FR1 FDD - 2TX, 2RX

|  | ULA Low |  | ULA High |  |
| --- | --- | --- | --- | --- |
| SNR [dB] | Gamma 1 | Gamma 2 | Gamma 1 | Gamma 2 |
| -6 | 0.99 | 3.11 | 1.00 |  |
| -4 | 1.01 | 1.65 | 1.00 |  |
| -2 | 1.02 | 1.33 | 1.00 |  |
| 0 | 1.02 | 1.39 | 1.00 |  |
| 2 | 1.02 | 1.42 | 1.00 |  |
| 4 | 1.00 | 1.45 | 1.00 |  |
| 6 | 1.03 | 1.31 | 1.00 |  |
| 8 | 1.05 | 1.22 | 1.00 |  |
| 10 | 1.03 | 1.17 | 1.00 |  |
| 12 | 1.07 | 1.12 | 1.00 |  |
| 14 | 1.10 | 1.09 | 1.00 |  |
| 16 | 1.14 | 1.05 | 1.00 |  |
| 18 | 1.15 | 1.04 | 1.00 |  |
| 20 | 1.18 | 1.03 | 0.99 |  |
| 22 | 1.20 | 1.02 | 0.98 |  |
| 24 | 1.27 | 1.01 | 0.91 |  |
| 26 | 1.35 | 1.00 | 0.79 |  |

Table 2.1.2 Test 1 FR1 FDD – 2TX, 4RX

|  | ULA Low |  | ULA High |  |
| --- | --- | --- | --- | --- |
| SNR [dB] | Gamma 1 | Gamma 2 | Gamma 1 | Gamma 2 |
| -6 | 0.44 | 1.02 | 0.48 |  |
| -4 | 1.06 | 1.05 | 0.58 |  |
| -2 | 1.13 | 1.03 | 0.58 |  |
| 0 | 1.34 | 1.01 | 0.58 |  |
| 2 | 1.43 | 1.00 | 0.67 |  |
| 4 | 1.60 | 1.00 | 0.68 |  |
| 6 | 1.73 | 1.00 | 0.75 |  |
| 8 | 1.76 | 1.00 | 0.81 |  |
| 10 | 1.74 | 1.00 | 0.86 |  |
| 12 | 1.76 | 1.00 | 0.85 |  |
| 14 | 1.84 | 1.00 | 0.91 |  |
| 16 | 1.91 | 1.00 | 0.92 |  |
| 18 | 1.83 | 1.00 | 0.91 |  |
| 20 | 1.77 | 1.00 | 0.82 |  |
| 22 | 1.85 | 1.00 | 0.74 |  |
| 24 | 1.98 | 1.00 | 0.64 |  |
| 26 | 2.10 | 1.00 | 0.52 |  |

Table 2.1.3 Test 1 FR1 FDD – 4TX, 4RX

|  | ULA Low |  |
| --- | --- | --- |
| SNR [dB] | Gamma 1 | Gamma 2 |
| -6 |  | 0.75 |
| -4 |  | 0.83 |
| -2 |  | 1.02 |
| 0 |  | 1.02 |
| 2 |  | 1.03 |
| 4 |  | 1.00 |
| 6 |  | 1.00 |
| 8 |  | 1.00 |
| 10 |  | 1.03 |
| 12 |  | 1.07 |
| 14 |  | 1.09 |
| 16 |  | 1.13 |
| 18 |  | 1.19 |
| 20 |  | 1.19 |
| 22 |  | 1.14 |
| 24 |  | 1.15 |
| 26 |  | 1.25 |
| 28 |  |  |
| 30 |  |  |

## 2.2 FR1 TDD (Test 2)

Table 2.2.1~Table 2.2.3 are the simulation results of RI Test 2.

Table 2.2.1 Test 2 FR1 TDD - 2TX, 2RX

|  | ULA Low |  | ULA High |  |
| --- | --- | --- | --- | --- |
| SNR [dB] | Gamma 1 | Gamma 2 | Gamma 1 | Gamma 2 |
| -6 | 1.00 | 5.33 | 1.00 |  |
| -4 | 1.02 | 1.70 | 1.00 |  |
| -2 | 1.04 | 1.16 | 1.00 |  |
| 0 | 1.04 | 1.31 | 1.00 |  |
| 2 | 1.02 | 1.48 | 1.00 |  |
| 4 | 1.02 | 1.56 | 1.00 |  |
| 6 | 1.02 | 1.35 | 1.00 |  |
| 8 | 1.02 | 1.25 | 1.00 |  |
| 10 | 1.04 | 1.16 | 1.00 |  |
| 12 | 1.05 | 1.09 | 1.00 |  |
| 14 | 1.08 | 1.03 | 1.00 |  |
| 16 | 1.14 | 1.01 | 1.00 |  |
| 18 | 1.18 | 1.02 | 1.00 |  |
| 20 | 1.17 | 1.03 | 1.00 |  |
| 22 | 1.14 | 1.00 | 0.98 |  |
| 24 | 1.19 | 1.00 | 0.94 |  |
| 26 | 1.15 | 1.00 | 0.90 |  |

Table 2.2.2 Test 2 FR1 TDD – 2TX, 4RX

|  | ULA Low |  | ULA High |  |
| --- | --- | --- | --- | --- |
| SNR [dB] | Gamma 1 | Gamma 2 | Gamma 1 | Gamma 2 |
| -6 | 0.33 | 1.34 | 0.65 |  |
| -4 | 1.40 | 1.02 | 0.70 |  |
| -2 | 1.49 | 1.03 | 0.79 |  |
| 0 | 1.50 | 1.02 | 0.81 |  |
| 2 | 1.52 | 1.00 | 0.81 |  |
| 4 | 1.72 | 1.00 | 0.90 |  |
| 6 | 1.72 | 1.00 | 0.88 |  |
| 8 | 1.78 | 1.00 | 0.91 |  |
| 10 | 1.74 | 1.00 | 0.95 |  |
| 12 | 1.74 | 1.00 | 0.95 |  |
| 14 | 1.83 | 1.00 | 0.95 |  |
| 16 | 1.91 | 1.00 | 0.97 |  |
| 18 | 1.86 | 1.00 | 0.89 |  |
| 20 | 1.73 | 1.00 | 0.85 |  |
| 22 | 1.79 | 1.00 | 0.78 |  |
| 24 | 1.96 | 1.00 | 0.72 |  |
| 26 | 2.10 | 1.00 | 0.50 |  |

Table 2.2.3 Test 2 FR1 TDD – 4TX, 4RX

|  | ULA Low |  |
| --- | --- | --- |
| SNR [dB] | Gamma 1 | Gamma 2 |
| -6 |  | 0.97 |
| -4 |  | 1.03 |
| -2 |  | 1.10 |
| 0 |  | 1.01 |
| 2 |  | 1.07 |
| 4 |  | 1.00 |
| 6 |  | 1.00 |
| 8 |  | 1.00 |
| 10 |  | 1.00 |
| 12 |  | 1.00 |
| 14 |  | 1.05 |
| 16 |  | 1.08 |
| 18 |  | 1.06 |
| 20 |  | 1.08 |
| 22 |  | 1.10 |
| 24 |  | 1.10 |
| 26 |  | 1.13 |
| 28 |  |  |
| 30 |  |  |

## 2.3 FR2 TDD (Test 3)

Table 2.3.1 are the simulation results of RI Test 3.

Table 2.3.1 Test 3 FR2 TDD - 2TX, 2RX

|  | XP High |  |
| --- | --- | --- |
| SNR [dB] | Gamma 1 | Gamma 2 |
| -6 | 0.94 |  |
| -4 | 0.89 |  |
| -2 | 0.89 |  |
| 0 | 0.96 |  |
| 2 | 1.03 |  |
| 4 | 1.07 |  |
| 6 | 1.08 |  |
| 8 | 1.12 |  |
| 10 | 1.17 |  |
| 12 | 1.23 |  |
| 14 | 1.21 |  |
| 16 | 1.21 |  |
| 18 | 1.24 |  |
| 20 | 1.27 |  |
| 22 | 1.20 |  |
| 24 | 1.15 |  |
| 26 | 1.03 |  |

# 3	Conclusions

In the contribution, we provide our simulation results of CSI RI tests.

# 4	Reference

[1] R4-1816491, “Way forward on NR UE CSI requirements”, Samsung

Appendix

Table A.1 RI test cases

| Parameter | Test 1 (FR1 FDD) | Test 2 (FR1 FDD) | Test 3 FR2 |
| --- | --- | --- | --- |
| Bandwidth/SCS | 10MHz /15kHz | 40MHz/30kHz | 100MHz/120kHz |
| Transmission Scheme | 1 | 1 | 1 |
| Duplex Mode | FDD | TDD | TDD |
| MCS table | 256QAM | 256QAM | 64QAM |
| DLUL configuration | N.A | 7D1S2U (FR1.30-1) | DDSU (FR2.120-2) |
| Propagation channel | TDL_A 30ns, 5Hz | TDL_A 30ns, 5Hz | TDL_A 30ns, 35Hz |
| Correlation and antenna configuration | 2RX: 2*2 ULA Low, ULA High 4Rx: 2*4 ULA Low, ULA High. 4*4 ULA Low | 2Rx: 2*2 ULA Low, ULA High 4Rx: 2*4 ULA Low, ULA High 4*4 ULA Low | 2Rx 2*2 [ULA Low, XP High] |
| resourceConfigType | Periodic | Periodic | Aperiodic |
| NZP CSI-RS ports | 2/4 | 2/4 | 2 |
| reportConfigType | periodic | periodic | Aperiodic |
| Reporting periodicity and slot offset | 5/1 | 10/1 | 8/1 |
| Codebook Restriction | 2Tx: 000011 for fixed RI = 1 010000 for fixed RI = 2 010011 for UE reported RI 4Tx: 11111111 with N1 =2, N2 =1 | 2Tx: 000011 for fixed RI = 1 010000 for fixed RI = 2 010011 for UE reported RI 4Tx: 11111111 with N1 =2, N2 =1 | 000011 for fixed RI = 1 010000 for fixed RI = 2 010011 for UE reported RI |
| RI Rrestriction | N/A for 2Tx 4Tx: 00000010 for fixed Rank 2 and 00001111 for follow RI | N/A for 2Tx 4Tx: 00000010 for fixed Rank 2 and 00001111 for follow RI | N/A |
| Reporting granularity | Wide-band | Wide-band | Wide-band |
| Physical channel for reporting | PUSCH | PUSCH | PUSCH |
| PUCCH reporting format | FFS | FFS | FFS |
| CQI table | Table 2 with 256QAM | Table 2 with 256QAM | Table 1 without 256QAM |
| Minimum CSI delay | 8ms | 9.5ms | 1.375ms |

Table A.2 CSI simulation detailed parameter setting

| Items | Setting | Notes |
| --- | --- | --- |
| TDD UL/DL configurations | - FR1 TDD: 7D1S2U, S = 6D:4G:4U - FR2 TDD: DDSU, S=11D+3G |  |
| HARQ | - For PMI, RV sequence ={0,2,3,1} - For RI and CQI test cases ,no retransmission |  |
| CSI-RS resource/CSI reporting periodicity slot offset for periodic CSI reporting | - FR1 FDD 15 kHz: 5/1 - FR1 TDD 30 kHz: Option 1: 10/1 - FR2 TDD 120 kHz: - Option 1: 16/1 - Option 2: 8/1 |  |
| Minimum CSI delay for periodic CSI | 8ms |  |
| CSI-RS resource/CSI reporting interval and slot offset for aperiodic CSI reporting | - FR1 FDD 15 kHz: 3/1 with total CSI delay [8]slots - FR1 TDD 30 kHz: 5/1 with total CSI delay [12] slots - FR2 TDD 120 kHz: 12/1 with total CSI delay [27] slots |  |
| CSI reporting and CSI-RS resources Type for fading CQI test cases and RI test cases of FR2 | - Option 1: Periodic (previous meeting agreements) - Option 2: aperiodic (baseline) - Companies are encouraged to bring analysis for above options considering different CSI delay assumptions for periodic and aperiodic CSI reporting types |  |
| Number of HARQ process number | - FR1 FDD: 4 - FR1 TDD: 8 - FR2 TDD: 8 |  |

Table A.3 NZP CSI-RS configurations

| First subcarrier index in the PRB used for CSI-RS (k0, k1) | Row 3, (6,-) | Row 4,(0,-) | Row 8,(4,6) |
| --- | --- | --- | --- |
| First OFDM symbol in the PRB used for CSI-RS (l0) | 13 | 13 | 5 |
| Number of CSI-RS ports (X) | 2 | 4 | 8 |
| CDM Type | FD-CDM2 | FD-CDM2 | CDM4 (FD2,TD2) |
| Density (ρ) | 1 | 1 | 1 |

Table A.4 ZP CSI-RS configurations

| Parameters | FR1 | FR2 |
| --- | --- | --- |
| First subcarrier index in the PRB used for CSI-RS (k0) | Row 5, 4 | Row 4, 8 |
| First OFDM symbol in the PRB used for CSI-RS (l0) | 9 | 13 |
| Number of CSI-RS ports (X) | 4 | 4 |
| CDM Type | FD-CDM2 | FD-CDM2 |
| Density (ρ) | 1 | 1 |

Table A.5 CSI-IM configurations

| Parameters | FR1 | FR2 |
| --- | --- | --- |
| CSI-IM-RE-pattern | Pattern 0 | Pattern 1 |
| CSI-IM-ResourceMapping (kCSI-IM,lCSI-IM) | (4,9) | (8,13) |