3GPP TSG-RAN WG4 Meeting #90	R4-1901247

Athens, Greece, 26 February – 1 March 2019

Agenda Item:	5.6.6.2

Source:	Ericsson

Title:	Simulation result of sTTI CQI reporting tests

Document for:	Discussion

# 1	Introduction

RAN4#89 agreed with the CR on CQI reporting test [1], however the SNR test points are TBD. This contribution shows our simulation results to decide the SNR test points.

# 2	CRS-based PDSCH

## 2.1	Test points

The test metrics of the existing wideband CQI reporting requirements are specified as follows:

a)	a CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least  % of the time;

b)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ ;

c)	when transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater or equal to δ

To decide the test points, we choose CQI indexes 7 and 11 for both slot-PDSCH and subslot-PDSCH because their CQI indexes mapping to MCS whose coding rate is close to the target coding rate. We could choose lower CQI indexes such as 3 or 4, however our simulation result shows the corresponding SNR is -6dB or lower. We also want to avoid setting low SNR region considering the less reliability of SNR estimation.

Table 1 and Table 2 show our simulation results of wideband CQI reporting with slot-PDSCH and subslot-PDSCH, respectively, with regard to the metrics above. Figure 1 and Figure 2 show the median CQI and throughput ratio values for information.

Proposal 1: For CRS-based PDSCH, set two sets of test points {1, 2} and {9, 10} for slot-based wideband CQI reporting test.

Proposal 2: For CRS-based PDSCH, set two sets of test points {1, 2} and {9, 10} for subslot-based wideband CQI reporting test.

Table 1	Simulation results of wideband CQI with slot-PDSCH.

| SNR | Median CQI | 1 - Prob(medianCQI-1, medianCQI, medianCQI+1) > [20]% | BLER with follow CQI > [0.02] | Tput ratio > [1.05] |
| --- | --- | --- | --- | --- |
| 1 | 7 | 41.4 | 0.09 | 1.40 |
| 2 | 7 | 42.8 | 0.08 | 1.35 |
| 9 | 11 | 37.6 | 0.09 | 1.50 |
| 10 | 11 | 39.6 | 0.10 | 1.35 |

Table 2	Simulation results of wideband CQI with subslot-PDSCH.

| SNR | Median CQI | 1 - Prob(medianCQI-1, medianCQI, medianCQI+1) > [20]% | BLER with follow CQI > [0.02] | Tput ratio > [1.05] |
| --- | --- | --- | --- | --- |
| 1 | 7 | 41.1 | 0.25 | 1.28 |
| 2 | 7 | 42.1 | 0.26 | 1.22 |
| 9 | 11 | 35.7 | 0.18 | 1.43 |
| 10 | 11 | 37.0 | 0.17 | 1.30 |

Figure 1	Median CQI.

Figure 2	Throughput ratio between the follow CQI and Fixed median CQI.

# 3	DMRS-based PDSCH

## 3.1	Test points

The test metrics of the existing wideband CQI reporting requirements are specified as follows:

a)	a CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least  % of the time;

b)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ ;

c)	when transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater or equal to δ

As we discussed in CRS-based CQI test, we should avoid setting SNR test points whose CQI indexes result in a big difference between target coding rate and effective coding rate. We propose to choose CQI indexes around 8 and 12 because there CQI indexes map to MCS whose coding rate is close to the target. Like CRS-based transmission, we don’t choose lower CQI index to avoid unreliable SNR estimation.

Table 3 and Table 4 show our simulation results of wideband CQI reporting with slot-PDSCH and subslot-PDSCH, respectively, with regard to the metrics above. Figure 3 and Figure 4 show the median CQI and throughput ratio values for reference. It is observed that our simulation results have enough margin to the current metrics, i.e., α=20%, γ=1.05, and δ=0.02.

Proposal 3: For DMRS-based PDSCH, set two sets of test points {3, 4} and {12, 13} for both slot-based wideband CQI reporting test.

Proposal 4: For DMRS-based PDSCH, set two sets of test points {4, 5} and {12, 13} for subslot-based wideband CQI reporting test.

Table 3	Simulation results of wideband CQI with slot-PDSCH.

| SNR | Median CQI | 1 - Prob(medianCQI-1, medianCQI, medianCQI+1) > [20]% | BLER with follow CQI > [0.02] | Tput ratio > [1.05] |
| --- | --- | --- | --- | --- |
| 3 | 8 | 40.6 | 0.09 | 1.40 |
| 4 | 8 | 40.9 | 0.09 | 1.31 |
| 12 | 12 | 36.2 | 0.09 | 1.40 |
| 13 | 13 | 36.3 | 0.08 | 1.56 |

Table 4	Simulation results of wideband CQI with subslot-PDSCH.

| SNR | Median CQI | 1 - Prob(medianCQI-1, medianCQI, medianCQI+1) > [20]% | BLER with follow CQI > [0.02] | Tput ratio > [1.05] |
| --- | --- | --- | --- | --- |
| 4 | 8 | 37.7 | 0.18 | 1.27 |
| 5 | 8 | 37.2 | 0.17 | 1.24 |
| 12 | 12 | 37.7 | 0.16 | 1.31 |
| 13 | 12 | 36.2 | 0.16 | 1.22 |

Figure 3	Median CQI.

Figure 4	Throughput ratio between the follow CQI and Fixed median CQI.

# 4	Conclusion

Proposal 1: For CRS-based PDSCH, set two sets of test points {1, 2} and {9, 10} for slot-based wideband CQI reporting test.

Proposal 2: For CRS-based PDSCH, set two sets of test points {1, 2} and {9, 10} for subslot-based wideband CQI reporting test.

Proposal 3: For DMRS-based PDSCH, set two sets of test points {3, 4} and {12, 13} for both slot-based wideband CQI reporting test.

Proposal 4: For DMRS-based PDSCH, set two sets of test points {4, 5} and {12, 13} for subslot-based wideband CQI reporting test.

Table below summarizes our proposal.

|  |  | SNR test points (dB) | Alpha | Gamma | Delta |
| --- | --- | --- | --- | --- | --- |
| CRS-based | Slot-PDSCH | {1, 2}, {9,10} | 20% | 1.05 | 0.02 |
|  | Subslot-PDSCH | {1, 2}, {9,10} | 20% | 1.05 | 0.02 |
| DMRS-based | Slot-PDSCH | {3, 4}, {12, 13} | 20% | 1.05 | 0.02 |
|  | Subslot-PDSCH | {4, 5}, {12, 13} | 20% | 1.05 | 0.02 |

# 5	References

- R4-1816325, “Introduction of CQI tests for sTTI”, Ericsson.

# Appendix

Table 5 through Table 9 show the CQI-to-MCS mapping tables and show the absolute difference between the target coding rate and effective coding rate. We highlighted in yellow for coding rates exceeding 0.05.

Table 5	CQI-to-MCS table and difference to the target coding rate for CRS-based Slot-PDSCH.

|  |  |  | Slot 0 |  |  | Slot 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PRB | 50 |  |  | 50 |  |  |
|  |  | Available REs | 2800 |  |  | 3672 |  |  |
| CQI index | Code rate | Modulation |  | CR | Diff |  | CR | Diff |
| 1 | 0.08 | 2 | 0 | 0.13 | 0.05 | 0 | 0.10 | 0.02 |
| 2 | 0.12 | 2 | 0 | 0.13 | 0.01 | 1 | 0.13 | 0.01 |
| 3 | 0.19 | 2 | 2 | 0.2 | 0.01 | 3 | 0.20 | 0.01 |
| 4 | 0.3 | 2 | 4 | 0.32 | 0.02 | 5 | 0.31 | 0.00 |
| 5 | 0.44 | 2 | 6 | 0.47 | 0.03 | 7 | 0.43 | 0.01 |
| 6 | 0.59 | 2 | 7 | 0.56 | 0.03 | 9 | 0.55 | 0.04 |
| 7 | 0.37 | 4 | 10 | 0.36 | 0.01 | 13 | 0.39 | 0.02 |
| 8 | 0.48 | 4 | 12 | 0.45 | 0.03 | 15 | 0.48 | 0.00 |
| 9 | 0.6 | 4 | 14 | 0.51 | 0.02 | 16 | 0.53 | 0.07 |
| 10 | 0.46 | 6 | 17 | 0.46 | 0.01 | 20 | 0.45 | 0.00 |
| 11 | 0.55 | 6 | 19 | 0.55 | 0.01 | 23 | 0.58 | 0.02 |
| 12 | 0.65 | 6 | 21 | 0.64 | 0.01 | 25 | 0.64 | 0.01 |
| 13 | 0.75 | 6 | 23 | 0.75 | 0.00 | 27 | 0.72 | 0.03 |
| 14 | 0.85 | 6 | 25 | 0.85 | 0.01 | 28 | 0.84 | 0.02 |
| 15 | 0.93 | 6 | 26 | 0.91 | 0.01 | 28 | 0.84 | 0.09 |

Table 6	CQI-to-MCS table and difference to the target coding rate for CRS-based Subslot-PDSCH.

|  |  |  | Subslot 1 |  |  | Subslot 2 |  |  | Subslot 3 |  |  | Subslot 4 | Subslot 5 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PRB | 50 |  |  | 50 |  |  | 50 |  |  | 50 | 50 |  |  |
|  |  | Available REs | 1408 |  |  | 1008 |  |  | 872 |  |  | 1008 | 1472 |  |  |
| CQI index | Code rate | Modulation |  | CR | Diff |  | CR | Diff |  | CR | Diff |  |  | CR | Diff |
| 1 | 0.08 | 2 | 0 | 0.09 | 0.01 | 0 | 0.12 | 0.05 | 0 | 0.14 | 0.07 | 0 | 0 | 0.08 | 0.01 |
| 2 | 0.12 | 2 | 1 | 0.11 | 0.00 | 0 | 0.12 | 0.01 | 0 | 0.14 | 0.03 | 0 | 1 | 0.11 | 0.01 |
| 3 | 0.19 | 2 | 3 | 0.18 | 0.01 | 2 | 0.20 | 0.01 | 1 | 0.18 | 0.01 | 2 | 3 | 0.17 | 0.02 |
| 4 | 0.3 | 2 | 6 | 0.32 | 0.02 | 4 | 0.31 | 0.01 | 3 | 0.28 | 0.02 | 4 | 6 | 0.30 | 0.00 |
| 5 | 0.44 | 2 | 8 | 0.42 | 0.02 | 6 | 0.44 | 0.01 | 5 | 0.44 | 0.00 | 6 | 9 | 0.46 | 0.02 |
| 6 | 0.59 | 2 | 9 | 0.48 | 0.11 | 8 | 0.59 | 0.00 | 7 | 0.61 | 0.02 | 8 | 9 | 0.46 | 0.13 |
| 7 | 0.37 | 4 | 14 | 0.39 | 0.02 | 11 | 0.37 | 0.00 | 10 | 0.39 | 0.02 | 11 | 14 | 0.37 | 0.00 |
| 8 | 0.48 | 4 | 16 | 0.45 | 0.02 | 13 | 0.48 | 0.01 | 12 | 0.49 | 0.01 | 13 | 16 | 0.43 | 0.04 |
| 9 | 0.6 | 4 | 16 | 0.45 | 0.15 | 15 | 0.59 | 0.01 | 14 | 0.62 | 0.02 | 15 | 16 | 0.43 | 0.17 |
| 10 | 0.46 | 6 | 22 | 0.46 | 0.01 | 18 | 0.46 | 0.00 | 17 | 0.49 | 0.03 | 18 | 22 | 0.44 | 0.01 |
| 11 | 0.55 | 6 | 24 | 0.55 | 0.01 | 20 | 0.56 | 0.01 | 18 | 0.53 | 0.03 | 20 | 25 | 0.54 | 0.01 |
| 12 | 0.65 | 6 | 27 | 0.64 | 0.01 | 22 | 0.65 | 0.00 | 20 | 0.65 | 0.00 | 22 | 27 | 0.61 | 0.04 |
| 13 | 0.75 | 6 | 28 | 0.74 | 0.01 | 24 | 0.76 | 0.01 | 22 | 0.75 | 0.01 | 24 | 28 | 0.71 | 0.04 |
| 14 | 0.85 | 6 | 28 | 0.74 | 0.11 | 26 | 0.86 | 0.00 | 24 | 0.88 | 0.03 | 26 | 28 | 0.71 | 0.14 |
| 15 | 0.93 | 6 | 28 | 0.74 | 0.18 | 27 | 0.89 | 0.04 | 25 | 0.92 | 0.01 | 27 | 28 | 0.71 | 0.22 |

Table 7	CQI-to-MCS table and difference to the target coding rate for DMRS-based Slot-PDSCH.

|  |  |  | Non CSI-RS |  |  |  |  |  | 2 CSI-RS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Slot 0 |  |  | Slot 1 |  |  | Slot 0 |  |  | Slot 1 |
|  |  | PRB | 50 |  |  | 50 |  |  | 50 |  |  | 50 |
|  |  | Available REs | 2600 |  |  | 3348 |  |  | 2500 (CSI-RS) |  |  | 3348 |
| CQI index | Code rate | Modulation |  | CR | Diff |  | CR | Diff |  | CR | Diff |  |
| 1 | 0.08 | 2 | 0 | 0.14 | 0.06 | 0 | 0.11 | 0.03 | 0 | 0.14 | 0.07 | 0 |
| 2 | 0.12 | 2 | 0 | 0.14 | 0.02 | 0 | 0.11 | 0.01 | 0 | 0.14 | 0.03 | 0 |
| 3 | 0.19 | 2 | 1 | 0.18 | 0.01 | 2 | 0.17 | 0.02 | 1 | 0.19 | 0.00 | 2 |
| 4 | 0.3 | 2 | 3 | 0.28 | 0.02 | 4 | 0.27 | 0.03 | 3 | 0.29 | 0.01 | 4 |
| 5 | 0.44 | 2 | 5 | 0.43 | 0.01 | 7 | 0.47 | 0.01 | 5 | 0.45 | 0.01 | 7 |
| 6 | 0.59 | 2 | 7 | 0.50 | 0.02 | 9 | 0.60 | 0.00 | 7 | 0.63 | 0.04 | 9 |
| 7 | 0.37 | 4 | 10 | 0.39 | 0.02 | 12 | 0.37 | 0.01 | 10 | 0.40 | 0.03 | 12 |
| 8 | 0.48 | 4 | 12 | 0.48 | 0.00 | 14 | 0.49 | 0.02 | 12 | 0.50 | 0.02 | 14 |
| 9 | 0.6 | 4 | 14 | 0.63 | 0.03 | 16 | 0.58 | 0.02 | 13 | 0.58 | 0.03 | 16 |
| 10 | 0.46 | 6 | 17 | 0.50 | 0.05 | 19 | 0.46 | 0.00 | 17 | 0.52 | 0.07 | 19 |
| 11 | 0.55 | 6 | 18 | 0.53 | 0.02 | 21 | 0.54 | 0.02 | 18 | 0.55 | 0.00 | 21 |
| 12 | 0.65 | 6 | 20 | 0.64 | 0.01 | 23 | 0.63 | 0.02 | 20 | 0.67 | 0.02 | 23 |
| 13 | 0.75 | 6 | 22 | 0.74 | 0.02 | 26 | 0.76 | 0.01 | 22 | 0.77 | 0.01 | 26 |
| 14 | 0.85 | 6 | 24 | 0.87 | 0.02 | 27 | 0.79 | 0.06 | 23 | 0.84 | 0.01 | 27 |
| 15 | 0.93 | 6 | 25 | 0.91 | 0.02 | 28 | 0.92 | 0.01 | 24 | 0.91 | 0.02 | 28 |

Table 8	CQI-to-MCS table and difference to the target coding rate for DMRS-based Subslot-PDSCH (non-CSI-RS subframe).

|  |  |  | Subslot 1 |  |  | Subslot 2 |  |  | Subslot 3 |  |  | Subslot 4 | Subslot 5 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PRB | 50 |  |  | 50 |  |  | 50 |  |  | 50 | 50 |  |  |
|  |  | Available REs | 1180 |  |  | 680 |  |  | 612 |  |  | 680 | 1212 |  |  |
| CQI index | Code rate | Modulation |  | CR | Diff |  | CR | Diff |  | CR | Diff |  |  | CR | Diff |
| 1 | 0.08 | 2 | 0 | 0.11 | 0.03 | 0 | 0.18 | 0.11 | 0 | 0.20 | 0.13 | 0 | 0 | 0.10 | 0.02 |
| 2 | 0.12 | 2 | 0 | 0.11 | 0.01 | 0 | 0.18 | 0.07 | 0 | 0.20 | 0.09 | 0 | 1 | 0.13 | 0.01 |
| 3 | 0.19 | 2 | 2 | 0.17 | 0.02 | 0 | 0.18 | 0.01 | 0 | 0.02 | 0.01 | 0 | 3 | 0.20 | 0.02 |
| 4 | 0.3 | 2 | 5 | 0.33 | 0.02 | 2 | 0.29 | 0.01 | 2 | 0.32 | 0.02 | 2 | 5 | 0.32 | 0.02 |
| 5 | 0.44 | 2 | 7 | 0.45 | 0.01 | 4 | 0.46 | 0.02 | 3 | 0.41 | 0.03 | 4 | 7 | 0.44 | 0.00 |
| 6 | 0.59 | 2 | 9 | 0.57 | 0.02 | 5 | 0.56 | 0.02 | 5 | 0.63 | 0.04 | 5 | 9 | 0.55 | 0.03 |
| 7 | 0.37 | 4 | 12 | 0.36 | 0.01 | 10 | 0.49 | 0.12 | 10 | 0.55 | 0.18 | 10 | 12 | 0.35 | 0.02 |
| 8 | 0.48 | 4 | 14 | 0.46 | 0.02 | 10 | 0.49 | 0.02 | 10 | 0.55 | 0.07 | 10 | 15 | 0.49 | 0.01 |
| 9 | 0.6 | 4 | 16 | 0.54 | 0.06 | 12 | 0.62 | 0.02 | 11 | 0.61 | 0.01 | 12 | 16 | 0.53 | 0.07 |
| 10 | 0.46 | 6 | 19 | 0.44 | 0.01 | 17 | 0.63 | 0.17 | 17 | 0.70 | 0.24 | 17 | 20 | 0.47 | 0.01 |
| 11 | 0.55 | 6 | 22 | 0.55 | 0.00 | 17 | 0.63 | 0.07 | 17 | 0.70 | 0.14 | 17 | 22 | 0.54 | 0.02 |
| 12 | 0.65 | 6 | 24 | 0.65 | 0.00 | 17 | 0.63 | 0.02 | 17 | 0.70 | 0.05 | 17 | 25 | 0.66 | 0.01 |
| 13 | 0.75 | 6 | 27 | 0.76 | 0.00 | 19 | 0.77 | 0.01 | 18 | 0.75 | 0.00 | 19 | 27 | 0.74 | 0.01 |
| 14 | 0.85 | 6 | 28 | 0.89 | 0.03 | 20 | 0.83 | 0.02 | 19 | 0.85 | 0.00 | 20 | 28 | 0.86 | 0.01 |
| 15 | 0.93 | 6 | 28 | 0.89 | 0.04 | 22 | 0.89 | 0.03 | 20 | 0.92 | 0.00 | 22 | 28 | 0.86 | 0.06 |

Table 9	CQI-to-MCS table and difference to the target coding rate for DMRS-based Subslot-PDSCH (CSI-RS subframe).

|  |  |  | Subslot 1 | Subslot 2 | Subslot 3 | Subslot 4 | Subslot 5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PRB | 50 | 50 | 50 | 50 | 50 |
|  |  | Available REs | 1180 | 612 (CSI-RS) | 612 | 680 | 1212 |
| CQI index | Code rate | Modulation |  |  |  |  |  |
| 1 | 0.08 | 2 | 0 | 0 | 0 | 0 | 0 |
| 2 | 0.12 | 2 | 0 | 0 | 0 | 0 | 1 |
| 3 | 0.19 | 2 | 2 | 0 | 0 | 0 | 3 |
| 4 | 0.3 | 2 | 5 | 2 | 2 | 2 | 5 |
| 5 | 0.44 | 2 | 7 | 3 | 3 | 4 | 7 |
| 6 | 0.59 | 2 | 9 | 5 | 5 | 5 | 9 |
| 7 | 0.37 | 4 | 12 | 10 | 10 | 10 | 12 |
| 8 | 0.48 | 4 | 14 | 10 | 10 | 10 | 15 |
| 9 | 0.6 | 4 | 16 | 11 | 11 | 12 | 16 |
| 10 | 0.46 | 6 | 19 | 17 | 17 | 17 | 20 |
| 11 | 0.55 | 6 | 22 | 22 | 17 | 17 | 22 |
| 12 | 0.65 | 6 | 24 | 24 | 17 | 17 | 25 |
| 13 | 0.75 | 6 | 27 | 27 | 18 | 19 | 27 |
| 14 | 0.85 | 6 | 28 | 28 | 19 | 20 | 28 |
| 15 | 0.93 | 6 | 28 | 28 | 20 | 22 | 28 |