3GPP TSG-RAN WG4 Meeting #90	R4-1901431

Athens, Greece, 25 February – 1 March 2019

Source:	Ericsson, BT plc

Title:	TP for TR 37.716-21-11: 1A-3A_n7A, 1A-3C_n7A

Agenda item:			9.4.3

Document for:	Approval

# 1. Introduction

Rel-16 LTE-NR DC combinations LTE 2 band + NR 1 band combination are defined in the WID [1]. This contribution is a text proposal for TR 37.716-21-11 to add 1A-3A_n7A and 1A-3C_n7A.

# 2. Text Proposal

# ---Start of changes---

## 5.2.x	DC_1A-3A_n7A
DC_1A-3C_n7A

### 5.2.x.1	Operating bands for EN-DC

Table 5.2B.5.3-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 1-3_n7 | CA_1-3 | n7 | No |

### 5.2.x.2	Configuration for DC

Table 5.5B.5.3-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-3A_n7A DC_1A-3C_n7A | DC_1A_n7A DC_3A_n7A DC_3C_n7A | CA_1A-3A CA_1A-3C | n7 |

### 5.2.x.3		∆TIB and ∆RIB values

For DC_1-3_n7, the TIB,c and RIB,c values are reused from the LTE combination CA_1-3-7, and are given in the tables below.

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n7 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n7 | 0.6 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3_n7 | 1 | 0 |
|  | 3 | 0 |
|  | n7 | 0 |

---End of changes---

# Reference

[1]	RP-182557, “Revised WID on Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL)”, Huawei