3GPP TSG-RAN4 Meeting #90	R4-1901513

# Athens, Greece, 25 February - 1 March, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR |  | rev |  | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR to 38.101-3: SUL clarifications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | SUL related requirements and constraints are not clear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following corrections and clarifications are made: - SUL related abbreviations are added. - UE behaviour for TDM based ULSUP is defined. - Single UL operations are allowed for TDM based ULSUP in EN-DC. In case of TDM based ULSUP single UL is always allowed, not only for difficult configurations of difficult EN-DC band combinations - E-UTRA and NR switching time mask is only defined for the intra-band scenarios, for which single UL is allowed. - Clarifications are added for the applicability for switching time mask requirements - Allowed reference sensitivity relaxation due to support for CA or DC operation reference sensitivity (ΔRIB,c) is removed from the SUL bands, which do not have any DL |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Network may have wrong interpretations of various different UE supports and requirements related to SUL, UL sharing from the UE perspective and also switching time masks applicability for the intra-band EN-DC scenarios. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 4.2, 4.3, 5.2B.4.2, 6.3B.1.1, 6.3B.2 and 7.3B.3.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### ---------------------------------------Start of Change 1-------------------------------

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

<defined term>: <definition>.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

ΔRIB,c	Allowed reference sensitivity relaxation due to support for CA or DC operation, for serving cell c.

ΔTIB,c	Allowed maximum configured output power relaxation due to support for CA or DC operation, for serving cell c

BWLTE_Channel	Channel bandwidth of E-UTRA carrier

BWLTE_Channel_CA	Channel bandwidth of E-UTRA sub-block which is composed of intra-band contiguous CA E-UTRA carriers

BWNR_Channel	Channel bandwidth of NR carrier

BWNR_Channel_CA	Channel bandwidth of NR sub-block which is composed of intra-band contiguous CA NR carriers

Ceil(x)	Rounding upwards; ceil(x) is the smallest integer such that ceil(x) ≥ x

EN-DCACLR	The ratio of the filtered mean power centred on the aggregated sub-block bandwidth ENBW to the filtered mean power centred on an adjacent bandwidth of the same size ENBW

E-UTRAACLR	E-UTRA ACLR

FC	RF reference frequency for the carrier center on the channel raster

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

FOOB	The boundary between the NR out of band emission and spurious emission domains

LCRB	Transmission bandwidth which represents the length of a contiguous resource block allocation

expressed in units of resources blocks

Max()	The largest of given numbers

Min()	The smallest of given numbers

NRACLR	NR ACLR

NRB	Transmission bandwidth configuration, expressed in units of resource blocks

PCMAX	The configured maximum UE output power

RBstart	Indicates the lowest RB index of transmitted resource blocks

Wgap	The sub-block gap between the two sub-blocks

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

A-MPR	Additional Maximum Power Reduction

BCS	Bandwidth Combination Set

CA	Carrier Aggregation

CC	Component carrier

DC	Dual Connectivity

EN-DC	E-UTRA/NR DC

EVM	Error Vector Magnitude

FDM	Frequency Division Multiplexing

FR	Frequency Range

ENBW	The aggregated bandwidth of an E-UTRA sub-block and an adjacent NR sub-block

ITU R	Radiocommunication Sector of the International Telecommunication Union

MBW	Measurement bandwidth defined for the protected band

MPR	Allowed maximum power reduction

MSD	Maximum Sensitivity Degradation

MCG	Master Cell Group

NR	New Radio

NS	Network Signalling

NSA	Non-Standalone, a mode of operation where operation of an other radio is assisted with an other radio

OOB	Out-of-band

OOBE	Out-of-band emission

OTA	Over The Air

PRB	Physical Resource Block

RE	Resource Element

REFSENS	Reference Sensitivity

RF	Radio Frequency

Rx	Receiver

SCG	Secondary Cell Group

SCS	Subcarrier spacing

SEM	Spectrum Emission Mask

SUL	Supplementary uplink

TDM	Time Division Multiplex

Tx	Transmitter

UE	User Equipment

UL-MIMO	Up Link Multiple Antenna transmission

ULSUP	Uplink sharing from UE perspective

ULSUP-FDM	FDM based ULSUP where NR UL and E-UTRA UL may be transmitted simultaneously on the same channel, which is shared by E-UTRA and NR

ULSUP-TDM	TDM based ULSUP where NR UL and E-UTRA UL are not transmitted simultaneously on the same channel, which is shared by E-UTRA and NR

# 4	General

## 4.1	Relationship between minimum requirements and test requirements

The present document is interwork specification for NR UE, covering RF characteristics and minimum performance requirements. Conformance to the present specification is demonstrated by fulfilling the test requirements specified in the conformance specification 3GPP TS 38.521-3 [5].

The Minimum Requirements given in this specification make no allowance for measurement uncertainty. The test specification TS 38.521-3 [5] defines test tolerances. These test tolerances are individually calculated for each test. The test tolerances are used to relax the minimum requirements in this specification to create test requirements. For some requirements, including regulatory requirements, the test tolerance is set to zero.

The measurement results returned by the test system are compared - without any modification - against the test requirements as defined by the shared risk principle.

The shared risk principle is defined in Recommendation ITU-R M.1545 [6].

## 4.2	Applicability of minimum requirements

a)	In this specification the Minimum Requirements are specified as general requirements and additional requirements. Where the Requirement is specified as a general requirement, the requirement is mandated to be met in all scenarios

b)	For specific scenarios for which an additional requirement is specified, in addition to meeting the general requirement, the UE is mandated to meet the additional requirements.

c)	The spurious emissions power requirements are for the long-term average of the power. For the purpose of reducing measurement uncertainty it is acceptable to average the measured power over a period of time sufficient to reduce the uncertainty due to the statistical nature of the signal

d)	Terminal that supports EN-DC configuration shall meet E-UTRA requirements as specified in TS 36.101 [4] and NR requirements as in TS 38.101-1 [2] and TS 38.101-2 [3] unless otherwise specified in this specification

e)	All the requirements for intra-band contiguous and non-contiguous CA apply under the assumption of the same uplink-downlink and special subframe configurations in the PCell and SCells for SA.

f)	For a terminal that supports TDM based ULSUP, the current version of the specification assumes the terminal is not required to transmit simultaneously NR SUL and E-UTRA UL on the same channel, which is shared by E-UTRA and NR and the UE requirements for single carrier shall apply for the active NR SUL or E-UTRA UL accordingly.

A terminal which supports an inter-band EN-DC configuration shall support all apecified E-UTRA bandwidth combination set that belong to the E-UTRA CA configuration part of E-UTRA – NR DC and shall support all apecified NR bandwidth combination set that belong to the NR CA configuration part of E-UTRA – NR DC.

A terminal which supports an inter-band EN-DC configuration with a certain UL configuration shall support the all lower order DL configurations of the lower order EN-DC combinations, which have this certain UL configuration and the fallbacks of this UL configuration.

## 4.3	Specification suffix information

Unless stated otherwise the following suffixes are used for indicating at 2nd level subclause, shown in Table 4.3-1.

Table 4.3-1: Definition of suffixes

| Clause suffix | Variant |
| --- | --- |
| None | Single Carrier |
| A | Carrier Aggregation (CA) |
| B | Dual-Connectivity (DC) with and without SUL including TDM and FDM based UL sharing from UE perspective |
| C |  |
| D | UL MIMO |

### ---------------------------------------End of Change 1-------------------------------

### ---------------------------------------Start of Change 2-------------------------------

#### 5.2B.4.2	EN-DC (three bands)

Table 5.2B.4.2-1: Band combinations for inter-band EN-DC within FR1 (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3_n28 | CA_1-3 | n28 | No |
| DC_1-3_n772 | CA_1-3 | n77 | DC_1_n77, DC_3_n77 |
| DC_1-3_n782 | CA_1-3 | n78 | DC_3_n78 |
| DC_1-3_n792 | CA_1-3 | n79 | No |
| DC_1-5_n782 | CA_1-5 | n78 | No |
| DC_1-7_n282 | CA_1-7 | n28 | No |
| DC_1-7_n78 2 | CA_1-7 | n78 | No |
| DC_1-7-7_n782 | CA_1-7-7 | n78 | No |
| DC_1-8_n782 | CA_1-8 | n78 | No |
| DC_1-18_n772 | CA_1-18 | n77 | DC_1_n77 |
| DC_1-18_n782 | CA_1-18 | n78 | No |
| DC_1-18_n79 | CA_1-18 | n79 | No |
| DC_1-19_n772 | CA_1-19 | n77 | DC_1_n77 |
| DC_1-19_n782 | CA_1-19 | n78 | No |
| DC_1-19_n792 | CA_1-19 | n79 | No |
| DC_1-20_n283 | CA_1-20 | n28 | No |
| DC_1-20_n782 | CA_1-20 | n78 | No |
| DC_1-21_n772 | CA_1-21 | n77 | DC_1_n77 |
| DC_1-21_n782 | CA_1-21 | n78 | No |
| DC_1-21_n792 | CA_1-21 | n79 | No |
| DC_1-28_n772 | CA_1-28 | n77 | DC_1_n77 |
| DC_1-28_n782 | CA_1-28 | n78 | No |
| DC_1-28_n79 | CA_1-28 | n79 | No |
| DC_1_n28-n782 | 1 | CA_n28-n78 | No |
| DC_1_n77-n79 | 1 | CA_n77-n79 | No |
| DC_1_n78-n79 | 1 | CA_n78-n79 | No |
| DC_1-41_n77 | CA_1-41 | n77 | DC_1_n77 |
| DC_1-41_n78 | CA_1-41 | n78 | No |
| DC_1-41_n79 | CA_1-41 | n79 | No |
| DC_1-42_n77 | CA_1-42 | n77 | DC_1_n77 |
| DC_1-42_n78 | CA_1-42 | n78 | No |
| DC_1-42_n79 | CA_1-42 | n79 | No |
| DC_1_SUL_n78-n842 | 1 | SUL_n78-n84 | No4 |
| DC_2-5_n66 | CA_2-5 | n66 | No |
| DC_2-12_n66 | CA_2-12 | n66 | No |
| DC_2-30_n66 | CA_2-30 | n66 | No |
| DC_2-(n)71 | CA_2-71 | n71 | No |
| DC_2-66_n71 | CA_2-66 | n71 | No |
| DC_3_n3-n77 | 3 | CA_n3-n77 | DC_3_n3 |
| DC_3_n3-n78 | 3 | CA_n3-n78 | DC_3_n3 |
| DC_3-5_n782 | CA_3-5 | n78 | DC_3_n78 |
| DC_3-7_n28 | CA_3-7 | n28 | No |
| DC_3-7_n782 | CA_3-7 | n78 | DC_3_n78 |
| DC_3-7-7_n782 | CA_3-7-7 | n78 | DC_3_n78 |
| DC_3-8_n78 | CA_3-8 | n78 | DC_3_n78 |
| DC_3-19_n772 | CA_3-19 | n77 | DC_3_n77 |
| DC_3-19_n782 | CA_3-19 | n78 | DC_3_n78 |
| DC_3-19_n792 | CA_3-19 | n79 | No |
| DC_3-20_n282,3 | CA_3-20 | n28 | No |
| DC_3-20_n782 | CA_3-20 | n78 | DC_3_n78 |
| DC_3-21_n772 | CA_3-21 | n77 | DC_3_n77 |
| DC_3-21_n782 | CA_3-21 | n78 | DC_3_n78 |
| DC_3-21_n792 | CA_3-21 | n79 | No |
| DC_3-28_n782 | CA_3-28 | n78 | No |
| DC_3_n28-n782 | 3 | CA_n28-n78 | DC_3_n78 |
| DC_3-28_n79 | 3 | CA_n28-n79 | No |
| DC_3-38_n78 | CA_3-38 | n78 | DC_3_n78 |
| DC_3-41_n78 | CA_3-41 | n78 | DC_3_n78 |
| DC_3-42_n77 | CA_3-42 | n77 | DC_3_n77 |
| DC_3-42_n78 | CA_3-42 | n78 | DC_3_n78 |
| DC_3-42_n79 | CA_3-42 | n79 | No |
| DC_3_n77-n79 | 3 | CA_n77-n79 | No |
| DC_3_n78-n79 | 3 | CA_n78-n79 | DC_3_n78 |
| DC_3_SUL_n78-n802 | 3 | SUL_n78-n80 | DC_3_n784 |
| DC_3_SUL_n78-n822 | 3 | SUL_n78-n821 | DC_3_n784 |
| DC_3_SUL_n79-n802 | 3 | SUL_n79-n80 | No4 |
| DC_5-7-7_n78 | CA_5-7-7 | n78 | No |
| DC_5-7_n78 | CA_5-7 | n78 | No |
| DC_5-30_n66 | CA_5-30 | n66 | DC_5_n66 |
| DC_7-20_n283 | CA_7-20 | n28 | No |
| DC_7-20_n782 | CA_7-20 | n78 | No |
| DC_7-28_n782 | CA_7-28 | n78 | No |
| DC_7_n28-n782 | 7 | CA_n28-n78 | No |
| DC_7-46_n78 | CA_7-46 | n78 | No |
| DC_8_SUL_n78-n812 | 8 | SUL_n78-n81 | No4 |
| DC_8_SUL_n79-n812 | 8 | SUL_n79-n81 | No4 |
| DC_12-30_n66 | CA_12-30 | n66 | No |
| DC_18-28_n772 | CA_18-28 | n77 | No |
| DC_18-28_n782 | CA_18-28 | n78 | No |
| DC_18-28_n792 | CA_18-28 | n79 | No |
| DC_19-21_n772 | CA_19-21 | n77 | No |
| DC_19-21_n782 | CA_19-21 | n78 | No |
| DC_19-21_n792 | CA_19-21 | n79 | No |
| DC_19-42_n77 | CA_19-42 | n77 | No |
| DC_19-42_n78 | CA_19-42 | n78 | No |
| DC_19-42_n79 | CA_19-42 | n79 | No |
| DC_19_n77-n79 | 19 | CA_n77-n79 | No |
| DC_19_n78-n79 | 19 | CA_n78-n79 | No |
| DC_20_n8-n75 | 20 | CA_n8-n75 | DC_20_n8 |
| DC_20_n28-n753 | 20 | CA_n28-n75 | No |
| DC_20_n28-n782,3 | 20 | CA_n28-n78 | No |
| DC_20_n75-n782 | 20 | CA_n75-n78 | No |
| DC_20_n76-n782 | 20 | CA_n76-n78 | No |
| DC_20_SUL_n78-n822 | 20 | SUL_n78-n82 | No4 |
| DC_20_SUL_n78-n832 | 20 | SUL_n78-n831 | No4 |
| DC_21-42_n77 | CA_21-42 | n77 | No |
| DC_21-42_n78 | CA_21-42 | n78 | No |
| DC_21-42_n79 | CA_21-42 | n79 | No |
| DC_21_n77-n79 | 21 | CA_n77-n79 | No |
| DC_21_n78-n79 | 21 | CA_n78-n79 | No |
| DC_28-42_n77 | CA_28-42 | n77 | No |
| DC_28-42_n78 | CA_28-42 | n78 | No |
| DC_28-42_n79 | CA_28-42 | n79 | No |
| DC_41-42_n77 | CA_41-42 | n77 | No |
| DC_41-42_n78 | CA_41-42 | n78 | No |
| DC_41-42_n79 | CA_41-42 | n79 | No |
| DC_28_SUL_n78-n832 | 28 | SUL_n78-n83 | No |
| DC_66_(n)71 | CA_66-71 | n71 | No |
| DC_66_SUL_n78-n862 | 66 | SUL_n78-n86 | DC_66_n784 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier can be up to 140us and placed in SUL resources. NOTE 2: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability NOTE 3: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. NOTE 4: If a UE supports ULSUP-TDM and is configured with ULSUP-TDM for the same channel, which is shared by E-UTRA and NR , single UL is allowed between E-UTRA UL and NR SUL on this channel. |  |  |  |

### ---------------------------------------End of Change 2-------------------------------

### ---------------------------------------Start of Change 3-------------------------------

## 6.3	Output power dynamics

Output power dynamics for CA operations in FR1 and FR2 as specified in TS 38.101-1 [2] and TS 38.101-2 [3], respectively.

Output power dynamics for EN-DC operations in FR1 and FR2 as specified in TS 38.101-1 [2] and TS 38.101-2 [3], respectively. E-UTRA as specified in TS 36.101 [4]. For intra-band contiguous EN-DC operation in FR1, minimum output power requirements specified in sub-clause 6.3.1 of TS 38.101-1 [2] and sub-clause 6.3.2 of TS 36.101 [4] shall only apply when the power of all NR and E-UTRA carriers are set to minimum value.  Similarly, OFF power requirements specified in sub-clause 6.3.2 of TS 38.101-1 [2] and sub-clause 6.3.3 of TS 36.101 [4] shall only apply when the power of all NR and E-UTRA carriers are OFF.  The OFF power condition in transmit ON/OFF time mask requirements specified in sub-clause 6.3.3 of TS 38.101-1 [2] and sub-clause 6.3.4 of TS 36.101 [4] is applicable only when all NR and E-UTRA carriers are OFF.  If both E-UTRA and NR transition between ON and OFF states simultaneously, the longer transient time shall apply to both.  If either E-UTRA or NR is OFF and the other carrier transitions from OFF to ON, then the transiet time associated with that carrier applies.

6.3B	Output power dynamics for DC

### 6.3B.1	Output power dynamics for EN-DC with UL sharing from UE perspective and intra-band contiguous scenario

#### 6.3B.1.1	E-UTRA and NR switching time mask for TDM based UL sharing from UE perspective and intra-band contiguous scenario

The E-UTRA and NR switching time mask is only applicable for non-simultaneous transmissions between E-UTRA and NR in TDM based UL sharing from the UE perspective in the same channel, which is shared by E-UTRA and NR, and intra-band contiguous scenario in adjacent channels when single UL is allowed.

The E-UTRA and NR switching time mask defines the observation period between E-UTRA subframe and NR slot/mini-slot boundary. Both E-UTRA subframe and NR slot/mini-slot have ON power transmissions. The ON power is defined as the mean power over the symbol duration excluding any transient period. For E-UTRA subframe or NR slot/mini-slot having OFF power transmission, the general time mask for E-UTRA or NR shall apply.

For UEs reporting E-UTRA and NR switching time capability of type 1 with switching time <0.5us for TDM based UL sharing from UE perspective and for intra-band contiguous scenario within FR1 when single UL is allowed, time masks in Figure 6.3B.1.1-1 and Figure 6.3B.1.1-2 shall apply. For UEs reporting E-UTRA and NR switching time capability of type 2 with switching time <20us for TDM based UL sharing from UE perspective within FR1, time masks in Figure 6.3B.1.1-3 and Figure 6.3B.1.1-4 shall apply.

Figure 6.3B.1.1-1: E-UTRA to NR switching time mask for type 1 for TDM based UL sharing from UE perspective and for intra-band contiguous scenario within FR1 when single UL is allowed

Figure 6.3B.1.1-2: NR to E-UTRA switching time mask for type 1 for TDM based UL sharing from UE perspective and for intra-band contiguous scenario within FR1 when single UL is allowed

Figure 6.3B.1.1-3: E-UTRA to NR switching time mask for type 2 for TDM based UL sharing from UE perspective within FR1

Figure 6.3B.1.1-4: NR to E-UTRA switching time mask for type 2 for TDM based UL sharing from UE perspective within FR1

### 6.3B.2	Output power dynamics for intra-band non-contigious switching time

For DC_3A_n3A single switched UL operation in Rel.15, maximum UL switching time is defined as 120 us and DL reception interruption is allowed during UL switching. Time masks in Figure 6.3B.2-1 and Figure 6.3B.2-2 shall apply.

Figure 6.3B.2-1: E-UTRA to NR switching time mask for intra-band non-contigious scenario when single UL is allowed

Figure 6.3B.2-2: NR to E-UTRA switching time mask for intra-band non-contigious scenario when single UL is allowed

### ---------------------------------------End of Change 3-------------------------------

### ---------------------------------------Start of Change 4-------------------------------

##### 7.3B.3.3.2	ΔRIB,c for EN-DC three bands

Table 7.3B.3.3.2-1: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-3_n28 | n28 | 0.2 |
| DC_1-3_n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n77 | 0.5 |
| DC_1-3_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_1-5_n78 | 1 | 0.2 |
|  | 5 | 0.2 |
|  | n78 | 0.5 |
| DC_1-7_n28 | n28 | 0.2 |
| DC_1-7_n78 DC_1-7-7_n78 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
| DC_1-8_n78 | 8 | 0.2 |
|  | n78 | 0.5 |
| DC_1-18_n77 | n77 | 0.5 |
| DC_1-18_n78 | n78 | 0.5 |
| DC_1-19_n77 | n77 | 0.5 |
| DC_1-19_n78 | n78 | 0.5 |
| DC_1-19_n79 | 1 | 0.3 |
|  | 19 | 0.3 |
| DC_1-20_n28 | 1 | 0.0 |
|  | 20 | 0.2 |
|  | n28 | 0.2 |
| DC_1-20_n78 | n78 | 0.5 |
| DC_1-21_n77 | n77 | 0.5 |
| DC_1-21_n78 | 1 | 0.2 |
|  | n78 | 0.5 |
| DC_1-28_n77 | 28 | 0.2 |
|  | n77 | 0.5 |
| DC_1-28_n78 | 28 | 0.2 |
|  | n78 | 0.5 |
| DC_1_n28-n78 | 1 | 0 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| DC_1_n28-n79 | 1 | 0.3 |
|  | 28 | 0.3 |
| DC_1-42_n77 | 1 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-41_n77 | n77 | 0.5 |
| DC_1-41_n78 | n78 | 0.5 |
| DC_1-42_n78 | 1 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_1-42_n79 | 42 | 0.5 |
| DC_1_n77-n79 | 1 | 0.2 |
|  | n77 | 0.5 |
|  | n79 | 0.0 |
| DC_1_n78-n79 | 1 | 0.0 |
|  | n78 | 0.5 |
|  | n79 | 0.0 |
| DC_1-SUL_n78-n84 | n78 | 0.5 |
| DC_2_5_n66 | 2 | 0.3 |
|  | n66 | 0.3 |
| DC_2_30_n66 | 2 | 0.4 |
|  | 30 | 0.5 |
|  | n66 | 0.4 |
| DC_2-66_n71B | 2 | 0.3 |
|  | 66 | 0.3 |
| DC_3_n3-n77 | 3 | 0.2 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |
| DC_3_n3-n78 | 3 | 0.2 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |
| DC_3-5_n78 | 3 | 0.2 |
|  | 5 | 0.2 |
|  | n78 | 0.5 |
| DC_3-7_n78, DC_3-7-7_n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
| DC_3-8_n78 | 3 | 0.2 |
|  | 8 | 0.2 |
|  | n78 | 0.5 |
| DC_3-19_n77 | 3 | 0.2 |
|  | n77 | 0.5 |
| DC_3-19_n78 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_3-20_n28 | 20 | 0.1 |
|  | n28 | 0.1 |
| DC_3-19_n79 |  |  |
| DC_3-20_n78 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_3-21_n77 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n77 | 0.5 |
| DC_3-21_n78 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n78 | 0.5 |
| DC_3-21_n79 | 3 | 0.3 |
|  | 21 | 0.5 |
| DC_3-28_n78 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_3_n28-n78 | 3 | 0.2 |
|  | n28 | 0 |
|  | n78 | 0.5 |
| DC_3-38_n78 | 3 | 0.2 |
|  | 38 | 0.4 |
|  | n78 | 0.5 |
| DC_3-41_n78 | 3 | 0.2 |
|  | 41 | 01 |
|  |  | 0.52 |
|  | n78 | 0.5 |
| DC_3-42_n77 | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_3-42_n78 | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_3-42_n79 | 3 | 0.2 |
|  | 42 | 0.5 |
| DC_3_n77-n79 | 3 | 0.2 |
|  | n77 | 0.5 |
|  | n79 | 0.0 |
| DC_3_n78-n79 | 3 | 0.2 |
|  | n78 | 0.5 |
|  | n79 | 0.0 |
| DC_3-SUL_n78-n80 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_3-SUL_n78-n82 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_5-7_n78 | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
| DC_5_30_n66 | 30 | 0.5 |
|  | n66 | 0.4 |
| DC_7-7_n78 | 7 | 0.0 |
|  | n78 | 0.5 |
| DC_7-20_n28 | 20 | 0.2 |
|  | n28 | 0.2 |
| DC_7-20_n78 | n78 | 0.5 |
| DC_7-28_n78 | n78 | 0.5 |
| DC_7_n28-n78 | n78 | 0.5 |
| DC_7-46_n78 | n78 | 0.5 |
| DC_8A-SUL_n78-n81 | 8 | 0.2 |
|  | n78 | 0.2 |
| DC_18-28_n77 | n77 | 0.5 |
| DC_18-28_n78 | n78 | 0.5 |
| DC_19-21_n77 | n77 | 0.5 |
| DC_19-21_n78 | n78 | 0.5 |
| DC_19-42_n77 | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_19-42_n78 | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_19-42_n79 | 42 | 0.5 |
| DC_19_n77-n79 | 19 | 0.0 |
|  | n77 | 0.5 |
|  | n79 | 0.0 |
| DC_19_n78-n79 | 19 | 0.0 |
|  | n78 | 0.5 |
|  | n79 | 0.0 |
| DC_20_n8-n75 | 20 | 0.0 |
|  | n8 | 0.0 |
|  | n75 | 0.0 |
| DC_20_n28-n75 | 20 | 0.0 |
|  | n28 | 0.2 |
|  | n75 | 0.0 |
| DC_20_n28-n78 | 20 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| DC_20_n75-n78 | 20 | 0.0 |
|  | n75 | 0.0 |
|  | n78 | 0.5 |
| DC_20_n76-n78 | 20 | 0.0 |
|  | n76 | 0.0 |
|  | n78 | 0.5 |
| DC_20-SUL_n78-n82 | n78 | 0.5 |
| DC_20-SUL_n78-n83 | 20 | 0.2 |
|  | n78 | 0.5 |
| DC_21-42_n77 | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_21-42_n78 | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_21-42_n79 | 42 | 0.5 |
| DC_21_n77-n79 | 21 | 0.0 |
|  | n77 | 0.5 |
|  | n79 | 0.0 |
| DC_21_n78-n79 | 21 | 0.0 |
|  | n78 | 0.5 |
|  | n79 | 0.0 |
| DC_28-SUL_n78-n83 | 28 | 0.2 |
|  | n78 | 0.5 |
| DC_28-42_n77 | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_28-42_n78 | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_28-42_n79 | 28 | 0.2 |
|  | 42 | 0.5 |
| DC_41-42_n77 | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_41-42_n78 | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_41-42_n79 | 42 | 0.5 |
| DC_41_n77 | n77 | 0.5 |
| DC_41_n78 | n78 | 0.5 |
| DC_41_n79 | n79 | 0.5 |
| DC_66-SUL_n78-n86 | 66 | 0.2 |
|  | n78 | 0.5 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

### ---------------------------------------End of Change 4-------------------------------