# 3GPP TSG-RAN WG4 Meeting #90	                                                         R4-1901562

# Athens, GR, 25th  Feb – 1st March, 2019

# Agenda Item:       6.10.4.3.1.2

# Source:                CAICT

# Title: 	2D Dynamic Beamforming Compact Range for OTA Testing of 5G NR AAU

# Document for:     Approval

Introduction

In this paper, a new 2D dynamic beamforming compact range（DBCR） testing system for 5G NR AAU OTA test is proposed.

The system mainly controls the 256 probes array by amplitude and phase modulation and generates 1 m*1 m quiet zone at 2.4M distance. It can test both the uplink and downlink specifications of AAU, such as TRP, EIRP, ACLR, EVM and EIS.

Discussion

Description

Different from the traditional far-field test,2D DBCR system can greatly reduce the cost at the same size of quiet zone.

Figure 1: DBFC construction and Setup

2D DBFC consists of the following equipment for recommended setup:

- Full anechoic shielded room

- Probes

  - 2D antenna array such as 256

  - Signal distribution network (SDN) for signal split or combination together with control of complex weighting of individual RF channels

  - Interface

- DUT positioner

- System bench including:

  - Cables，amplitude and phase modulation matrix between different measurement instruments

  - Vector Network Analyser: For calibration and passive AUT measurements

  - Vector Signal Generator: For generation of CW or modulated signals to test DUT receiver parameters

  - Vector Signal Analyser: For measurement of CW or modulated signals generated by the DUT transmitter

  - Oscilloscope: For measurement of CW or modulated signals generated by the DUT transmitter

- Controller PC

Simulation

In order to generate plane wave in a relatively short distance, we design and control each probe by a set of algorithms. Through the electromagnetic numerical simulation design, taking into account the frequency band, the amplitude and phase control, the spacing of probes, array arrangement and other factors, the following design results can be obtained at a distance of 2.4 meters.

Figure 2 shows the magnitude and phase distributions in the quiet zone with from the face view (longitudinal yz plane).

Figure 2: Variation of electric field magnitude (left) and phase (right) in the transverse plane

Testing capability and results

# The overall plan is including: Chamber@4.5m*3.5m*3m ，256 probs@2m*2m，4 Axis 80kg turn table， test software, spectrum generator  and spectrum analyzer  etc. The test distance is 2.4m.

  - Comparison with CATR using a real  NR BS passive antenna array

| 2-DATR CATR 2-DCATR CATR | Comparison sites：CATR @Tsinghua University Chamber Size：20m×10m×10m; FB：400 MHz–40 GHz; QZ Level：<-40dB@1GHz~2GHz, <- 50dB@2GHz~40GHz; QZ size： Forward section 10m ; Backward section13m. |
| --- | --- |

3.2Testing capability and data

BS configuration

|  | TX/RX channels | Antenna Array | Polarization | TX power/channel | TRP |
| --- | --- | --- | --- | --- | --- |
| gNBconfig | 64 | 192 | Dual 45° ,line | 3W | 200W |

Test results

| Traffic Beam @Max Gain Point (manufacturers declaration) | EIRP（dBm） | EVM-64QAM | EVM-256QAM | Occupied Bandwidth（MHz） | Frequency Error -64QAM（Hz） | Frequency Error -256QAM（Hz） | EIS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 76.82 | 2.00% | 1.68% | 97.6 | 1.29 | -1.37 | -119.5@qpsk mcs4 51RB |

Conclusion

The 2D DBFC test system can obtain the same test ability and similar accuracy to FF and CATR. And with the additional advantages of path loss, dynamic range and so on, the test can be completed at very low cost. It is a good measurement method, hoping to provide an alternative low-cost solution for the industry.

Proposal 1: Consider the 2D DBCR as an OTA method for measurement of radiated parameters of active antenna basestations, including both antenna and transceiver performance metrics.

# 4. References

[1] 3GPP TS 38.141-1: “NR, Base Station (BS) conformance testing, Part 1: Conducted conformance testing”

[2] 3GPP TS 38.141-2: “NR, Base Station (BS) conformance testing, Part 2: Radiated conformance testing”

[3] 3GPP TR 37.843: “Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS) radiated requirements ”

[4] 3GPP TR 38.817-02:” NR; General aspects for Base Station (BS) Radio Frequency (RF) for NR”

[5] 3GPP TS 38.104 NR: “Base Station (BS) radio transmission and reception”

[6] R4-1806605 2D Compact Range for Testing of AAS Basestations