3GPP TSG-RAN WG4 Meeting #90 	R4-1901688

# Athens, Greece, 25 February – 1 March, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.817-01 | CR | 0010 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TR 38.817-01 on GSCN raster ranges |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The TR contains a description of how the valid GSCN range for each operating band is calculated in 4.3.1.4. In paralell in RAN4, a spread sheet was developed that performs these calculations automatically for each operating band. It would be very useful to make the latest version of the spread sheet available through the TR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The spread sheet (attached) is included in the TR, with a short introduction text. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spread sheet would not be available throguh the 3GPP documentation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | The spread sheet is presented in more detail in R4-1901687. |  |  |  |  |  |  |  |  |

#### 4.3.1.4	Synchronization Channel raster

Sync raster is defined such that there is a minimum number of entries for each band. Sync raster entries will be included in the specifications for each band. Sync raster entries will be defined for initial system acquisition, sync blocks can be transmitted in other frequency locations if the position is signalled to the UE

The sync block is not RB aligned with the data RBs in the channel as shown in Figure 4.3.1.4-1. Instead, there is an arbitrary offset between the edge of the sync block RBs and the edge of the data RBs in the channel, this offset can be up to 11 REs. This enables multiple channels that are subcarrier grid aligned but not RB grid aligned to use the same sync block location. Different channels that are offset by up to 11 REs in frequency can re-use the same sync block frequency location.

Figure 4.3.1.4-1: Alignment between SS block and channel RBs.

Subcarrier mapping:

-	Sync raster will indicate the position of RE=#0(subcarrier #0) of RB#10 of the SS block.

SS Raster Definition for the 0 – 3.0 GHz range:

-	SS Raster is defined as

-	Raster entry = N * 1200 kHz + M * 50 kHz, N=1:2499, M ϵ {1,3,5}
(For operating bands with SCS spaced channel raster, the value M=3 is used.)

-	Raster entry numbering in this range is in increasing order of frequency, starting numbering at #2 (entry #2=1250 kHz, entry #3=1350 kHz, entry #4=1450 kHz, entry#5=2450 kHz…)

-	Total number of entries 7497

-	Raster applies to bands with 100 kHz channel raster and 15 kHz channel raster with 5 MHz minimum bandwidth (bands might be defined in the future) in this frequency range

SS Raster Definition for the 3.0 - 24.25 GHz range:

-	SS Raster is defined as:

-	Raster entry = 3000 MHz + N * 1.44 MHz, N= 0: 14756

-	Raster entry numbering is in increasing order of frequency, starting at 7499

-	Total number of entries 14757

-	Raster applies to bands with 15 kHz channel raster and 10 MHz minimum bandwidth (n77,n78, n79, etc) in this frequency range

SS Raster Definition for the 24.25-100GHz range

-	SS Raster is defined as:

-	Raster entry = 24250.08 MHz + N * 17.28 MHz, N= 0:4383

-	Raster entry numbering is in increasing order of frequency, starting at 22256

-	Total number of entries 4384

-	Raster applies to bands with 60 kHz channel raster in this frequency range

#### 4.3.1.5	Calculations of sync raster GSCN per operating band

For each operating band, the GSCN that can be used in the band are tabulated in clause 5.4.3.3 of TS 38.104 [7], TS 38.101-1 [4] and TS 38.101-2 [5]. Selection of GSCN for Table(s) is done the following way:

-	Include GSCN that correspond to SS block that completely fit within the channel bandwidth, accounting for guard bands needed.

-	Guard bands are calculated based on the minimum channel BW, the SCS for the SS block and the corresponding NRB (spectrum utilization), assuming that the SS block can be in any position within the transmission BW configuration, including at positions adjacent to the edges.

-	For GSCN ranges with step size <N>, the GSCN numbers selected should be multiples of N (this ensures that overlapping bands will have the same GSCN sequences)

For specific combinations of minimum channel bandwidth and SS block SCS, the GSCN ranges are down-selected using a step size <N>, as shown in Table 4.3.1.5-1.

Table 4.3.1.5-1: Down selection factors (step size).

| Frequency range | Minimum channel bandwidth | SS block SCS | Down selection factor (step size) |
| --- | --- | --- | --- |
| 0 – 3.0 GHz | 10 MHz | 15 kHz | <3> |
| 3.0 - 24.25 GHz | 40 MHz | 30 kHz | <16> |
| 24.25 – 100 GHz | 100 MHz | 240 kHz | <2> |

R4-1808269 [11] is a spreadsheet that was used to generate the tables in clause 5.4.3.3 of TS 38.104 [7], TS 38.101-1 [4] and TS 38.101-2 [5]. The following description for the determination of the GSCN ranges are aligned with the calculations in the spreadsheet.

To determine the GSCN range, let fmin denote the lowest frequency location of the SS block within a band after accounting for the guard band G and the bandwidth encompassing the width of all subcarriers from RE#0 of RB#0 to the centre of RE#0 of RB#10 of the SS block. Let fmax denote the high frequency location of the SS block within a band after accounting for the guard band and the bandwidth from the centre of RE#0 of RB#10 and encompassing the width of all subcarriers until RE#11 of RB#19 of the SS block. Let Flow represent the lowest frequency of the frequency range in clause 4.3.1.4, Fraster represent the raster spacing in MHz, Fshift represent the raster shift in MHz (applicable for the frequency range 0 to 3000MHz).

The lowest frequency location fmin is computed as

fmin=Flow+G+∆fNRBSSNRERB+12

where ∆f is the subcarrier spacing, NRERB=12 is the number of REs per RB, NRBSS=20 is the number of RBs in the SS block.

The highest frequency location fmax is computed as

fmax=Flow+WBand-G-∆fNRBSSNRERB-12

where WBand is the width of the operating band.

The first possible raster location within a band (prior to applying the step size) is given by

fmin≤Flow+N∆Fraster+M∆Fshift

while the last possible raster location within a band (prior to applying the step size) is given by

fmax≥Flow+N∆Fraster+M∆Fshift

where Fshift is defined to be 0 outside the frequency range 0 to 3000MHz and where N (and M) are defined in clause 4.3.1.4.

For the first possible raster location, Nmin is the smallest integer satisfying

Nmin≥1∆Frasterfmin-Flow-max⁡M∆Fshift

and the corresponding value of M (if defined and if multiple values are defined)

M≥1∆Fshiftfmin-Flow-Nmin∆Fraster

For the last possible raster location, Nmax is the largest integer satisfying

Nmax≤fmax-Flow-minM∆Fshift∆Fraster

and the corresponding value of M (if defined and if multiple values are defined)

M≤fmax-Flow-Nmax∆Fraster∆Fshift

For the frequency ranges in clause 4.3.1.4, table 4.3.1.5-2 indicates the formulas used to compute the values of N and M (if necessary).

Table 4.3.1.5-2: Formulas to compute the minimum and maximum values of GSCN

| Type | Parameter | Range | Formula |
| --- | --- | --- | --- |
| For GSCNmin | N (compute first) | 0-3000 MHz | N=fmin-Flow-maxM∆Fshift∆Fraster,Mϵ1,3,53Note |
|  |  | ≥ 3000 MHz | N=fmin-Flow∆Fraster |
|  | M | 0-3000 MHz | M'=maxfmin-Flow-N∆Fraster∆Fshift,1 M=M'+M'+1mod2 |
|  |  | 0-3000 MHz, NOTE | 3 |
|  |  | ≥ 3000 MHz | N/A |
| For GSCNmax | N (compute first) | 0-3000 MHz | N=fmax-Flow-minM∆Fshift∆Fraster,Mϵ1,3,53Note |
|  |  | ≥ 3000 MHz | N=fmax-Flow∆Fraster |
|  | M | 0-3000 MHz | M=minfmax-Flow-N∆Fraster∆Fshift,5 M=M'+M'+1mod2 |
|  |  | 0-3000 MHz, NOTE | 3 |
|  |  | ≥ 3000 MHz | N/A |
| NOTE: Refers to Note in Table 5.4.3.1-1 in TS 38.101-1 [4] |  |  |  |

To compute the GSCN ranges with step size <N>, the N and M (when defined) values computed with the formulas in table 4.3.1.5-2 are used to determine GSCNmin and GSCNmax according to the appropriate raster entry formula from section 4.3.1.4. The first entry in the GSCN range, GSCNmin, is determined by

GSCNmin=NGSCNmin'N

and the last entry in the range, GSCNmax, is determined by

GSCNmax=NGSCNmax'N.

The method described above for calculating the GSCN range [GSCNmin, GSCNmax] for an operating band is implemented in the Excel spread sheet NR sync raster calculations (2019-03).xlsx, which is attached to the Technical Report.