3GPP TSG-RAN WG4 Meeting #90		R4-1901767

Athens, GR, 25th Feb – 1st March, 2019

Agenda Item:	10.2.3.2

Source: 		Rohde & Schwarz

Title: 			Setup considerations for FR2

Document for:	Approval

# Introduction

During RAN4#89, the Way Forward on NR MIMO OTA was approved in [1]. In this way forward, 3D MPAC was agreed as the starting point methodology to be studied for FR2, with special attention to the far field criteria and the test zone definition.

Even though those are critical parameters to define the OTA environment, there are some other that will limit the feasibility of the methodology.

# Feasible SNR

Despite the fact that Noise-limited environment was deemed a priority in the NR MIMO OTA SI due to the implementation challenges the noise-controlled environment introduces, the limitations of achievable SNR in a OTA environment for FR2 can also limit the feasibility of the 3D MPAC implementation.

As shown in [3], the feasible SNR range for FR2 Demod testing has been already discussed for a DFF system and a device with a radiating aperture size of 5 cm. The results of the analysis are shown below in table 1.

| Channel Bandwidth | Maximum SNR |
| --- | --- |
| 100MHz | [18.5dB] |
| 200MHz | [15.5dB] |

Table 1: Maximum feasible SNR for DFF system according to TS 38.810

The SNR has been derived under the assumption that the AWGN power is 6dB higher than the thermal noise floor of the receiver in the DUT and the test is performed in the beam-peak of the UE. Under noise free conditions it can be assumed that the SNR is possibly up to 6dB higher.

However these values have been derived for a single AoA system, for a 3D MPAC system there may be further limitations, i.e. range length, longer required cables, etc. that further reduce the available SNR for the test. Therefore, further studies are needed to analyse the impact of a multiple antenna test system on the feasible SNR range.

Observation 1: The maximum feasible SNR for beam-peak AoA is 18.5 dB.

Proposal 1: further investigate the feasible SNR range for a multiple antenna test system.

# Channel model KPI

The definition of the test volume for a MIMO OTA environment is essentially related to the chosen metrics to evaluate how well the channel models are emulated in a 3D MPAC methodology.

Previous works for LTE MIMO OTA in [2] cover extensive research and results on the verification of Channel Model implementation for sub-6GHz, using the following metrics:

Power delay profile (PDP)

Doppler/Temporal correlation

Spatial Correlation

Cross-polarization

where the Spatial Correlation is the main one to evaluate the test volume.

It has been argued (e.g. in [4]) that the Spatial Correlation becomes less meaningful for mmWave testing due to the fact that beamforming (both at BS and UE sides) reduces the effective number of clusters to simulate. Therefore, the UE beamforming effect in the spherical power spectrum shall be considered in the metric.

In addition to the metrics in [2], [4] and [5] have presented in the Total Variation Distance of PAS an alternative metric to the Spatial Correlation in order to evaluate the similarity of the Power Angular Spectrum produced by the OTA system to the reference PAS, but the main issue is that it requires information (or specific assumptions) on:

UE size

Antenna array aperture

Antenna array structure

Antenna array locations

All the above is against the black-box approach agreed for the NR FR2 Testability SI [3], so further definition of the assumptions to be used for the UE are required.

Proposal 2: further define UE assumptions in order to narrow down the metrics to be used for channel model implementation evaluation.

# Summary

The following observations and proposal have been made in this contribution:

Observation 1: The maximum feasible SNR for beam-peak AoA is 18.5 dB.

Proposal 1: further investigate the feasible SNR range for a multiple antenna test system.

Proposal 2: further define UE assumptions in order to narrow down the metrics to be used for channel model implementation evaluation.

# References

- R4-1815933, "WF on MIMO OTA“, CAICT, RAN4 #89, November 2018

- TR 37.977, Universal Terrestrial Radio Access (UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRA); Verification of radiated multi-antenna reception performance of User Equipment (UE), V15.0.0 (2018-10)

- TR 38.810, NR; Study on test methods, V16.1.0 (2019-01)

- R4-1705831, “Metrics for evaluating RRM/Demodulation Measurement Setup”, Keysight Technologies UK Ltd, RAN4 #83, May 2017

- R4-1814835, “On Feasibility of MPAC Systems for FR2”, Keysight Technologies UK Ltd, RAN4 #89, November 2018