3GPP TSG-RAN WG4 Meeting #90                 	R4-1901860

Athens, Greece, Feb 25th – Mar 1st,2019

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on TCI states for RRM test cases (Section A.3.16) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  |  | Date: |  | 2018-11-02 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Adding TCI states for RRM tests |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding TCI states for RRM tests |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TCI State config will be missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section A.3.16 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS/TR…CR... TS38.521-3 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### A.3.16 TCI State Configuration

#### A.3.16.1 Introduction

This section provides the configuration for TCI sates with both SSB and CSI-RS. This section also provides the configuration for NZP-CSIRS resources to which the physical channels shall be QCL’d.

##### A.3.16.2 TCI states

Table A.3.16.2-1: TCI States

| Parameter | TCI.State.0 | TCI.State.1 | TCI.State.2 | TCI.State.3 |
| --- | --- | --- | --- | --- |
| tci-StateId | Id0 | Id1 | Id2 | Id3 |
| qcl-Type1 | typeC | typeC | typeA | typeA |
| qcl-Type2Note1 | typeD | typeD | typeD | typed |
| referenceSignal | SSB0 | SSB1 | CSI-RS0 | CSI-RS1 |
| Note1: qcl-Type2 of typeD only where applicable. For RRM test cases, this will be only in FR2 |  |  |  |  |

Table A.3.16.2-1: NZP-CSI-RS Resources

| Parameter | CSI-RS.Config.0 | CSI-RS.Config.1 |
| --- | --- | --- |
| nzp-CSI-RS-ResourceId | CSI-RSID0 | CSI-RSID0 |
| resourceMapping | TBD | TBD |
| powerControlOffset | 0 | 0 |
| scramblingID | 0 | 0 |
| periodicityAndOffset | Slots640 | Slots640 |
| qcl-InfoPeriodicCSI-RS | Id0 | Id1 |