3GPP TSG-RAN WG4 Meeting #90               	R4-1902078

Athens, GR, 25th Feb – 1st March 2019

Agenda item:	6.14.2.4.1

Source:	Samsung

Title:	Simulation results for NR PRACH

Document for:	Discussion

# 1	Introduction

During RAN#82 meeting, the status report of Rel-15 NR WI is presented. As mentioned in the report, for NR BS performance requirements, due to lack of simulation results, some requirements, test parameters and configurations are still remaining TBD. Additional work is needed to further alignment simulation results with aiming to specify the requirement, and correct the requirement due to unachievable performance measures. For PRACH, the test parameters with timing offset and timing estimation error tolerance are discussed and agreed as follows:

- Timing offset: For preambles with short sequence, set the timing offset base value as 0, and this offset is increased within the loop, by adding in each step a value of 0.1us, until the end of the tested range, which is 0.8us. Then the loop is being reset and the timing offset is set again to 0us

- Timing estimation error tolerance

Table 1: Timing estimation error tolerance

| PRACH Preamble | PRACH SCS | Proposed time estimation error tolerance for AWGN | Proposed time estimation error tolerance for TDL channel |
| --- | --- | --- | --- |
| Long sequence | 1.25kHz | 1.04 us | 1.04 us + 1.51 us = 2.55 us |
| Short sequence | 15kHz | 0.52 us | 0.52 us + 1.51 us = 2.03 us |
|  | 30kHz | 0.26 us | 0.26 us + 1.51 us = 1.77 us |
|  | 60kHz | 0.13 us | 0.13 us + 0.15 us = 0.28 us |
|  | 120kHz | 0.07 us | 0.07us + 0.15 us = 0.22us |

In this contribution, based on the agreement, the ideal and impairment simulation results are presented.

# 2	Simulation Results

In this section, the ideal and impairment simulation results are provided as Table 2 and Table 5 for FR1 and FR2, based on the agreement, respectively.

Table 2: ideal and impairment simulation results for NR PRACH in FR1 with 2Rx

| Format | SCS(KHz) | AWGN |  | TDLC300-100 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Missed detection probability (<=1%) (ideal) | Missed detection probability (<=1%) (impairment) | Missed detection probability (<=1%) (ideal) | Missed detection probability (<=1%) (impairment) |
| Format 0 | 1.25 | -16.38 | -14.38 | -8.48 | -6.48 |
| Format A1 | 15 | -10.86 | -8.86 | -3.72 | -1.72 |
| Format A1 | 30 | -10.73 | -8.73 | -4.71 | -2.71 |
| Format A2 | 15 | -13.30 | -11.30 | -6.08 | -4.08 |
| Format A2 | 30 | -13.18 | -11.18 | -6.99 | -4.99 |
| Format A3 | 15 | -14.60 | -12.60 | -7.43 | -5.43 |
| Format A3 | 30 | -14.47 | -12.47 | -8.25 | -6.25 |
| Format B4 | 15 | -16.87 | -14.87 | -9.57 | -7.57 |
| Format B4 | 30 | -16.69 | -14.69 | -10.30 | -8.30 |
| Format C0 | 15 | -8.29 | -6.29 | -1.34 | 0.66 |
| Format C0 | 30 | -8.15 | -6.15 | -2.21 | -0.21 |
| Format C2 | 15 | -13.23 | -11.23 | -6.13 | -4.13 |
| Format C2 | 30 | -13.18 | -11.18 | -7.06 | -5.06 |

Table 3: ideal and impairment simulation results for NR PRACH in FR1 with 4Rx

| Format | SCS(KHz) | AWGN |  | TDLC300-100 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Missed detection probability (<=1%) (ideal) | Missed detection probability (<=1%) (impairment) | Missed detection probability (<=1%) (ideal) | Missed detection probability (<=1%) (impairment) |
| Format 0 | 1.25 | -18.88 | -16.88 | -13.47 | -11.47 |
| Format A1 | 15 | -13.27 | -11.27 | -8.60 | -6.60 |
| Format A1 | 30 | -13.17 | -11.17 | -8.59 | -6.59 |
| Format A2 | 15 | -15.49 | -13.49 | -10.67 | -8.67 |
| Format A2 | 30 | -15.42 | -13.42 | -10.71 | -8.71 |
| Format A3 | 15 | -16.77 | -14.77 | -11.91 | -9.91 |
| Format A3 | 30 | -16.73 | -14.73 | -11.93 | -9.93 |
| Format B4 | 15 | -18.89 | -16.89 | -13.90 | -11.90 |
| Format B4 | 30 | -18.77 | -16.77 | -13.77 | -11.77 |
| Format C0 | 15 | -10.84 | -8.84 | -6.25 | -4.25 |
| Format C0 | 30 | -10.71 | -8.71 | -6.28 | -4.28 |
| Format C2 | 15 | -15.49 | -13.49 | -10.67 | -8.67 |
| Format C2 | 30 | -15.41 | -13.41 | -10.69 | -8.69 |

Table 4: ideal and impairment simulation results for NR PRACH in FR1 with 8Rx

| Format | SCS(KHz) | AWGN |  | TDLC300-100 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Missed detection probability (<=1%) (ideal) | Missed detection probability (<=1%) (impairment) | Missed detection probability (<=1%) (ideal) | Missed detection probability (<=1%) (impairment) |
| Format 0 | 1.25 | -21.25 | -19.25 | -17.57 | -15.57 |
| Format A1 | 15 | -15.52 | -13.52 | -12.42 | -10.42 |
| Format A1 | 30 | -15.41 | -13.41 | -11.91 | -9.91 |
| Format A2 | 15 | -17.65 | -15.65 | -14.51 | -12.51 |
| Format A2 | 30 | -17.51 | -15.51 | -13.91 | -11.91 |
| Format A3 | 15 | -18.83 | -16.83 | -15.56 | -13.56 |
| Format A3 | 30 | -18.78 | -16.78 | -15.03 | -13.03 |
| Format B4 | 15 | -20.79 | -18.79 | -17.45 | -15.45 |
| Format B4 | 30 | -20.67 | -18.67 | -16.83 | -14.83 |
| Format C0 | 15 | -13.28 | -11.28 | -10.22 | -8.22 |
| Format C0 | 30 | -13.16 | -11.16 | -9.80 | -7.80 |
| Format C2 | 15 | -17.67 | -15.67 | -14.39 | -12.39 |
| Format C2 | 30 | -17.57 | -15.57 | -13.97 | -11.97 |

Table 5: ideal and impairment simulation results for NR PRACH in FR2 (

| Format | SCS(KHz) | AWGN |  | TDLC300-100 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Missed detection probability (<=1%) (ideal) | Missed detection probability (<=1%) (impairment) | Missed detection probability (<=1%) (ideal) | Missed detection probability (<=1%) (impairment) |
| Format A1 | 60 | -10.7 | -8.7 | -3.4 | -1.4 |
| Format A1 | 120 | -10.8 | -8.8 | -3.7 | -1.7 |
| Format A2 | 60 | -13.1 | -11.1 | -5.5 | -3.5 |
| Format A2 | 120 | -12.8 | -10.8 | -5.8 | -3.8 |
| Format A3 | 60 | -14.5 | -12.5 | -7.4 | -5.4 |
| Format A3 | 120 | -14.4 | -12.4 | -7.5 | -5.5 |
| Format B4 | 60 | -16.7 | -14.7 | -.7 | -7.7 |
| Format B4 | 120 | -16.7 | -14.7 | -9.9 | -7.9 |
| Format C0 | 60 | -8.1 | -6.1 | -0.7 | 1.3 |
| Format C0 | 120 | -8.1 | -6.1 | -1.1 | 0.9 |
| Format C2 | 60 | -13.1 | -11.1 | -5.7 | -3.7 |
| Format C2 | 120 | -13.1 | -11.1 | -6 | -4 |

# 3	Conclusion

In this contribution, both ideal and impairment simulation results are presented in section 2.

# References

R4-1816345, Adhoc minutes for NR BS demodulation performance, Huawei.