3GPP TSG-RAN WG4 Meeting #90	R4-1902456

# Athens, Greece, February 25 – March 1, 2019

Agenda item:	8.1.2

Source: 	Intel Corporation

Title: 	Out-of-Band Blocking for remaining BW Classes in Rel-16

Document for:	Approval

# Introduction

In the TS38.101-1 out of band blocking for intra-band contiguous CA is defined only up to 200 MHz in Rel-15. For Rel-16 the rest of the BW Classes are still under discussion. Furthermore, the OBB parameters are defined in terms of frequency range instead of BW Class as for the ACS and IBB intra-band contiguous CA requirements. In this contribution we provide our view for this requirement on all the BW Classes.

# Discussion

## Background

- Definition of Out-of-band Blocking for NR band for BW Class C in TS38.101-1 [1] :

- Out-of-band blocking parameters for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 110 MHz | 120 MHz | 130 MHz | 140 MHz | 150 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel specific value below |  |  |  |  |
|  | dB | 9 | 9 | 9 | 9 | 9 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 160 MHz | 180 MHz | 200 MHz |  |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel specific value below |  |  |  |  |
|  | dB | 9 | 9 | 9 |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. |  |  |  |  |  |  |

- Out of-band blocking for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| NR band | Parameter | Unit | Range1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
| n77, n78 (NOTE 3) | Pinterferer | dBm | -45 | -30 | -15 |
|  | Finterferer (CW) | MHz | N/A | N/A | 1 ≤ f ≤ FDL_low – MAX(200,3CBW) or FDL_high + MAX(200,3CBW) ≤ f ≤ 12750 |
| n79 (NOTE 4) | Finterferer (CW) | MHz | N/A | N/A | 1 ≤ f ≤ FDL_low – MAX(150,3CBW) or FDL_high + MAX(150,3CBW) ≤ f ≤ 12750 |
| NOTE 1: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 6000 MHz. NOTE 2: CBW denotes the channel bandwidth of the wanted signal NOTE 3: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 2700 MHz and FInterferer < 4800 MHz. For CBW > 15 MHz, the requirement for Range 1 is not applicable and Range 2 applies from the frequency offset of 3CBW from the band edge. For CBW larger than 60 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3CBW from the band edge. NOTE 4: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 3650 MHz and FInterferer < 5750 MHz. For CBW ≥ 40 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3CBW from the band edge. |  |  |  |  |  |

## Definition of Out-of-Band blocking for intra-band contiguous CA for all BW classes

In the background subsection shows that in the spec OBB intra-band contiguous CA is only defined up to 200 MHz, which is up to BW Class C. The next table provides the parameters for the rest of the BW classes.

In order to keep consistency in the specification, we propose to define the OBB parameters in terms of the BW Classes instead of the channel bandwidth as shown in Table.

| BW Class | AggBW | Max BW (MHz) | ∆OBB sig | OBB_Range |
| --- | --- | --- | --- | --- |
| B | 20M-50M | 50 | 9 | max(200,3AggrBW) |
| F | 60M-100M | 100 | 9 | max(200,3AggrBW) |
| C | 110M-200M | 200 | 9 | max(200,3AggrBW) |
| D | 210M-300M | 300 | 9 | max(200,3AggrBW) |
| E | 310M-400M | 400 | 9 | max(200,3AggrBW) |
| G | 110M-150M | 150 | 9 | max(200,3AggrBW) |
| H | 160M-200M | 200 | 9 | max(200,3AggrBW) |
| I | 210M-250M | 250 | 9 | max(200,3AggrBW) |
| J | 260M-300M | 300 | 9 | max(200,3AggrBW) |
| K | 310M-350M | 350 | 9 | max(200,3AggrBW) |
| L | 360M-400M | 400 | 9 | max(200,3AggrBW) |

Table 1: Out-of-band blocking parameters for intra-band contiguous CA for all BW Classes

| RX parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E | F | G |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel specific value below |  |  |  |  |  |
|  | dB | 9 | 9 | 9 | 9 | 9 | 9 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |  |
|  |  | H | I | J | K | L |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel specific value below |  |  |  |  |  |
|  | dB | 9 | 9 | 9 | 9 | 9 |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. |  |  |  |  |  |  |  |

Table 2: Out of-band blocking for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| NR band | Parameter | Unit | Range1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
| n77, n78 (NOTE 3) | Pinterferer | dBm | -45 | -30 | -15 |
|  | Finterferer (CW) | MHz | N/A | N/A | 1 ≤ f ≤ FDL_low – MAX(200,3CBW) or FDL_high + MAX(200,3CBW) ≤ f ≤ 12750 |
| n79 (NOTE 4) | Finterferer (CW) | MHz | N/A | N/A | 1 ≤ f ≤ FDL_low – MAX(150,3CBW) or FDL_high + MAX(150,3CBW) ≤ f ≤ 12750 |
| NOTE 1: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 6000 MHz. NOTE 2: CBW denotes the channel bandwidth of the wanted signal NOTE 3: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 2700 MHz and FInterferer < 4800 MHz. For CBW > 15 MHz, the requirement for Range 1 is not applicable and Range 2 applies from the frequency offset of 3CBW from the band edge. For CBW larger than 60 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3CBW from the band edge. NOTE 4: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 3650 MHz and FInterferer < 5750 MHz. For CBW ≥ 40 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3CBW from the band edge. |  |  |  |  |  |

Proposal 1:	The introduction of the out band blocking requirement for intra-band contiguous CA for all BW classes as provided in Table 1 and 2.

# Conclusion

In this contribution we have provided our view for Out-band Blocking for intra-band contiguous CA for all BW classes. In summary, we have made the following observations and proposal:

Proposal 1:	The introduction of the out band blocking requirement for intra-band contiguous CA for all BW classes as provided in Table 1 and 2.

# References

- 3GPP TS 38.101-1 V15.4.0 (2018-12)

- R4-1814599 “Out-of-Band blocking for intra-band contiguous CA requirements for all BW Classes”, Intel, RAN4 #89