3GPP TSG-RAN4 Meeting #90Bis	R4-1902922

# Xi’an, China,  Apr. 8th – 12th 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | CRNum | rev | - | Current version: | V15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.101-2: Correction to ACS and In-band Blocking notes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT_Core |  |  |  |  |  | Date: |  |  | 2019-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current notes in ACS, IBB have typos |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct typos: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to FInterferer/SCS+0.5SCS ([│FInterferer│/SCS] + 0.5)SCS FInterferer/SCS+0.5SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ACS and IBB requirements are not correctly implemented |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5, 7.5A, 7.6.2, 7.6A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

## 7.5	Adjacent channel selectivity

Adjacent Channel Selectivity (ACS) is a measure of a receiver's ability to receive a NR signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s).

The requirement applies at the RIB when the AoA of the incident wave of the wanted signal and the interfering signal are both from the direction where peak gain is achieved.

The wanted and interfering signals apply to all supported polarizations, under the assumption of polarization match.

The UE shall fulfil the minimum requirement specified in Table 7.5-1 for all values of an adjacent channel interferer up to –25 dBm. However, it is not possible to directly measure the ACS, instead the lower and upper range of test parameters are chosen in Table 7.5-2 and Table 7.5-3 where the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.3.2, with QPSK, R=1/3 and one sided dynamic OCNG Pattern for the DL-signal as described in Annex A. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link angle).

Table 7.5-1: Adjacent channel selectivity

|  |  | Adjacent channel selectivity / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating band | Units | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257, n258, n261 | dB | 23 | 23 | 23 | 23 |
| n260 | dB | 22 | 22 | 22 | 22 |

Table 7.5-2: Test parameters for adjacent channel selectivity, Case 1

| Rx Parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14 dB |  |  |  |
| PInterferer for band n257, n258, n261 | dBm | REFSENS + 35.5 dB | REFSENS +35.5dB | REFSENS +35.5dB | REFSENS +35.5dB |
| PInterferer for band n260 | dBm | REFSENS + 34.5 dB | REFSENS +34.5dB | REFSENS +34.5dB | REFSENS +34.5dB |
| BWInterferer | MHz | 50 | 100 | 200 | 400 |
| FInterferer (offset) | MHz | 50 / -50 NOTE 3 | 100 / -100 NOTE 3 | 200 / -200 NOTE 3 | 400 / -400 NOTE 3 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern as described in Annex A.3.2 and set-up according to Annex C. NOTE 2: The REFSENS power level is specified in Section 7.3.2, which are applicable to different UE power classes. NOTE 3: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to FInterferer/SCS+0.5SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. |  |  |  |  |  |

Table 7.5-3: Test parameters for adjacent channel selectivity, Case 2

| Rx Parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in Transmission Bandwidth Configuration for band n257, n258, n261 | dBm | -46.5 | -46.5 | -46.5 | -46.5 |
| Power in Transmission Bandwidth Configuration for band n260 | dBm | -45.5 | -45.5 | -45.5 | -45.5 |
| PInterferer | dBm | -25 |  |  |  |
| BWInterferer | MHz | 50 | 100 | 200 | 400 |
| FInterferer (offset) | MHz | 50 / -50 NOTE 2 | 100 / -100 NOTE 2 | 200 / -200 NOTE 2 | 400 / -400 NOTE 2 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex 3.2 with one sided dynamic OCNG Pattern TDD as described in Annex A and set-up according to Annex C. NOTE 2: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to FInterferer/SCS+0.5SCS FInterferer/SCS+0.5SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. |  |  |  |  |  |

## 7.5A	Adjacent channel selectivity for CA

For intra-band contiguous carrier aggregation, the SCC(s) shall be configured at nominal channel spacing to the PCC. The UE shall fulfil the minimum requirement specified in Table 7.5.1A-1 for an adjacent channel interferer on either side of the aggregated downlink signal at a specified frequency offset and for an interferer power up to -25 dBm.

The throughput of each carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.3.2 with one sided dynamic OCNG Pattern for the DL-signal as described in Annex A. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link angle).

Table 7.5A-1: Adjacent channel selectivity for CA

| Operating band | Units | Adjacent channel selectivity / CA bandwidth class |
| --- | --- | --- |
|  |  | All CA bandwidth class |
| n257, n258, n261 | dB | 23 |
| n260 | dB | 22 |

Table 7.5A-2: Adjacent channel selectivity test parameters for CA, Case 1

| Rx Parameter | Units | CA Bandwidth Class |
| --- | --- | --- |
|  |  | All CA bandwidth Classes |
| Pw in Transmission Bandwidth Configuration, per CC |  | REFSENS + 14 dB |
| PInterferer for band n257, n258, n261 | dBm | Aggregated power + 21.5 |
| PInterferer for band n260 | dBm | Aggregated power + 20.5 |
| BWInterferer | MHz | BWChannel_CA |
| FInterferer (offset) | MHz | BWChannel_CA NOTE 3 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex 3.2 with one sided dynamic OCNG Pattern as described in Annex A and set-up according to Annex C. NOTE 2: The Finterferer (offset) is the frequency separation between the center of the aggregated CA bandwidth and the center frequency of the Interferer signal NOTE 3: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to FInterferer/SCS+0.5SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. |  |  |

Table 7.5A-3: Adjacent channel selectivity test parameters for CA, Case 2

| Rx Parameter | Units | CA bandwidth class |
| --- | --- | --- |
|  |  | All CA bandwidth classes |
| Pw in Transmission Bandwidth Configuration, aggregated power for band n257, n258, n261 | dBm | - 46.5 |
| Pw in Transmission Bandwidth Configuration, aggregated power for band n260 | dBm | -45.5 |
| Pinterferer | dBm | -25 |
| BWInterferer | MHz | BWChannel_CA |
| FInterferer (offset) | MHz | BWChannel_CA NOTE 3 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex 3.2 with one sided dynamic OCNG Pattern as described in Annex A and set-up according to Annex C. NOTE 2: The Finterferer (offset) is the frequency separation between the center of the aggregated CA bandwidth and the center frequency of the Interferer signal NOTE 3: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to FInterferer/SCS+0.5SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. |  |  |

< end of changes >

< start of changes >

### 7.6.2	In-band blocking

In-band blocking is a measure of a receiver's ability to receive a NR signal at its assigned channel frequency in the presence of an interferer at a given frequency offset from the centre frequency of the assigned channel.

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annex A with one sided dynamic OCNG Pattern for the DL-signal as described in Annex A. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link angle).

Table 7.6.2-1: In band blocking requirements

| Rx parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14dB |  |  |  |
| BWInterferer | MHz | 50 | 100 | 200 | 400 |
| PInterferer for bands n257, n258, n261 | dBm | REFSENS + 35.5 dB | REFSENS + 35.5 dB | REFSENS + 35.5 dB | REFSENS + 35.5 dB |
| PInterferer for band n260 | dBm | REFSENS + 34.5 dB | REFSENS + 34.5 dB | REFSENS + 34.5 dB | REFSENS + 34.5 dB |
| FIoffset | MHz | ≤ -100 & ≥ 100 NOTE 5 | ≤ -200 & ≥ 200 NOTE 5 | ≤ -400 & ≥ 400 NOTE 5 | ≤ -800 & ≥ 800 NOTE 5 |
| FInterferer | MHz | FDL_low + 25 to FDL_high - 25 | FDL_low + 50 to FDL_high - 50 | FDL_low + 100 to FDL_high - 100 | FDL_low + 200 to FDL_high - 200 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern as described in Annex A and set-up according to Annex C. NOTE2: The REFSENS power level is specified in Section 7.3.2, which are applicable according to different UE power classes. NOTE 3: The wanted signal consists of the reference measurement channel specified in Annex A.3.2 QPSK, R=1/3 with one sided dynamic OCNG pattern as described in Annex A and set-up according to Annex C. NOTE 4: FIoffset is the frequency separation between the center of the aggregated CA bandwidth and the center frequency of the Interferer signal. NOTE 5: The absolute value of the interferer offset FIoffset shall be further adjusted to SCSFInterferer/SCS+0.5SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. NOTE 6: FInterferer range values for unwanted modulated interfering signals are interferer center frequencies. |  |  |  |  |  |

< end of changes >

< start of changes >

### 7.6A.2	In-band blocking

For intra-band contiguous carrier aggregation, the SCC(s) shall be configured at nominal channel spacing to the PCC. The UE shall fulfil the minimum requirement specified in Table 7.6A.2-1 for in the presence of an interferer at a given frequency offset from the centre frequency of the assigned channel  and for an interferer power up to -25 dBm. The throughput of each carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.3.2 with one sided dynamic OCNG Pattern for the DL-signal as described in Annex A. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link angle).

Table 7.6A.2-1: In band blocking minimum requirements for intra-band contiguous CA

| Rx Parameter | Units | CA bandwidth class |
| --- | --- | --- |
|  |  | All CA bandwidth classes |
| Power in Transmission Bandwidth Configuration, per CC |  | REFSENS + 14 dB |
| Pinterferer for band n257, n258, n261 | dBm | Aggregated power + 21.5 |
| Pinterferer for band n260 | dBm | Aggregated power + 20.5 |
| BWInterferer | MHz | BWChannel_CA |
| FIoffset | MHz | +2*BWChannel_CA / -2*BWChannel_CA NOTE 5 |
| FInterferer | MHz | FDL_low + 0.5*BWChannel_CA To FDL_high - 0.5*BWChannel_CA |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex 3.2 with one sided dynamic OCNG Pattern as described in Annex A. and set-up according to Annex C. NOTE 2: The REFSENS power level is specified in Table 7.3.2-1. NOTE 3: The wanted signal consists of the reference measurement channel specified in Annex A.3.2 QPSK, R=1/3 with one sided dynamic OCNG pattern as described in Annex A and set-up according to Annex C. NOTE 4: The FInterferer (offset) is the frequency separation between the center of the aggregated CA bandwidth and the center frequency of the Interferer signal. NOTE 5: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to FInterferer/SCS+0.5SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. NOTE 6: FInterferer range values for unwanted modulated interfering signals are interferer center frequencies. |  |  |

< end of changes >