3GPP TSG-RAN WG4 Meeting #90bis 	R4-1902981

Xian, China, 8th– 12th April, 2019

Title:	Reply LS on RRC processing delay for BWP switching

Reply To:	R2-1818901

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN WG 4

To:	RAN WG 2

Cc:

Contact Person:

Name:	Jie Cui

E-mail Address:	jie.cui@intel.com

Attachments:	N.A.

1. Overall Description:

In LS R2-1818901, RAN2 asks RAN4 whether the BWP switching delays defined for DCI-based BWP switching also apply to the cases of RRC signalling based BWP switch and BWP switching by MAC entity upon initiation of random access procedure.

According to above question, RAN4 provides the following answers:

- The delay requirement for RRC based BWP switching is different from DCI-based BWP switching delay requirement. This new defined delay requirement will be captured in TS38.133 sec 8.6.3, duplicated as below,

TBWPswitchDelayRRC is defined as the values for - Scenario 1 in Table 8.6.3-1, if UE switches between the BWPs in disjoint channel bandwidths or in partially overlapping channel bandwidths; - Scenario 2 in Table 8.6.3-1, if UE switches between the BWPs in fully overlapping channel bandwidths or in the two channel bandwidths where one is fully contained in the other. Table 8.6.3-1: RRC based BWP swich delay NR Slot length (ms); RRC based BWP switch delay TBWPswitchDelayRRC (slots). Scenario 1; Scenario 2. 0; 1; 4+2*SMTC; 4. 1; 0.5; 8+2*SMTC; 8. 2; 0.25; 16+2*SMTC; 16. 3; 0.125; 32+2*SMTC; 32. Note: If the BWP switch involves changing of SCS, the BWP switch delay is determined by the larger one between the SCS before BWP switch and the SCS after BWP switch..

- In RAN4 meeting #90, it is approved that no RRM requirement will be specified for RACH-based BWP switch.

2. Actions:

To RAN WG1, WG2 group.

ACTION: 	RAN4 kindly asks RAN2 to take the above information into account.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN WG4 Meeting #91		May 13 – 17, 2019   		Reno, Nevada 	US

TSG-RAN WG4 Meeting #92		Aug  26 – 30,  2019	     	Ljubljana, Slovenia