3GPP TSG-RAN WG4 Meeting #90-bis                                 R4-1903019

Xi’an, China, 8 – 12 April, 2019

Agenda Item:		9.20.2

Source: 		CMCC

Title: 	TP for TR 37.825 for PC2 EN-DC_41A-79A

Document for:		Approval

- Introduction

New WID on Power Class 2 UE for EN-DC (1 LTE band +1 NR band) were approved in RAN#82 meeting [1]. This TP summarizes the Band combination specific RF requirements for PC2 EN-DC_41A-79A.

- Reference

[1]  RP-182877 New WID on Power Class 2 UE for EN-DC (1 LTE band +1 NR band). CMCC

- Text Proposal

<Start of Text Proposal for TR 37.825>

  - Operating bands for PC2 EN_DC

# Table 2.1-1: DC band combination of LTE 1DL/1UL + one NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_41A-n79A1 | 41 | n79 | No |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. |  |  |  |

  - Channel bandwidths per operating band for PC2 EN_DC

# Table 2.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_41A-n79A | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 120 |
|  | n79 | 15 |  |  |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |

  - Co-existence studies for PC2 EN_DC

Table 2.3-1 lists up to 7th order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis for Band 41+Band n79 2UL DC.

# Table 2.3-1: Band 41 and Band n79 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 |  | 4400 | 5000 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |  |
| 2nd harmonics frequency limits (MHz) | 4992-5380 |  |  | 8800-10000 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low |  | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7488-8070 |  |  | 13200-15000 |  |  |
| 4th harmonics frequency limits | 4*fx_low |  | 4*fx_high | 4* fy_low |  | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 9984-10760 |  |  | 17600-20000 |  |  |
| 5th harmonics frequency limits | 5*fx_low |  | 5*fx_high | 5*fy_low |  | 5*fy_high |
| 5th harmonics frequency limits (MHz) | 12480-13450 |  |  | 22000-25000 |  |  |
| 6th harmonics frequency limits | 6*fx_low |  | 6*fx_high | 6*fy_low |  | 6*fy_high |
|  | 14976-16140 |  |  | 26400-30000 |  |  |
| 7th harmonics frequency limits | 7*fx_low |  | 7*fx_high | 7*fy_low |  | 7*fy_high |
| 7th harmonics frequency limits (MHz) | 17472-18830 |  |  | 30800-35000 |  |  |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1710-2504 |  |  | 6896-7690 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 8-980 |  |  | 6110-7504 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 9392-10380 |  |  | 11296-12690 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) |  | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 2396-2790 |  |  | 4380-5020 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ |  | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2488-3670 |  |  | 10510-12504 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 5008-3420 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ |  | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 11888-13070 |  |  | 15696-17690 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |  |
| IMD frequency limits (MHz) | 13792-15380 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ |  | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 17504-14910 |  |  | 6360-4984 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ |  | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 10008-7820 |  |  | 730-2512 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ |  | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 20096-22690 |  |  | 14384-15760 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ |  | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 18192-20380 |  |  | 16288-18070 |  |  |

Based on Table 2.3-1,

- The 2nd harmonic of Band 41 may fall into Band n79 due to UE transmitting in the lower part of the Band 41. However, the lower part of Band 41 is only used in US where there is no plan on using n79. So the harmonic issue will not be a problem for this combination.

- The 2nd, 3rd, 4th and 5th order IMD may fall into Band 41 and the 3rd, 4th, 5th order IMD product may fall into Band n79. However it should be noted that IMD will not be an issue for TDD bands (no self-interference for the TDD band) even through the IMD products may fall into the concerning band.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table2.3-2 lists if up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

# Table 2.3-2: Band 41 and Band n79 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | 5th IMD |
| Galileo | 1559 | - | 1591 | Yes |  | 5th IMD |
| GLONASS | 1591 | - | 1610 | Yes |  | 5th IMD |
| GPS | 1563 | - | 1587 | Yes |  | 5th IMD |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | 2nd, 3rd, 4th, 5th IMD |
|  | 2400 | - | 2494 | Yes | Asia | 2nd, 3rd, 4th, 5th IMD |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 2nd harmonic, 5th IMD |
|  | 5150 | - | 5350 | Yes | Europe | 2nd harmonic, 5th IMD |
|  | 5470 | - | 5725 | Yes |  | 5th IMD |
|  | 5150 | - | 5825 | Yes | Asia | 2nd harmonic, 5th IMD |

Table 2.3-3 lists the protected bands required for the dual connectivity configuration.

# Table 2.3-3: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_41A-n79A | E-UTRA Band 1, 3, 5, 8, 9, 11, 18, 19, 21, 28, 34, 40, 42, 44, 45, 65 or NR Band n1, n3, n8, n28, n34, n40, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  |  |  | - |  |  |  |  |
| NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. |  |  |  |  |  |  |  |

  - ∆TIB and ∆RIB values for PC2 EN_DC

For PC2 EN_DC_41A-n79A, the TIB,c and RIB values are given in Table 2.4-1 Table 2.4-2

# Table 2.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_41A-n79A | 41 | 0.3 |
|  | n79 | 0.8 |

# Table 2.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_41A-n79A | 41 | 0 |
|  | n79 | 0.5 |

  - MSD for PC2 EN-DC

There will be no harmonic and/or harmonic mixing issue as long as the frequency in Band 41 is above 2506. According to the spectrum situation in China and Japan, the lowest frequency to be used for this combination is 2515MHz. So there will be no harmonic and/or harmonic mixing issue. A note is noted to clarify the frequency to be used in Band 41as “Note: The lowest frequency is 2506 in Band 41 for this band combination.”

No specific REFSENS requirements need to be specified

<Start of Text Proposal for TR 37.825>