3GPP TSG RAN4 #90BIS Meeting                                        	R4-1903034

Xi’an, China, 8 – 12 April, 2019

Source: 	LG Electronics

Title: 	TP on summary of interference studies for new x bands DL (x=3, 4, 5) with 2 bands UL inter-band CA combinations in rel-16

Agenda item:	7.6.2

Document for:	Approval

1	Introduction

This contribution provides self-interference analysis results for new adding x bands DL/2 bands UL with x=3, 4, 5 CA combinations in rel-16.

For 3 bands DL/2 bands UL CA combinations, we analyze the impact to the 3rd own receiving band by the harmonics and IMD products from the dual uplink transmission.

For 4 bands DL/2 bands UL CA combinations, we analyze the impact to the 3rd and 4th own receiving bands by the harmonics and IMD products from the dual uplink transmission.

For 5 bands DL/2 bands UL CA combinations, we analyze the impact to the 3rd, 4th and 5th own receiving bands by the harmonics and IMD products from the dual uplink transmission.

Based on the self-desense analysis, we propose that RAN4 needs to define the MSD requirements for only 3 bands DL/2 bands UL CA combinations since all of the self-desense problems were covered in low band CA combinations for 4 bands DL/2 bands UL CA and 5 bands DL/2 bands UL CA with self-desense problems.

****************** Start of the TP in subclause 5.1.5 of TR36.716-03-02 ************************

### 5.1.5	Summary of interference studies

Table 5.1.5-1 summarizes the CA band combinations with self-interference problems for LTE 3 bands DL and 2 bands UL CA.

Table 5.1.5-1: Summary of self-interference analysis for 3 bands DL/2 bands UL CA

| Downlink CA configuration | Uplink CA configuration | Harmonic relation to 3rd band without uplink | Intermodulation to 3rd band without uplink | Interference due to small frequency separation | MSD (Maximum Sensitivity Degradation) |
| --- | --- | --- | --- | --- | --- |
| CA_3A-11A-18A | CA_3A-11A | - | 5th IMD | - | 4.9 dB |
| CA_3A-11A-26A | CA_3A-11A | - | 5th IMD | - | 4.9 dB |
| CA_1A-3A-42C | CA_1A-42C | - | - | - | N/A |
| CA_1A-3A-42C | CA_3A-42C | - | - | - | N/A |
| CA_2A-4A-13A | CA_2A-13A | - | 4th IMD | - | 7.6 dB |
|  | CA_4A-13A | - | 4th IMD | - | 6.2 dB |
| CA_2A-2A-4A-5A | CA_2A-5A | - | 4th IMD | - | 7.6 dB |
|  | CA_4A-5A | - | 2nd, 5th IMDs | - | - 2nd and 5th IMD problems were already covered in Table 7.3.1A-0f:2DL/2UL_4A-5A of TS36.101. |
| CA_2A-5A-66A, CA_2A-5A-66B, CA_2A-5A-66C, CA_2A-5B-66A, CA_2A-5B-66B, CA_2A-5B-66C, CA_2A-2A-5A-66A, CA_2A-2A-5A-66B, CA_2A-2A-5A-66C, CA_2A-5A-66A-66A, CA_2A-2A-5A-66A-66A, CA_2A-5B-66A-66A | CA_2A-5A | - | 4th IMD | - | 7.2 dB |
|  | CA_5A-66A | - | - | - | N/A |
| CA_2A-5A-46D | CA_2A-5A | 3rd Harmonic | 4th, 5th IMDs | - | - No need to study for 3rd harmonic impact from B2 to B46 since B46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS36.101. - 2.4 dB for IMD4 - 2.7 dB for IMD5 |
| CA_5A-46D-66A | CA_5A_46A | - | 5th IMD | - | 0.3 dB |
|  | CA_5A_66A | 3rd Harmonic | 4th, 5th IMDs | - | - No need to study for 3rd harmonic impact from B66 to B46 since B46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS36.101. - 2.5 dB for IMD4 - 0 dB for IMD5 |
| CA_2A-13A-66A, CA_2A-2A-13A-66A, CA_2A-13A-66A-66A, CA_2A-13A-66B, CA_2A-13A-66C, CA_2A-13A-66A-66B | CA_2A-13A | - | 4th IMD | - | 7.2 dB |
|  | CA_13A-66A | - | 4th IMD | - | FFS |
| CA_2A-13A-48A-48C | CA_2A-13A | 2nd Harmonic at high frequency band edge | - | - | 2nd harmonic impact from B2 to B48 was covered in Table 7.3.1A-0a of TS 36.101. |
| CA_13A-46D-66A | CA_13A-66A | 3rd Harmonic | 4th, 5th IMDs | - | - No need to study for 3rd harmonic impact from B66 to B46 since B46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS36.101. - 7.2 dB for IMD4 - 0 dB for IMD5 |
| CA_2A-13A-46D | CA_2A-13A | 3rd Harmonic | 4th IMD | - | - No need to study for 3rd harmonic impact from B2 to B46 since B46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS36.101. - 2.5 dB |
| CA_1A-3A-38A | CA_1A-3A | - | 3rd IMD into B1 | - | - 3rd IMD problem was already covered in Table 7.3.1A-0f: 2DL/2UL_1A-3A of TS36.101. |
| CA_2A-12A-66A | CA_2A-12A | 3rd Harmonic impact from B12 to B66 | - | - | - 3rd harmonic impact from B12 to B66 was covered in Table 7.3.1A-0a of TS36.101. |
|  | CA_2A-66A | - | 3rd IMD into B2 5th IMD into B66 | - | - 3rd and 5th IMD problems were already covered in Table 7.3.1A-0f:2DL/2UL_2A-66A of TS36.101. |
|  | CA_12A-66A | 3rd Harmonic impact from B12 to B66 | 4th IMD into B66 4th IMD into B2 | - | - 3rd harmonic impact from B12 to B66 was covered in Table 7.3.1A-0a of TS36.101. - For 4th IMD into B66, no impact when considering fixed Tx-Rx band separation of 400 MHz in Band 66. - Need to study for the side-lobe impact of 4th IMD product |
| CA_1A-3A-42D | CA_1A-3A | 2nd Harmonic impact from B3 to B42 | 3rd IMD into B1 4th IMD into B42 | - | - 2nd harmonic impact from B3 to B42 was covered in Table 7.3.1A-0a of TS36.101. - 3rd IMD problem was already covered in Table 7.3.1A-0f:2DL/2UL_1A-3A of TS36.101. - 4th IMD problem was already covered in Table 7.3.1A-0g:3DL/2UL_CA_1A-3A-42A/CA_1A-3A of TS36.101. |
|  | CA_1A-42A | - | - | - | N/A |
|  | CA_3A-42A | - | - | - | N/A |
|  | CA_1A-42C | - | - | - | N/A |
|  | CA_3A-42C | - | - | - | N/A |
| CA_2A-46E-48A, CA_2A-46D-48A | CA_2A-48A | 3rd Harmonic | 2nd and 3rd IMDs | - | - No need to study for 3rd harmonic impact from B2 to B46 since B46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS 36.101 - No need to study for IMDs since Reference sensitivity for CA with band 46 is specified in Table 7.3.1A-0eA of TS 36.101 |
| CA_2A-48A-66A | CA_48A-66A | - | 2nd and 5th IMDs | - | FFS |
| CA_3A-8A-38A | CA_3A-8A | - | 2nd and 3rd IMDs | - | FFS |
| CA_1A-1A-3C-5A | CA_1A-3A | - | - | - | N/A |
|  | CA_1A-5A | - | - | - | N/A |
|  | CA_3A-5A | - | - | - | N/A |
| CA_1A-1A-3C-28A | CA_1A-3A | - | - | - | N/A |
|  | CA_1A-28A | - | 5th IMD | - | FFS |
|  | CA_3A-28A | 3rd Harmonic | 4th IMD | - | FFS |
| CA_1A-3A-3A-7A, CA_1A-3A-3A-7A-7A | CA_1A-3A | - | - | - | N/A |
|  | CA_1A-7A | - | - | - | N/A |
|  | CA_3A-7A | - | - | - | N/A |

Table 5.1.5-2 summarizes the CA_ band combinations with self-interference problems for CA_4 bands DL/2 bands UL CA.

Table 5.1.5-2: Summary of self-interference analysis for 4 bands DL/2 bands UL CA

| Downlink CA configuration | Uplink CA configuration | Harmonic relation to 3rd band without uplink | Intermodulation to 3rd band without uplink | Interference due to small frequency separation | MSD (Maximum Sensitivity Degradation) |
| --- | --- | --- | --- | --- | --- |
| CA_2A-13A-48C-66A | CA_2A-13A | 2nd Harmonic at high frequency band edge | 4th IMD into B66 | - | - 2nd harmonic impact from B2 to B48 was covered in Table 7.3.1A-0a of TS 36.101. - 4th IMD problem will be covered in 3DL_2A-13A-66A_2UL_2A-13A |
|  | CA_13A-66A | 2nd Harmonic | 4th IMD into B2 5th IMD into B48 | - | - 2nd harmonic impact from B66 to B48 was covered in Table 7.3.1A-0a of TS 36.101. - 4th IMD problem will be covered in 3DL_2A-13A-66A_2UL_13A-66A - 5th IMD problem will be covered in 3DL_13A-48A-66A_2UL_13A-66A |
| CA_2A-13A-48A-48A-66A | CA_2A-13A | 2nd Harmonic at high frequency band edge | 4th IMD into B66 | - | - 2nd harmonic impact from B2 to B48 was covered in Table 7.3.1A-0a of TS 36.101. - 4th IMD problem will be covered in 3DL_2A-13A-66A_2UL_13A-66A |
|  | CA_13A-66A | 2nd Harmonic | 4th IMD into B2 5th IMD into B48 | - | - 2nd harmonic impact from B66 to B48 was covered in Table 7.3.1A-0a of TS 36.101. - 4th IMD problem will be covered in 3DL_2A-13A-66A_2UL_13A-66A - 5th IMD problem will be covered in 3DL_13A-48A-66A_2UL_13A-66A |
| CA_1A-3A-8A-38A | CA_1A-3A | - | 3rd IMD into B1 | - | - 3rd IMD problem was already covered in Table 7.3.1A-0f:2DL/2UL_1A-3A of TS36.101. |
|  | CA_1A-8A | 2nd Harmonic impact from B8 to B3 | 4th IMD into B1 | - | - 2nd harmonic impact from B8 to B3 was covered in Table 7.3.1A-0a of TS36.101. - 4th IMD problem was already covered in Table 7.3.1A-0f:2DL/2UL_1A-8A of TS36.101. |
|  | CA_3A-8A | 2nd Harmonic impact from B8 to B3 | 2nd,3rd IMDs into B38 4th IMD into B8 5th IMD into B8 4th IMD into B3 | - | - 2nd harmonic impact from B8 to B3 was covered in Table 7.3.1A-0a of TS36.101. - MSD for 2nd and 3rd IMDs into B38 will be covered in 3DL_3A-8A-38A_2UL_3A-8A. - 4th IMD into B8 problem was already covered in Table 7.3.1A-0f:2DL/2UL_3A-8A of TS36.101. - In many cases, while the 2 UL channel pairing is indeed within B3 and B5, IMD does not fall on the DL(TR36.860-12). - 4th IMD into B3 problem was already covered in Table 7.3.1A-0f:2DL/2UL_3A-8A of TS36.101. |
| CA_2A-46E-48A-66A | CA_2A-48A | 3rd Harmonic impact from B2 to B46 | 2nd, 3rd IMDs into B46 4th IMD into B2 and B66 4th IMD into B48 | - | - No need to study for 3rd harmonic impact from B2 to B46 since B46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS36.101. - No need to study for 2nd and 3rd IMDS into B46 since Reference sensitivity for CA with band 46 is specified in Table 7.3.1A-0eA of TS36.101. - MSD for 4th IMD into B2 and B66 will be analyzed in 3 bands DL_2A-48A-66A_2 bands UL_2A-48A. - MSD for 4th IMD into B48 will be analyzed in 2 bands DL/2 bands UL_2A-48A. |
|  | CA_66A-48A | 2nd, 3rd Harmonic impact from B66 to B48 and B46, respectively | 2nd,3rd IMDs into B46 4th, 5th IMDs into B48 5th IMD into B2 and B66 |  | - 2nd harmonic impact from B66 to B48 was covered in Table 7.3.1A-0a of TS36.101.- - No need to study for 3rd harmonic impact from B66 to B46 since B46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS36.101. - No need to study for 2nd and 3rd IMDS into B46 since Reference sensitivity for CA with band 46 is specified in Table 7.3.1A-0eA of TS36.101. - MSD for 4th and 5th IMDs into B48 will be analyzed in 2 bands UL/2 bands DL_48A-66A. - MSD for 5th IMD into B2 and B66 will be analyzed in 3 bands DL_2A-48A-66A_2 bands UL_48A-66A. |
| CA_1A-3A-41C-42C | CA_1A-3A | 2nd Harmonic impact from B3 to B42 | 3rd IMD into B1 4th IMD into B42 5th IMD into B41 | - | - 2nd harmonic impact from B3 to B42 was covered in Table 7.3.1A-0a of TS36.101. - 3rd IMD into B1 problem was already covered in Table 7.3.1A-0f:2DL/2UL_1A-3A of TS36.101. - 4th IMD into B42 problem was already covered in Table 7.3.1A-0g: 3 bands DL_1A-3A-42A/2 bands UL_1A-3A of TS36.101. - 5th IMD into B41 problem was already covered in Table 7.3.1A-0g: 3bands DL_1A-3A-41/2 bands UL_1A-3A. |
|  | CA_1A-42A | - | 4th IMD into B1 and B41 | - | - For 4th IMD into B1, no impact when considering fixed Tx-Rx band separation of 190 MHz in Band 1. - For 4th IMD into B41, no need to study since B41 and B42 are always synchronized. |
|  | CA_1A-42C | - | 4th IMD into B1 and B41 | - | - For 4th IMD into B1, no impact when considering fixed Tx-Rx band separation of 190 MHz in Band 1. - For 4th IMD into B41, no need to study since B41 and B42 are always synchronized. |
|  | CA_3A-42A | 2nd Harmonic impact from B3 to B42 | 2nd ,4th IMDs into B3 5th IMD into B3 | - | - 2nd harmonic impact from B3 to B42 was covered in Table 7.3.1A-0a of TS36.101. - MSDs for 2nd and 4th IMDS into B3 were already covered in Table 7.3.1A-0f. -MSD for 5th IMD into B3 was already covered in TR36.714-02-02. |
|  | CA_3A-42C | 2nd Harmonic impact from B3 to B42 | 2nd ,4th IMDs into B3 5th IMD into B3 | - | - 2nd harmonic impact from B3 to B42 was covered in Table 7.3.1A-0a of TS36.101. - MSDs for 2nd and 4th IMDS into B3 were already covered in Table 7.3.1A-0f. -MSD for 5th IMD into B3 was already covered in TR36.714-02-02. |
| CA_1A-3A-41C-42A | CA_1A-3A | 2nd Harmonic impact from B3 to B42 | 3rd IMD into B1 4th IMD into B42 5th IMD into B41 | - | - 2nd harmonic impact from B3 to B42 was covered in Table 7.3.1A-0a of TS36.101. - 3rd IMD into B1 problem was already covered in Table 7.3.1A-0f:2DL/2UL_1A-3A of TS36.101. - 4th IMD into B42 problem was already covered in Table 7.3.1A-0g: 3 bands DL_1A-3A-42A/2 bands UL_1A-3A of TS36.101. - 5th IMD into B41 problem was already covered in Table 7.3.1A-0g: 3bands DL_1A-3A-41/2 bands UL_1A-3A. |
|  | CA_1A-42A | - | 4th IMD into B1 and B41 | - | - For 4th IMD into B1, no impact when considering fixed Tx-Rx band separation of 190 MHz in Band 1. - For 4th IMD into B41, no need to study since B41 and B42 are always synchronized. |
|  | CA_3A-42A | 2nd Harmonic impact from B3 to B42 | 2nd ,4th IMDs into B3 5th IMD into B3 | - | - 2nd harmonic impact from B3 to B42 was covered in Table 7.3.1A-0a of TS36.101. - MSDs for 2nd and 4th IMDS into B3 were already covered in Table 7.3.1A-0f. -MSD for 5th IMD into B3 was already covered in TR36.714-02-02. |
| CA_1A-3A-41A-42C | CA_1A-3A | 2nd Harmonic impact from B3 to B42 | 3rd IMD into B1 4th IMD into B42 5th IMD into B41 | - | - 2nd harmonic impact from B3 to B42 was covered in Table 7.3.1A-0a of TS36.101. - 3rd IMD into B1 problem was already covered in Table 7.3.1A-0f:2DL/2UL_1A-3A of TS36.101. - 4th IMD into B42 problem was already covered in Table 7.3.1A-0g: 3 bands DL_1A-3A-42A/2 bands UL_1A-3A of TS36.101. - 5th IMD into B41 problem was already covered in Table 7.3.1A-0g: 3bands DL_1A-3A-41/2 bands UL_1A-3A. |
|  | CA_1A-42A | - | 4th IMD into B1 and B41 | - | - For 4th IMD into B1, no impact when considering fixed Tx-Rx band separation of 190 MHz in Band 1. - For 4th IMD into B41, no need to study since B41 and B42 are always synchronized. |
|  | CA_1A-42C | - | 4th IMD into B1 and B41 | - | - For 4th IMD into B1, no impact when considering fixed Tx-Rx band separation of 190 MHz in Band 1. - For 4th IMD into B41, no need to study since B41 and B42 are always synchronized. |
|  | CA_3A-42A | 2nd Harmonic impact from B3 to B42 | 2nd, 4th IMDs into B3 5th IMD into B3 | - | - 2nd harmonic impact from B3 to B42 was covered in Table 7.3.1A-0a of TS36.101. - MSDs for 2nd and 4th IMDS into B3 were already covered in Table 7.3.1A-0f. -MSD for 5th IMD into B3 was already covered in TR36.714-02-02. |
|  | CA_3A-42C | 2nd Harmonic impact from B3 to B42 | 2nd, 4th IMDs into B3 5th IMD into B3 | - | - 2nd harmonic impact from B3 to B42 was covered in Table 7.3.1A-0a of TS36.101. - MSDs for 2nd and 4th IMDS into B3 were already covered in Table 7.3.1A-0f. -MSD for 5th IMD into B3 was already covered in TR36.714-02-02. |
| CA_1A-3A-41A-42A | CA_1A-3A | 2nd Harmonic impact from B3 to B42 | 3rd IMD into B1 4th IMD into B42 5th IMD into B41 | - | - 2nd harmonic impact from B3 to B42 was covered in Table 7.3.1A-0a of TS36.101. - 3rd IMD into B1 problem was already covered in Table 7.3.1A-0f:2DL/2UL_1A-3A of TS36.101. - 4th IMD into B42 problem was already covered in Table 7.3.1A-0g: 3 bands DL_1A-3A-42A/2 bands UL_1A-3A of TS36.101. - 5th IMD into B41 problem was already covered in Table 7.3.1A-0g: 3bands DL_1A-3A-41/2 bands UL_1A-3A. |
|  | CA_1A-42A | - | 4th IMD into B1 and B41 | - | - For 4th IMD into B1, no impact when considering fixed Tx-Rx band separation of 190 MHz in Band 1. - For 4th IMD into B41, no need to study since B41 and B42 are always synchronized. |
|  | CA_3A-42A | 2nd Harmonic impact from B3 to B42 | 2nd ,4th IMDs into B3 5th IMD into B3 | - | - 2nd harmonic impact from B3 to B42 was covered in Table 7.3.1A-0a of TS36.101. - MSDs for 2nd and 4th IMDS into B3 were already covered in Table 7.3.1A-0f. -MSD for 5th IMD into B3 was already covered in TR36.714-02-02. |

Table 5.1.5-3 summarizes the CA_ band combinations with self-interference problems for CA_5 bands DL/2 bands UL CA.

Table 5.1.5-3: Summary of self-interference analysis for 5 bands DL/2 bands UL CA

<Table will be added if there are proposed in related basket WID>

Based on the MSD test configuration as summarized in Table 5.1.5.1, 5.1.5.2, and 5.1.5.3, it is proposed to define the following REFSENS exceptions and test configurations in the core specifications for 3 bands DL/2 bands UL inter-band CA with IMD problems.

Table 5.1.5.4 Proposed 3 bands DL/ 2 bands UL inter-band CA REFSENS exceptions and test configurations

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_3A-11A-18A | CA_3A-11A | 3 | 1730 | 5 | 25 | 1825 | 5 | N/A | FDD | IMD5 |
|  |  | 11 | 1442.9 | 5 | 25 | 1490.9 | 5 |  |  |  |
|  |  | 18 | 823.7 | 5 | 25 | 868.7 | 5 | 4.9 |  |  |
| CA_3A-11A-26A | CA_3A-11A | 3 | 1730 | 5 | 25 | 1825 | 5 | N/A | FDD | IMD5 |
|  |  | 11 | 1442.9 | 5 | 25 | 1490.9 | 5 |  |  |  |
|  |  | 26 | 823.7 | 5 | 25 | 868.7 | 5 | 4.9 |  |  |
| CA_2A-4A-13A | CA_2A-13A | 2 | 1855 | 5 | 25 | 1935 | 5 | N/A | FDD | IMD4 |
|  |  | 13 | 782 | 5 | 25 | 751 | 5 |  |  |  |
|  |  | 4 | 1746 | 5 | 25 | 2146 | 5 | 7.6 |  |  |
|  | CA_4A-13A | 4 | 1750 | 5 | 25 | 2150 | 5 | N/A | FDD | IMD4 |
|  |  | 13 | 780 | 5 | 25 | 749 | 5 |  |  |  |
|  |  | 2 | 1860 | 5 | 25 | 1940 | 5 | 6.2 |  |  |
| CA_2A-2A-4A-5A | CA_2A-5A | 2 | 1900 | 5 | 25 | 1980 | 5 | N/A | FDD | IMD4 |
|  |  | 5 | 834 | 5 | 25 | 879 | 5 |  |  |  |
|  |  | 4 | 1732 | 5 | 25 | 2132 | 5 | 7.6 |  |  |
| CA_2A-5B-66A-66A, CA_2A-5A-66A, CA_2A-5A-66B, CA_2A-5A-66C, CA_2A-5B-66A, CA_2A-5B-66B, CA_2A-5B-66C, CA_2A-2A-5A-66A, CA_2A-2A-5A-66B, CA_2A-2A-5A-66C, CA_2A-5A-66A-66A, CA_2A-2A-5A-66A-66A, CA_2A-5B-66A-66A | CA_2A-5A | 2 | 1900 | 5 | 25 | 1980 | 5 | N/A | FDD | IMD4 |
|  |  | 5 | 834 | 5 | 25 | 879 | 5 |  |  |  |
|  |  | 66 | 1712 | 5 | 25 | 2132 | 5 | 7.2 |  |  |
| CA_2A-5A-46D | CA_2A-5A | 2 | 1900 | 5 | 25 | 1980 | 5 | N/A | FDD-TDD | IMD4 |
|  |  | 5 | 844 | 5 | 25 | 889 | 5 |  |  |  |
|  |  | 46 | 5488 | 20 | 100 | 5488 | 20 | 2.5 |  |  |
|  | CA_2A-5A | 2 | 1900 | 5 | 25 | 1980 | 5 | N/A | FDD-TDD | IMD5 |
|  |  | 5 | 844 | 5 | 25 | 889 | 5 |  |  |  |
|  |  | 46 | 5276 | 20 | 100 | 5276 | 20 | 2.7 |  |  |
| CA_5A-46D-66A | CA_5A_46D | 5 | 834 | 5 | 25 | 879 | 5 | N/A | FDD-TDD | IMD5 |
|  |  | 46 | 5491 | 20 | 100 | 5491 | 20 |  |  |  |
|  |  | 66 | 1755 | 5 | 25 | 2155 | 5 | 0.3 |  |  |
|  | CA_5A_66A | 5 | 834 | 5 | 25 | 879 | 5 | N/A | FDD-TDD | IMD4 |
|  |  | 66 | 1770 | 5 | 25 | 2190 | 5 |  |  |  |
|  |  | 46 | 5208 | 20 | 100 | 5208 | 20 | 2.5 |  |  |
|  | CA_5A_66A | 5 | 846.5 | 5 | 25 | 891.5 | 5 | N/A | FDD-TDD | IMD5 |
|  |  | 66 | 1774 | 5 | 25 | 2194 | 5 |  |  |  |
|  |  | 46 | 5160 | 20 | 100 | 5160 | 20 | 0 |  |  |
| CA_2A-13A-66A, CA_2A-2A-13A-66A, CA_2A-13A-66A-66A, CA_2A-13A-66B, CA_2A-13A-66C, CA_2A-13A-66A-66B | CA_2A-13A | 2 | 1860 | 5 | 25 | 1940 | 5 | N/A | FDD | IMD4 |
|  |  | 13 | 782 | 5 | 25 | 751 | 5 |  |  |  |
|  |  | 66 | 1736 | 5 | 25 | 2156 | 5 | 7.2 |  |  |
|  | CA_13A-66A | 2 | 1880 | 5 | 25 | 1960 | 5 | TBD | FDD | IMD4 |
|  |  | 13 | 782 | 5 | 25 | 751 | 5 | N/A |  |  |
|  |  | 66 | 1762 | 5 | 25 | 2162 | 5 |  |  |  |
| CA_13A-46D-66A | CA_13A-66A | 13 | 782 | 5 | 25 | 751 | 5 | N/A | FDD-TDD | IMD4 |
|  |  | 66 | 1713.5 | 5 | 25 | 2113.5 | 5 |  |  |  |
|  |  | 46 | 5922.5 | 20 | 100 | 5922.5 | 20 | 7.2 |  |  |
|  | CA_13A-66A | 13 | 782 | 5 | 25 | 751 | 5 | N/A | FDD-TDD | IMD5 |
|  |  | 66 | 1727 | 5 | 25 | 2127 | 5 |  |  |  |
|  |  | 46 | 5800 | 20 | 100 | 5800 | 20 | 0 |  |  |
| CA_2A-13A-46D | CA_2A-13A | 2 | 1905 | 5 | 25 | 1985 | 5 | N/A | FDD-TDD | IMD4 |
|  |  | 13 | 782 | 5 | 25 | 751 | 5 |  |  |  |
|  |  | 46 | 5374 | 20 | 100 | 5374 | 20 | 2.5 |  |  |
| CA_2A-48A-66A | CA_48A-66A | 2 | 1880 | 5 | 25 | 1960 | 5 | TBD | FDD-TDD | IMD2 |
|  |  | 48 | 3695 | 5 | 25 | 3695 | 5 | N/A |  |  |
|  |  | 66 | 1735 | 5 | 25 | 2135 | 5 | N/A |  |  |
|  |  | 2 | 1895 | 5 | 25 | 1975 | 5 | TBD | FDD-TDD | IMD5 |
|  |  | 48 | 3620 | 5 | 25 | 3620 | 5 | N/A |  |  |
|  |  | 66 | 1755 | 5 | 25 | 2155 | 5 | N/A |  |  |
| CA_3A-8A-38A | CA_3A-8A | 3 | 1720 | 5 | 25 | 1815 | 5 | N/A | FDD-TDD | IMD2 |
|  |  | 8 | 885 | 5 | 25 | 930 | 5 | N/A |  |  |
|  |  | 38 | 2605 | 5 | 25 | 2605 | 5 | TBD |  |  |
|  |  | 3 | 1745 | 5 | 25 | 1840 | 5 | N/A | FDD-TDD | IMD3 |
|  |  | 8 | 895 | 5 | 25 | 940 | 5 | N/A |  |  |
|  |  | 38 | 2595 | 5 | 25 | 2595 | 5 | TBD |  |  |
| CA_1A-1A-3C-28A | CA_1A-28A | 1 | 1975 | 5 | 25 | 2165 | 5 | N/A | FDD | IMD5 |
|  |  | 3 | 1731 | 5 | 25 | 1826 | 5 | TBD |  |  |
|  |  | 28 | 708 | 5 | 25 | 763 | 5 | N/A |  |  |
|  | CA_3A-28A | 1 | 1949 | 5 | 25 | 2139 | 5 | TBD | FDD | IMD4 |
|  |  | 3 | 1780 | 5 | 25 | 1875 | 5 | N/A |  |  |
|  |  | 28 | 710.5 | 5 | 25 | 765.5 | 5 | N/A |  |  |

5.2	RRM specific

[Editor Note] It will be updated in future

<End of TP>