3GPP TSG-RAN WG4 Meeting #90Bis                 	R4-1903205

Xi’an, China, 8th – 12th April 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	NR PUCCH simulation results

Agenda item:	6.12.2.3.1

Document for:	Discussion

Introduction

In the RAN4#88-bis and RAN4#89 meetings, the way forward for contributing the input for NR PUCCH demodulation performance requirements was agreed [1, 2]. A procedure to arrive at performance requirement SNR numbers from the contributions of all companies was furthermore agreed in [3] and new test case configurations (DMRS TDRA 1+1 in FR2 for format 3 and 4) have been added in RAN4#90 [4].
In this contribution we add almost all the remaining simulation results with the simulation assumptions agreed in the WFs and correct some of our previous results.

Simulation Results

Simulation parameters

The simulation results in the following section are based on the simulation assumptions that were agreed in the WF [1, 2] and [4]:

Table 1: Simulation parameters

| Parameters |  | PUCCH Format 0 | PUCCH Format 1 | PUCCH Format 2 | PUCCH Format 3 | PUCCH Format 4 |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth/SCS |  | FR1: 15kHz: 5MHz, 10MHz, 20MHz; 30kHz: 10MHz, 20MHz, 40MHz, 100MHz FR2: 60kHz: 50MHz, 100MHz; 120kHz: 50MHz, 100MHz, 200MHz |  |  |  |  |
| Antenna |  | 1T2R, 1T4R and 1T8R for FR1; 1T2R for FR2 |  |  |  |  |
| Propagation condition |  | FR1: TDLC300-100; FR2: TDLA30-300 |  |  |  |  |
| Carrier frequency (GHz) |  | FR1: 4GHz; FR2: 30GHz |  |  |  |  |
| (#Bits, #Symbols, #PRBs) |  | (1, 1, 1) (1, 2, 1) | (2, 14,1) | (4, 1, 4) (22, 2, 9) | (16, 14, 1) (16, 4, 3) | (22, 14, 1) |
| DMRS pattern |  |  |  |  | FR1 and FR2: - Without additional DMRS for all cases - With additional DMRS for cases with the number of OFDM symbols more than 9 |  |
| Modulation |  |  |  | QPSK | QPSK | QPSK |
| initialCyclicShift |  | 0 | 0 |  |  |  |
| StartingSymbolIndex |  | 13 for 1 symbol; 12 for 2 symbols; | 0 | 13 for 1 symbol; 12 for 2 symbols; | 0 | 0 |
| Index of orthogonal sequence (time-domain-OCC) |  |  | 0 |  |  |  |
| occ-Length |  |  |  |  |  | n2 |
| occ-Index |  |  |  |  |  | n0 |
| Test metric | DTX to Ack probability <1% | √ | √ | √ for bits <=11 | × | × |
|  | Missed Ack probability < 1% | √ | √ | √ | × | × |
|  | NACK2ACK < 0.1% | × | √ | x | × | × |
|  | BLER < 1% | × | × | √ for bits >11 | √ | √ |

Simulation results PUCCH FR1 1T2R

Simulation results for PUCCH FR1 1T2R and with the parameters defined are shown in Table 2.

Table 2: Simulation results for PUCCH FR1 1T2R

| PUCCH FR1 1T2R, TDLC300-100 |  | Number of bits | Number of OFDM symbols | Number of PRBs | Note | SNR[dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Ideal | Impairment |
| Format 0 | 15kHz, 10MHz | 1 | 1 | 1 |  | 6,56 | 9,06 |
|  |  | 1 | 2 | 1 |  | 0,68 | 3,18 |
|  | 30kHz, 40MHz | 1 | 1 | 1 |  | 7,20 | 9,70 |
|  |  | 1 | 2 | 1 |  | 0,60 | 3,10 |
| Format 1 | 15kHz, 10MHz | 2 | 14 | 1 | ACK miss | -6,90 | -4,40 |
|  |  |  |  |  | NACK to ACK | -5,50 | -3,00 |
|  | 30kHz, 40MHz |  |  |  | ACK miss | -7,40 | -4,90 |
|  |  |  |  |  | NACK to ACK | -6,20 | -3,70 |
| Format 2 | 15kHz, 10MHz | 4 | 1 | 4 | ACK miss | 3,70 | 6,20 |
|  |  | 22 | 2 | 9 | BLER@1% | -1,60 | 0,90 |
|  | 30kHz, 40MHz | 4 | 1 | 4 | ACK miss | 2,80 | 5,30 |
|  |  | 22 | 2 | 9 | BLER@1% | -2,60 | -0,10 |
| Format 3 | 15kHz, 10MHz | 16 | 14 | 1 | Without additional DMRS | -1,80 | 0,70 |
|  |  |  |  |  | With additional DMRS | -2,40 | 0,10 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -0,60 | 1,90 |
|  | 30kHz, 40MHz | 16 | 14 | 1 | Without additional DMRS | -2,20 | 0,30 |
|  |  |  |  |  | With additional DMRS | -2,90 | -0,40 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -1,60 | 0,90 |
| Format 4 | 15kHz, 10MHz | 22 | 14 | 1 | Without additional DMRS | 0,00 | 2,50 |
|  |  |  |  |  | With additional DMRS | -0,30 | 2,20 |
|  | 30kHz, 40MHz |  |  |  | Without additional DMRS | -0,50 | 2,00 |
|  |  |  |  |  | With additional DMRS | -0,80 | 1,70 |
| Format 0 | 15kHz, 5MHz | 1 | 1 | 1 |  | 6,92 | 9,42 |
|  |  | 1 | 2 | 1 |  | 0,39 | 2,89 |
|  | 15kHz, 20MHz | 1 | 1 | 1 |  | 7,04 | 9,54 |
|  |  | 1 | 2 | 1 |  | 0,53 | 3,03 |
|  | 30kHz, 10MHz | 1 | 1 | 1 |  | 7,50 | 10,00 |
|  |  | 1 | 2 | 1 |  | 1,88 | 4,38 |
|  | 30kHz, 20MHz | 1 | 1 | 1 |  | 7,06 | 9,56 |
|  |  | 1 | 2 | 1 |  | 0,92 | 3,42 |
|  | 30kHz, 100MHz | 1 | 1 | 1 |  | 7,17 | 9,67 |
|  |  | 1 | 2 | 1 |  | 0,46 | 2,96 |
| Format 1 | 15kHz, 5MHz | 2 | 14 | 1 | ACK miss | -7,50 | -5,00 |
|  |  |  |  |  | NACK to ACK | -6,00 | -3,50 |
|  | 15kHz, 20MHz |  |  |  | ACK miss | -7,60 | -5,10 |
|  |  |  |  |  | NACK to ACK | -6,10 | -3,60 |
|  | 30kHz, 10MHz |  |  |  | ACK miss | -6,50 | -4,00 |
|  |  |  |  |  | NACK to ACK | -5,50 | -3,00 |
|  | 30kHz, 20MHz |  |  |  | ACK miss | -7,20 | -4,70 |
|  |  |  |  |  | NACK to ACK | -6,00 | -3,50 |
|  | 30kHz, 100MHz |  |  |  | ACK miss | -6,90 | -4,40 |
|  |  |  |  |  | NACK to ACK | -6,00 | -3,50 |
| Format 2 | 15kHz, 5MHz | 4 | 1 | 4 | ACK miss | 3,90 | 6,40 |
|  |  | 22 | 2 | 9 | BLER@1% | -1,80 | 0,70 |
|  | 15kHz, 20MHz | 4 | 1 | 4 | ACK miss | 3,70 | 6,20 |
|  |  | 22 | 2 | 9 | BLER@1% | -1,70 | 0,80 |
|  | 30kHz, 10MHz | 4 | 1 | 4 | ACK miss | 2,90 | 5,40 |
|  |  | 22 | 2 | 9 | BLER@1% | -2,40 | 0,10 |
|  | 30kHz, 20MHz | 4 | 1 | 4 | ACK miss | 3,30 | 5,80 |
|  |  | 22 | 2 | 9 | BLER@1% | -1,60 | 0,90 |
|  | 30kHz, 100MHz | 4 | 1 | 4 | ACK miss | 3,20 | 5,70 |
|  |  | 22 | 2 | 9 | BLER@1% | -2,50 | 0,00 |
| Format 3 | 15kHz, 5MHz | 16 | 14 | 1 | Without additional DMRS | -2,30 | 0,20 |
|  |  |  |  |  | With additional DMRS | -3,10 | -0,60 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -1,60 | 0,90 |
|  | 15kHz, 20MHz | 16 | 14 | 1 | Without additional DMRS | -2,30 | 0,20 |
|  |  |  |  |  | With additional DMRS | -3,10 | -0,60 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -1,40 | 1,10 |
|  | 30kHz, 10MHz | 16 | 14 | 1 | Without additional DMRS | -1,60 | 0,90 |
|  |  |  |  |  | With additional DMRS | -2,20 | 0,30 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -0,70 | 1,80 |
|  | 30kHz, 20MHz | 16 | 14 | 1 | Without additional DMRS | -2,10 | 0,40 |
|  |  |  |  |  | With additional DMRS | -2,80 | -0,30 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -1,00 | 1,50 |
|  | 30kHz, 100MHz | 16 | 14 | 1 | Without additional DMRS | -1,80 | 0,70 |
|  |  |  |  |  | With additional DMRS | -2,60 | -0,10 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -1,60 | 0,90 |
| Format 4 | 15kHz, 5MHz | 22 | 14 | 1 | Without additional DMRS | -1,00 | 1,50 |
|  |  |  |  |  | With additional DMRS | -1,40 | 1,10 |
|  | 15kHz, 20MHz |  |  |  | Without additional DMRS | -0,80 | 1,70 |
|  |  |  |  |  | With additional DMRS | -1,00 | 1,50 |
|  | 30kHz, 10MHz |  |  |  | Without additional DMRS | 0,40 | 2,90 |
|  |  |  |  |  | With additional DMRS | 0,20 | 2,70 |
|  | 30kHz, 20MHz |  |  |  | Without additional DMRS | -0,40 | 2,10 |
|  |  |  |  |  | With additional DMRS | -0,60 | 1,90 |
|  | 30kHz, 100MHz |  |  |  | Without additional DMRS | -0,20 | 2,30 |
|  |  |  |  |  | With additional DMRS | -0,50 | 2,00 |

Simulation results PUCCH FR1 1T4R

Simulation results for PUCCH FR1 1T4R and with the parameters defined are shown in Table 3.

Table 3: Simulation results for PUCCH FR1 1T4R

| PUCCH FR1 1T4R, TDLC300-100 |  | Number of bits | Number of OFDM symbols | Number of PRBs | Note | SNR[dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Ideal | Impaired |
| Format 0 | 15kHz, 10MHz | 1 | 1 | 1 |  | 0,54 | 3,04 |
|  |  | 1 | 2 | 1 |  | -3,64 | -1,14 |
|  | 30kHz, 40MHz | 1 | 1 | 1 |  | 0,99 | 3,49 |
|  |  | 1 | 2 | 1 |  | -3,27 | -0,77 |
| Format 1 | 15kHz, 10MHz | 2 | 14 | 1 | ACK miss | -10,70 | -8,20 |
|  |  |  |  |  | NACK to ACK | -10,00 | -7,50 |
|  | 30kHz, 40MHz |  |  |  | ACK miss | -11,10 | -8,60 |
|  |  |  |  |  | NACK to ACK | -10,00 | -7,50 |
| Format 2 | 15kHz, 10MHz | 4 | 1 | 4 | ACK miss | -1,20 | 1,30 |
|  |  | 22 | 2 | 9 | BLER@1% | -5,80 | -3,30 |
|  | 30kHz, 40MHz | 4 | 1 | 4 | ACK miss | -1,60 | 0,90 |
|  |  | 22 | 2 | 9 | BLER@1% | -6,30 | -3,80 |
| Format 3 | 15kHz, 10MHz | 16 | 14 | 1 | Without additional DMRS | -5,80 | -3,30 |
|  |  |  |  |  | With additional DMRS | -6,70 | -4,20 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -5,50 | -3,00 |
|  | 30kHz, 40MHz | 16 | 14 | 1 | Without additional DMRS | -6,30 | -3,80 |
|  |  |  |  |  | With additional DMRS | -7,20 | -4,70 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -6,00 | -3,50 |
| Format 4 | 15kHz, 10MHz | 22 | 14 | 1 | Without additional DMRS | -4,60 | -2,10 |
|  |  |  |  |  | With additional DMRS | -5,20 | -2,70 |
|  | 30kHz, 40MHz |  |  |  | Without additional DMRS | -5,10 | -2,60 |
|  |  |  |  |  | With additional DMRS | -5,60 | -3,10 |
| Format 0 | 15kHz, 5MHz | 1 | 1 | 1 |  | 0,53 | 3,03 |
|  |  | 1 | 2 | 1 |  | -3,78 | -1,28 |
|  | 15kHz, 20MHz | 1 | 1 | 1 |  | 0,34 | 2,84 |
|  |  | 1 | 2 | 1 |  | -3,67 | -1,17 |
|  | 30kHz, 10MHz | 1 | 1 | 1 |  | 0,98 | 3,48 |
|  |  | 1 | 2 | 1 |  | -2,87 | -0,37 |
|  | 30kHz, 20MHz | 1 | 1 | 1 |  | 0,78 | 3,28 |
|  |  | 1 | 2 | 1 |  | -3,06 | -0,56 |
|  | 30kHz, 100MHz | 1 | 1 | 1 |  | 1,23 | 3,73 |
|  |  | 1 | 2 | 1 |  | -3,55 | -1,05 |
| Format 1 | 15kHz, 5MHz | 2 | 14 | 1 | ACK miss | -11,30 | -8,80 |
|  |  |  |  |  | NACK to ACK | -10,70 | -8,20 |
|  | 15kHz, 20MHz |  |  |  | ACK miss | -11,20 | -8,70 |
|  |  |  |  |  | NACK to ACK | -10,20 | -7,70 |
|  | 30kHz, 10MHz |  |  |  | ACK miss | -10,50 | -8,00 |
|  |  |  |  |  | NACK to ACK | -9,50 | -7,00 |
|  | 30kHz, 20MHz |  |  |  | ACK miss | -10,90 | -8,40 |
|  |  |  |  |  | NACK to ACK | -10,30 | -7,80 |
|  | 30kHz, 100MHz |  |  |  | ACK miss | -10,70 | -8,20 |
|  |  |  |  |  | NACK to ACK | -9,90 | -7,40 |
| Format 2 | 15kHz, 5MHz | 4 | 1 | 4 | ACK miss | -1,4 | 1,1 |
|  |  | 22 | 2 | 9 | BLER@1% | -6,1 | -3,6 |
|  | 15kHz, 20MHz | 4 | 1 | 4 | ACK miss | -1,6 | 0,9 |
|  |  | 22 | 2 | 9 | BLER@1% | -6,1 | -3,6 |
|  | 30kHz, 10MHz | 4 | 1 | 4 | ACK miss | -1,8 | 0,7 |
|  |  | 22 | 2 | 9 | BLER@1% | -6,3 | -3,8 |
|  | 30kHz, 20MHz | 4 | 1 | 4 | ACK miss | -1,6 | 0,9 |
|  |  | 22 | 2 | 9 | BLER@1% | -5,9 | -3,4 |
|  | 30kHz, 100MHz | 4 | 1 | 4 | ACK miss | -1,7 | 0,8 |
|  |  | 22 | 2 | 9 | BLER@1% | -6,4 | -3,9 |
| Format 3 | 15kHz, 5MHz | 16 | 14 | 1 | Without additional DMRS | -6,60 | -4,10 |
|  |  |  |  |  | With additional DMRS | -7,50 | -5,00 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -5,90 | -3,40 |
|  | 15kHz, 20MHz | 16 | 14 | 1 | Without additional DMRS | -6,50 | -4,00 |
|  |  |  |  |  | With additional DMRS | -7,50 | -5,00 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -6,00 | -3,50 |
|  | 30kHz, 10MHz | 16 | 14 | 1 | Without additional DMRS | -5,70 | -3,20 |
|  |  |  |  |  | With additional DMRS | -6,70 | -4,20 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -5,30 | -2,80 |
|  | 30kHz, 20MHz | 16 | 14 | 1 | Without additional DMRS | -6,30 | -3,80 |
|  |  |  |  |  | With additional DMRS | -7,10 | -4,60 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -5,60 | -3,10 |
|  | 30kHz, 100MHz | 16 | 14 | 1 | Without additional DMRS | -6,10 | -3,60 |
|  |  |  |  |  | With additional DMRS | -7,00 | -4,50 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -5,80 | -3,30 |
| Format 4 | 15kHz, 5MHz | 22 | 14 | 1 | Without additional DMRS | -5,50 | -3,00 |
|  |  |  |  |  | With additional DMRS | -6,00 | -3,50 |
|  | 15kHz, 20MHz |  |  |  | Without additional DMRS | -5,30 | -2,80 |
|  |  |  |  |  | With additional DMRS | -5,80 | -3,30 |
|  | 30kHz, 10MHz |  |  |  | Without additional DMRS | -4,40 | -1,90 |
|  |  |  |  |  | With additional DMRS | -4,90 | -2,40 |
|  | 30kHz, 20MHz |  |  |  | Without additional DMRS | -4,90 | -2,40 |
|  |  |  |  |  | With additional DMRS | -5,50 | -3,00 |
|  | 30kHz, 100MHz |  |  |  | Without additional DMRS | -4,80 | -2,30 |
|  |  |  |  |  | With additional DMRS | -5,40 | -2,90 |

Simulation results PUCCH FR1 1T8R

Simulation results for PUCCH FR1 1T8R and with the parameters defined are shown in Table 4.

Table 4: Simulation results for PUCCH FR1 1T8R

| PUCCH FR1 1T8R, TDLC300-100 |  | Number of bits | Number of OFDM symbols | Number of PRBs | Note | SNR[dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Ideal | Impaired |
| Format 0 | 15kHz, 10MHz | 1 | 1 | 1 |  | -3,43 | -0,93 |
|  |  | 1 | 2 | 1 |  | -6,45 | -3,95 |
|  | 30kHz, 40MHz | 1 | 1 | 1 |  | -3,31 | -0,81 |
|  |  | 1 | 2 | 1 |  | -6,49 | -3,99 |
| Format 1 | 15kHz, 10MHz | 2 | 14 | 1 | ACK miss | -13,60 | -11,10 |
|  |  |  |  |  | NACK to ACK | -13,60 | -11,10 |
|  | 30kHz, 40MHz |  |  |  | ACK miss | -13,80 | -11,30 |
|  |  |  |  |  | NACK to ACK | -13,50 | -11,00 |
| Format 2 | 15kHz, 10MHz | 4 | 1 | 4 | ACK miss | -5,60 | -3,10 |
|  |  | 22 | 2 | 9 | BLER@1% | -9,40 | -6,90 |
|  | 30kHz, 40MHz | 4 | 1 | 4 | ACK miss | -5,70 | -3,20 |
|  |  | 22 | 2 | 9 | BLER@1% | -9,60 | -7,10 |
| Format 3 | 15kHz, 10MHz | 16 | 14 | 1 | Without additional DMRS | -9,10 | -6,60 |
|  |  |  |  |  | With additional DMRS | -10,10 | -7,60 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -9,00 | -6,50 |
|  | 30kHz, 40MHz | 16 | 14 | 1 | Without additional DMRS | -9,40 | -6,90 |
|  |  |  |  |  | With additional DMRS | -10,50 | -8,00 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -9,20 | -6,70 |
| Format 4 | 15kHz, 10MHz | 22 | 14 | 1 | Without additional DMRS | -8,10 | -5,60 |
|  |  |  |  |  | With additional DMRS | -9,00 | -6,50 |
|  | 30kHz, 40MHz |  |  |  | Without additional DMRS | -8,40 | -5,90 |
|  |  |  |  |  | With additional DMRS | -9,20 | -6,70 |
| Format 0 | 15kHz, 5MHz | 1 | 1 | 1 |  | -3,31 | -0,81 |
|  |  | 1 | 2 | 1 |  | -7,01 | -4,51 |
|  | 15kHz, 20MHz | 1 | 1 | 1 |  | -3,28 | -0,78 |
|  |  | 1 | 2 | 1 |  | -6,98 | -4,48 |
|  | 30kHz, 10MHz | 1 | 1 | 1 |  | -3,22 | -0,72 |
|  |  | 1 | 2 | 1 |  | -6,15 | -3,65 |
|  | 30kHz, 20MHz | 1 | 1 | 1 |  | -3,17 | -0,67 |
|  |  | 1 | 2 | 1 |  | -6,48 | -3,98 |
|  | 30kHz, 100MHz | 1 | 1 | 1 |  | -3,21 | -0,71 |
|  |  | 1 | 2 | 1 |  | -6,66 | -4,16 |
| Format 1 | 15kHz, 5MHz | 2 | 14 | 1 | ACK miss | -13,90 | -11,40 |
|  |  |  |  |  | NACK to ACK | -13,80 | -11,30 |
|  | 15kHz, 20MHz |  |  |  | ACK miss | -13,90 | -11,40 |
|  |  |  |  |  | NACK to ACK | -13,60 | -11,10 |
|  | 30kHz, 10MHz |  |  |  | ACK miss | -13,50 | -11,00 |
|  |  |  |  |  | NACK to ACK | -13,40 | -10,90 |
|  | 30kHz, 20MHz |  |  |  | ACK miss | -13,70 | -11,20 |
|  |  |  |  |  | NACK to ACK | -13,10 | -10,60 |
|  | 30kHz, 100MHz |  |  |  | ACK miss | -13,60 | -11,10 |
|  |  |  |  |  | NACK to ACK | -13,30 | -10,80 |
| Format 2 | 15kHz, 5MHz | 4 | 1 | 4 | ACK miss | -5,5 | -3 |
|  |  | 22 | 2 | 9 | BLER@1% | -9,5 | -7 |
|  | 15kHz, 20MHz | 4 | 1 | 4 | ACK miss | -5,5 | -3 |
|  |  | 22 | 2 | 9 | BLER@1% | -9,5 | -7 |
|  | 30kHz, 10MHz | 4 | 1 | 4 | ACK miss | -5,7 | -3,2 |
|  |  | 22 | 2 | 9 | BLER@1% | -9,5 | -7 |
|  | 30kHz, 20MHz | 4 | 1 | 4 | ACK miss | -5,7 | -3,2 |
|  |  | 22 | 2 | 9 | BLER@1% | -9,2 | -6,7 |
|  | 30kHz, 100MHz | 4 | 1 | 4 | ACK miss | -5,4 | -2,9 |
|  |  | 22 | 2 | 9 | BLER@1% | -9,6 | -7,1 |
| Format 3 | 15kHz, 5MHz | 16 | 14 | 1 | Without additional DMRS | -9,60 | -7,10 |
|  |  |  |  |  | With additional DMRS | -10,70 | -8,20 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -9,10 | -6,60 |
|  | 15kHz, 20MHz | 16 | 14 | 1 | Without additional DMRS | -9,60 | -7,10 |
|  |  |  |  |  | With additional DMRS | -10,70 | -8,20 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -9,20 | -6,70 |
|  | 30kHz, 10MHz | 16 | 14 | 1 | Without additional DMRS | -9,10 | -6,60 |
|  |  |  |  |  | With additional DMRS | -10,10 | -7,60 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -9,00 | -6,50 |
|  | 30kHz, 20MHz | 16 | 14 | 1 | Without additional DMRS | -9,40 | -6,90 |
|  |  |  |  |  | With additional DMRS | -10,40 | -7,90 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -9,00 | -6,50 |
|  | 30kHz, 100MHz | 16 | 14 | 1 | Without additional DMRS | -9,30 | -6,80 |
|  |  |  |  |  | With additional DMRS | -10,40 | -7,90 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -9,00 | -6,50 |
| Format 4 | 15kHz, 5MHz | 22 | 14 | 1 | Without additional DMRS | -8,60 | -6,10 |
|  |  |  |  |  | With additional DMRS | -9,50 | -7,00 |
|  | 15kHz, 20MHz |  |  |  | Without additional DMRS | -8,60 | -6,10 |
|  |  |  |  |  | With additional DMRS | -9,40 | -6,90 |
|  | 30kHz, 10MHz |  |  |  | Without additional DMRS | -8,00 | -5,50 |
|  |  |  |  |  | With additional DMRS | -8,80 | -6,30 |
|  | 30kHz, 20MHz |  |  |  | Without additional DMRS | -8,40 | -5,90 |
|  |  |  |  |  | With additional DMRS | -9,10 | -6,60 |
|  | 30kHz, 100MHz |  |  |  | Without additional DMRS | -8,10 | -5,60 |
|  |  |  |  |  | With additional DMRS | -9,00 | -6,50 |

Simulation results PUCCH FR2 1T2R

Simulation results for PUCCH FR2 1T2R and with the parameters defined are shown in Table 5.

Table 5: Simulation results for PUCCH FR2 1T2R

| PUCCH FR2 1T2R, TDLA30-300 |  | Number of bits | Number of OFDM symbols | Number of PRBs | Note | SNR[dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Ideal | Impaired |
| Format 0 | 60kHz, 100MHz | 1 | 1 | 1 |  | 6,40 | 9,50 |
|  |  | 1 | 2 | 1 |  | 1,72 | 4,82 |
|  | 120kHz, 100MHz | 1 | 1 | 1 |  | 6,68 | 9,78 |
|  |  | 1 | 2 | 1 |  | 0,74 | 3,84 |
| Format 1 | 60kHz, 100MHz | 2 | 14 | 1 | ACK miss | -7,10 | -4,00 |
|  |  |  |  |  | NACK to ACK | -5,90 | -2,80 |
|  | 120kHz, 100MHz |  |  |  | ACK miss | -7,00 | -3,90 |
|  |  |  |  |  | NACK to ACK | -5,90 | -2,80 |
| Format 2 | 60kHz, 100MHz | 4 | 1 | 4 | ACK miss | 4,70 | 7,80 |
|  |  | 22 | 2 | 9 | BLER@1% | -1,00 | 2,10 |
|  | 120kHz, 100MHz | 4 | 1 | 4 | ACK miss | 4,30 | 7,40 |
|  |  | 22 | 2 | 9 | BLER@1% | -1,70 | 1,40 |
| Format 3 | 60kHz, 100MHz | 16 | 14 | 1 | Without additional DMRS | -1,90 | 1,20 |
|  |  |  |  |  | With additional DMRS | -1,90 | 1,20 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -0,90 | 2,20 |
|  | 120kHz, 100MHz | 16 | 14 | 1 | Without additional DMRS | -2,00 | 1,10 |
|  |  |  |  |  | With additional DMRS | -1,80 | 1,30 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -1,30 | 1,80 |
| Format 4 | 60kHz, 100MHz | 22 | 14 | 1 | Without additional DMRS | -0,50 | 2,60 |
|  |  |  |  |  | With additional DMRS | -0,20 | 2,90 |
|  | 120kHz, 100MHz |  |  |  | Without additional DMRS | -0,40 | 2,70 |
|  |  |  |  |  | With additional DMRS | -0,20 | 2,90 |
| Format 0 | 60kHz, 50MHz | 1 | 1 | 1 |  | 6,29 | 9,39 |
|  |  | 1 | 2 | 1 |  | 2,18 | 5,28 |
|  | 120kHz, 50MHz | 1 | 1 | 1 |  | 6,57 | 9,67 |
|  |  | 1 | 2 | 1 |  | 0,81 | 3,91 |
|  | 120kHz, 200MHz | 1 | 1 | 1 |  | 6,61 | 9,71 |
|  |  | 1 | 2 | 1 |  | 0,56 | 3,66 |
| Format 1 | 60kHz, 50MHz | 2 | 14 | 1 | ACK miss | -6,60 | -3,50 |
|  |  |  |  |  | NACK to ACK | -5,30 | -2,20 |
|  | 120kHz, 50MHz |  |  |  | ACK miss | -7,50 | -4,40 |
|  |  |  |  |  | NACK to ACK | -6,20 | -3,10 |
|  | 120kHz, 200MHz |  |  |  | ACK miss | -7,60 | -4,50 |
|  |  |  |  |  | NACK to ACK | -6,30 | -3,20 |
| Format 2 | 60kHz, 50MHz | 4 | 1 | 4 | ACK miss | 4,7 | 7,8 |
|  |  | 22 | 2 | 9 | BLER@1% | 0,3 | 3,4 |
|  | 120kHz, 50MHz | 4 | 1 | 4 | ACK miss | 4,1 | 7,2 |
|  |  | 22 | 2 | 9 | BLER@1% | -0,9 | 2,2 |
|  | 120kHz, 200MHz | 4 | 1 | 4 | ACK miss | 4,3 | 7,4 |
|  |  | 22 | 2 | 9 | BLER@1% | -1,7 | 1,4 |
| Format 3 | 60kHz, 50MHz | 16 | 14 | 1 | Without additional DMRS | -1,50 | 1,60 |
|  |  |  |  |  | With additional DMRS | -0,90 | 2,20 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -0,30 | 2,80 |
|  | 120kHz, 50MHz | 16 | 14 | 1 | Without additional DMRS | -2,40 | 0,70 |
|  |  |  |  |  | With additional DMRS | -2,00 | 1,10 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -1,60 | 1,50 |
|  | 120kHz, 200MHz | 16 | 14 | 1 | Without additional DMRS | -2,70 | 0,40 |
|  |  |  |  |  | With additional DMRS | -1,80 | 1,30 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -1,70 | 1,40 |
| Format 4 | 60kHz, 50MHz | 22 | 14 | 1 | Without additional DMRS | 0,10 | 3,20 |
|  |  |  |  |  | With additional DMRS | 0,80 | 3,90 |
|  | 120kHz, 50MHz |  |  |  | Without additional DMRS | -0,90 | 2,20 |
|  |  |  |  |  | With additional DMRS | -0,30 | 2,80 |
|  | 120kHz, 200MHz |  |  |  | Without additional DMRS | -1,10 | 2,00 |
|  |  |  |  |  | With additional DMRS | 0,00 | 3,10 |

Conclusion

In this contribution we have provided simulation results for NR PUCCH.

- References

- R4-1813943, Way forward on NR PUCCH demodulation requirements, Huawei, 3GPP TSG-RAN WG4 RAN#88bis, Chengdu, China, 8 – 12 October 2018

- R4-1816348, Way Forward for NR BS PUCCH demodulation performance, Huawei, HiSilicon, 3GPP TSG-RAN WG4 RAN#89, Spokane, US, 12 – 16 November 2018.

- R4-1816594, WF for test case SNR derivation procedure, Nokia, Nokia Shanghai Bell, 3GPP TSG-RAN WG4 RAN#89, Spokane, US, 12 – 16 November 2018.

- R4-1902440, Way forward on NR PUCCH demodulation performance in Rel-15, Huawei, 3GPP TSG-RAN WG4 Meeting #90, Athens, Greece, Feb.25 – Mar.01 2019.