3GPP TSG-RAN WG4 Meeting #90b                    	   R4-1903269

Xi’an, China, Apr 8th – 12th, 2019

Agenda item:            6.12.1.4.2

Source:	MediaTek Inc.

Title:	NR CSI CQI simulation result

Document for:          Discussion

# 1	Introduction

In this paper, we present the simulation results of CSI CQI.

# 2	Discussion

Based on the simulation assumption in [1], we provide the simulation results of CSI CQI test cases. Table A.1~Table A.3 (in Appendix) are the simulation assumption for CQI. Table A.4~Table A.7 contain other detailed parameter setting in the simulation.

## 2.1 Static CQI

Table 2.1.1~Table 2.1.5 are the simulation results (CQI report distribution and BLER performance of median CQI, median CQI+1 and median CQI-1) for Static CQI test 1~3 with 2RX and 4RX. The SNR range is from 0 to 20dB.

Table 2.1.1-1 Static CQI (Distribution) – Test 1 2TX x 2RX Rank2

| SNR [dB] | Median CQI | MCS | Prob. In Median CQI-1, Median CQI, Median CQI+1 |
| --- | --- | --- | --- |
| 0 | 3 | 3 | 100% |
| 2 | 3 | 3 | 100% |
| 4 | 4 | 5 | 100% |
| 6 | 5 | 7 | 100% |
| 8 | 6 | 9 | 100% |
| 9 | 6 | 9 | 100% |
| 10 | 7 | 11 | 100% |
| 12 | 7 | 11 | 100% |
| 14 | 9 | 15 | 100% |
| 15 | 9 | 15 | 100% |
| 16 | 9 | 15 | 100% |
| 18 | 10 | 17 | 100% |
| 20 | 12 | 21 | 100% |

Table 2.1.1-2 Static CQI (BLER) – Test 1 2TX x 2RX Rank2

| SNR [dB] | Median CQI | MCS | BLER with Median CQI-1 | BLER with Median CQI | BLER with Median CQI+1 |
| --- | --- | --- | --- | --- | --- |
| 0 | 3 | 3 | 0 | 0 | 1 |
| 2 | 3 | 3 | 0 | 0 | 1 |
| 4 | 4 | 5 | 0 | 1 | 1 |
| 6 | 5 | 7 | 0 | 1 | 1 |
| 8 | 6 | 9 | 0 | 1 | 1 |
| 9 | 6 | 9 | 0 | 0 | 1 |
| 10 | 7 | 11 | 0 | 0.16 | 1 |
| 12 | 7 | 11 | 0 | 0 | 0.07 |
| 14 | 9 | 15 | 0 | 0.11 | 1 |
| 15 | 9 | 15 | 0 | 0 | 1 |
| 16 | 9 | 15 | 0 | 0 | 0.08 |
| 18 | 10 | 17 | 0 | 0 | 0.07 |
| 20 | 12 | 21 | 0 | 0.06 | 1 |

Figure 2.1.2-1 Static CQI (Distribution) – Test 1 2TX x 4RX Rank2

| SNR [dB] | Median CQI | MCS | Prob. In Median CQI-1, Median CQI, Median CQI+1 |
| --- | --- | --- | --- |
| 0 | 4 | 5 | 100% |
| 2 | 5 | 7 | 100% |
| 4 | 6 | 9 | 100% |
| 5 | 6 | 9 | 100% |
| 6 | 7 | 11 | 100% |
| 8 | 8 | 13 | 100% |
| 10 | 9 | 15 | 100% |
| 11 | 10 | 17 | 100% |
| 12 | 10 | 17 | 100% |
| 14 | 11 | 19 | 100% |
| 16 | 12 | 21 | 100% |
| 18 | 13 | 23 | 100% |
| 20 | 14 | 25 | 100% |

Figure 2.1.2-2 Static CQI (BLER) – Test 1 2TX x 4RX Rank2

| SNR [dB] | Median CQI | MCS | BLER with Median CQI-1 | BLER with Median CQI | BLER with Median CQI+1 |
| --- | --- | --- | --- | --- | --- |
| 0 | 4 | 5 | 0 | 1 | 1 |
| 2 | 5 | 7 | 0 | 1. | 1 |
| 4 | 6 | 9 | 0 | 1. | 1 |
| 5 | 6 | 9 | 0 | 1 | 1 |
| 6 | 7 | 11 | 0 | 1. | 1 |
| 8 | 8 | 13 | 0 | 1 | 1 |
| 10 | 9 | 15 | 0 | 1 | 1 |
| 11 | 10 | 17 | 0 | 0.095 | 1 |
| 12 | 10 | 17 | 0 | 1 | 1 |
| 14 | 11 | 19 | 0 | 1 | 1 |
| 16 | 12 | 21 | 0 | 1 | 1 |
| 18 | 13 | 23 | 0 | 1 | 1 |
| 20 | 14 | 25 | 0 | 1 | 1 |

Figure 2.1.3-1 Static CQI (Distribution) – Test 2 2TX x 2RX Rank2

| SNR [dB] | Median CQI | MCS | Prob. In Median CQI-1, Median CQI, Median CQI+1 |
| --- | --- | --- | --- |
| 0 | 2 | 1 | 100% |
| 2 | 3 | 3 | 100% |
| 4 | 3 | 3 | 100% |
| 6 | 5 | 7 | 100% |
| 8 | 6 | 9 | 100% |
| 9 | 6 | 9 | 100% |
| 10 | 7 | 11 | 100% |
| 12 | 7 | 11 | 100% |
| 14 | 9 | 15 | 100% |
| 15 | 9 | 15 | 100% |
| 16 | 9 | 15 | 100% |
| 18 | 10 | 17 | 100% |
| 20 | 12 | 21 | 100% |

Figure 2.1.3-2 Static CQI (BLER) – Test 2 2TX x 2RX Rank2

| SNR [dB] | Median CQI | MCS | BLER with Median CQI-1 | BLER with Median CQI | BLER with Median CQI+1 |
| --- | --- | --- | --- | --- | --- |
| 0 | 2 | 1 | 0 | 0 | 1 |
| 2 | 3 | 3 | 0 | 0 | 1 |
| 4 | 3 | 3 | 0 | 0 | 1 |
| 6 | 5 | 7 | 0 | 1 | 1 |
| 8 | 6 | 9 | 0 | 1 | 1 |
| 9 | 6 | 9 | 0 | 0 | 1 |
| 10 | 7 | 11 | 0 | 0.6591 | 1 |
| 12 | 7 | 11 | 0 | 0 | 0.07 |
| 14 | 9 | 15 | 0 | 0.0451 | 1 |
| 15 | 9 | 15 | 0 | 0 | 1 |
| 16 | 9 | 15 | 0 | 0.0 | 0.08 |
| 18 | 10 | 17 | 0 | 0.0 | 0.31 |
| 20 | 12 | 21 | 0.0 | 0.04 | 1 |

Figure 2.1.4-1 Static CQI (Distribution) – Test 2 2TX x 4RX Rank2

| SNR [dB] | Median CQI | MCS | Prob. In Median CQI-1, Median CQI, Median CQI+1 |
| --- | --- | --- | --- |
| 0 | 4 | 5 | 100% |
| 2 | 5 | 7 | 100% |
| 4 | 6 | 9 | 100% |
| 5 | 6 | 6 | 100% |
| 6 | 6 | 9 | 100% |
| 8 | 7 | 11 | 100% |
| 10 | 8 | 13 | 100% |
| 11 | 9 | 15 | 100% |
| 12 | 9 | 15 | 100% |
| 14 | 10 | 17 | 100% |
| 16 | 12 | 21 | 100% |
| 18 | 12 | 21 | 100% |
| 20 | 13 | 23 | 100% |

Figure 2.1.4-2 Static CQI (BLER) – Test 2 2TX x 4RX Rank2

| SNR [dB] | Median CQI | MCS | BLER with Median CQI-1 | BLER with Median CQI | BLER with Median CQI+1 |
| --- | --- | --- | --- | --- | --- |
| 0 | 4 | 5 | 0 | 1 | 1 |
| 2 | 5 | 7 | 0 | 1 | 1 |
| 4 | 6 | 9 | 0 | 1 | 1 |
| 5 | 6 | 9 | 0 | 1 | 1 |
| 6 | 6 | 9 | 0 | 0. | 1 |
| 8 | 7 | 11 | 0 | 0. | 1 |
| 10 | 8 | 13 | 0 | 0 | 1 |
| 11 | 9 | 15 | 0 | 0.045 | 1 |
| 12 | 9 | 15 | 0 | 0 | 1 |
| 14 | 10 | 17 | 0 | 0 | 1 |
| 16 | 12 | 21 | 0.03 | 1 | 1 |
| 18 | 12 | 21 | 0.0 | 0 | 1 |
| 20 | 13 | 23 | 0.0 | 0 | 1 |

Figure 2.1.5-1 Static CQI (Distribution) – Test 3 2TX x 2RX Rank2

| SNR [dB] | Median CQI | MCS | Prob. In Median CQI-1, Median CQI, Median CQI+1 |
| --- | --- | --- | --- |
| 0 | 5 | 6 | 1 |
| 2 | 6 | 8 | 1 |
| 4 | 6 | 8 | 1 |
| 6 | 7 | 11 | 1 |
| 8 | 8 | 13 | 1 |
| 9 | 9 | 15 | 1 |
| 10 | 10 | 18 | 1 |
| 12 | 11 | 20 | 1 |
| 14 | 12 | 22 | 1 |
| 15 | 12 | 22 | 1 |
| 16 | 13 | 24 | 1 |
| 18 | 14 | 26 | 1 |
| 20 | 15 | 28 | 1 |

Figure 2.1.5-2 Static CQI (BLER) – Test 3 2TX x 2RX Rank2

| SNR [dB] | Median CQI | MCS | BLER with Median CQI-1 | BLER with Median CQI | BLER with Median CQI+1 |
| --- | --- | --- | --- | --- | --- |
| 0 | 5 | 6 | 0 | 1 | 1 |
| 2 | 6 | 8 | 0 | 1 | 1 |
| 4 | 6 | 8 | 0 | 0 | 1 |
| 6 | 7 | 11 | 0 | 0 | 1 |
| 8 | 8 | 13 | 0 | 0 | 0.99 |
| 9 | 9 | 15 | 0 | 0 | 1 |
| 10 | 10 | 18 | 0 | 0.27 | 1 |
| 12 | 11 | 20 | 0 | 0.039 | 1 |
| 14 | 12 | 22 | 0 | 0.017 | 1 |
| 15 | 12 | 22 | 0 | 0 | 1 |
| 16 | 13 | 24 | 0. | 0.059 | 1 |
| 18 | 14 | 26 | 0 | 0.5 | 1 |
| 20 | 15 | 28 | 0 | 0.02 | 0.02(Max CQI) |

## 2.2 Fading Wideband CQI

Table 2.2.1~Table 2.2.5 are the simulation results for Fading Wideband CQI test 1~3 with 2RX and 4RX. The SNR range is from 0 to 20dB.

Table 2.2.1 Fading Wideband CQI – Test 1 2TX x 2RX

| SNR [dB] | Percentage of Reported CQI Index not in {median CQI1 -1, median CQI1 +1} (%) | BLER with following CQI | TP ratio with following CQI and Median CQI |
| --- | --- | --- | --- |
| 0 | 16 | 0.09 | 1.28 |
| 2 | 18 | 0.09 | 1.31 |
| 4 | 37 | 0.08 | 1.33 |
| 6 | 50 | 0.11 | 1.21 |
| 7 | 43 | 0.09 | 1.35 |
| 8 | 41 | 0.07 | 1.44 |
| 10 | 29 | 0.13 | 1.38 |
| 12 | 31 | 0.11 | 1.46 |
| 13 | 37 | 0.11 | 1.35 |
| 14 | 35 | 0.10 | 1.54 |
| 16 | 37 | 0.14 | 1.57 |
| 18 | 31 | 0.14 | 1.34 |
| 20 | 27 | 0.09 | 1.38 |

Table 2.2.1 Fading Wideband CQI – Test 1 2TX x 4RX

| SNR [dB] | Percentage of Reported CQI Index not in {median CQI1 -1, median CQI1 +1} (%) | BLER with following CQI | TP ratio with following CQI and Median CQI |
| --- | --- | --- | --- |
| 0 | 18 | 0.07 | 1.03 |
| 2 | 9 | 0.06 | 1.10 |
| 3 | 19 | 0.06 | 1.14 |
| 4 | 22 | 0.09 | 1.12 |
| 6 | 18 | 0.13 | 1.09 |
| 8 | 16 | 0.09 | 1.19 |
| 9 | 21 | 0.09 | 1.19 |
| 10 | 16 | 0.06 | 1.19 |
| 12 | 10 | 0.06 | 1.17 |
| 14 | 18 | 0.06 | 1.19 |
| 16 | 36 | 0.06 | 1.16 |
| 18 | 27 | 0.01 | 1.19 |
| 20 | 15 | 0.03 | 1.17 |

Table 2.2.3 Fading Wideband CQI – Test 2 2TX x 2RX

| SNR [dB] | Percentage of Reported CQI Index not in {median CQI1 -1, median CQI1 +1} (%) | BLER with following CQI | TP ratio with following CQI and Median CQI |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 1.37 |
| 2 | 11 | 0.01 | 1.16 |
| 4 | 13 | 0.02 | 1.20 |
| 6 | 34 | 0.01 | 1.32 |
| 7 | 45 | 0.02 | 1.17 |
| 8 | 43 | 0.01 | 1.31 |
| 10 | 34 | 0.04 | 1.30 |
| 12 | 23 | 0.04 | 1.28 |
| 13 | 37 | 0.03 | 1.26 |
| 14 | 23 | 0.04 | 1.38 |
| 16 | 28 | 0.08 | 1.46 |
| 18 | 29 | 0.09 | 1.59 |
| 20 | 23 | 0.09 | 1.19 |

Table 2.2.4 Fading Wideband CQI – Test 2 2TX x 4RX

| SNR [dB] | Percentage of Reported CQI Index not in {median CQI1 -1, median CQI1 +1} (%) | BLER with following CQI | TP ratio with following CQI and Median CQI |
| --- | --- | --- | --- |
| 0 | 0 | 0.08 | 1.31 |
| 2 | 13 | 0.07 | 1.44 |
| 3 | 12 | 0.06 | 1.18 |
| 4 | 19 | 0.07 | 1.07 |
| 6 | 22 | 0.04 | 1.53 |
| 8 | 4 | 0.08 | 1.42 |
| 9 | 7 | 0.04 | 1.13 |
| 10 | 4 | 0.02 | 1.42 |
| 12 | 10 | 0.04 | 1.60 |
| 14 | 13 | 0.10 | 1.64 |
| 16 | 11 | 0.14 | 1.15 |
| 18 | 10 | 0.04 | 1.33 |
| 20 | 0 | 0.01 | 1.02 |

Table 2.2.5 Fading Wideband CQI – Test 3 2TX x 2RX

| SNR [dB] | Percentage of Reported CQI Index not in {median CQI1 -1, median CQI1 +1} (%) | BLER with following CQI | TP ratio with following CQI and Median CQI |
| --- | --- | --- | --- |
| 0 | 13 | 0.15 | 1.27 |
| 2 | 17 | 0.12 | 1.27 |
| 4 | 17 | 0.12 | 1.30 |
| 6 | 15 | 0.09 | 1.28 |
| 7 | 18 | 0.07 | 1.41 |
| 8 | 15 | 0.09 | 1.24 |
| 10 | 15 | 0.06 | 1.24 |
| 12 | 14 | 0.05 | 1.28 |
| 13 | 11 | 0.07 | 1.32 |
| 14 | 16 | 0.03 | 1.28 |
| 16 | 19 | 0.03 | 1.23 |
| 18 | 7 | 0.03 | 1.26 |
| 20 | 5 | 0.01 | 1.21 |

## 2.3 Fading Sub-band CQI

Table 2.3.1~Table 2.3.4 are the simulation results for Fading Sub-band CQI test 1~2 with 2RX and 4RX. The SNR range is from 0 to 20dB.

Table 2.2.1 Fading Sub-band CQI – Test 1 2TX x 2RX

| SNR [dB] | Percentage of Reported sub-band CQI offset equals 0, % | BLER with following CQI on selected Sub-band | TP ratio with following CQI on selected Sub-band and median wideband CQI on random sub-band |
| --- | --- | --- | --- |
| 0 | 24 | 0.28 | 2.24 |
| 2 | 17 | 0.25 | 1.85 |
| 4 | 19 | 0.17 | 1.79 |
| 6 | 11 | 0.25 | 1.63 |
| 8 | 14 | 0.26 | 1.62 |
| 10 | 14 | 0.16 | 1.71 |
| 12 | 12 | 0.18 | 1.62 |
| 14 | 9 | 0.10 | 1.72 |
| 16 | 9 | 0.10 | 1.69 |
| 18 | 8 | 0.10 | 1.77 |
| 20 | 7 | 0.07 | 1.65 |

Table 2.2.1 Fading Sub-band CQI – Test 1 2TX x 4RX

| SNR [dB] | Percentage of Reported sub-band CQI offset equals 0, % | BLER with following CQI on selected Sub-band | TP ratio with following CQI on selected Sub-band and median wideband CQI on random sub-band |
| --- | --- | --- | --- |
| 0 | 16 | 0.32 | 1.50 |
| 2 | 15 | 0.32 | 1.44 |
| 4 | 15 | 0.27 | 1.62 |
| 6 | 19 | 0.22 | 1.63 |
| 8 | 13 | 0.17 | 1.70 |
| 10 | 17 | 0.17 | 1.63 |
| 12 | 17 | 0.08 | 1.71 |
| 14 | 11 | 0.08 | 1.70 |
| 16 | 5 | 0.10 | 1.71 |
| 18 | 5 | 0.08 | 1.56 |
| 20 | 5 | 0.13 | 1.47 |

Table 2.2.3 Fading Sub-band CQI – Test 2 2TX x 2RX

| SNR [dB] | Percentage of Reported sub-band CQI offset equals 0, % | BLER with following CQI on selected Sub-band | TP ratio with following CQI on selected Sub-band and median wideband CQI on random sub-band |
| --- | --- | --- | --- |
| 0 | 25 | 0.10 | 2.85 |
| 2 | 12 | 0.15 | 2.14 |
| 4 | 17 | 0.09 | 1.92 |
| 6 | 9 | 0.10 | 2.00 |
| 8 | 10 | 0.12 | 1.94 |
| 10 | 11 | 0.12 | 1.85 |
| 12 | 9 | 0.09 | 1.79 |
| 14 | 10 | 0.05 | 1.75 |
| 16 | 10 | 0.03 | 1.93 |
| 18 | 7 | 0.05 | 1.77 |
| 20 | 5 | 0.0 | 1.77 |

Table 2.2.4 Fading Sub-band CQI – Test 2 2TX x 4RX

| SNR [dB] | Percentage of Reported sub-band CQI offset equals 0, % | BLER with following CQI on selected Sub-band | TP ratio with following CQI on selected Sub-band and median wideband CQI on random sub-band |
| --- | --- | --- | --- |
| 0 | 13 | 0.08 | 2.31 |
| 2 | 11 | 0.1 | 1.72 |
| 4 | 10 | 0.1 | 1.68 |
| 6 | 12 | 0.09 | 1.75 |
| 8 | 14 | 0.08 | 1.52 |
| 10 | 11 | 0.06 | 1.53 |
| 12 | 16 | 0.04 | 1.50 |
| 14 | 14 | 0.06 | 1.68 |
| 16 | 7 | 0.09 | 1.74 |
| 18 | 6 | 0.04 | 1.51 |
| 20 | 4 | 0.09 | 1.46 |

# 3	Conclusions

In the contribution, we provide our simulation results of CSI CQI tests.

# 4	Reference

[1] R4-1902404, “Way forward on NR UE CSI requirements”, Samsung

Appendix

Table A.1 Static Wideband CQI test cases

| Parameter | Test 1 (FR1 FDD) | Test 2 (FR1 TDD) | Test 3 (FR2) |
| --- | --- | --- | --- |
| Bandwidth/SCS | 10MHz /15kHz | 40MHz/30kHz | 100MHz/120kHz |
| Transmission Scheme | 1 | 1 | 1 |
| Duplex Mode | FDD | TDD | TDD |
| DLUL configuration | N.A | 7D1S2U (FR1.30-1) | DDSU (FR2.120-2) |
| Propagation channel | AWGN | AWGN | AWGN |
| Correlation and antenna configuration | 2*2, 2*4 with static channel in TS38.101-4 Annex B | 2*2, 2*4 with static channel in TS38.101-4 Annex B | 2Rx 2*2 with static channel in TS38.101-4 Annex B |
| MCS Table | 256QAM | 256QAM | 64QAM |
| resourceConfigType | Periodic | Periodic | Aperiodic |
| CSI-RS/CSI-IM periodicity and slot offset | 5slot 1 slot offset | 10slot, 1 slot offset | 8slot, 1 slot offset |
| NZP CSI-RS ports | 2 port {3000,3001} | 2 port {3000,3001} | 2 port {3000,3001} |
| CSI-IM configuration | FFS | FFS | FFS |
| reportConfigType | periodic | periodic | periodic |
| Reporting periodicity and slot offset | 5/1 | 10/1 | Opt1: 16/1 Opt2: 8/1 |
| Codebook Restriction | 010000 (fixed rank2) | 010000 (fixed rank2) | 010000 (fixed rank2) |
| Reporting granularity | Wide-band | Wide-band | Wide-band |
| Physical channel for reporting | PUSCH | PUSCH | PUSCH |
| PUCCH reporting format | FFS | FFS | FFS |
| CQI table | Table 2 with 256QAM | Table 2 with 256QAM | Table 1 without 256QAM |
| Minimum CSI delay | 8ms | 9.5ms | 8.375ms |

Table A.2 Fading Wideband CQI test cases

| Parameter | Test 1 (FR1 FDD) | Test 2 (FR1 TDD) | Test 3 (FR2) |
| --- | --- | --- | --- |
| Bandwidth/SCS | 10MHz /15kHz | 40MHz/30kHz | 100MHz/120kHz |
| Transmission Scheme | 1 | 1 | 1 |
| Duplex Mode | FDD | TDD | TDD |
| DLUL configuration | N.A | 7D1S2U | DDSU |
| Propagation channel | TDL_A 30ns, 5Hz | TDL_A 30ns, 5Hz | TDL_A 30ns, 35Hz |
| Correlation and antenna configuration | 2Rx 2*2 4Rx: 2*4 [ULA High] for 2Rx [XP High] for 4Rx | 2Rx 2*2 4Rx: 2*4 [ULA High] for 2Rx [XP High ]for 4Rx | 2Rx 2*2 [XP High] |
| resourceConfigType | Periodic | Periodic | Aperiodic |
| NZP CSI-RS ports | 2 port {3000,3001} | 2 port {3000,3001} | 2 port {3000,3001} |
| reportConfigType | periodic | periodic | Aperiodic |
| Reporting periodicity and slot offset | 5/1 | 10/1 | 8/1 |
| Codebook Restriction | 000001 | 000001 | 000001 |
| Reporting granularity | Wide-band | Wide-band | Wide-band |
| Physical channel for reporting | PUSCH | PUSCH | PUSCH |
| PUCCH reporting format | FFS | FFS | FFS |
| CQI table | Table 2 with 256QAM | Table 2 with 256QAM | Table 1 without 256QAM |
| Minimum CSI delay | 8ms | 9.5ms | 1.375ms |

Table A.3 Fading Sub-band CQI test cases

| Parameter | Test 1 (FR1 FDD) | Test 2 (FR1 TDD) |
| --- | --- | --- |
| Bandwidth/SCS | 10MHz /15kHz | 40MHz/30kHz |
| Transmission Scheme | 1 | 1 |
| Duplex Mode | FDD | TDD |
| DLUL configuration | N.A | 7D1S2U (FR1.30-1) |
| Propagation channel | Clause B.2.4 in TS 38.101-4, with a =1,fd=5Hz and td =0.45us | Clause B.2.4 in TS 38.101-4, with a =1,fd=5Hz and td=0.1125us |
| Correlation and antenna configuration | Option 1: 1*2 and 1*4 Option2: 2*2 and 2*4 (baseline) | Option 1: 1*2 and 1*4 Option2: 2*2 and 2*4 (baseline) |
| MCS Table | 256QAM | 256QAM |
| resourceConfigType | Periodic | Periodic |
| NZP CSI-RS ports | 2 port {3000,3001} | 2 port {3000,3001} |
| reportConfigType | periodic | periodic |
| Reporting periodicity and slot offset | 5/1 | 10/1 |
| Codebook Restriction | 000001 | 000001 |
| Reporting granularity | Sub-band | Sub-band |
| Physical channel for reporting | PUSCH | PUSCH |
| PUCCH reporting format | FFS | FFS |
| CQI table | Table 2 with 256QAM | Table 2 with 256QAM |
| Minimum CSI delay | 8ms | 9.5ms |

Table A.4 CSI simulation detailed parameter setting

| Items | Setting | Notes |
| --- | --- | --- |
| TDD UL/DL configurations | - FR1 TDD: 7D1S2U, S = 6D:4G:4U - FR2 TDD: DDSU, S=11D+3G |  |
| HARQ | - For PMI, RV sequence ={0,2,3,1} - For RI and CQI test cases ,no retransmission |  |
| CSI-RS resource/CSI reporting periodicity slot offset for periodic CSI reporting | - FR1 FDD 15 kHz: 5/1 - FR1 TDD 30 kHz: Option 1: 10/1 - FR2 TDD 120 kHz: - Option 1: 16/1 - Option 2: 8/1 |  |
| Minimum CSI delay for periodic CSI | 8ms |  |
| CSI-RS resource/CSI reporting interval and slot offset for aperiodic CSI reporting | - FR1 FDD 15 kHz: 3/1 with total CSI delay [8]slots - FR1 TDD 30 kHz: 5/1 with total CSI delay [12] slots - FR2 TDD 120 kHz: 12/1 with total CSI delay [27] slots |  |
| CSI reporting and CSI-RS resources Type for fading CQI test cases and RI test cases of FR2 | - Option 1: Periodic (previous meeting agreements) - Option 2: aperiodic (baseline) - Companies are encouraged to bring analysis for above options considering different CSI delay assumptions for periodic and aperiodic CSI reporting types |  |
| Number of HARQ process number | - FR1 FDD: 4 - FR1 TDD: 8 - FR2 TDD: 8 |  |

Table A.5 NZP CSI-RS configurations

| First subcarrier index in the PRB used for CSI-RS (k0, k1) | Row 3, (6,-) | Row 4,(0,-) | Row 8,(4,6) |
| --- | --- | --- | --- |
| First OFDM symbol in the PRB used for CSI-RS (l0) | 13 | 13 | 5 |
| Number of CSI-RS ports (X) | 2 | 4 | 8 |
| CDM Type | FD-CDM2 | FD-CDM2 | CDM4 (FD2,TD2) |
| Density (ρ) | 1 | 1 | 1 |

Table A.6 ZP CSI-RS configurations

| Parameters | FR1 | FR2 |
| --- | --- | --- |
| First subcarrier index in the PRB used for CSI-RS (k0) | Row 5, 4 | Row 4, 8 |
| First OFDM symbol in the PRB used for CSI-RS (l0) | 9 | 13 |
| Number of CSI-RS ports (X) | 4 | 4 |
| CDM Type | FD-CDM2 | FD-CDM2 |
| Density (ρ) | 1 | 1 |

Table A.7 CSI-IM configurations

| Parameters | FR1 | FR2 |
| --- | --- | --- |
| CSI-IM-RE-pattern | Pattern 0 | Pattern 1 |
| CSI-IM-ResourceMapping (kCSI-IM,lCSI-IM) | (4,9) | (8,13) |