# 3GPP TSG-RAN WG4 Meeting #90bis                                                        R4-1903281

# XiAN, China, 8th - 12th April, 2019

Agenda Item:	6.12.2.3.1

Source:	ZTE Corporation

Title:	Simulation results for multi-slot PUCCH

Document for:	Approval

# Introduction

In RAN4 #90, a WF was approved on multi-slot NR PUCCH with two open issues:

- PUCCH format

  - Option 1: PUCCH format 3

  - Option 2: PUCCH format 1.

- Frequency hopping

  - Option 1: Inter-slot frequency hopping enabled and intra-slot frequency hopping disabled

  - Option 2: both intra and inter slot frequency hopping are disabled

In this contribution, we provide some numerical results related to frequency hopping.

# Discussion

According to TS 38.213 [2], if inter-slot frequency hopping enable, intra-slot frequency hoping must be disabled, or if inter-slot frequency hoping is disabled, intra-slot frequency hopping can be either enabled or disabled. We simulate the three allowed cases for PUCCH format 1 with similar simulation setup to that for non-repetition PUCCH format 1 (Due to limited time, simulation on format 3 is not finished yet):

Case 1: intra-slot frequency hopping enabled, and inter-slot frequency hopping disabled

Case 2: intra-slot frequency hopping disabled, and inter-slot frequency hopping disabled

Case 3: intra-slot frequency hopping disabled, and inter-slot frequency hopping enabled

The following table shows numerical results for these three cases for PUCCH format 1:

The second open issue in the WF on multi-slot PUCCH is the choice of either case 2 or case 3. From the simulation results, case 3 shows quite high performance gains over case 2. Therefore, we propose to take option 1 for specifying performance requirements for multi-slot PUCCH, which enables inter-slot frequency hopping and disables intra-slot frequency hopping.

Observation 1: Inter-slot frequency hopping enables quite high performance gains.

Proposal 1: Select option 1 which enables inter-slot frequency hopping to define multi-slot PUCCH performance requirements.

# Conclusion

In this contribution, we provide some numerical results for multi-slot PUCCH and have the following observations and proposals:

Observation 1: Inter-slot frequency hopping enables quite high performance gains.

Proposal 1: Select option 1 which enables inter-slot frequency hopping to define multi-slot PUCCH performance requirements.

# Reference

[1] R4-1902441 WF on multi-slot PUCCH, ZTE

[2] 3GPP TS 38.213 v15.4.0