3GPP TSG-RAN WG4 Meeting #90-Bis	R4-1903291

Xi’an, China, 8 – 12 April, 2019

Agenda item:	7.12.3

Source: 	Qualcomm Incorporated

Title: 	Simulation results for RSS based RRM measurements for MTC

Document for:	Discussion

# Introduction

In the RAN4#90 meeting, simulation assumptions for RSS based RRM measurements for MTC was agreed which is reproduced here for reference [1]:

Table 1: Simulation parameters for Rel-16 MTC RSRP measurement studies using RSS

| Parameters | Value | Comments |
| --- | --- | --- |
| RSS bandwidth | 2 consecutive resource blocks |  |
| RSS durations | {8, 40} subframes |  |
| RSS periodicity | 160 ms | Option 1: non-colliding RSS Option 2: Colliding RSS, |
| Ratio of RSS RE energy to CRS RE energy | 0 dB |  |
| Power level of neighbour cell relative to serving cell | {0, -3} dB |  |
| Measurement bandwidth | 2 resource blocks | RSRP measured over the 2 RSS PRBs. |
| System bandwidth | 6 resource blocks |  |
| L1 measurement period | Single shot, 480 ms, 800 ms | Single shot measurement does not include any time-domain averaging over multiple samples |
| Measurement sampling rate |  | Implementation dependent (NOTE 1) |
| L3 filtering | Disabled |  |
| Transmit antenna | 1 |  |
| Receive antennas | 1, 2 | Single and double Rx branches, respectively |
| Mobility | Stationary UEs, mobile UEs |  |
| Propagation conditions | AWGN, ETU and EPA |  |
| Doppler Frequency for stationary UEs: ETU and EPA | 1 Hz and 1 Hz, respectively |  |
| Doppler Frequency for mobile UEs: ETU and EPA | 30 Hz and 5 Hz, respectively |  |
| Channel estimation techniques | Measurement averaging techniques used currently for category M1 | Implementation dependent (NOTE 2) |
| Number of RSS subframes used for measurement |  | Implementation dependent (NOTE 3) |
| CP length | Normal |  |
| Carrier frequency | 2 GHz |  |
| Ec/Iot | -15 dB, …, 5 dB | AWGN noise |
| NOTE 1: Companies are requested to provide the details of the measurement sampling rate for interpretation and comparison of the results NOTE 2: Companies are requested to provide the details of the RS averaging techniques for interpretation and comparison of the results. NOTE 3: The number of RSS subframes used for measurement is implementation dependent, and can range from 1 to the RSS duration. Companies are requested to provide the details of the RSS subframes used for interpretation and comparison of the results. |  |  |

In this paper, we provide partial simulation results based on the above agreements.

# Simulation Methodology

Only single shot measurement results with single Rx are presented in this paper. The methodology consists of the followings:

- RSS duration with 8ms length and periodicity of 160ms is used.

- For colliding cell scenario, power level of neighbor is assumed to be -3 dB.

- Measurement sampling rate is 1.92 MHz.

- Two methods for single-shot RSRP measurement is assumed: a) 1 subframe (1SF) samples with coherent processing of all CRS (for CRS-based) or RSS (for RSS-based) tones; b) two subframes (2SF) of samples with non-coherent processing cross subframes for all CRS and RSS tones where non-coherent combining across subframes are with equal weights.

- In each RSS occasion, two measurements are done for the same channel realization: one based on CRS and the other based on RSS. Same number of subframes are used for fair comparison of the two measurements.

- For each SNR point, 500 RSS occasions were simulated to obtain CDF.

# Simulation Results

In the following figures, CDF of measured RSRP error using CRS or RSS for Es/Iot = {-15, -6} dB are presented for AWGN, EPA1, ETU1, EPA5, and ETU30 in single cell scenario using the methodology in Section 2.

Figure 1 shows the CDF of RSRP error for AWGN channel. RSS measurement with 1SF shows much better performance compared to CRS in low SNR both in median and 90-percentile points. In high SNR point, the performance different is less prominent. With 2SF processing, the gain of RSS measurement is much less and mostly seen in 90-percentile point.

Figure 1 CDF of RSRP error for AWGN

Figure 2 shows the CDF of RSRP error for EPA1 channel. Similar to AWGN, RSS measurement using 1SF has better performance compared to CRS but with 2SF processing, the gap in performance is mostly covered. In some cases (e.g., median point of Es/Iot = -15 dB), CRS using 2SF performs better than RSS.

Figure 2 CDF of RSRP error for EPA1

Figure 3 CDF of RSRP error for ETU1

Figure 4 CDF of RSRP error in EPA5

Figure 5 CDF of RSRP error in ETU30

Figures 3-5 show the CDF of RSRP error for ETU1, EPA5, and ETU30. The behaviour seen in these plots is consistent with previous ones. RSS with only 1SF processing has significant performance improvement over CRS particularly in Es/Iot = -15 dB but with 2SF processing and increasing SNR, the performance improvement over CRS is negligible.

Tables 1 and 2 below summarize the observations from Figures 1-5 for median and 90-percentile point in each channel and each Es/Iot level. Performance improvement of RSS over CRS is tabulated in these tables. For example, a positive value X in median row for a channel means that RSS measurement accuracy has X dB improvement over CRS measurement accuracy in the median point. A negative value means CRS measurement has better accuracy (smaller error) compared to RSS measurement. Table 1 tabulates the improvement of RSS-based measurement over CRS-based measurement with 1SF processing and Table 2 tabulates the same for 2SF processing.

These two tables show that RSS-based measurement has merit over CRS-based measurement only when 1-subframe is captured in low SNR conditions (Es/Iot < - 6 dB). With increasing SNR or sample capture time to 2 subframes, there is much smaller gain from RSS-based measurement accuracy in the 90-percentile and no gain in median. In other words, using RSS for measurement is helpful when UE is in deep coverage mode and wishes to preserve power by limiting sample capture time. In such conditions, it is better for UE to align its sample capture time with RSS occasions. However, if UE is supposed to capture more than 1 subframe for measurement, then there is no need to align its sample capture for measurement purpose with RSS occasion as CRS-based measurement delivers the same accuracy as RSS and CRS is present in every subframe.

Table 1 Measurement accuracy improvement (RSS-based vs. CRS-based) with 1SF processing

| Single Cell |  | AWGN | EPA1 | ETU1 | EPA5 | ETU30 |
| --- | --- | --- | --- | --- | --- | --- |
| Es/Iot = -15 | 50% | 4.1 | 5 | 4.4 | 5.1 | 4.5 |
|  | 90% | 4.9 | 4.6 | 4.2 | 4.1 | 4.2 |
| Es/Iot = -12 | 50% | 2.8 | 3.3 | 3.4 | 3.6 | 3.3 |
|  | 90% | 3.8 | 2.4 | 2.2 | 2.2 | 2.4 |
| Es/Iot = -9 | 50% | 1.7 | 1.3 | 1.1 | 0.4 | 1.1 |
|  | 90% | 2.7 | 1.2 | 1 | 1.7 | 1.3 |
| Es/Iot = -6 | 50% | 0.9 | -1 | -0.9 | -0.9 | -0.9 |
|  | 90% | 1.9 | 0.8 | 0.6 | 0.8 | 0.2 |
| Es/Iot = -3 | 50% | 0.7 | -0.5 | -0.6 | -0.6 | -0.7 |
|  | 90% | 1.4 | 0.5 | 0 | 0.5 | 0 |
| Es/Iot = 0 | 50% | 0.1 | -0.7 | -0.2 | -0.4 | -0.1 |
|  | 90% | 0.7 | 0 | -0.1 | 0.2 | -0.2 |

Table 2 Measurement accuracy improvement (RSS-based vs. CRS-based) with 2SF processing

| Single Cell |  | AWGN | EPA1 | ETU1 | EPA5 | ETU30 |
| --- | --- | --- | --- | --- | --- | --- |
| Es/Iot = -15 | 50% | 0 | 0.7 | -0.5 | 0 | -0.3 |
|  | 90% | 2.8 | 2 | 1.7 | 1.6 | 1.8 |
| Es/Iot = -12 | 50% | 0.3 | -0.5 | 0.6 | -0.4 | 0.7 |
|  | 90% | 2.2 | 1.3 | 0.4 | 1.3 | 0.9 |
| Es/Iot = -9 | 50% | 0.5 | 0 | 0.8 | -0.9 | 0.3 |
|  | 90% | 1.7 | 0.5 | 0.3 | 0.2 | 0.4 |
| Es/Iot = -6 | 50% | 0.2 | -0.5 | -0.3 | -0.6 | -0.1 |
|  | 90% | 1 | 0 | 0.1 | 0.4 | 0.2 |
| Es/Iot = -3 | 50% | 0.1 | -0.5 | -0.2 | -0.2 | -0.1 |
|  | 90% | 0.8 | 0.3 | -0.4 | 0.2 | -0.5 |
| Es/Iot = 0 | 50% | 0.1 | 0.2 | 0 | -0.1 | 0.1 |
|  | 90% | 0.5 | 0.1 | -0.7 | 0.1 | 0 |

Observation 1. RSS-based measurement shows improvement over CRS-based measurement when:

- Es/Iot < -6 dB and sample capture for measurement is limited to 1 subframe with significant gains in both median and 90-percentile points.

- Es/Iot <= -12 dB and 2-subframe sample capture with much smaller gain in 90-percentile point.

Observation 2. Based on the results of Tables 1 and 2, further simulations of RSS duration with 40 subframes or L1 measurement period beyond single shot is not necessary.

Next, results with 2 cells (with neighbour cell 3 dB weaker than serving cell) are presented. For the sake of brevity, we only present the summary tables as the behaviour pattern is consistent with single cell scenario. The observations in 2-cell scenario with neighbour cell 3 dB weaker than serving cell is consistent with single cell scenarios.

Observation 3. Performance of RSS-based and CRS-based measurement accuracy in 2-cell scenario (with neighbour cell 3 dB weaker than serving cell) is consistent with single cell scenarios.

Table 3 Measurement accuracy improvement for 2-cell with 1SF processing

| Two Cells Diff = -3dB |  | AWGN | EPA1 | ETU1 | EPA5 | ETU30 |
| --- | --- | --- | --- | --- | --- | --- |
| Es/Iot = -15 | 50% | 3.5 | 4.9 | 4.5 | 4.6 | 5 |
|  | 90% | 4.4 | 4.1 | 3.9 | 4.1 | 4.2 |
| Es/Iot = -12 | 50% | 2 | 3.9 | 3.5 | 3.7 | 3.2 |
|  | 90% | 3.1 | 2.9 | 2.6 | 2.6 | 2.7 |
| Es/Iot = -9 | 50% | 0.7 | 2.5 | 1.8 | 2.1 | 1.8 |
|  | 90% | 2.1 | 1.2 | 1.4 | 1.3 | 1.3 |
| Es/Iot = -6 | 50% | 0 | -0.1 | -0.1 | 0.1 | 0.8 |
|  | 90% | 0.9 | 0.5 | 0.8 | 0.5 | 0.9 |
| Es/Iot = -3 | 50% | -0.6 | 0.8 | 0.5 | 0.3 | 0.1 |
|  | 90% | 0.2 | 0.3 | 0.4 | 0.3 | 0.2 |
| Es/Iot = 0 | 50% | -0.9 | 0.9 | 0.6 | 0.6 | 0.9 |
|  | 90% | -0.2 | 0 | 0 | 0 | -0.2 |

Table 4 Measurement accuracy improvement for 2-cell with 2SF processing

| Two Cells Diff = -3dB |  | AWGN | EPA1 | ETU1 | EPA5 | ETU30 |
| --- | --- | --- | --- | --- | --- | --- |
| Es/Iot = -15 | 50% | -0.6 | 0.3 | -0.4 | 0 | -0.7 |
|  | 90% | 2 | 2.2 | 1.8 | 1.7 | 1.3 |
| Es/Iot = -12 | 50% | -0.6 | -0.2 | -0.7 | 0.1 | 0.4 |
|  | 90% | 1.4 | 1.4 | 1.1 | 0.8 | 0.6 |
| Es/Iot = -9 | 50% | -0.6 | 0.1 | -0.7 | -0.1 | 0.5 |
|  | 90% | 0.8 | 0.7 | 0.6 | 0.3 | -0.1 |
| Es/Iot = -6 | 50% | -0.7 | -1.2 | -0.8 | -0.7 | 0.1 |
|  | 90% | 0.3 | 0.1 | 0 | 0 | 0 |
| Es/Iot = -3 | 50% | -0.8 | -0.3 | -0.5 | -0.4 | 0 |
|  | 90% | 0 | -0.1 | 0.1 | 0 | -0.6 |
| Es/Iot = 0 | 50% | -0.8 | -0.3 | 0.3 | 0.2 | -0.1 |
|  | 90% | -0.4 | 0 | -0.1 | 0 | -0.5 |

Based on these results, we propose RAN4 to consider improved RSS-based measurement accuracy requirements only for operating range of Es/Iot < 6 dB and idle state. In higher SNR regime or connected state, UE can achieve the same accuracy level as RSS-based measurement by capturing more successive samples. These conclusions are consistent with our earlier analysis paper [2].

It is worth noting that in idle mode, different UEs may wake up at different times (UEs with different UE_ID will, in general, monitor different paging occasions), so it may not be possible to align the RSS transmissions with every UE in the cell.

Observation 4. For UEs in idle mode, it is not possible (in most cases) to align the RSS transmission to every UE’s PO.

Proposal 1. RAN4 to consider improved RSS-based measurement accuracy requirements only if UE’s Es/Iot < 6 dB and UE is in idle state and UE’s wake up period aligns with RSS occasion.

# Conclusions

With the simulation methodology of Section 2, RSS-based measurement accuracy improvement compared to CRS-based measurement can be summarized:

Table 1 Measurement accuracy improvement (RSS-based vs. CRS-based) with 1SF processing

| Single Cell |  | AWGN | EPA1 | ETU1 | EPA5 | ETU30 |
| --- | --- | --- | --- | --- | --- | --- |
| Es/Iot = -15 | 50% | 4.1 | 5 | 4.4 | 5.1 | 4.5 |
|  | 90% | 4.9 | 4.6 | 4.2 | 4.1 | 4.2 |
| Es/Iot = -12 | 50% | 2.8 | 3.3 | 3.4 | 3.6 | 3.3 |
|  | 90% | 3.8 | 2.4 | 2.2 | 2.2 | 2.4 |
| Es/Iot = -9 | 50% | 1.7 | 1.3 | 1.1 | 0.4 | 1.1 |
|  | 90% | 2.7 | 1.2 | 1 | 1.7 | 1.3 |
| Es/Iot = -6 | 50% | 0.9 | -1 | -0.9 | -0.9 | -0.9 |
|  | 90% | 1.9 | 0.8 | 0.6 | 0.8 | 0.2 |
| Es/Iot = -3 | 50% | 0.7 | -0.5 | -0.6 | -0.6 | -0.7 |
|  | 90% | 1.4 | 0.5 | 0 | 0.5 | 0 |
| Es/Iot = 0 | 50% | 0.1 | -0.7 | -0.2 | -0.4 | -0.1 |
|  | 90% | 0.7 | 0 | -0.1 | 0.2 | -0.2 |

Table 2 Measurement accuracy improvement (RSS-based vs. CRS-based) with 2SF processing

| Single Cell |  | AWGN | EPA1 | ETU1 | EPA5 | ETU30 |
| --- | --- | --- | --- | --- | --- | --- |
| Es/Iot = -15 | 50% | 0 | 0.7 | -0.5 | 0 | -0.3 |
|  | 90% | 2.8 | 2 | 1.7 | 1.6 | 1.8 |
| Es/Iot = -12 | 50% | 0.3 | -0.5 | 0.6 | -0.4 | 0.7 |
|  | 90% | 2.2 | 1.3 | 0.4 | 1.3 | 0.9 |
| Es/Iot = -9 | 50% | 0.5 | 0 | 0.8 | -0.9 | 0.3 |
|  | 90% | 1.7 | 0.5 | 0.3 | 0.2 | 0.4 |
| Es/Iot = -6 | 50% | 0.2 | -0.5 | -0.3 | -0.6 | -0.1 |
|  | 90% | 1 | 0 | 0.1 | 0.4 | 0.2 |
| Es/Iot = -3 | 50% | 0.1 | -0.5 | -0.2 | -0.2 | -0.1 |
|  | 90% | 0.8 | 0.3 | -0.4 | 0.2 | -0.5 |
| Es/Iot = 0 | 50% | 0.1 | 0.2 | 0 | -0.1 | 0.1 |
|  | 90% | 0.5 | 0.1 | -0.7 | 0.1 | 0 |

Observation 1. RSS-based measurement shows improvement over CRS-based measurement when:

- Es/Iot < -6 dB and sample capture for measurement is limited to 1 subframe with significant gains in both median and 90-percentile points.

- Es/Iot <= -12 dB and 2-subframe sample capture with much smaller gain in 90-percentile point.

Observation 2. Based on the results of Tables 1 and 2, further simulations of RSS duration with 40 subframes or L1 measurement period beyond single shot is not necessary.

Observation 3. Performance of RSS-based and CRS-based measurement accuracy in 2-cell scenario (with neighbour cell 3 dB weaker than serving cell) is consistent with single cell scenarios.

Table 3 Measurement accuracy improvement for 2-cell with 1SF processing

| Two Cells Diff = -3dB |  | AWGN | EPA1 | ETU1 | EPA5 | ETU30 |
| --- | --- | --- | --- | --- | --- | --- |
| Es/Iot = -15 | 50% | 3.5 | 4.9 | 4.5 | 4.6 | 5 |
|  | 90% | 4.4 | 4.1 | 3.9 | 4.1 | 4.2 |
| Es/Iot = -12 | 50% | 2 | 3.9 | 3.5 | 3.7 | 3.2 |
|  | 90% | 3.1 | 2.9 | 2.6 | 2.6 | 2.7 |
| Es/Iot = -9 | 50% | 0.7 | 2.5 | 1.8 | 2.1 | 1.8 |
|  | 90% | 2.1 | 1.2 | 1.4 | 1.3 | 1.3 |
| Es/Iot = -6 | 50% | 0 | -0.1 | -0.1 | 0.1 | 0.8 |
|  | 90% | 0.9 | 0.5 | 0.8 | 0.5 | 0.9 |
| Es/Iot = -3 | 50% | -0.6 | 0.8 | 0.5 | 0.3 | 0.1 |
|  | 90% | 0.2 | 0.3 | 0.4 | 0.3 | 0.2 |
| Es/Iot = 0 | 50% | -0.9 | 0.9 | 0.6 | 0.6 | 0.9 |
|  | 90% | -0.2 | 0 | 0 | 0 | -0.2 |

Table 4 Measurement accuracy improvement for 2-cell with 2SF processing

| Two Cells Diff = -3dB |  | AWGN | EPA1 | ETU1 | EPA5 | ETU30 |
| --- | --- | --- | --- | --- | --- | --- |
| Es/Iot = -15 | 50% | -0.6 | 0.3 | -0.4 | 0 | -0.7 |
|  | 90% | 2 | 2.2 | 1.8 | 1.7 | 1.3 |
| Es/Iot = -12 | 50% | -0.6 | -0.2 | -0.7 | 0.1 | 0.4 |
|  | 90% | 1.4 | 1.4 | 1.1 | 0.8 | 0.6 |
| Es/Iot = -9 | 50% | -0.6 | 0.1 | -0.7 | -0.1 | 0.5 |
|  | 90% | 0.8 | 0.7 | 0.6 | 0.3 | -0.1 |
| Es/Iot = -6 | 50% | -0.7 | -1.2 | -0.8 | -0.7 | 0.1 |
|  | 90% | 0.3 | 0.1 | 0 | 0 | 0 |
| Es/Iot = -3 | 50% | -0.8 | -0.3 | -0.5 | -0.4 | 0 |
|  | 90% | 0 | -0.1 | 0.1 | 0 | -0.6 |
| Es/Iot = 0 | 50% | -0.8 | -0.3 | 0.3 | 0.2 | -0.1 |
|  | 90% | -0.4 | 0 | -0.1 | 0 | -0.5 |

Observation 4. For UEs in idle mode, it is not possible (in most cases) to align the RSS transmission to every UE’s PO.

Proposal 1. RAN4 to consider improved RSS-based measurement accuracy requirements only if UE’s Es/Iot < 6 dB and UE is in idle state and UE’s wake up period aligns with RSS occasion.

# References

[1] R4-1902018

[2] R4-1900334