3GPP TSG-RAN4 Meeting #90bis 	R4-1903403

Xi’an, China, 8th-12th April, 2019

Agenda item:	6.10.3.1.2

Source:	Intel Corporation

Title:	LS on capability of measurement gap patterns

Document for:	Discussion

# Introduction

In RAN2 #105 meeting, one LS[1] was approved to send to RAN4, duplicated as below

1. Overall Description RAN2 would like to inform RAN4 that RAN2 has discussed the capability signalling of measurement gap patterns. RAN2 agreed that current RRC signaling for gap pattern capabilities as following: - Gap patterns 0 and 1 are mandatory for all cases. - In LTE SA - LTE RRC signalling shortMeasurementGap is used to signal whether UE supports gap patterns 2 and 3. - LTE RRC signalling measGapPatterns (8 bits) is used to signal whether UE supports gap patterns 4 to 11. - In EN-DC - LTE RRC signalling shortMeasurementGap is used to signal whether UE supports gap patterns 2 and 3 (for per-UE cap and FR1 gap of per-FR gap). - LTE RRC signalling measGapPatterns is used to signal whether UE supports gap patterns 4 to 11 (for per-UE cap and FR1 gap of per-FR gap). - NR RRC signalling supportedGapPattern (22 bits) is used to signal whether UE supports gap patterns 2 to 23 (for FR2 gap of per-FR gap). - In NR SA - NR RRC signalling supportedGapPattern (22 bits) is used to signal whether UE supports gap patterns 2 to 23. - If the UE supports an optional gap pattern (#2 to #23), it supports using the gap pattern according to the corresponding applicability defined in RAN4 specifications. RAN2 would like to confirm with RAN4 that the above agreements are consistent with TS 36.133 and TS 38.133. 2. Actions: ACTION: RAN2 respectfully asks RAN4 to take above RAN2 agreements into consideration and check if they are consistent with TS 36.133 and TS 38.133.

In this contribution, we discuss whether RAN2 agreement is aligned with RAN4 spec and whether RAN4 need to revise TS38.133/36.133 to align with the RAN2 agreement.

# Discussion

- UE MG capability by using LTE RRC shortMeasurementGap-r14 + measGapPatterns

In LTE SA and EN-DC, there is two LTE RRC signalling used in RAN2 to indicate UE capability of measurement gap patterns: shortMeasurementGap and measGapPatterns. However, in the current TS38.133, the clarification is:

NOTE 2:	In E-UTRA-NR dual connectivity mode, the gap patterns with short MGL (gap pattern #2, 3, 6, 7, 8, 10)  are supported by UEs which support shortMeasurementGap-r14.

According to RAN2 LS, shortMeasurementGap is only used to signal whether UE supports gap patterns 2 and 3, but UE still needs measGapPatterns to indicate the supporting of pattern #6, 7, 8, 10. Since in RAN4 people understand that as long as UE can report shortMeasurementGap-r14=true to network, it means UE can support all the MG patterns with short MGL=3ms/4ms except the patterns with MGRP=160ms. We are fine to follow RAN2 agreement to use this extra signalling of measGapPatterns which as difined in TS36.331:

measGapPatterns

Indicates whether the UE that supports NR supports gap patterns 4 to 11. The first/ leftmost bit covers pattern 4, and so on. Value 1 indicates that the UE supports the concerned gap pattern. See TS 36.133 [16].

So based on the discussion above, the RAN4 spec TS38.133/TS36.133 shall be revised accordingly, as below,

TS38.133:

Table 9.1.2-2: Applicability for Gap Pattern Configurations supported by the E-UTRA-NR dual connectivity UE or NR-E-UTRA dual connectivity UE

| Measurement gap pattern configuration | Serving cell | Measurement Purpose | Applicable Gap Pattern Id |
| --- | --- | --- | --- |
| Per-UE measurement gap | E-UTRA + FR1, or E-UTRA + FR2, or E-UTRA + FR1 + FR2 | non-NR RAT Note1,2 | 0,1,2,3 |
|  |  | FR1 and/or FR2 | 0-11 |
|  |  | non-NR RATNote1,2 and FR1 and/or FR2 | 0, 1, 2, 3, 4, 6, 7, 8,10 |
| Per FR measurement gap | E-UTRA and, FR1 if configured | non-NR RAT Note1,2 | 0,1,2,3 |
|  | FR2 if configured |  | No gap |
|  | E-UTRA and, FR1 if configured | FR1 only | 0-11 |
|  | FR2 if configured |  | No gap |
|  | E-UTRA and, FR1 if configured | FR2 only | No gap |
|  | FR2 if configured |  | 12-23 |
|  | E-UTRA and, FR1 if configured | non-NR RAT Note1,2 and FR1 | 0, 1, 2, 3, 4, 6, 7, 8,10 |
|  | FR2 if configured |  | No gap |
|  | E-UTRA and, FR1 if configured | FR1 and FR2 | 0-11 |
|  | FR2 if configured |  | 12-23 |
|  | E-UTRA and, FR1 if configured | non-NR RAT Note1,2 and FR2 | 0, 1, 2, 3, 4, 6, 7, 8,10 |
|  | FR2 if configured |  | 12-23 |
|  | E-UTRA and, FR1 if configured | non-NR RAT Note1,2 and FR1 and FR2 | 0, 1, 2, 3, 4, 6, 7, 8,10 |
|  | FR2 if configured |  | 12-23 |
| Note: In E-UTRA-NR dual connectivity mode, if GSM or UTRA TDD or UTRA FDD inter-RAT frequency layer is configured to be monitered, only measurement gap pattern #0 and #1 can be used for per-FR gap in E-UTRA and FR1 if configured, or for per-UE gap. NOTE 1: In E-UTRA-NR dual connectivity mode, non-NR RAT includes E-UTRA, UTRA and/or GSM. In NR-E-UTRA dual connectivity mode, non-NR RAT means E-UTRA. NOTE 2: In E-UTRA-NR dual connectivity mode, the gap patterns with short MGL (gap pattern #2, 3, 6, 7, 8, 10) are supported by UEs which support shortMeasurementGap-r14 and indicate the capability signalling of measGapPatterns to network. In NR-E-UTRA dual connectivity mode, the measurement gap pattern #2, 3, 6, 7, 8, 10 are supported by the UEs which indicate the capability signalling of supportedGapPattern to network. NOTE 3: When E-UTRA inter-frequency RSTD measurements are configured and the UE requires measurement gaps for performing such measurements, only Gap Pattern #0 can be used. |  |  |  |

TS36.133:

Table 8.1.2.1-1: Gap Pattern Configurations supported by the UE

| Gap Pattern Id | MeasurementGap Length (MGL, ms) | Measurement Gap Repetition Period (MGRP, ms) | Minimum available time for inter-frequency and inter-RAT measurements during 480ms period (Tinter1, ms) | Measurement Purpose |
| --- | --- | --- | --- | --- |
| 0 | 6 | 40 | 60 | Inter-Frequency E-UTRAN FDD and TDD, UTRAN FDD, GERAN, LCR TDD, HRPD, CDMA2000 1x, inter-RAT NR |
| 1 | 6 | 80 | 30 | Inter-Frequency E-UTRAN FDD and TDD, UTRAN FDD, GERAN, LCR TDD, HRPD, CDMA2000 1x, inter-RAT NR |
| 2 | 3 | 40 | 24NOTE 1,2 | Inter-Frequency E-UTRAN FDD and TDD for cells with time difference as specified below. inter-RAT NR |
| 3 | 3 | 80 | 12NOTE 1,2 | Inter-Frequency E-UTRAN FDD and TDD for cells with time difference according as specified below. inter-RAT NR |
| [4] | [6] | [20] | 120 Note 1, 6 | inter-RAT NR, Inter-Frequency E-UTRAN FDD and TDD |
| [5] | 6 | 160 | Note 3 | inter-RAT NR |
| [6] | [4] | [20] | 72 Note 1, 5, 7 | inter-RAT NR, Inter-Frequency E-UTRAN FDD and TDD |
| [7] | 4 | 40 | 36 Note 1, 5, 8 | inter-RAT NR, Inter-Frequency E-UTRAN FDD and TDD |
| [8] | 4 | 80 | 18Note 1, 5, 9 | inter-RAT NR, Inter-Frequency E-UTRAN FDD and TDD |
| [9] | [4] | [160] | Note 3 | inter-RAT NR |
| [10] | 3 | 20 | 48 Note 1, 5 | inter-RAT NR, Inter-Frequency E-UTRAN FDD and TDD |
| [11] | 3 | 160 | Note 3 | inter-RAT NR |
| NOTE 1: When determing UE requirements using Tinter1 for GP2, 3, 4, 6, 7, 8, 10, Tinter1 = [60] for GP2, GP4, GP6, GP7, GP10 and Tinter1 = [30] for GP3 and GP8 shall be used. NOTE 2: This gap pattern can be configured for E-UTRA measurement when UEs support shortMeasurementGap-r14 or can be configured for inter-RAT NR measurement only. NOTE 3: This gap pattern can be used only for measurement of NR carriers, and Tinter is not applicable. NOTE 4: Void NOTE 5: This gap pattern is supported by UEs which are configured to perform both E-UTRA inter-frequency measurement and inter-RAT NR measurement when UEs indicate the capability in signalling measGapPatterns to network or supported by UEs configured to perform inter-RAT NR measurement only. NOTE 6: This gap pattern is supported by UEs which are configured to perform both E-UTRA inter-frequency measurement and inter-RAT NR measurement or supported by UEs configured to perform inter-RAT NR measurement only. NOTE 7: When this gap pattern is used, the Tinter for E-UTRA interfrequency measurements is 48ms corresponding to the first 3ms of the 4ms gap NOTE 8: When this gap pattern is used, the Tinter for E-UTRA interfrequency measurements is 24ms corresponding to the first 3ms of the 4ms gap NOTE 9: When this gap pattern is used, the Tinter for E-UTRA interfrequency measurements is 12ms corresponding to the first 3ms of the 4ms gap |  |  |  |  |

Proposal 1: revise TS38.133 and TS36.133 MG applicability tables to capture the UE capability signalling of both shortMeasurementGap-r14 and measGapPatterns via LTE RRC.

- Mandatory MG patterns

RAN2 captured mandatory MG patterns in their LS as: Gap patterns 0 and 1 are mandatory for all cases. However, in RAN4 TS36.133 and TS38.133, we have clear definition that for gap based RSTD measurements only MG pattern #0 can be used/mandatory. We think gap patterns 0 and 1 are mandatory for all cases except MG based RSTD measurements, and we need to reply the LS to clarify this exception case to RAN2.

Proposal 2: MG patterns 0 and 1 are mandatory for all cases except MG based RSTD measurements. Only MG pattern #0 is mandatory for MG based RSTD measurements.

# Draft reply LS to RAN2 [2]

RAN4 would like to thank RAN2 for the LS on capability of measurement gap patterns (R2-1902732). After discussion in RAN4, the feedback to RAN2 for their future work is as below,

- MG patterns 0 and 1 are mandatory for all cases except MG based RSTD measurements. Only MG pattern #0 is mandatory for MG based RSTD measurements.

- For LTE SA and EN-DC, RAN4 updated TS36.133 and TS38.133 to capture the UE capability signalling of both shortMeasurementGap-r14 and measGapPatterns via LTE RRC.

# Conclusion

In this contribution, we discuss whether RAN2 agreement is aligned with RAN4 spec and whether RAN4 need to revise TS38.133/36.133 to align with the RAN2 agreement.

Proposal 1: revise TS38.133 and TS36.133 MG applicability tables to capture the UE capability signalling of both shortMeasurementGap-r14 and measGapPatterns via LTE RRC.

Proposal 2: MG patterns 0 and 1 are mandatory for all cases except MG based RSTD measurements. Only MG pattern #0 is mandatory for MG based RSTD measurements.

# References

[1] R2-1902732, LS on capability of measurement gap patterns, MediaTek, RAN2 #105 meeting

[2] R4-1903404, Reply LS on capability of measurement gap patterns, Intel, RAN4 #90bis meeting