3GPP TSG-RAN4 Meeting #90bis	R4-1903454

# Xian, China, 8th – 12th April 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.817-02 | CR | 0035 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TR 38.817-02: Addition of RC MU tables for FR2 for spurious emission in subclause 12.7.1.2.2A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | At last meeting the general description for measuring spurious emission in a reverberation chamber was approved in TR 37.843. This CR adds MU evaluation table for RC for spurious emission in TR 38.817-02, sub-clause 12.7. This is a new version of R4-1900821 presented at RAN4#90. In companion contributions all open issues have been resolved. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new subclause 12.7.1.2.2A have been created to capture the measurement evaluation for RC measureing spurious emission for FR2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not approved, TR 38.817-02 will not described the capabilites of the reverberation chamber methodology to measure spurious emission. The reverberation chamber test methodology brings some interesting aspects related to test time to consider for spurious emission. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Subclause 12.7.1.2.2A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

##### 12.7.1.2.2A	Reverberation Chamber MU Assessment

The detailed descriptions of uncertainty sources relevant for the reverberation chamber test method can be found in [26].

Table 12.7.1.2.2A-1: Reverberation Chamber uncertainty assessment spurious emissions 380 MHz-18 GHz

| UID | Uncertainty source | Uncertainty value 380 MHz < f ≦ 3 GHz | Uncertainty value 3 GHz < f ≦ 3.8 GHz | Uncertainty value 3.8 GHz < f ≦ 12.75 GHz | Uncertainty value 12.75 GHz < f ≦ 18 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] 380 MHz < f ≦ 3 GHz | Standard uncertainty ui [dB] 3 GHz < f ≦ 3.8 GHz | Standard uncertainty ui [dB] 3.8 GHz < f ≦ 12.75 GHz | Standard uncertainty ui [dB] 12.75 GHz < f ≦ 18 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Uncertainty of the measurement equipment | 0.14 | 0.26 | 0.26 | 0.37 | Gaussian | 1 | 1 | 0.14 | 0.26 | 0.26 | 0.37 |
| 2 | Impedance mismatch in the receiving chain | 0.2 | 0.2 | 0.45 | 0.45 | U-shaped | √2 | 1 | 0.14 | 0.14 | 0.32 | 0.32 |
| 3 | Random uncertainty | 0.1 | 0.1 | 0.1 | 0.1 | Rectangular | √3 | 1 | 0.06 | 0.06 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | Reference antenna radiation efficiency | 0.50 | 0.50 | 0.50 | 0.50 | Gaussian | 1 | 1 | 0.50 | 0.50 | 0.50 | 0.50 |
| 5 | Mean value estimation of reference antenna mismatch efficiency | 0.27 | 0.27 | 0.27 | 0.27 | Gaussian | 1 | 1 | 0.27 | 0.27 | 0.27 | 0.27 |
| 6 | Uncertainty of the Network Analyzer | 0.13 | 0.20 | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.13 | 0.20 | 0.20 | 0.20 |
| 7 | Influence of the reference antenna feed cable | 0.20 | 0.20 | 0.20 | 0.20 | Gaussian | 1 | 1 | 0.20 | 0.20 | 0.20 | 0.20 |
| 8 | Mean value estimation of power transfer function | 0.27 | 0.27 | 0.27 | 0.27 | Gaussian | 1 | 1 | 0.27 | 0.27 | 0.27 | 0.27 |
| 9 | Uniformity of transfer function | 1.5 | 1.5 | 1.5 | 1.5 | Gaussian | 1 | 1 | 1.5 | 1.5 | 1.5 | 1.5 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  |  |  | 1.66 | 1.68 | 1.70 | 1.72 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  |  |  | 3.25 | 3.29 | 3.33 | 3.37 |

Table 12.7.1.2.2A-2: Reverberation Chamber uncertainty assessment spurious emissions 18 GHz – 60 GHz

| UID | Uncertainty source | Uncertainty value 18 GHz < f ≦ 26.5 GHz | Uncertainty value 26.5 GHz < f ≦ 40 GHz |  | Uncertainty value 40 GHz < f ≦ 60 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Uncertainty value 18 GHz < f ≦ 26.5 GHz | Uncertainty value 26.5 GHz < f ≦ 40 GHz | Uncertainty value 40 GHz < f ≦ 60 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Uncertainty of the measurement equipment | 0.52 | 0.84 |  | 1.15 | Gaussian | 1 | 1 | 0.52 | 0.84 | 1.15 |
| 2 | Impedance mismatch in the receiving chain | 0.45 | 0.45 |  | 0.45 | U-shaped | √2 | 1 | 0.32 | 0.32 | 0.32 |
| 3 | Random uncertainty | 0.1 | 0.1 |  | 0.1 | Rectangular | √3 | 1 | 0.06 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |
| 4 | Reference antenna radiation efficiency | 0.50 | 0.50 | 0.50 |  | Gaussian | 1 | 1 | 0.50 | 0.50 | 0.50 |
| 5 | Mean value estimation of reference antenna radiation efficiency | 0.27 | 0.27 | 0.27 |  | Gaussian | 1 | 1 | 0.27 | 0.27 | 0.27 |
| 6 | Uncertainty of the Network Analyzer | 0.20 | 0.30 | 0.30 |  | Gaussian | 1 | 1 | 0.20 | 0.30 | 0.30 |
| 7 | Influence of the reference antenna feed cable | 0.20 | 0.20 | 0.20 |  | Gaussian | 1 | 1 | 0.20 | 0.20 | 0.20 |
| 8 | Mean value estimation of transfer function | 0.27 | 0.27 | 0.27 |  | Gaussian | 1 | 1 | 0.27 | 0.27 | 0.27 |
| 9 | Uniformity of transfer function | 1.5 | 1.5 | 1.5 |  | Gaussian | 1 | 1 | 1.5 | 1.5 | 1.5 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  |  |  | 1.76 | 1.89 | 2.05 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  |  |  | 3.45 | 3.71 | 4.02 |