3GPP TSG-RAN4 Meeting #90bis	R4-1903593

# Xi’an, China, 8th – 12th  April, 2019

Agenda item:	          	6.10.8.3.1

Source:		MediaTek Inc.

Title:		Discussion on CSI-RS based L1-RSRP in FR2

Document for:	Discussion

- Introduction

In this paper, it provides our views on the remaining issues for CSI-RS L1-RSRP reporting in FR2.

- CSI-RS with repetition set to “off” in FR2

If the TCI state is not configured for the CSI-RS resource, then the CSI-RS resource is not QCLed to other RSs. In our understanding, such a CSI-RS resource is used for P1 procedure like SSB. Hence, it should provide UE opportunities to find its RX beam direction in FR2 and scheduling restriction should be applied.

Proposal 1: For CSI-RS configured with repetition set to “off”, if the TCI state is not configured for the CSI-RS resource, N=1 does not apply and scheduling restriction should be applied.

In other words, for CSI-RS configured with repetition set to “off”, if it is Type-D QCLed and TDMed to SSB for L1-RSRP reporting or CSI-RS with repetition “ON”, its RX beam direction can reference to the indicated QCL source,  so N=1 can apply.

Proposal 2: For CSI-RS configured with repetition set to “off”, N=1 if the CSI-RS is Type-D QCLed and TDMed to SSB for L1-RSRP reporting or CSI-RS with repetition “ON”.

- CSI-RS with repetition set to “on” in FR2

Currently the scheduling restriction is depending on whether N=1 is applying. However, for the CSI-RS configured with repetition parameter “on”, it has agreed the scheduling restriction should be always applied in FR2, regardless whether N=1 is applying or not.

Observation 1: Scheduling restriction for CSI-RS configured with repetition “on” should be always applied in FR2, regardless whether N=1 is applying or not.

- Conclusion

In the contribution, we discuss the requirement for CSI-RS based L1-RSRP measurement for reporting in FR2. We have the following observations and proposals:

Proposal 1: For CSI-RS configured with repetition set to “off”, if the TCI state is not configured for the CSI-RS resource, N=1 does not apply and scheduling restriction should be applied.

Proposal 2: For CSI-RS configured with repetition set to “off”, N=1 if the CSI-RS is Type-D QCLed and TDMed to SSB for L1-RSRP reporting or CSI-RS with repetition “ON”.

Observation 1: Scheduling restriction for CSI-RS configured with repetition “on” should be always applied in FR2, regardless whether N=1 is applying or not.

- Reference

[1] TS38.133, NR; Requirements for support of radio resource management, V15.4.0.