3GPP TSG RAN WG4 Meeting #90bis		R4-1903629

Xi'an, China, 8th – 12th April 2019

Source: 	NTT DOCOMO, INC.

Title: 	Remaining issues on measurement period for L1-RSRP reporting

Agenda item:	6.10.8.3.1

Document for:	Discussion

- Introduction

At the last RAN4 meeting, most parts of requirements for measurement period of L1-RSRP reporting were agreed including number of measurement samples. However, scaling factor related to Rx beam sweeping, i.e. N, are still under discussion. In this contribution, we provide our views on the remaining issues.

- Discussion

- Agreements in RAN4#90: - For L1-RSRP measurement period, - If the IE timeRestrictionForChannelMeasurement is configured, then only one sample is defined as the measurement period. - Otherwise, the maximum delay is 3 samples for measurements - For measurement accuracy, - The accuracy is defined based on the single sample.

At the last meeting, RAN4 made good progress on measurement period for L1-RSRP reporting. As captured above, number of samples deriving measurement period was agreed, but there are still some remaining issues, e.g. scaling factor N. In the previous meetings, RAN4 has discussed about condition where N=1 can apply, and we have proposed that UE does not need to sweep Rx beam to perform L1-RSRP measurement for reporting if CSI-RS resources configured for b reporting are QCL-Type D and TDMed with CSI-RS with repetition on or for other RS resources configured for beam reporting. On the other hands, at the last RAN4 meeting, some companies proposed that UE does not need to perform Rx beam sweeping, i.e. N =1, if CSI-RS configured for L1-RSRP reporting is configured with TCI state [1]. From our perspective, this proposal would make sense, and hence, condition for N could be as following.

Proposal 1: For CSI-RS based L1-RSRP reporting in FR2, scaling factor N should be defined as following:

- N = 1 could apply if TCI state is configured to the CSI-RS resources.

- N = 8, otherwise.

- Conclusion

In this contribution, we provided our views on measurement period for L1-RSRP reporting, and we made following proposal.

Proposal 1: For CSI-RS based L1-RSRP reporting in FR2, scaling factor N should be defined as following:

- N = 1 could apply if TCI state is configured to the CSI-RS resources.

- N = 8, otherwise.

# Reference

- 3GPP, R4-1901171, Huawei, HiSilicon, “Requirements for L1-RSRP based beam reporting,” Feb. 2019