3GPP TSG-RAN4 Meeting #90bis	R4-1903714

# Xi’an, China, 8th – 12th April, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 38.133 | CR |  | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Maintenance draftCR on re-establishment test cases for FR1 (section A.6.3.2.1) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-03-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Correction is needed on FR1 RRC re-establishment test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add active UL/DL BWP configurations - Editorial changes in the tables to remove the errors. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases configurations are not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A.6.3.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

A.6.3.2.1	SA: RRC Re-establishment

A.6.3.2.1.1	Intra-frequency RRC Re-establishment in FR1

A.6.3.2.1.1.1	Test Purpose and Environment

The purpose is to verify that the NR intra-frequency RRC re-establishment delay in FR1 with known target cell is within the specified limits. These tests will verify the requirements in clause 6.2.1.

The test parameters are given in table A.6.3.2.1.1.1-1, table A.6.3.2.1.1.1-2 and table A.6.3.2.1.1.1-3 below. The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, cell 1, which is the active cell, is deactivated. The time period T3 starts after the occurrence of the radio link failure.

Table A.6.3.2.1.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.3.2.1.1.1-2: General test parameters for NR intra-frequency RRC Re-establishment test case in FR1

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3 | Cell1 |  |
|  | Neighbour cells |  | 1, 2, 3 | Cell2 |  |
| Final condition | Active cell |  | 1, 2, 3 | Cell2 |  |
| RF Channel Number |  |  | 1, 2, 3 | 1 |  |
| Time offset between cells |  |  | 1 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| N310 |  | - | 1, 2, 3 | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1, 2, 3 | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 1, 2, 3 | 0 | Radio link failure timer; T310 is disabled |
| T311 |  | ms | 1, 2, 3 | 3000 | RRC re-establishment timer |
| Access Barring Information |  | - | 1, 2, 3 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC pattern 2 |  |
|  |  |  | 2 | SMTC pattern 1 |  |
|  |  |  | 3 | SMTC pattern 1 |  |
| DRX cycle length |  | s | 1, 2, 3 | OFF |  |
| PRACH configuration index |  |  | 1, 2, 3 | 87 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| T1 |  | s | 1, 2, 3 | 5 |  |
| T2 |  | ms | 1, 2, 3 | 200 | Time for the UE to detect RLF |
| T3 |  | s | 1, 2, 3 | 2 |  |

Table A.6.3.2.1.1.1-3: Cell specific test parameters for NR intra-frequency RRC Re-establishment test case in FR1

| Parameter | Unit | Test configuration | Cell 1 |  |  |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 |  | T2 |  | T3 | T1 | T2 |  | T3 |
| TDD configuration |  | 1 | N/A |  |  |  |  | N/A |  |  |  |
|  |  | 2 | TDDConf.1.1 |  |  |  |  | TDDConf.1.1 |  |  |  |
|  |  | 3 | TDDConf.2.1 |  |  |  |  | TDDConf.2.1 |  |  |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  |  |  |  | N/A |  |  |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |  |  |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |  |  |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  |  |  |  | CR.1.1 FDD |  |  |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |  | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |  | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  |  |  |  | CCR.1.1 FDD |  |  |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |  | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |  | CCR.2.1 TDD |  |  |  |
| OCNG Pattern |  | 1, 2, 3 | OP.1 defined in A.3.2.1 |  |  |  |  | OP.1 defined in A.3.2.1 |  |  |  |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0.1 |  |  |  |  | DLBWP.0.1 |  |  |  |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0.1 |  |  |  |  | ULBWP.0.1 |  |  |  |
| Active DL BWP confgiuration |  | 1, 2, 3 | DLBWP.1.1 | N/A |  | N/A |  | N/A | N/A | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 | N/A |  | N/A |  | N/A | N/A | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  |  |  |  | SSB |  |  |  |
|  | dB | 1 | 1.54 |  | -infinity |  | -infinity | -3.79 | 4 |  | 4 |
|  |  | 2 |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |  |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -98 |  |  |  |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |
|  | dB | 1 | 7 |  | -infinity |  | -infinity | 4 | 4 |  | 4 |
|  |  | 2 |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -91 |  | -infinity |  | -infinity | -94 | -94 |  | -94 |
|  |  | 2 | -91 |  | -infinity |  | -infinity | -94 | -94 |  | -94 |
|  |  | 3 | -88 |  | -infinity |  | -infinity | -91 | -91 |  | -91 |
| Io | dBm/9.36 MHz | 1 | -60.74 |  | -infinity |  | -infinity | -60.74 | -64.59 |  | -64.59 |
|  | dBm/9.36 MHz | 2 | -60.74 |  | -infinity |  | -infinity | -60.74 | -64.59 |  | -64.59 |
|  | dBm/38.16 MHz | 3 | -54.65 |  | -infinity |  | -infinity | -54.65 | -58.50 |  | -58.50 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |

A.6.3.2.1.1.2	Test Requirements

The RRC re-establishment delay is defined as the time from the start of time period T3, to the moment when the UE starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2.

The RRC re-establishment delay to a known NR intra frequency cell shall be less than [1.6] s.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90%.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay= TUL_grant + TUE_re-establish_delay.

Where:

TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The PRACH reception at the system simulator is used as a trigger for the completion of the test; hence TUL_grant is not used.

TUE_re-establish_delay=50+Tidentify_intra_NR+i=1Nfreq-1Tidentify_inter_NR,i+TSI-NR+TPRACH

Nfreq = 1

Tidentify_intra_NR = 200 ms

TSI = [1280] ms; it is the time required for receiving all the relevant system information as defined in TS 38.331 for the target intra-frequency NR cell.

TPRACH = [15] ms; it is the additional delay caused by the random access procedure.

This gives a total of [1545] ms, allow [1.6] s in the test case.

A.6.3.2.1.2	Inter-frequency RRC Re-establishment in FR1

A.6.3.2.1.2.1	Test Purpose and Environment

The purpose is to verify that the NR inter-frequency RRC re-establishment delay in FR1 without known target cell is within the specified limits. These tests will verify the requirements in clause 6.2.1.

The test parameters are given in table A.6.3.2.1.2.1-1, table A.6.3.2.1.2.1-2 and table A.6.3.2.1.2.1-3 below. The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, cell 1, which is the active cell, becomes inactive. The time period T3 starts after the occurrence of the radio link failure. During T1, the UE shall be configured with the carrier frequency of cell 2 (with RF Channel Number #2) to ensure that the UE has the context of the carrier frequency of cell 2 by the end of T1.

Table A.6.3.2.1.2.1-1: Supported test configurations

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 1 | 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode | 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode | 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode | 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Table A.6.3.2.1.2.1-2: General test parameters for NR inter-frequency RRC Re-establishment test case in FR1

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3 | Cell1 |  |
|  | Neighbour cells |  | 1, 2, 3 | Cell2 |  |
| Final condition | Active cell |  | 1, 2, 3 | Cell2 |  |
| RF Channel Number |  |  | 1, 2, 3 | 1, 2 |  |
| Time offset between cells |  |  | 1 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| N310 |  | - | 1, 2, 3 | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1, 2, 3 | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 1, 2, 3 | 0 | Radio link failure timer; T310 is disabled |
| T311 |  | ms | 1, 2, 3 | 5000 | RRC re-establishment timer |
| Access Barring Information |  | - | 1, 2, 3 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC pattern 2 |  |
|  |  |  | 2 | SMTC pattern 1 |  |
|  |  |  | 3 | SMTC pattern 1 |  |
| DRX cycle length |  | s | 1, 2, 3 | OFF |  |
| PRACH configuration index |  |  | 1, 2, 3 | 87 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| T1 |  | s | 1, 2, 3 | 5 |  |
| T2 |  | ms | 1, 2, 3 | 200 | Time for the UE to detect RLF |
| T3 |  | s | 1, 2, 3 | 5 |  |

Table A.6.3.2.1.2.1-3: Cell specific test parameters for NR inter-frequency RRC Re-establishment test case in FR1

| Parameter | Unit |  | Test configuration |  | Cell 1 |  |  |  |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 |  | T2 |  | T3 |  | T1 | T2 |  | T3 |
| RF Channel Number |  |  | 1, 2, 3 |  | 1 |  |  |  |  |  | 2 |  |  |  |
| TDD configuration |  |  | 1 |  | N/A |  |  |  |  |  | N/A |  |  |  |
|  |  |  | 2 |  | TDDConf.1.1 |  |  |  |  |  | TDDConf.1.1 |  |  |  |
|  |  |  | 3 |  | TDDConf.2.1 |  |  |  |  |  | TDDConf.2.1 |  |  |  |
| PDSCH RMC configuration |  |  | 1 |  | SR.1.1 FDD |  |  |  |  |  | N/A |  |  |  |
|  |  |  | 2 |  | SR.1.1 TDD |  |  |  |  |  |  |  |  |  |
|  |  |  | 3 |  | SR.2.1 TDD |  |  |  |  |  |  |  |  |  |
| RMSI CORESET RMC configuration |  |  | 1 |  | CR.1.1 FDD |  |  |  |  |  | CR.1.1 FDD |  |  |  |
|  |  |  | 2 |  | CR.1.1 TDD |  |  |  |  |  | CR.1.1 TDD |  |  |  |
|  |  |  | 3 |  | CR.2.1 TDD |  |  |  |  |  | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  |  | 1 |  | CCR.1.1 FDD |  |  |  |  |  | CCR.1.1 FDD |  |  |  |
|  |  |  | 2 |  | CCR.1.1 TDD |  |  |  |  |  | CCR.1.1 TDD |  |  |  |
|  |  |  | 3 |  | CCR.2.1 TDD |  |  |  |  |  | CCR.2.1 TDD |  |  |  |
| OCNG Pattern |  |  | 1, 2, 3 |  | OP.1 defined in A.3.2.1 |  |  |  |  |  | OP.1 defined in A.3.2.1 |  |  |  |
| Initial DL BWP configuration |  |  | 1, 2, 3 |  | DLBWP.0 |  |  |  |  |  | DLBWP.0 |  |  |  |
| Initial UL BWP configuration |  |  | 1, 2, 3 |  | ULBWP.0 |  |  |  |  |  | ULBWP.0 |  |  |  |
| Active DL BWP confgiuration |  | 1, 2, 3 |  | DLBWP.1.1 |  | N/A |  | N/A |  | N/A |  | N/A | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 |  | ULBWP.1.1 |  | N/A |  | N/A |  | N/A |  | N/A | ULBWP.1.1 |  |
| RLM-RS |  |  | 1, 2, 3 |  | SSB |  |  |  |  |  | SSB |  |  |  |
|  | dB |  | 1 |  | 4 |  | -infinity |  | -infinity |  | -infinity | -infinity |  | 7 |
|  |  |  | 2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | dBm/SCS |  | 1 |  | -98 |  |  |  |  |  |  |  |  |  |
|  |  |  | 2 |  | -98 |  |  |  |  |  |  |  |  |  |
|  |  |  | 3 |  | -95 |  |  |  |  |  |  |  |  |  |
| Note2 | dBm/15 kHz |  | 1 |  | -98 |  |  |  |  |  |  |  |  |  |
|  |  |  | 2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |
|  | dB |  | 1 |  | 4 |  | -infinity |  | -infinity |  | -infinity | -infinity |  | 7 |
|  |  |  | 2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS |  | 1 |  | -94 |  | -infinity |  | -infinity |  | -infinity | -infinity |  | -91 |
|  |  |  | 2 |  | -94 |  | -infinity |  | -infinity |  | -infinity | -infinity |  | -91 |
|  |  |  | 3 |  | -91 |  | -infinity |  | -infinity |  | -infinity | -infinity |  | -88 |
| Io | dBm/9.36 MHz |  | 1 |  | -64.59 |  | -infinity |  | -infinity |  | -infinity | -infinity |  | -62.26 |
|  | dBm/9.36 MHz |  | 2 |  | -64.59 |  | -infinity |  | -infinity |  | -infinity | -infinity |  | -62.26 |
|  | dBm/38.16 MHz |  | 3 |  | -58.50 |  | -infinity |  | -infinity |  | -infinity | -infinity |  | -57.17 |
| Propagation Condition |  |  | 1, 2, 3 |  | AWGN |  |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |

A.6.3.2.1.2.2	Test Requirements

The RRC re-establishment delay is defined as the time from the start of time period T3, to the moment when the UE starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2.

The RRC re-establishment delay to an unknown NR inter frequency cell shall be less than [3] s.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90%.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay= TUL_grant + TUE_re-establish_delay.

Where:

TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The PRACH reception at the system simulator is used as a trigger for the completion of the test; hence TUL_grant is not used.

TUE_re-establish_delay=50+Tidentify_intra_NR+i=1Nfreq-1Tidentify_inter_NR,i+TSI-NR+TPRACH

Nfreq = 2

Tidentify_intra_NR = 800 ms

Tidentify_inter_NR = 800 ms

TSI = [1280] ms; it is the time required for receiving all the relevant system information as defined in TS 38.331 for the target inter-frequency NR cell.

TPRACH = [15] ms; it is the additional delay caused by the random access procedure.

This gives a total of [2945] ms, allow [3] s in the test case.

<End of Changes>